3GPP TSG-RAN WG4 Meeting #101-e	R4-2118164

# Electronic meeting, November 1st – 12th, 2021

Source:		Ericsson

Title:			900MHz RMR band – BS RF

Agenda item:		7.3.3

Document for:		Approval

# Introduction

The 900MHz NR band for European RMR WI ([1]) has been revised in RAN#92e meeting to introduce a new 900MHz band for Railway in Europe.

Following the discussion from last RAN4#100-e meeting, this contribution is addressing one of the major blocking issue to progress on this WI and makes proposal on how to capture EC Decision(20)02 limits.

# Discussion

## EIRP limits management

From last RAN4#100-e meeting, one of the major issue that should be discussed is related to how EIRP limits specified in ECC Decision (20)02 should be considered in RAN4 specifications:

To make progress, RAN4 should then first agree on this issue.

In the past, such EIRP limit coming from Regulatory authorities were handled in RAN4 by:

- Introducing a manufacturer declaration of the maximum emission level for DTT protection.

- Referring to an annex (e.g. Annex G in TS 36.104) describing how compliance with the EIRP regional requirement could be checked.

This approach is correct but it has triggered many discussion with the European Commission in ETSI TFES, while drafting the corresponding Harmonized Standard. In the scope of the Radio Equipment Directive, such requirement specified based on manufacturer declaration are indeed not allowed. In the DTT protection case, as this was an additional limit, the compromised was then to just refer informatively to TS 36.141 (see EN 301 908-14 or EN 301 908-18).

Similar issue was recently discussed in ETSI TFES when considering protection of EC Decision 2019/235 and the protection of FSS/FS for frequency range above 3 800MHz. Again, the specified limits are expressed as EIRP limits. In this later case, the European Commission was very clear and its recommendation was to specify conducted limits considering a certain antenna gain. Facing the strong position from the European Commission, ETSI TFES decided then to follow the recommendation and specify conducted limits assuming a 21 dBi antenna gain. Even if those limits are mentioned in these Harmonized Standards ([7] and [8]), it will not prevent a Base Station supporting higher limits but intended to be deployed with a lower antenna gain to get certified.

In the case of the RMR 900 MHz and 1900 MHz bands, we think we should follow the same approach and specify a clear limit adding this limit is set assuming a certain antenna gain, and not using a manufacturer declaration based approach. The reasons for that are:

- RMR 900 MHz and 1900 MHz bands are European bands and shall then comply with EC Decision(20)02.

- Those bands and their associated requirements shall be added later in the European Harmonized Standards.

- To avoid any misinterpretation or any confusion, it’s always better to keep 3GPP specifications and European Harmonized Standards aligned, as much as possible.

- This will not prevent to design and deploy a Base Station supporting higher limits but intended to be deployed with lower antenna gain.

Noting that CEPT used a 17 dBi antenna gain to perform their studies and come to those EIRP limits, we would make the following proposal:

Proposal1: When relevant, convert EIRP limits from EC Decision(20)02 in conducted limits assuming a 17 dBi antenna gain.

## ECC Decision(20)02

CEPT ([2]) has specified following Least Restrictive Technical Conditions for BS operating in the new 900MHz RMR band:

- General in-block requirement mandatory for uncoordinated deployment:

- Out of Band requirement

- Baseline requirement:

- Receiver characteristics:

It should be noted that ECC Decision(20)02 also specified LRTC for cab-radio equipment, but this is out of scope of this WI ([1]).

## TS 38.104 impacts

### BS maximum output power

The BS maximum output power is currently a manufacturer declaration, and there is no cap on the declared value. It’s left up to the operator (and the manufacturer) to deploy BS + antenna which don’t exceed any maximum power limit fixed by the Regulator.

CEPT made studies considering uncoordinated deployment (which was a strong wish for FRMCS) and specified a maximum EIRP limit for such deployment. Still, according to ECC Decision ([2]), higher EIRP limit might still be possible, but this should be done via coordinated procedure and/or using some other mitigation techniques.

In the context of this WI, based on our proposal made in 2.2.1, we would propose:

Proposal2:  Capture in TS 38.104 that for BS operating in [900MHz RMR band], for uncoordinated deployment, the BS rated output power Prated,c,AC  shall not exceed: 47.5dBm/5MHz + (fDL-922.1) x 40/3 dB (considering a 17 dBi antenna gain, and with fDL being the centre frequency in MHz).

### Out of Band requirement

The CEPT our of band requirement should be captured a OBUE category B option 2 requirement:

Proposal3: Capture the following OBUE category B option 2 requirement for band [900MHz RMR] in TS 38.104:

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits (Note 1) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.2 MHz | 0.1 MHz f_offset < 0.3 MHz | 15.5 dBm | 200 kHz |
| 0.2 MHz f < 1 MHz | 0.6MHz f_offset < 1.4 MHz | -3 dBm | 800 kHz |
| 1 MHz f 10 MHz | 1.5 MHz f_offset < 10.5 | -12 dBm | 1 MHz |
| NOTE 1: Assuming a 17dBi antenna gain |  |  |  |

### Additional baseline

CEPT specified an EIRP baseline requirement of -49dBm/5MHz in the 880-915MHz frequency range. For the new 900MHz RMR band (ΔfOBUE = 10MHz), this baseline would fall partly in the OBUE domain and partly in the spurious domain.

For BS type 1-C, the existing Category B requirement for spurious in that frequency range is equal to -36dBm/1MHz, which is less stringent than the CEPT baseline.

Proposal4: Capture the following additional requirement (most likely as an additional spurious limit) for band [900 MHz RMR] in TS 38.104:

| Spurious frequency range | Basic limit | Measurement bandwidth |
| --- | --- | --- |
| 880 MHz – 915 MHz | -66 dBm (NOTE) | 5 MHz |
| NOTE: considering a 17 dBi antenna gain |  |  |

### Blocking requirement

CEPT Report 313 specified an additional BS blocking requirement considering a 3 dB desens wanted signal and a 200kHz interferer, while RAN4 usually consider a 6 dB desens value and a CW interferer.

As this additional blocking requirement is only applicable to the new 900MHz RMR band, we propose to capture the CEPT blocking requirement as an additional blocking requirement, and as specified by CEPT with 3 dB desens in TS 38.104.

This blocking requirement is motivated by the coexistence with SRD devices below 874.4 MHz ([3]). CEPT has not specified the characteristics of such interferer, but the study was done considering a 500mW EPR device with a bandwidth of 200kHz. One approach would be to consider 1 NR PRB as interferer, and define so a new narrowband blocking requirement but further study would be needed to characterize this interferer in TS 38.104.

Proposal5: Capture the following additional BS blocking requirement for band [900 MHz RMR] in TS 38.104 and further study the interferer’s characteristics:

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) (Note 1) | Interfering signal mean power (dBm) | Center Frequency of Interfering Signal (MHz) (Note 2) | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 5 | PREFSENS + 3 dB | Wide Area BS: -34 | TBD | TBD |
| NOTE 1: PREFSENS depends on the RAT. For NR, PREFSENS depends also on the BS channel bandwidth as specified in tables 7.2.2-1, 7.2.2-2 and 7.2.2-3. NOTE 2: Considering a 200kHz interferer. |  |  |  |  |

# Conclusion

In this contribution, we studied the impacts of introducing the 900MHz RMR band in TS 38.104, based on ECC Decision (20)02. We made the following proposals:

Proposal1: When relevant, convert EIRP limits from EC Decision(20)02 in conducted limits assuming a 17 dBi antenna gain.

Proposal2:  Capture in TS 38.104 that for BS operating in [900MHz RMR band], for uncoordinated deployment, the BS rated output power Prated,c,AC  shall not exceed: 47.5dBm/5MHz + (fDL-922.1) x 40/3 dB (considering a 17 dBi antenna gain, and with fDL being the centre frequency in MHz).

Proposal3: Capture the following OBUE category B option 2 requirement for band [900MHz RMR] in TS 38.104:

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits (Note 1) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.2 MHz | 0.1 MHz f_offset < 0.3 MHz | 15.5 dBm | 200 kHz |
| 0.2 MHz f < 1 MHz | 0.6MHz f_offset < 1.4 MHz | -3 dBm | 800 kHz |
| 1 MHz f 10 MHz | 1.5 MHz f_offset < 10.5 | -12 dBm | 1 MHz |
| NOTE 1: Assuming a 17 dBi antenna gain |  |  |  |

Proposal4: Capture the following additional requirement (most likely as an additional spurious limit) for band [900 MHz RMR] in TS 38.104:

| Spurious frequency range | Basic limit | Measurement bandwidth |
| --- | --- | --- |
| 880 MHz – 918 MHz | -66 dBm (NOTE) | 5 MHz |
| NOTE: considering a 17 dBi antenna gain |  |  |

Proposal5: Capture the following additional BS blocking requirement for band [900 MHz RMR] in TS 38.104 and further study the interferer’s characteristics:

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) (Note 1) | Interfering signal mean power (dBm) | Center Frequency of Interfering Signal (MHz) (Note 2) | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 5 | PREFSENS + 3 dB | Wide Area BS: -34 | TBD | TBD |
| NOTE 1: PREFSENS depends on the RAT. For NR, PREFSENS depends also on the BS channel bandwidth as specified in tables 7.2.2-1, 7.2.2-2 and 7.2.2-3. NOTE 2: Considering a 200kHz interferer. |  |  |  |  |

# References

- RP-211495, Introduction of 900MHz NR band for Europe for Rail Mobile Radio (RMR), UIC

- ECC Decision(20)02,  Harmonised use of the paired frequency bands 874.4-880.0 MHz and 919.4-925.0 MHz and of the unpaired frequency band 1900-1910 MHz for Railway Mobile Radio (RMR), 20 November 2020

- ECC Report 313, Technical study for co-existence between RMR in the 900 MHz range and other applications in adjacent bands

- ECC Report 318, Compatibility between RMR and MFCN in the 900 MHz range, the 1900-1920 MHz band and the 2290-2300 MHz band

- R4-2108609, WF on BS RF requirements for the RMR 900 and RMR 1900 WI, Huawei

- R4-2118167, 1900MHz RMR band – BS RF, Ericsson

- EN 301 908-14, IMT cellular networks; Harmonised Standard for access to radio spectrum; Part 14: Evolved Universal Terrestrial Radio Access (E-UTRA) Base Stations (BS) Release 15 - E-UTRA BS v15.1.1

- EN 301 908-18, IMT cellular networks; Harmonised Standard for access to radio spectrum; Part 18: E-UTRA, UTRA and GSM/EDGE Multi-Standard Radio (MSR) Base Station (BS) Release 15 - 3GPP Rel-15 update including NR