3GPP TSG-RAN WG4 Meeting # 101-e 												R4-2120211

Electronic Meeting, November 1-12, 2021

Agenda item:			8.9.4.1, 8.9.4.2, 8.9.4.3

Source:	Moderator (Nokia, Nokia Shanghai Bell)

Title:	Email discussion summary for [101-e][216] NR_HST_FR2_RRM_1

Document for:	Approval

# Introduction

Briefly introduce background, the scope of this email discussion (e.g. list of treated agenda items) and provide some guidelines for email discussion if necessary.

List of candidate target of email discussion for 1st round and 2nd round

- 1st round: TBA

- 2nd round: TBA

## Background and scope

This T-doc will be used to guide and summarize the email discussion for the topic of Rel-17 NR HST FR2 enhancements RRM core requirements, with the email thread identifier “[101-e][216] NR_HST_FR2_RRM_1”.

In this email thread, the following agenda items will be discussed:

- 8.9.4.1 General

- 8.9.4.2 Number of RX beams

- 8.9.4.3 Idle/Inactive and connected state mobility requirements

In the previous RAN4#100-e meeting, the discussion about RRM requirements for HST operation in FR2 has continued. The discussions are summarized in [R4-2115431], and the ways forward were captured in [R4-2115334].
Between the significant agreements at RAN4#100-e were:

- HST FR2 CPE has a capacity to support both uni- and bi-directional operation.

- RX beam number for RRM requirements definition

  - Define two set of requirements for Scenario A and Scenario B in terms of number of RX beams per UE

    - Scenario A: [2] RX beams for all scenarios

    - Scenario B: [6] RX beams for all scenarios

- Apply existing R16 requirements for long DRX cycles, i.e., above the upper bound of DRX cycle.

As a moderator for FR2 HST enhancements RRM discussion, we would like to suggest the following candidate target of 1st and 2nd round email discussion:

- 1st round: Further discuss the requirements based on the achieved agreement on the number of Rx beams. Discussion of network signaling needed to support and enhance HST FR2 requirements shall continue.

- 2nd round: Based on the results from the 1st round, identify a few issues that have the potential to achieve agreements and discuss them further. Achieve agreements as much as possible.

## Email discussion guidelines

Unless different guidance is received from the session chairs, the moderator would like to ask companies to adhere to the following guidelines, when taking part in [101-e][216] NR_HST_FR2_RRM_1.

Please also check the “RAN4#101-e meeting arrangements and guidelines”, available on the reflector, for fundamental guidelines and deadlines [R4-2117001].

The preferred method of commenting is to add/update your company’s view directly in this email summary document (use change marks if appropriate) and upload it to [100-e][218] NR_HST_FR2_RRM draft folder.

- Draft folder:
[101-e][216] NR_HST_FR2_RRM_1
https://www.3gpp.org/ftp/tsg_ran/WG4_Radio/TSGR4_101-e/Inbox/Drafts/%5B101-e%5D%5B216%5D%20NR_HST_FR2_RRM_1

- It is expected that delegates will download the latest version (including other companies’ versions) of the summary document, insert comments and upload it again.

- To ensure the comments are captured timely and correctly, delegates are encouraged to:

  - Rename the file by adding your company name and changing the file version.
Example: “Summary_218_HST_FR2_RRM_1_1st_round_v05_CATT_Nokia.docx” -> “Summary_218_HST_FR2_RRM_1_1st_round_v06_Nokia_QC.docx”

  - There is no need to send e-mails in the reflector when comments in the summary are added.

  - Please, check for possibly updated base document versions, right before uploading your updates.

- Please, do not hesitate to mark your company as supporting a certain option directly in this document.
Please refrain from rewriting existing options and proposed WFs; ask the moderator (in your company’s comment) to modify/add.

- It is encouraged to give a short reasoning for each view expressed (1-2 sentences are recommended).
Please avoid statements like “Option X”, without further explication or reasoning.

- Please, update your company contact information in the Annex.
It is also recommended to explicitly mention delegate’s name next to company name in the comments if multiple delegates from the same company are commenting.
Another way is to identify different delegates in the Track changes username.

# Topic #1: General aspects

Main technical topic overview. The structure can be done based on sub-agenda basis.

In the previous RAN4#100-e meeting the discussion in this topic mainly focused on signaling of deployment types and UE capabilities. This RAN4#101-e meeting, a similar list of sub-topics can be defined based on company contributions. Additionally, a LS on UE capability and network signaling were submitted that require a separate consideration. Finally, it is proposed to share the CR work load between the companies.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2117299 | Apple | Discussion on general aspects for FR2 HST Proposal 1: Enable network assisted signaling, either implicit or explicit, to differentiate scenario A and scenario B for enhanced RRM requirement. Proposal 2: Network signals the type of deployment (uni- or bi- direction) to UE. Proposal 3: Support option 1 and option 2 for network assisted information, i.e., enable network assisted signaling of SSB index and order per RRH, and whether this is uni-directional or bi-directional deployment. Proposal 4: Support option 4. Network can determine the proper RRM requirement and signal to UE directly. Proposal 5: Support UE capability of report different Rx beam sweeping number in uni/bi-directional operation. |
| R4-2117364 | CATT | Discussion on general issues for FR2 HST Proposal 1: Define NW signalling to differentiate scenarios for better power consumption performance from UE perspective. Proposal 2: Signalling of uni-/bi-directional operation is not needed. Proposal 3: It is not necessary to introduce UE capability to indicate the support of FR2 HST. No need to define separate UE capability for Scenario A and Scenario B. Proposal 4: After the conclusion of CPE in RF session, the RRM requirements can be specified based on Power class. Proposal 5: No need for CPE capability to change characteristics, e.g. RX beam sweep number in uni-/bi-directional operation. |
| R4-2117627 | Qualcomm, Inc. | On NR FR2 HST RRM Requirements Proposal 1: The 2Rx beam sweep based requirement applies to the cases in which Dmin and Hdiff are both no greater than 10m. [Moderator]: Treated in Topic#2 Proposal 2: Define different UE capabilities for the two sets of requirements. The 2Rx and 6Rx sweep agreements only apply to neighboring cell and L1-RSRP measurement requirement Proposal 3: Network signals uni-direction or bi-direction deployment to UE. In uni-directional deployment, network signals the beam direction of new RRH, to which UE is switching, w.r.t. UE moving direction when it changes. Observation 1: Performance degradation right after TCI state switch and handover is expected when 2Rx beam sweep requirements are applied. Proposal 4: Network assistant signaling in section 2.2 should be introduced to ensure the feasibility of 6Rx beam sweeping. Proposal 5: Network can indicate different SSBs on adjacent RRHs having the same QCL property: signal the mapping between the repeated sets of beams from the adjacent RRHs. Network only signals this mapping when the QCL property relationship can be established across RRHs. If deployment scenario doesn’t satisfy the QCL property similarity across RRHs, network doesn’t have to signal these flags. Proposal 6: The following additional beam coverage related information can be signaled to UE - Distance between the projections of adjacent beam peaks on the track - Beam peak direction angle relative to track - The 6 dB beam-width projection on track In addition, UE can report speed to the network. Proposal 7: For FR2 HST neighboring cell search enhancement in scenario B, consider 6Rx instead of 8Rx sweeping in the legacy requirement and keep 1.5 relaxation factor and number of samples the same if network can provide assistant information to UE as proposed in section 2.2. The enhancement is applicable only to DRx cycle <= 80ms. Proposal 8: For FR2 HST neighboring cell search enhancement in scenario A, replace 8Rx sweeping in the legacy requirement by 2Rx sweeping if applicability rule in Proposal 1 is agreed. Proposal 9: Known TCI state condition can be shorter if network can provide assistant information to UE as proposed in section 2.2. Proposal 10: Apply agreed Rx sweeping number to enhance L1-RSRP measurement requirement if network can provide assistant information to UE as proposed in section 2.2. Keep the 1.5 factor at least for scenario A. [Moderator]: Three Proposals above not treated in this Summary. Proposal 11: Define UE capability for FR2 HST enhancement support. Observation 2: PSS/SSS detection is robust enough to accommodate the ISI and timing change due to large propagation delay difference between beams from different RRHs. Proposal 12: Add a MAC-CE command to inform UE that the TCI state switch is across RRH. No requirement after TCI state switch across different RRHs before the first TRS is received by UE, including TA adjustment limitation, Tq. Proposal 13: Impose UL scheduling restriction between TCI state switch across different RRHs and the first TRS after the switch. [Moderator]: Two Proposals above not treated in this Summary. Proposal 14: Restrict idle mode enhancement to DRx cycle within 320ms. Proposal 15: Allow 24 DRx cycles detection time when DRx cycle length is 320ms in idle mode enhancement. |
| R4-2117692 | CMCC | Discussion on general RRM requirements for FR2 HST Proposal 1: it is preferred to have unified requirements to cover scenario A and scenario B. [Moderator]: Treated in Topic#2. Proposal 2: if unified requirements are specified for scenario A and scenario B, it is not necessary to introduce network signaling to help UE differentiate the deployment scenario. Proposal 3: if same requirements are applied for both uni-directional deployment and bi-directional deployment, it is not necessary have the network signaling to indicate type of deployment (uni- or bi-direction) to UE. Proposal 4: considering that the power class for highs-peed CPEs will be specified, it is not necessary to introduce UE capability additionally to indicate the support of FR2 HST. Proposal 5: it is not necessary to introduce different UE capabilities to indicate the support of Scenario A and B. |
| R4-2118344 | Ericsson | General requirements for HST FR2 Proposal 1: Support network signaling in LS R4-2114568. Defer the signaling of UE capacity flag before agreement on issue ‘CPE support for HST FR2 deployment’. Proposal 2: The definition of UE capacity in RRM hypothetically assumes non-CPE exist or present in HST FR2 scenario. How to handle non-CPE shall be discussed in other session and be reflected into the issue on UE capacity and this issue in turn. Proposal 3: Support Option 1: Network signals type of deployment (uni- or bi-direction) to UE. Proposal 4: With respect to scenario A/B and Dmin, the signaling combined may use the format with 2 dimensions, to cover requirement differentiation among Uni- or Bi-direction are tightly together with [Scenario A/B]: Direction; Deployment scenario. [Scenario A] ( Distance_railtoRRH<[50]m); [Scenario B] ( Distance_railtoRRH<[50]m). Unidirection; 00; 01. Bi-idirection; 10; 11. Proposal 5: To our understanding, detailed targets of signaling of network assistance information may comprise: - Optimized TCI state switching in condition of sudden SNR drop if network can acquire position of UE relative to RRHs(based on any possible direct or indirect calculation) and/or Dmin. - The issue on RRH position at various side of trail rail can be overcame by explicit signaling from network if network send signaling of known neighbor RRH side position information. - To correctly signaling UE moving direction to UE in uni-directional deployment, RRH needs to have capacity to identify UE’s moving towards RRH’s direction or backward. It can be discussed more detailly after agreement on beam sweep number in uni-direcitional deployment. Proposal 6: UE shall support Scenario A and B both, no capacity is needed to distinguish them. Proposal 7: Support Option 1: It is not necessary to introduce UE capability to indicate the support of FR2 HST. Proposal 8: Regarding UE without enhancement support, if it must be checked, the issue is how to deal with UE without enhancement support in FR2 HST scenario essentially, which shall be discussed in other session. Proposal 9: Related to differentiating HST FR2 capable CPEs from any other UEs in the specification, we suggest to get agreement on proposal 8 firstly. Proposal 10: CPE shall have capacity to change beam sweep number in uni-/bi-directional operation from implementation point of view. And if flag is agreed to signal uni-/bi-directional case, UE shall change beam sweep number following the signaling. |
| R4-2118803 | Huawei, Hisilicon | General discussion on network signaling and UE capability in FR2 HST Proposal 1: Signalling of uni-/bi-directional operation is not needed from RRM perspective. Proposal 2: To facilitate UE Rx beam sweeping, it is suggested that network can indicate scenario A/ scenario B to UE. Proposal 3: Define UE capability to indicate support scenario A or scenario B or both. |
| R4-2119108 | ZTE Corporation | General RRM requirements for HST FR2 Proposal 1: Considering for different Rx beam number for uni- and bi-directional deployments, defining different RRM requirements for uni-directional and bi-directional deployments is needed. [Moderator]: Treated in Topic#2. Proposal 2: Network can signal whether uni- or bi-directional deployment to UE. Proposal 3: At least for Scenario A, the necessity of introducing NW assistance information signalling is not obvious. Observation 1: For Scenario B, some simple NW assistance information signalling can be further studied. Proposal 4: Operators’ suggestions should be firstly collected referring to the deploy of Scenario A and Scenario B. |
| R4-2119167 | Nokia, Nokia Shanghai Bell | LS on network signalling for Rel-17 NR HST RRM |
| R4-2119478 | Nokia, Nokia Shanghai Bell | On HST FR2 RRM CR work split TS 38.133 section number; TS 38.133 section title; Responsible company. 4; SA: RRC_IDLE state mobility. 4.1; Cell Selection. 4.2; Cell Re-selection. 5; SA: RRC_INACTIVE state mobility. 5.1; Cell Re-selection. 6; RRC_CONNECTED state mobility. 6.1; Handover. 6.2; RRC Connection Mobility Control. 7; Timing. 7.1; UE transmit timing. 7.2; UE timer accuracy. 7.3; Timing advance. 7.4; Cell phase synchronization accuracy. 7.7; deriveSSB-IndexFromCell tolerance. 8; Signalling characteristics. 8.1; Radio Link Monitoring. 8.5; Link Recovery Procedures. 8.10; Active TCI state switching delay. 8.12; Uplink spatial relation switch delay. 9; Measurement Procedure. 9.2; NR intra-frequency measurements. 9.5; L1-RSRP measurements for Reporting. 9.8; L1-SINR measurements for Reporting. 9.10; CSI-RS based L3 measurements. RAN 4 to agree on the HST FR2 RRM CR work split based on the table above. Companies are encouraged to sign up for desired items. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 1-1: Network signaling

Sub-topic description:

This sub-topic is devoted to network signaling needed for the indication of different/enhanced requirements related to the HST FR2 deployments.

Open issues and candidate options before e-meeting:

Presented as background in the corresponding issues.

#### Issue 1-1-1: Signaling for differentiation between Scenario-A and Scenario-B

- Background

At the RAN4#100-e meeting it was agreed to

Define two set of requirements for Scenario A and Scenario B in terms of number of RX beams per UE:

  - Scenario A: [2] RX beams for all scenarios

  - Scenario B: [6] RX beams for all scenarios

  - FFS on feasibility and methods to differentiate scenarios from UE perspective

- Proposals and/or Observations

  - Proposal 1 (Apple): Enable network assisted signaling, either implicit or explicit, to differentiate scenario A and scenario B for enhanced RRM requirement

  - Proposal 2 (CATT): Define NW signalling to differentiate scenarios for better power consumption performance from UE perspective.

  - Proposal 3 (CMCC): if unified requirements are specified for scenario A and scenario B, it is not necessary to introduce network signaling to help UE differentiate the deployment scenario.

  - Proposal 4 (Ericsson): With respect to scenario A/B and Dmin, the signaling combined may use the format with 2 dimensions, to cover requirement differentiation among Uni- or Bi-direction are tightly together with [Scenario A/B]:

| Direction | Deployment scenario |  |
| --- | --- | --- |
|  | [Scenario A] ( Distance_railtoRRH<[50]m) | [Scenario B] ( Distance_railtoRRH<[50]m) |
| Unidirection | 00 | 01 |
| Bi-idirection | 10 | 11 |

  - Proposal 4 (Huawei): To facilitate UE Rx beam sweeping, it is suggested that network can indicate scenario A/ scenario B to UE.

- Candidate options:

  - Option 1: Enable network signaling

  - Option 2: It is not necessary to introduce network signaling, if unified requirements are specified

  - Option 3: Combined 2-dimentional signaling: Uni- or Bi-directional, and Scenario-A or -B

- Recommended WF

  - Continue the discussion in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Nokia | We’re open to discuss the proposals and would like to understand the potential benefits of network signaling. Could network signaling result in more stringent RRM requirements than without signaling? And what are these RRM requirements? |
| QC | We prefer to define requirement with 6Rx beam assumption and option 2. If RAN4 agrees to have two sets of requirements based on 2Rx and 6Rx beams, we support option 3 for the information to be signaled, but detailed signaling discussion should be left to RAN2. |
| ZTE | We support Option 2. |
| Huawei | We think at least scenario A and scenario B shall signals to UE. |
| Ericsson | Because our understanding is differentiation of requirements is necessary, therefore we support Option 1 and Option 3. The format in option 3 can be FFS. Scenario-A or -B bring different RX beam number and scaling factor. Uni-direction or Bi-direction is important signaling to UE. UE can use less RX beam number in Uni-direction if differentiation of Uni-direction and Bi-direction is allowed. Beside of that, we can use Uni-direction or Bi-direction signaling to assist mitigation of SNR drop in un-direction, occasional direction changes in uni-direction deployment and change between Uni-direction or Bi-direction. We believe that keeping two opposite direction measurements for Uni-direction, especially in scenario A. always is inefficient. |
| CATT | We support option 1 to add network signaling. In addition, we think different signalling for Scenario A and Scenario B is necessary. |
| Apple | Support option 1 to add network signaling. We also support option 3 to enable uni-directional and bi-directional signaling. It does not change Scenario A/B for Rx beam number here since we currently agree to have the same number of Rx beam for both uni-directional and bi-directional deployment. However the uni-directional signaling can be helpful to handle the one time large TA adjustment. |
| Intel | Support Option 1. We think that Scenario A/B signaling is necessary. We also think that uni/bi-directional scenario signaling can be helpful in many cases mentioned by other companies, so we also support Option 3 with the exact format discussion left to RAN2 |
| CMCC | We are open to discussion. If unified requirements are specified, no signaling is needed. If two sets of requirements are specified for scenario A and scenario B, it is better to have signaling to help UE to identify the scenario. |
| Samsung | We support option 1, but of course option 3 in which the same intention to differentiate Scn-A and B can also be accepted, if the necessity of introducing the signaling for uni-/bi-directional option is confirmed. The detailed signaling is of course in RAN2 scope, and RAN4 can only discuss the necessity here. With signaling Scenario-A and B, NW have different expectation on the RRM procedure’s RX beam scaling factor, which is already agreed in last RAN4 meeting. If no such signaling introduced, the worst performance (to match Scn-B) have to be followed. |

#### Issue 1-1-2: Signaling of uni-/bi-directional operation

- Background

The Options identified in the previous meetings [R4-2115334]:

  - Option 1: Network signals type of deployment (uni- or bi-direction) to UE.

  - Option 2: Signalling of uni-/bi-directional operation is not needed.

- Proposals and/or Observations

  - Proposal 1 (Apple): Network signals the type of deployment (uni- or bi- direction) to UE.

  - Proposal 2 (CATT): Signalling of uni-/bi-directional operation is not needed.

  - Proposal 3 (Qualcomm): Network signals uni-direction or bi-direction deployment to UE. In uni-directional deployment, network signals the beam direction of new RRH, to which UE is switching, w.r.t. UE moving direction when it changes.

  - Proposal 4 (CMCC): if same requirements are applied for both uni-directional deployment and bi-directional deployment, it is not necessary have the network signaling to indicate type of deployment (uni- or bi-direction) to UE.

  - Proposal 5 (Ericsson): Support Option 1: Network signals type of deployment (uni- or bi-direction) to UE.

  - Proposal 6 (Huawei): Signalling of uni-/bi-directional operation is not needed from RRM perspective.

  - Proposal 7 (ZTE): Network can signal whether uni- or bi-directional deployment to UE.

- Recommended WF

  - The Options from the previous meeting can be kept.

  - Continue the discussion taking Issue 2-1-2 on the difference of requirements into account.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Nokia | The analysis on our contribution shows that inter-cell mobility and L1 beam management performance is worse when UE moving in the direction opposite to the pointing direction of RRH TX beams. Consequently, we have made the following observations and proposals: Observation 2: For unidirectional Scenarios A and B, no handover could be triggered at the switching point Ds_offset when UE is moving in the opposite direction to the pointing direction of RRH TX beams since the condition for handover cannot be satisfied. Proposal 1: RAN4 are encouraged to investigate the potential handover issues in unidirectional scenarios when UE is moving in the direction opposite to the pointing direction of RRH TX beams. Observation 6: For unidirectional Scenarios A and B, no beam switching could occur at the switching point when UE is moving in the opposite direction to the pointing direction of RRH TX beams since the target beam SNR is worse than the serving beam. Proposal 3: RAN4 are encouraged to investigate the potential beam switching issues in unidirectional scenarios when UE is moving in the direction opposite to the pointing direction of RRH TX beams. We would like to understand how signaling could address the above issue. |
| QC | Even if RAN4 agrees to have the same measurement requirements for uni and bi-directional deployment, network signaling to distinguish the two is still have from other perspective, e.g., UL timing and Doppler shift change in demod. The issue Nokia raised is tough to resolve, but proposal 5 can be helpful in this case. UE may be able to use fine beam to search for the beam from new RRHs from neighboring cells based on the mapping information, since UE already knows the fine Rx beam for the corresponding gNB Tx beam. This can help UE to identify RRH before regular search side condition is satisfied, which provides more time for search and measurement on target cell and the HO procedure can be triggered earlier. |
| ZTE | If unified Rx beam number assumed for uni- and bi- direction deployments, which means unified RRM requirements identified for the both deployments, no need to signal of uni-/bi-direction deployment to UE. Otherwise, the signalling is needed. |
| Huawei | Now we are open to signal uni/bi-derectional scenario information to UE. As UE has some private implementation, these information may be useful. |
| Ericsson | To our understanding, differentiation of requirements for uni- or bi-direction is beneficial, meanwhile It introduces flexibility and enhancement possibility. We generally agree with first sentence by QC. To solve the question raised by Nokia, essentially, we shall have below information at least: - Explicit or implicit siganlling of uni/bi direction - Explicit or implicit siganlling of UE moving direction - UE shall know next TCI state/SSB beam/ Explicitly or implicitly Maybe the 3rd step is difficult to get agreement on how to realize it right now, we believe there are several solutions can realize it, QC has a suggestion also. But maybe it shall be checked in issue on network assistance information, if come back to this issue, we suggest having signaling of uni/bi direction because reliance on UE’s detection capability is difficult. |
| CATT | It depends on the outcome of conclusion of detailed RRM requirements. Uni-direcional and bi-directional have the same RRM requirement, it is not necessary to add such signalling from RRM perspective. But considering the issue from Nokia, we agree such information might be useful. In addition, we think by changing the power level of beams can solve it. |
| Apple | We also support enabling uni-directional and bi-directional signaling. However, the uni-directional signaling can be helpful to handle the one time large TA adjustment. |
| QC | One correction to our comment: the proposal 5 mentioned in our previous comment refers to proposal 5 in issue 1-1-4. |
| Intel | Support Option 1. Uni/bi-directional scenario signaling can be helpful in many cases. For the issue raised by Nokia: The Ds_offset was defined as a location with same SNR from neighboring RRHs/Cells, so we are a bit surprised to see the difference of -32.2dB between Serving and Target SNRs at Ds_offset in the analysis present in R4-2119168. Seems like antenna gains are not taken into account. Anyway, we agree that handover issue can still be the case when UE is moving in the opposite direction to the pointing direction of RRH TX beams. However, we still think that CHO can easily resolve this issue. There is no need for network to know the train movement direction to apply CHO, we can consider it for all cases of uni-directional deployment. |
| CMCC | According to the agreements in last meeting, it was agreed to define two set of requirements for Scenario A and Scenario B in terms of number of RX beams per UE: [2] RX beams for all scenarios of scenario A and [6] RX beams for all scenarios of scenario B. From the wording “for all scenarios”, our understanding is that same requirements will be applied for both uni-directional deployment and bi-directional deployment. Since same requirements are applied, we do not see the necessity to have the network signaling to indicate type of deployment (uni- or bi-direction) to UE. However, we are also open to have further discussion if benefits or necessity are identified. |
| Samsung | Suggest to simplify the discussion here only on the necessity of introducing bi-/uni-direcitional indication from RRM benefits perspective. If other benefit identified from Demodulation perspective, it should be provided and proposed in Demod session, but of course it don’t necessarily means the signaling can not be used for multiple purpose. From performance requirement perspective for the number of RX beams, the benefits seems not be able to be identified, because (as also mentioned by other companies), the agreed RX beam numbers are not yet differentiated by uni- and bi-directional deployment. |

#### Issue 1-1-3: Identification of enhanced RRM requirements in HST FR2 deployments

- Background

At RAN4#100-e the following agreement was achieved [R4-2115334]:

  - Add a flag in a form of cell-specific signalling to indicate UE to use different/enhanced RRM requirements in HST FR2 deployments.

The part of the proposal below on network signaling re-confirms the former agreement.

- Proposals and/or Observations

  - Proposal 1 (Ericsson): Support network signaling in LS R4-2114568.
Defer the signaling of UE capacity flag before agreement on issue ‘CPE support for HST FR2 deployment’.

- Recommended WF

  - It is recommended not to discuss this issue any further.

  - The LS can be discussed in Issue 1-3-1.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| XXX |  |
| YYY |  |
| ZZZ |  |
| Huawei | Agree with the recommended WF |
| Ericsson | Agree with Recommended WF |
| CATT | Support Recommended WF. |
| Apple | Support |
| Intel | Agree with Recommended WF |
| Samsung | Support Moderator’s recommended WF |

#### Issue 1-1-4: Signaling of network assistance information

- Background

The moderator’s observation is that, in general, the list of Options from the previous meeting has not changed much. However, neither the proponents of additional network signaling nor the other companies have not provided the analysis of potential gains.
Additionally, as it was discussed at the RAN4#100-e GtW, HST FR2 WI includes only RAN4 and does not assume direct involvement of other WGs. Therefore, the proposals that require additional signaling should be addressed only if they are resolving crustal related to HST FR2 network deployment, i.e., they resolve a problem that prohibit the deployment of the network.

- Proposals and/or Observations

  - Proposal 1 (Apple): Support option 1 and option 2 for network assisted information, i.e., enable network assisted signaling of SSB index and order per RRH, and whether this is uni-directional or bi-directional deployment.

  - Proposal 2 (Apple): Support option 4. Network can determine the proper RRM requirement and signal to UE directly.

  - Proposal 3 (Qualcomm): Network signals uni-direction or bi-direction deployment to UE. In uni-directional deployment, network signals the beam direction of new RRH, to which UE is switching, w.r.t. UE moving direction when it changes.

  - Proposal 4 (Qualcomm): Network assistant signaling in section 2.2 should be introduced to ensure the feasibility of 6Rx beam sweeping.

  - Proposal 5 (Qualcomm): Network can indicate different SSBs on adjacent RRHs having the same QCL property: signal the mapping between the repeated sets of beams from the adjacent RRHs. Network only signals this mapping when the QCL property relationship can be established across RRHs. If deployment scenario doesn’t satisfy the QCL property similarity across RRHs, network doesn’t have to signal these flags.

  - Proposal 6 (Qualcomm): The following additional beam coverage related information can be signaled to UE

    - Distance between the projections of adjacent beam peaks on the track

    - Beam peak direction angle relative to track

    - The 6 dB beam-width projection on track

In addition, UE can report speed to the network.

  - Proposal 7(Ericsson): To our understanding, detailed targets of signaling of network assistance information may comprise:

    - Optimized TCI state switching in condition of sudden SNR drop if network can acquire position of UE relative to RRHs(based on any possible direct or indirect calculation) and/or Dmin.

    - The issue on RRH position at various side of trail rail can be overcame by explicit signaling from network if network send signaling of known neighbor RRH side position information.

    - To correctly signaling UE moving direction to UE in uni-directional deployment, RRH needs to have capacity to identify UE’s moving towards RRH’s direction or backward.  It can be discussed more detailly after agreement on beam sweep number in uni-direcitional deployment.

  - Proposal 8 (ZTE): At least for Scenario A, the necessity of introducing NW assistance information signalling is not obvious.

  - Observation 1 (ZTE): For Scenario B, some simple NW assistance information signalling can be further studied.

- Candidate options:

  - Option1: Enable network assisted signaling of SSB index and order per RRH

  - Option 1a: Network can indicate different SSBs on adjacent RRHs having the same QCL property: signal the mapping between the repeated sets of beams from the adjacent RRHs

  - Option 2: In uni-directional deployment, network signals the beam direction of new RRH, to which UE is switching, w.r.t. UE moving direction when it changes

  - Option 3: The following additional beam coverage related information can be signaled to UE

    - Distance between the projections of adjacent beam peaks on the track

    - Beam peak direction angle relative to track

    - The 6 dB beam-width projection on track

    - In addition, UE can report speed to the network.

  - Option 4: Signaling of known neighbor RRH side position information

- Recommended WF

  - Discuss in the 1st round if any of proposed signaling is crucial for HST FR2

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Nokia | We’re open to discuss the proposals and the same comment as in Issue 1-1-2. |
| QC | The benefit of option 1a is clear, since UE can leverage the beam refinement from the current RRH to infer the Rx beam for the next RRH. However, the benefit of network signaling of SSB index without the mapping information, i.e., option 1, is not clear. The only thing we can infer from option 1 without option 1a is whether the new TCI state is from a new RRH. But this can be signaled by a flag with one bit instead of all the SSB indexes. The proponent of option 1 without option 1a needs to clarify the benefit. One issue for option 2 is how gNB infers the UE moving direction. Since network has to estimate frequency offset of the received signal on UL, the UE moving direction can infer from the sign of frequency offset. In fact, since UE moving direction is fixed, UE signaling is another option to inform gNB the UE moving direction to get the new (next) RRH beam direction. Nokia’s comment is addressed in issue 1-1-2. |
| ZTE | The gain of introducing network assistance information is not clear, especially for small RX beam number case. So we believe for Scenario A, network assistance information is not necessary. For Scenario B, Option 1 and Option 1a can be further studied. |
| Ericsson | Support Option 2, we understand QC’s comments, our worry is occasional change of direction, in this case UE or NW needs extra time maybe misses RRHs to detect direction of UE moving. Regarding Option 4, even current assumption of RX beam number in scenario B covers both RRH side positions to railway but it is a problematic that practical UE may ignore the hidden fact that RRH’s side positions are not always fixed. Regarding our understanding on the possible use cases and benefits of option 1 or variants of option 1, even it can be used for other use cases, it may be used to implicitly identify the angel/position to RRH. We can keep it open because it can help to mitigate issue in Issue 1-1-2, but we don’t think the solution is very mature to have a signaling, taking different Ds and Dmin into account. |
| CATT | For the moving direction, the BS can estimate the UL signal to achieve rough droppler results to know estimate the direction. But for the signalling in option 2, how does it sent to CPE in different trains if the trains are in opposite directions? |
| Apple | Option 1 provides multiple information that UE might use to improve performance. First, the number of SSBs per RRH can be used to implicitly derive the angle/position to RRH, as Ericsson commented. It is also an abstracted information of option 3, as direct signaling of all the physical parameters can be difficult. Second, based on order of SSB index along the track, UE can expect the next TCI state to switch too. This is similar to option 2, without gNB to estimate UE moving direction. And can possibly derive the receiver Rx beam. If new QCL type can be defined in RAN1 to enable 1a, then UE Rx beam is clearly defined. Even if 1a is not specified in R17 due to RAN1 impact, it is still helpful to derive the Rx beam as CPE runs on fixed track. Third, the SSB index/order per RRH signaling helps UE autonomous one time TA adjustment. With the assisted information UE knows when apply the large TA adjustment when the corresponding TCI state switching commend is received. Current specification signal SSB index bitmap, without RRH information. With very limited overhead, the SSB index/order per RRH can be signaled. |
| QC | To Apple: Number of SSB implies the angle and position only when the beam is separated evenly on angular domain, which is not necessarily true in HST, as we demonstrated in our previous analysis, uneven beams (in angular domain) works better than even distributed beams. The second benefit mentioned is also under the assumption that SSB is in order, which is again not necessarily guaranteed in HST. For comment to 1a, we address in the following: To Apple’s comment: this is a network assistant information, the QCL term we use is mainly for description purpose, referring to the same/similar RRH DL beam to train track angle. Therefore, it doesn’t require new definition from RAN1, as this is revealing network beam deployment information instead of defining new RS relationship. To Ericsson’s comment: option 1a signlaing is opportunistic. If Ds and Dmin is uniform across RRHs, we can utilize this signaling to assist UE. If they are different across RRH and therefore no mapping can be established, network doesn’t have to signal this mapping. |
| Intel | Even though we see the benefits of most of the proposals, we prefer to minimize the involvement of other WGs and limit the newly defined signaling. Considering trade-off between the benefits and other WG specs impact we can support Option 1. We agree with most of the advantages of this option mentioned in Apple’s comment. We also see the motivation of Option 2, we had similar proposal two meetings ago. However, our view is that the direction change is the corner case and it should not be treated separately. |
| Samsung | Similar view as Intel. We can identify some benefits form some complex design, but it should be noted there is not RAN1/2 TU for this work item when WID is approved in plenary: In other words, except some of simple RRC signaling work expected by RAN2, there is no intention to introduce new MAC-CE or other complex procedure in this work item. Seems the question needs to be answered by the group is: whether or not the system can work if the new MAC-CE or other new mechanism is not yet introduced. |

### Sub-topic 1-2: UE capabilities

Sub-topic description

This sub-topic is devoted to the discussion of CPE capabilities that may be needed in HST FR2 deployments.

Open issues and candidate options before e-meeting:

Presented as background in the corresponding issues.

#### Issue 1-2-1: Capability to support different RX beam sweeping number

- Background

The issue refers to the RAN4#100-e agreement on the number of supported RX beams:

  - Scenario A: [2] RX beams for all scenarios

  - Scenario B: [6] RX beams for all scenarios

- Proposals and/or Observations

  - Proposal 1(Apple): Support UE capability of report different Rx beam sweeping number in uni/bi-directional operation.

  - Proposal 2 (CATT): No need to define separate UE capability for Scenario A and Scenario B.

  - Proposal 3 (Qualcomm): Define different UE capabilities for the two sets of requirements. The 2Rx and 6Rx sweep agreements only apply to neighboring cell and L1-RSRP measurement requirement.

  - Proposal 4 (CMCC): it is not necessary to introduce different UE capabilities to indicate the support of Scenario A and B.

  - Proposal 5 (Ericsson): UE shall support Scenario A and B both, no capacity is needed to distinguish them.

  - Proposal 6 (Huawei): Define UE capability to indicate support scenario A or scenario B or both.

  - Proposal 7 (ZTE): Operators’ suggestions should be firstly collected referring to the deploy of Scenario A and Scenario B.

- Candidate options:

  - Option 1: No capacity is needed to distinguish Scenario-A and -B.

  - Option 2: Define different UE capabilities for Scenario-A and Scenario-B

  - Option 2a: Define UE capability to indicate support Scenario-A or Scenario-B or both

  - Option 3: Define UE capability of report different Rx beam sweeping number in uni/bi-directional operation.

- Recommended WF

  - Continue the discussion in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| QC | Since 2 and 6Rx beams correspond to different search codebook design, we can’t assume UE supports two sets of codebooks, and UE capability is needed if separate requirements are defined. |
| ZTE | For a railway line from the starting point to the ending point, if operator decides to deploy RRHs in the form of Scenario A in some sections of the railway line and deploy RRHs in the form of Scenario B in other sections of the railway line according to the geographical environment, even if the UE reports that it only supports Scenario A, can the UE stop running when the train reaches to the section where Scenario B is deployed? Therefore, we do not need to discuss this issue blindly, and we need to hear more suggestions from operators. |
| Huawei | Support option 2. For HST FR2 CPE, the deployment scenario A and scenario B differ widely. The UE behavior of RX sweeping in the two scenarios is diverse, which lead to different L3 measurement, beam management, handover operation etc. If mandate HST FR2 CPE to support both scenario A and B, UE needs to support two sets of implementation design. To provide implementation freedom for UE, we suggest to define UE capability for whether supporting scenario A and scenario B. |
| Ericsson | We understand the issue is only about capacity, e.g. the question is like: if A UE has capacity - Only support A, - Only support B - Support A and B but adaptively change number for A and B with signaling If above understanding is correct, we assume the new type CPE shall have capacity of option 3 to be compatible with practical network implementations, e.g. it is difficult to guarantee 10 meters distance between RRH and rail for a long-distance train. |
| CATT | We think the CPEs should support both Scenario A and Scenario B. The HST cells along the tail can be Scenario A and Scenario B in different area for network deployment. For the different requirements, UE can use NW ignaling. |
| Apple | Support different UE capability with different RRM requirement. |
| Intel | Support Option 2. Scenario A and B describe two different approaches for HST deployment: dedicated FR2 HST deployment (Dmin=10m) vs FR1 HST deployment reuse (Dmin=150). We don’t think that mixed A-B scenario should be the part of Rel-17 HST FR2. Based on that there is no need for UE to support both scenarios. The corresponding UE differentiation can be used. |
| CMCC | Option 1. Whether scenario A or scenario B is deployed is up to operator deployment, our preference is that both Scenario A and B are supported by the high-speed CPEs. |
| Samsung | Option 1: No capacity is needed to distinguish Scenario-A and –B Even there are two sets of requirements provided by different NW signaling for Scenario-A and B, but we still prefer to have UE mandatory to support both sets of requirement for A and B, and no need to differentiate UE type here. |

#### Issue 1-2-2: Capability to support HST FR2 scenario

- Background

At the RAN4#100-e meeting the two issues below were listed [R4-2117299].

  - Continue the discussion of CPE support for HST FR2 deployment:

    - Option 1: It is not necessary to introduce UE capability to indicate the support of FR2 HST.

    - Option 2: Define UE capability for FR2 HST enhancement support.

  - FFS the ways to differentiate HST FR2 capable CPEs from any other UEs in the specification:

    - Option 1: Apply enhanced RRM requirements for FR2 HST based on Power Class corresponding to FR2 UE in TS 38.101-2.

    - Other options are not precluded.

In principle, no new options were introduced since the previous meeting. Most of the companies except one still prefer Option 1.

- Proposals and/or Observations

  - Proposal 1 (CATT): It is not necessary to introduce UE capability to indicate the support of FR2 HST.

  - Proposal 2 (CATT): After the conclusion of CPE in RF session, the RRM requirements can be specified based on Power class.

  - Proposal 3 (Qualcomm): Define UE capability for FR2 HST enhancement support.

  - Proposal 4 (CMCC): considering that the power class for highs-peed CPEs will be specified, it is not necessary to introduce UE capability additionally to indicate the support of FR2 HST.

  - Proposal 5 (Ericsson): The definition of UE capacity in RRM hypothetically assumes non-CPE exist or present in HST FR2 scenario. How to handle non-CPE shall be discussed in other session and be reflected into the issue on UE capacity and this issue in turn.

  - Proposal 6 (Ericsson): Support Option 1: It is not necessary to introduce UE capability to indicate the support of FR2 HST.

  - Proposal 8 (Ericsson): Regarding UE without enhancement support, if it must be checked, the issue is how to deal with UE without enhancement support in FR2 HST scenario essentially, which shall be discussed in other session.

  - Proposal 9 (Ericsson): Related to differentiating HST FR2 capable CPEs from any other Ues in the specification, we suggest to get agreement on proposal 8 firstly.

- Recommended WF

  - Discussion in the 1st round whether Option 1 (It is not necessary to introduce UE capability to indicate the support of FR2 HST) is agreeable.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| QC | As we commented previously, UE capability is necessary to allow RRHs serving the CPE on train rooftop and the nearby non-highspeed Ues that are not on the train. Power class is still under discussion in RF, and there is a potential backward compatibility issue in the future: currently even if the power class is dedicated to FR2 HST, if we enlarge the applicability scenario sets in the future, gNB may not be able to distinguish Ues by power class. |
| Ericsson | Support option 1. Regarding power class, we have concern on the future utilization similar to QC. It’s not preferred to use power class or use power class only to distinguish HST FR2 CPE. |
| CATT | We support option 1. For the non-CPE in the future, firstly, we don’t know whether we can extend it to non-CPE with the requirements unmodified. |
| Apple | Support option 1. |
| Intel | Support Option 2. In our understanding the UE capability is much more straightforward way to differentiate HST FR2 capable CPEs from other UEs comparing to differentiation based on power class. |
| CMCC | Power class dedicated for high-speed CPEs are under discussion in RF session, which can be used to differentiate the HST-UE from other UEs (i.e. non-HST UE). It seems confusing to introduce both power class and UE capability, which have the similar function. In our view, if the power class dedicated for high-speed CPEs is defined, no need to additionally introduce new UE capability indicate the support of FR2 HST. |
| Samsung | Option 1: It is not necessary to introduce UE capability to indicate the support of FR2 HST. Based on current discussion in RF session, it is likely the new power class is introduced for FR2 HST UE. It should be noted that in TS38.101-2, the power class is linked with UE type (for certain application scenario). Another example is PC1/5’s RRM requirement is also defined based on its claimed PC support while no other signaling introduced. |

#### Issue 1-2-3: Capability to change RX beam sweep number

- Background

At the RAN4#101-e a need for CPE capability to change characteristics was discussed, e.g., RX beam sweep number in uni-/bi-directional operation.

- Proposals and/or Observations

  - Proposal 1 (CATT): No need for CPE capability to change characteristics, e.g. RX beam sweep number in uni-/bi-directional operation.

  - Proposal 2 (Ericsson): CPE shall have capacity to change beam sweep number in uni-/bi-directional operation from implementation point of view. And if flag is agreed to signal uni-/bi-directional case, UE shall change beam sweep number following the signaling.

- Candidate options:

  - Option 1: No need for CPE capability to change beam sweep number.

- Recommended WF

  - Confirm that Option 1 is agreeable.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Huawei | The issue is not quite clear. What’s the motivation or benefit of letting CPE change beam sweeping number? |
| Ericsson | We don’t see strong reason to keep same RX beam sweep number in uni-/bi-directional operation. Essentially, it is a rationale assumption bi-direction shall use double RX beam sweep number than uni-direction in scenario A. Our intention is to keep Scenario A simple as possible. For Scenario B, it can be flexible. |
| CATT | Option 1 is agreeable. |
| Intel | Agree with Option 1. |
| Samsung | I assume when the group discussion whether or not to introduce different RX beam number for uni-and bi-directional scenarios in last meeting, the group well know the performance can be better for uni- case, while still get the conclusion as last meeting. Anything changed? |

### Sub-topic 1-3: LSs and CR work split

Sub-topic description

In this subtopic tow LS and CR work load split is discussed.

Open issues and candidate options before e-meeting:

Presented as background in the corresponding issues.

#### Issue 1-3-1: LS on UE capability and network signalling

- Background

  - At RAN4#100-e meeting it was agreed [R4-2115334] to
“Add a flag in a form of cell-specific signalling to indicate UE to use different/enhanced RRM requirements in HST FR2 deployments.”

- Recommended WF

  - In the 1st round, companies are invited to discuss if it is possible to proceed with the LS.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Nokia | Nokia can take responsibility. |
| YYY |  |
| ZZZ |  |

#### Issue 1-3-2: LS on Beam Management Enhancement Signaling

- Background

  - The Draft LS is included as an Appendix in R4-2117627.

  - The discussion related to the LS takes place in Issue 1-1-3.

- Recommended WF

  - In the 1st round, companies are invited to discuss if it is possible to proceed with the LS.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Nokia | Pending the outcome of discussions in Issue 1-1-4 |
| ZTE | We are not sure any conclusion about network assistance signaling has been achieved in RAN4. Before sending LS to other group, we should discuss the necessity of beam management enhancement signaling in RAN4 firstly, i.e. wait for the conclusion in Issue 1-1-4. |
| ZZZ |  |

#### Issue 1-3-3 HST FR2 RRM CR work split

- Background

  - A template table for the work split is proposed in R4-2119478.

- Proposals:

  - Proposal 1 (Nokia): RAN 4 to agree on the HST FR2 RRM CR work split based on the table above.
Companies are encouraged to sign up for desired items.

| TS 38.133 section number | TS 38.133 section title | Responsible company |
| --- | --- | --- |
| 4 | SA: RRC_IDLE state mobility |  |
| 4.1 | Cell Selection |  |
| 4.2 | Cell Re-selection | ZTE |
| 5 | SA: RRC_INACTIVE state mobility |  |
| 5.1 | Cell Re-selection |  |
| 6 | RRC_CONNECTED state mobility |  |
| 6.1 | Handover |  |
| 6.2 | RRC Connection Mobility Control |  |
| 7 | Timing |  |
| 7.1 | UE transmit timing | Ericsson |
| 7.2 | UE timer accuracy |  |
| 7.3 | Timing advance |  |
| 7.4 | Cell phase synchronization accuracy |  |
| 7.7 | deriveSSB-IndexFromCell tolerance |  |
| 8 | Signalling characteristics |  |
| 8.1 | Radio Link Monitoring | CATT |
| 8.5 | Link Recovery Procedures | CATT |
| 8.10 | Active TCI state switching delay | Samsung |
| 8.12 | Uplink spatial relation switch delay | Samsung |
| 9 | Measurement Procedure |  |
| 9.2 | NR intra-frequency measurements | Ericsson |
| 9.5 | L1-RSRP measurements for Reporting | ZTE |
| 9.8 | L1-SINR measurements for Reporting |  |
| 9.10 | CSI-RS based L3 measurements |  |

- Recommended WF

  - Use the table above to track record of companies responsible for the preparation of draft CRs to TS 38.133.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| ZTE | We agree with moderator ’s work split suggestion. ZTE would like to undertake the sub-clauses 4.2 and 9.5, and we fill in our name in the last column on top of the table given by the moderator as above |
| Ericsson | 7.1 and 9.2 |
| CATT | CATT volunteers to take 8.1&8.5. |
| Samsung | 8.10 and 8.12, which is also included in the table of the email already. |

### CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing Wis, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2118340, CR On HST FR2 RRM, Ericsson | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

| Sub-topic | Status summary |
| --- | --- |
| Sub-topic #1-1: Network signaling | Issue 1-1-1: Signaling for differentiation between Scenario-A and Scenario-B Background: The companies still have different opinions on signaling need. However, the common understanding is that the signaling is dependent on the difference in the requirements. Additionally it was noted by several companies that the detailed signaling is in the scope of RAN2. Tentative agreements: None Candidate options: - Option 1 (Huawei, Ericsson, Apple, Intel, Samsung): Enable network signaling - Option 2 (QC, ZTE): It is not necessary to introduce network signaling, if unified requirements are specified - Option 3 (QC, Ericsson, Apple): Combined 2-dimentional signaling: Uni- or Bi-directional, and Scenario-A or -B Recommendations for 2nd round: Keep the discussion open in the second round to take into account possible agreements achieved in Topic#3, Issues 2-1-1, 2-1-2. Issue 1-1-2: Signaling of uni-/bi-directional operation Background: It can be observed that there is a group of companies supporting signaling of the type of deployment. However, other companies see a need to wait for a clarification on the difference of requirements in these deployments (Issue 2-1-2), especially taking into account the agreement to define “two set of requirements for Scenario A and Scenario B in terms of number of RX beams”. Additionally, one of the companies has raised an issue with HO when the train travels in the direction opposite to the serving beam orientation. The issue is confirmed by a number of companies but it seems that deployment signaling cannot resolve an issues on its own. Tentative agreements: None Candidate options: - Option 1 (Qualcomm, Ericsson, Apple, Intel): Network signals type of deployment (uni- or bi-direction) to UE. - Option 2 (Samsung): Signaling of uni-/bi-directional operation is not needed. Recommendations for 2nd round: - Continue the discussion in the second round - Limit the discussion in this issue only to uni-/bi-directional signaling. - Take into account the discussion/agreements in the Issues 2-1-2. - Define a new issue to discuss whether assistance signaling is needed to resolve an issues when the train is travelling direction is opposite to the serving beam orientation. Issue 1-1-3: Identification of enhanced RRM requirements in HST FR2 deployments Background: The companies have confirmed the agreements from the previous RAN#100-e meeting [R4-2115334]: - Add a flag in a form of cell-specific signalling to indicate UE to use different/enhanced RRM requirements in HST FR2 deployments. The corresponding combined LS to RAN2 is discussed but as a separate Issue. Tentative agreements: None Candidate options: - None Recommendations for 2nd round: None Issue 1-1-4: Signaling of network assistance information Background: The companies still have different opinions about needed assistance information from the networks side. However, the most active discussion was around Option 1, Option 1a, and Option2. However, it was noted that a need for Option 2 is dependent on a need to consider the change of RRH panel orientation in uni-directional scenario. One of the companies also had an opinion that assistance signaling is mostly useful in Scenario-B. Finally, it was reminded by one of the companies that there is not RAN1/2 TUs for this WI. Hance, it is necessary to answer the question: whether or not the system can work if the new MAC-CE or other new mechanism is not yet introduced Tentative agreements: None Candidate options: - Option1: Enable network assisted signaling of SSB index and order per RRH - Option 1a: Network can indicate different SSBs on adjacent RRHs having the same QCL property: signal the mapping between the repeated sets of beams from the adjacent RRHs - Option 2: The system can work without such assistance signaling Recommendations for 2nd round: - Continue the discussion only of the options above in this issue - Discussion Option 2: In uni-directional deployment, network signals the beam direction of new RRH, to which UE is switching, w.r.t. UE moving direction when it changes and a change of RRH panel orientation in a separate Issue. |
| Sub-topic #1-2: UE capabilities | Issue 1-2-1: Capability to support different RX beam sweeping number Background: Companies have different opinions: some companies think that if the requirements are different then the UE can support only one of those. Whereases, other companies have an opinion that UE shall support both scenarios and adopt the number of beams accordingly. One of the companies is wondering what will happen with a UE supporting only Scenario-A if it enters the area where Scenario-B is deployed? However, the other company thinks that mixed scenarios shall not be supported in Rel-17. Tentative agreements: None Candidate options: - Option 1 (QC, Huawei, Apple, Intel): Define different UE capabilities for Scenario-A and Scenario-B. - Option 2 (Ericsson, CATT, CMCC, Samsung): UE can support both Scenario-A and -B and adapt the number of Rx beams to those. No capacity is needed. Recommendations for 2nd round: Include both options directly to the WF. Issue 1-2-2: Capability to support HST FR2 scenario Background: Companies have different views on the issue. However, the discussion in the second round can take into account the RF agreement to introduce an new UE Power Class 6 (High Speed Train Roof-Mounted UE) Tentative agreements: None Candidate options: - Option 1(Ericsson, CATT, Apple, CMCC, Samsung): It is not necessary to introduce UE capability to indicate the support of FR2 HST. - Option 2(QC, Intel): Define UE capability for FR2 HST enhancement support. Recommendations for 2nd round: Continue the discussion taking RF agreement into account Issue 1-2-3: Capability to change RX beam sweep number Background: The majority of the companies do not see a need to introduce the differentiation and Tentative agreements: No need for CPE capability to change beam sweep number in uni-/bi-directional operation. Candidate options: - None Recommendations for 2nd round: Agree on tentative agreement. |
| Sub-topic #1-3: LSs and CR work split | Issue 1-3-1: LS on UE capability and network signalling Background: It does not look like the companies are ready to submit the LS yet. Tentative agreements: None Candidate options: - none Recommendations for 2nd round: None Issue 1-3-2: LS on Beam Management Enhancement Signaling Background: It was noted by two companies that more discussion are still needed in the Issue 1-1-3. Tentative agreements: None Candidate options: - None Recommendations for 2nd round: None Issue 1-3-3: HST FR2 RRM CR work split Background: The table for CR work split was shared, and some companies identified their interest. Tentative agreements: TS 38.133 section number; TS 38.133 section title; Responsible company. 4; SA: RRC_IDLE state mobility. 4.1; Cell Selection. 4.2; Cell Re-selection; ZTE. 5; SA: RRC_INACTIVE state mobility. 5.1; Cell Re-selection. 6; RRC_CONNECTED state mobility. 6.1; Handover. 6.2; RRC Connection Mobility Control. 7; Timing. 7.1; UE transmit timing; [Nokia, Ericsson]. 7.2; UE timer accuracy. 7.3; Timing advance. 7.4; Cell phase synchronization accuracy. 7.7; deriveSSB-IndexFromCell tolerance. 8; Signalling characteristics. 8.1; Radio Link Monitoring; CATT. 8.5; Link Recovery Procedures; CATT. 8.10; Active TCI state switching delay; Samsung. 8.12; Uplink spatial relation switch delay; Samsung. 9; Measurement Procedure. 9.2; NR intra-frequency measurements; [Nokia, Ericsson]. 9.5; L1-RSRP measurements for Reporting; ZTE. 9.8; L1-SINR measurements for Reporting. 9.10; CSI-RS based L3 measurements. Recommendations for 2nd round: Include in the table into the WF. |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

TBA

### Sub-topic 1-2: TBA

#### Issue 1-2-2: TBA

Agreements from round 1:

TBA

Candidate options:

- Option 1:

- Option 2:

Recommendations for 2nd round:

TBA

Contributor Comments:

| Company | Comments |
| --- | --- |
| XXX |  |
| YYY |  |
| ZZZ |  |

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

# Topic #2: Number of RX beams

Main technical topic overview. The structure can be done based on sub-agenda basis.

This topic is devoted to the discussion of difference in RRM requirements in relation to the number of RX beams at CPE in HST FR2 deployments from the RRM perspective.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2117300 | Apple | Discussion on number of Rx beam for FR2 HST Proposal 1: Support implicit or explicit network signaling and UE capability for Scenario A and Scenario B RRM requirements. [Moderator]: Already treated in Topic#1. Proposal 2: RRM requirement for scenario B applies to the case where RRH position at one or both sides of rail track. |
| R4-2117365 | CATT | Further discussion on number of RX beams for FR2 HST Proposal 1: Define two series RRM requirements for differentiate scenarios for better power consumption performance from UE perspective based on different RX beam numbers in different scenarios. No need to define different UE capabilities for Scenario A and Scenario B. |
| R4-2118345 | Ericsson | Number of RX beams for HST FR2 Proposal 1: Scenario A: 1 RX beams for uni-directional scenario, 2 RX beams for bi-directional scenario. Proposal 2: Scenario B: 4 RX beams for all scenarios. Proposal 3: UE capability shall support Scenario A and B, but UE shall be able to follow different requirements through changeable configurations according to scenario A and B if it is decided to use Scenario A and B as reference or one of references to specify different requirements in document. [Moderator]: treated in Topic#1. Observation 1: Various Dmin could cause SNR degradation if RX beam direction cannot change adaptively with Dmin. Proposal 4: Adaptive RX beam change with Dmin based on explicit or implicit signaling by network can provide more freedom and margin of Dmin in deployment. Proposal 5: If it is decided to include Dmin in condition(s) to specify different RRM requirements, [50]m is a compromised number to distinguish different RRM requirements. Proposal 6: Even number of Dmin is reference or one of references to distinguish different RRM requirements, we don’t suggest to use Dmin or distance number in definitions directly. We propose the scenarios for different RRM requirements in below: - [Scenario A]: Deployments in which the basestations can always be expected to be visible within a single beam at one of the two panels. This scenario is typical when the maximum BS to track distance is low; e.g. less than 50m. - [Scenario B]: Deployments in which the basestations are visible within one of 3 beams at one of the two panels. This scenario is typical when minimum BS to track distance is larger; e.g. greater than 50m. Proposal 7: [6] RX beams in scenario-B covers impact of RRH position at one/both sides of rail track according to GtW meeting. Further enhancement shall not be precluded, more efficient beam sweeping achieved by signaling from network can mitigate impact from unpredictable various RRH position. Proposal 8: Uni-direction and bi-direction shall also be included in conditions or benchmark to specify different RRM requirements, since we stand on that view: in scenario A: 1 RX beams for uni-directional case, 2 RX beams for bi-directional case. No reason to waste operation on useless directions in uni-directional case. |
| R4-2119109 | ZTE Corporation | Number of RX beams for HST FR2 Observation 1: In order to achieve better beam performance, two CPE panels are needed just for uni-directional two sides RRH deployment, so four CPE panels may be needed for bi-directional two sides RRH deployment. Proposal 1: 2 RX beam per CPE panel is enouth for both uni- and bi-directional deployments of Scenario A. Proposal 2: 6 RX beam per CPE panel is a bit redundant for Scenario B, especially for bi-directional deployment. Proposal 3: Firstly identify whether some update are needed for the previous agreements. If no, we do not need to discuss this issue. Proposal 4: If we identify update the previous agreements, RRM requirements are defined with adapted scaling factor based on explicit or implicit signalling from network to UE. Proposal 5: If considering the possibility of two sides deployment, not only adapted scaling factor should be supported, the NW signalling indication should also be studied further. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 2-1: The scope of RRM requirements

Sub-topic description:

Open issues and candidate options before e-meeting:

GtW agreements at RAN4#100-e:

- RX beam number for RRM requirements definition

  - Define two set of requirements for Scenario A and Scenario B in terms of number of RX beams per UE

    - Scenario A: [2] RX beams for all scenarios

    - Scenario B: [6] RX beams for all scenarios

    - FFS on feasibility and methods to differentiate scenarios from UE perspective

    - FFS if different UE capabilities shall be used for Scenario A and B support

  - Note: if there is insignificant difference between Scenario A and B requirements, then further discussion on unified requirements can take place

Way forward:

- Discuss the FFS issues from GTW agreement and the possibility to unify the requirements further.

#### Issue 2-1-1: Requirements for Scenario-A and Scenario-B

- Background

- Proposals and/or Observations

  - Proposal 1 (CMCC): it is preferred to have unified requirements to cover scenario A and scenario B.

  - Proposal 2 (CATT): Define two series RRM requirements for differentiate scenarios for better power consumption performance from UE perspective based on different RX beam numbers in different scenarios. No need to define different UE capabilities for Scenario A and Scenario B.

  - Observation 1 (Ericsson): Various Dmin could cause SNR degradation if RX beam direction cannot change adaptively with Dmin.

  - Proposal 3(Ericsson): Adaptive RX beam change with Dmin based on explicit or implicit signaling by network can provide more freedom and margin of Dmin in deployment.

  - Proposal 4(Ericsson): If it is decided to include Dmin in condition(s) to specify different RRM requirements, [50]m is a compromised number to distinguish different RRM requirements.

  - Proposal 5(Ericsson): Even number of Dmin is reference or one of references to distinguish different RRM requirements, we don’t suggest to use Dmin or distance number in definitions directly. We propose the scenarios for different RRM requirements in below:

    - [Scenario A]: Deployments in which the basestations can always be expected to be visible within a single beam at one of the two panels. This scenario is typical when the maximum BS to track distance is low; e.g. less than 50m.

    - [Scenario B]: Deployments in which the basestations are visible within one of 3 beams at one of the two panels. This scenario is typical when minimum BS to track distance is larger; e.g. greater than 50m.

- Candidate options:

  - Option 1: Define unified requirements

  - Option 2: Define two sets of requirements for Scenario-A and Scenario-B

  - Option 2a: Define two sets of requirements, including Dmin in condition(s) to specify different RRM requirements

- Recommended WF

  - Discuss candidate options in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Nokia | We discuss the above candidate options in detail in our contribution, focusing on cell mobility and beam management requirements. The comments in Topic 3 are relevant here. |
| QC | We support option 1. Since scenario A and B have the same ISD, if 6Rx beams assumption can work for scenario B, it should also work for scenario A. If RAN4 ends up with two sets of requirement for scenario A and B, Dmin conditions for scenario A should be captured. Since the 2Rx number is derived based on link budget and performance analysis under Dmin and height difference <= 10m condition, it should be captured as a side condition for scenario A requirement. |
| Huawei | Option 2 is preferred. A clarification question on option 2a: if the definition of scenario A and scenario B is decided by UE, i.e., base stations is expected to be visible within a single/ one of 3 beams at one of the two panels, whether network has the same understanding as UE. This question is related with the signaling issue. |
| Ericsson | Differentiation of requirements regarding Scenario-A and Scenario-B or distance between RRH and railway. Anyhow we expect different RX beam number shall be specified in different scenarios and the bound of different scenarios is listed in Issue 2-2-1. |
| CATT | We support option 2. because Scenario A and Scenario B are different. |
| Apple | Support option 2 |
| QC | To address Huawei’s comment: our proposal for capturing the Dmin and height difference as guidance to deployment. If network wants to signal this scenario A enhancement, we need to confirm that Dmin and height difference satisfies the constraint, so that the base station is visible within a single beam, and network should have symmetric information because UE only move along the track. |
| Intel | Support Option 2 |
| CMCC | Our preference is option 1, but we are open to discussion. Our consideration is that the difference between scenario A and scenario B is Dmin: Dmin = 10 meters for scenario A and Dmin = 150 meters for scenario B. In our understanding, no matter 10m or 150m, both are selected to reflect the deployment. However, we cannot say that Dmin is always 10 meters for scenario A, and neither that Dmin is always 150m for scenario B, since the practical deployment is also impacted by geography. From this point of view, it is preferred to have unified requirements. |
| Samsung | Support option 2 by following last meeting’s agreement. |

#### Issue 2-1-2: RRM requirements for uni-directional and bi-directional deployments

- Background

In the previous meeting the number of beams in Scenario-A and Scenario-B was agreed independent on the type of deployment (i.e., for all scenarios).

- Proposals and/or Observations

  - Proposal 1(ZTE): Considering for different Rx beam number for uni- and bi-directional deployments, defining different RRM requirements for uni-directional and bi-directional deployments is needed.

  - Proposal 2 (Ericsson): Uni-direction and bi-direction shall also be included in conditions or benchmark to specify different RRM requirements, since we stand on that view: in scenario A: 1 RX beams for uni-directional case, 2 RX beams for bi-directional case. No reason to waste operation on useless directions in uni-directional case.

- Candidate options:

  - Option 1: Define only two sets of requirements (for Scenario A and Scenario B) in terms of number of RX beams

  - Option 2: Define different RRM requirements for uni-directional and bi-directional deployments.

- Recommended WF

  - Keep the GtW agreement from RAN4#100-e (Option 1) and re-confirm understanding in the first round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Nokia | It is better to discuss per RRM requirement rather than in general sense. We provide detailed comments in Topic 3 on this issue. |
| QC | We support recommended WF, but signaling is still needed. |
| ZTE | We support Option 1. |
| Huawei | Option 1. From defining requirements perspective, scenario A and B request different beam numbers, then different requirements are supposed to be defined. However from network indication perspective, we are open whether uni-/bi directional deployment is signaled or not. |
| Ericson | Support Option 1. Regarding to Option2, at least, the consideration in scenario A is straightforward and doable: 1 RX beams for uni-directional case, 2 RX beams for bi-directional case. May we know the reason keeping 2 opposite RX beam in uni-directional deployment of scenario A? |
| CATT | Support Recommended WF. |
| Apple | Support option 1 to simplify the RRM requirement. Also signaling uni-directional/bi-directional is helpful to perform better than the minimum requirement by using 1 Rx beam in Scenario unidirectional deployment |
| Intel | Support Option 1 |
| Samsung | Support Moderator’s recommended WF. |

### Sub-topic 2-2: Scenario-A requirements

Sub-topic description

In this sub-topic, the Scenario-A specific parameters are discussed.

Open issues and candidate options before e-meeting:

Presented as background in the corresponding issues.

#### Issue 2-2-1: Applicability of requirements

- Background

TBA

- Proposals and/or Observations

  - Proposal 1 (Qualcomm): The 2Rx beam sweep based requirement applies to the cases in which Dmin and Hdiff are both no greater than 10m.

  - Proposal 2 (Ericsson): Even number of Dmin is reference or one of references to distinguish different RRM requirements, we don’t suggest to use Dmin or distance number in definitions directly. We propose the scenarios for different RRM requirements in below:

    - [Scenario A]: Deployments in which the basestations can always be expected to be visible within a single beam at one of the two panels. This scenario is typical when the maximum BS to track distance is low; e.g. less than 50m.

- Recommended WF

  - Discuss the Proposals in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| QC | We are open to discuss the idea in proposal 2, using “maximum BS to track distance” instead of Dmin. But the distance should align to the assumption for deriving 2Rx, i.e., 10m instead of 50m. Besides BS to track distance, RRH and UE height difference is another assumption that should be captured in the applicability rule for 2Rx beam sweeping requirement. |
| Huawei | Need further consideration on the value (10m) in proposal 1. We had a clarification question on proposal 2 (see comments in issue 2-1-1) |
| Ericsson | The purpose of Proposal 2 is to clarify ambiguity when distance between RRH and railway is not Dmin=10 or Dmin=150 and how to solve it. We more prefer no exact number in spec. to distinguish different scenarios. And we can keep the exact number and height in bracket or TBD. |
| QC | Huawei’s comment is addressed in issue 2-1-1. To Ericsson: in our opinion, between 10m and 150m, 150m requirement should apply because when 10m requirement applies, UE has limited number of beams and performance degradation is expected. Note that the conclusion of 1 beam is sufficient is based on 10m assumption, and when Dmin > 10m, the conclusion may not hold. Since Ds is the same, the requirement works for 150m case can work in Dmin < 150m, and since we have enough beams to cover 150m case, the same number of beams can easily cover Dmin < 150m case. |
| Intel | In our understanding its totally up to Network to decide which set of requirements should be used. If we need to have recommendations for Network than Proposal 2 can be used. |
| Samsung | We don’t see the necessity of introducing restriction on NW deployment. |

#### Issue 2-2-2: Number of Rx beams

- Background

The issues it related to Issue 2-1-2.

- Proposals and/or Observations

  - Proposal 1(Ericsson): Scenario A: 1 RX beams for uni-directional scenario, 2 RX beams for bi-directional scenario.

  - Proposal 2 (ZTE): 2 RX beam per CPE panel is enouth for both uni- and bi-directional deployments of Scenario A.

- Recommended WF

  - TBA

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| QC | We should follow previous meeting agreements on the assumption. Requirement derivation can be discussed separately. |
| YYY |  |
| ZZZ |  |

### Sub-topic 2-3: Scenario-B requirements

Sub-topic description

In this sub-topic, the Scenario-B specific parameters are discussed.

Open issues and candidate options before e-meeting:

Presented as background in the corresponding issues.

#### Issue 2-3-1: Number of Rx beams

- Proposals and/or Observations

  - Proposal 1 (Ericsson): Scenario B: 4 RX beams for all scenarios.

  - Proposal 2 (ZTE): 6 RX beam per CPE panel is a bit redundant for Scenario B, especially for bi-directional deployment.

- Recommended WF

  - Discuss Proposal 1 in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| QC | Option 1 contradict to the previous meeting agreement. We should follow previous meeting agreements on the assumption. Requirement derivation can be discussed separately. |
| ZTE | We support Rx beam reduction for Scenario B, smaller Rx beam number than 6 is enough. |
| Huawei | RF session is evaluating the RX beam number in this meeting. We can wait whether the conclusion on number of RX beams can be confirmed in RF session. |
| Ericson | Bracket was added in last meeting, so it doesn’t break agreement. |
| Intel | We support Proposal 1. |
| Samsung | To Huawei, there discussion in RF session is about spherical coverage, which is not necessarily linked to the number of RX beam used for RRM procedure discussed here. |

#### Issue 2-3-2: Requirements for RRH deployment on both sides of the track

- Background

  - Agreement on deployments from RAN4#98-e:

    - RAN4 primarily consider HST FR2 deployment with

      - One train moving over one railway track in one direction;

      - RRHs are located on one side of the track;

  - Agreement from RAN4#100-e:

    - There is no impact of RRH position at one/both sides of rail track in Scenario-A under the assumption that UE boresight direction (or the beam direction if there is only one beam) is parallel to the track

  - Way forward from RAN4#100-e:

    - Companies are encouraged to evaluate candidate options for Scenario-B considering the GtW agreements

      - Option 1: RRM requirements are defined with scaling factor which is double of number of RX sweep number in scenario study.

      - Option 2: RRM requirements are defined with adapted scaling factor based on explicit or implicit ignaling from network to UE.

      - Other options are not precluded.

- Proposals and/or Observations

  - Proposal 1 (Apple): RRM requirement for scenario B applies to the case where RRH position at one or both sides of rail track.

  - Proposal 2 (ZTE): Firstly identify whether some update are needed for the previous agreements. If no, we do not need to discuss this issue.

  - Proposal 3 (ZTE): If we identify update the previous agreements, RRM requirements are defined with adapted scaling factor based on explicit or implicit ignaling from network to UE.

  - Proposal 4 (ZTE): If considering the possibility of two sides deployment, not only adapted scaling factor should be supported, the NW ignaling indication should also be studied further.

  - Proposal 5: [6] RX beams in scenario-B covers impact of RRH position at one/both sides of rail track according to GtW meeting. Further enhancement shall not be precluded, more efficient beam sweeping achieved by signaling from network can mitigate impact from unpredictable various RRH position.

- Candidate options:

  - Option 1: Define adaptive scaling factor for deployments with RRHs on both sides of the track

  - Option 2: [6] RX beams in scenario-B covers impact of RRH position at one/both sides of rail track

- Recommended WF

  - Discuss in the 1st round whether Option 2 is agreeable.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| QC | We support option 2 from the unified requirement perspective. But so far the 6Rx beam conclusion is based on the analysis of one side, and RAN4 should address the fact that RRHs can be on both sides by increasing number of Rx beams or some scaling factor in the requirement. |
| ZTE | If we really want to update the previous following agreements, we prefer Option 1. - RAN4 primarily consider HST FR2 deployment with - One train moving over one railway track in one direction; - RRHs are located on one side of the track;. |
| Ericsson | With respect to agreement in GtW, we can agree that [6] RX beams in scenario-B covers impact of RRH position at one/both sides of rail track. But as we mentioned, unpredictable various RRH position is risky to UE design and implementation which may occur in practice. At least, we shall note it somewhere in spec. |
| Apple | Option 2 is agreeable. |
| Intel | Agree with Option 2 |
| Samsung | Agree with Option 2. |

### CRs/TPs comments collection

For close-to-finalize Wis and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing Wis, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |
| NONE |  |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

| Sub-topic | Status summary |
| --- | --- |
| Sub-topic #2-1: The scope of RRM requirements | Issue 2-1-1: Requirements for Scenario-A and Scenario-B Background: In general, most of the companies support Option 2 (Define two sets of requirements for Scenario-A and Scenario-B) that also follows the last meeting’s agreements. However, there are companies that still see a possibility to further unify the requirements, e.g., by defining requirements based on Scenario-B (6Rx beams) and with tight requirements on supported DRx (<=[40]ms) Tentative agreements: None Candidate options: - Option 1 (Ericsson, Apple, Intel, Samsung, CATT, Huawei): Define two sets of requirements for Scenario-A and Scenario-B - Option 2 (QC, CMCC): Define unified requirements Recommendations for 2nd round: - Further check if it could be possible to define requirements based on Scenario-B only - Clarify the understanding at GtW session. Issue 2-1-2: RRM requirements for uni-directional and bi-directional deployments Background: Most of the companies think that there is no need to define different RRM requirements for uni-/bi-directional deployments. However, on company thinks that it is better to decide about this issue on per-requirement basis. Some companies noted that signaling of deployment type shall be discussed separately. Tentative agreements: Define only two sets of RRM requirements (for Scenario A and Scenario B) in terms of number of RX beams. No separate requirements for uni-/bi-directional deployments are needed. Candidate options: - None Recommendations for 2nd round: Agree on tentative agreement. |
| Sub-topic #2-2: Scenario-A requirements | Issue 2-2-1: Applicability of requirements Background: Some companies see a need to restrict the applicability of Scenario-A/2Rx requirements to certain deployments. Whereases, other companies do not see a need introducing restriction on NW deployment Tentative agreements: None Candidate options: - Option 1: Restrict applicability of 2Rx requirements: - Dmin and Hdiff are both no greater than 10m - basestations can always be expected to be visible within a single beam at one of the two panels. - Option 2: No restrictions on NW deployment Recommendations for 2nd round: Include the issues FFS to the WF. Issue 2-2-2: Number of Rx beams Background: Seems that companies do not see a further need to discuss this issue. Previous agreements on the number of RX beams can be followed. Tentative agreements: None Candidate options: - None Recommendations for 2nd round: None |
| Sub-topic #2-3: Scenario-B requirements | Issue 2-3-1: Number of Rx beams Background: Some of the companies propose to reduce the number of Rx beams in Scenario-B further from the [6] Rx beams agreed before. Tentative agreements: None Candidate options: - Option 1: 6Rx beams in Scenario-B - Option 2: 4Rx beams in Scenario-B Recommendations for 2nd round: - Discuss if further reduction of Rx beams is needed Issue 2-3-2: Requirements for RRH deployment on both sides of the track Background: The companies agree that 6 RX beams are sufficient to cover the Sceanrio-B deployments with RRH place on one/both sides of the track. However, it is mentioned that the scaling factor in the requirements shall be discussed further. Tentative agreements: [6] RX beams in scenario-B cover the impact of RRH position at one/both sides of rail track Candidate options: - None Recommendations for 2nd round: Agree on tentative agreement. |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

TBA

### Sub-topic 2-2: TBA

#### Issue 2-2-2: TBA

Agreements from round 1:

TBA

Candidate options:

- Option 1: TBA

- Option 2: TBA

Recommendations for 2nd round:

TBA

Contributor Comments:

| Company | Comments |
| --- | --- |
| XXX |  |
| YYY |  |
| ZZZ |  |

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

# Topic #3: RRC Idle/Inactive and connected state mobility requirements

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2117301 | Apple | Discussion on mobility requirement for FR2 HST Proposal 1: M2 = 1.5 if SMTC periodicity > [40] ms, otherwise M2=1. Proposal 2: Requirement to identify target NR cell for RRC connection re-establishment to known NR intra-frequency cell MAX (400 ms, 5 x N1 x TSMTC). Proposal 3: Requirement to identify target NR cell for RRC connection re-establishment to unknown NR intra-frequency cell MAX (1000 ms, 10 x N1 x TSMTC). Proposal 4: Handover requirement can be enhanced depends on the number of Rx beam in general RRM requirement discussion. Proposal 5: For idle/inactive mode cell reselection enhancement, Use 320 ms DRX cycle as baseline for following analysis and requirement definition. The scaling factor N1 can reuse the general Rx beam agreement. |
| R4-2117366 | CATT | Further discussion on RRC Idle/Inactive and connected state mobility requirements for FR2 HST Proposal 1: DRX upper bound can be 80ms. Proposal 2: For time to identify target NR cell for RRC connection re-establishment to NR intra-frequency cell, the requirement can be defined as: Serving cell; FR of target NR; Tidentify_intra_NR [ms]. SSB Ês/Iot (dB); cell; Known NR cell; Unknown NR cell. ≥ -8; FR1; MAX (200 ms, 5 x TSMTC); MAX (800 ms, 10 x TSMTC). ≥ -8; FR2; MAX (200 ms, 5 x N1 x TSMTC); MAX (1000 ms, 10 x N1 x TSMTC)). < -8; FR1; N/A; 800Note1. < -8; FR2; N/A; 3520Note1. Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC > 20 ms and serving cell SSB Ês/Iot < -8 dB.. N1 can be enhanced by the consensus of number of RX beams for different scenarios. Proposal 3: Do not define HO requirements for unknown cell in HST FR2 deployments. Proposal 4: If only requirements for known cell are defined, the Tsearch for FR2 is zero. Proposal 5: With considering the maximum RX beam number is 2/ 6, use 320 ms DRX cycle as the feasible upper bound for FR2 HST idle mode enhancement requirement. N1 refers to agreed Rx numbers for different scenarios. |
| R4-2117693 | CMCC | Discussion on mobility requirements for FR2 HST CONNECTED State Mobility Proposal 1: for the enhancement of short DRX in connected mode, the scaling factor M2 introduced for the enhancement of scaling factor 1.5 specified in Rel-16 NR HST can be reused for FR2 HST: M2 = 1 when SMTC < = 40 ms, otherwise M2 = 1.5. Idle/inactive mode requirements Proposal 2: for the enhancement in idle/inactive mode, M2 = 1 when SMTC < = 40 ms, otherwise M2 = 1.5. |
| R4-2118343 | Ericsson | Mobility requirements for HST FR2 Proposal 1: Support Option2: [80] ms for DRX upper bound. Proposal 2: Support Option1, The way to check the question is: when SMTC >40ms, M2 =1 should be the default value following Rel-16 NR FR1 HST; there is no necessity to relax M2 from 1 to 1.5 in HST FR2 compared to HST FR1. Proposal 3: Regarding to Connection Re-establishment to known cell, support Option 1: MAX (TBD ms, 5 x N1 x TSMTC). Otherwise, MAX (1000 ms, 80 x TSMTC) for unknown NR intra-frequency cell shall be changed to MAX (1000 ms, 10 x N1 x TSMTC), where N1=RX beam number. Proposal 4: The scaling factor in HO requirement corresponding the number of RX beam sweep shall follow RX beam number decided. Proposal 5: We have not strong view on the upper bound of DRX cycle with enhanced requirements before N1 issue is fixed, but 320 ms is acceptable. But the table shall be enhanced up to DRX=1.28ms as listed in WF. Proposal 6: Support opion2, the scaling factor N1 refers to agreed Rx numbers. Observation 1: Unknow cell in HST FR2 deployments may happen where UE is at certain positions relative to RRHs with some configurations due to SNR drop and other practical issues along with railway. |
| R4-2118804 | Huawei, Hisilicon | Discussion on RRC Idle/Inactive and connected state mobility requirements for HST in FR2 Proposal 1: DRX upper bound for enhanced RRM HST FR2 requirements in connected mode can be 80ms. Proposal 2: Enhance RRM measurements requirements for small DRX (e.g., 320ms) in idle/inactive mode. Proposal 3: For 320ms DRX cycle, the scaling factor N1 need to be changed according to RX beam number and the sample number for Tdetect, Tmeasure and Tevaluate can refer to the existing enhancement requirements for R16 FR1 HST, i.e., 8,1,3 respectively. Proposal 4: The existing FR2 handover delay when target cell is known can be applicable in high speed scenario. Proposal 5: The existing definition of known cell for RRC connection re-establishment can be reused for FR2 HST. Proposal 6: The need of enhancement of RRC connection re-establishment delay for FR2 HST scenario is not clear. |
| R4-2119110 | ZTE Corporation | Discussion on RRC Idle Inactive and Connected state mobility requirements for HST FR2 Proposal 1: Reusing Rel-16 FR1 HST enhancement is enough, so we support M2 = 1.5 if SMTC periodicity > [40] ms, otherwise M2=1. Proposal 2: For the enhancement of connection re-establishment, enhanced as the following table. Serving cell; FR of target NR; Tidentify_intra_NR [ms]. SSB Ês/Iot (dB); cell; Known NR cell; Unknown NR cell. ≥ -8; FR1; MAX (200 ms, 5 x TSMTC); MAX (800 ms, 10 x TSMTC). ≥ -8; FR2; MAX(200ms, 5 x N1 x TSMTC); MAX (1000 ms, 10 x N1 x TSMTC)). < -8; FR1; N/A; 800 Note1. < -8; FR2; N/A; 3520 Note1. Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC > 20 ms and serving cell SSB Ês/Iot < -8 dB.. Proposal 3: Do not need to consider handover to unknown cell. Proposal 4: For handover to known cell case, the limitation of SMTC periodicity can be relaxed. Proposal 5: Use 320 ms DRX cycle as baseline for following analysis and requirement definition. Proposal 6: The value of N1 refers to agreed RX beam number. |
| For reference, papers submitted to the AI 8.9.4.6 |  |  |
| R4-2117304 | Apple | Discussion on measurement procedure requirement for FR2 HST Proposal 1: Reuse the Rel-16 FR1 HST scaling factor M2 for FR2 HST intra-freq measurement requirement. Same SMTC periodicity bound (i.e., 40ms) can be reused. Proposal 2: Mpss/sss_sync_w/o_gaps = 3N, where N is the number of Rx beam follows the general RRM requirement discussion. Mmeas_preriod_w/o_gaps = 3N, where N is number of Rx beam follows the general RRM requirement discussion. Proposal 3: Reuse the Rel-16 FR1 HST scaling factor K for FR2 HST L1-RSRP measurement requirement, with the same SMTC periodicity bound of 40ms. The number of Rx beams can be reduced follows the general RRM requirement discussion. |
| R4-2117369 | CATT | Further discussion on measurement procedure requirements for FR2 HST Proposal 1: Mpss/sss_sync_w/o_gaps and Mmeas_period_w/o_gaps can be enhanced by reducing RX beams from 8 in PSS/SSS detection and Measurement period. The number of RX beams can be used as 2 and 6 for Scenario A and Scenario B. Proposal 2: The same M2 in Rel-16 HST can be reused for FR2 HST. Proposal 3: For L1 RSRP measurement, reuse the Rel-16 FR1 HST scaling factor K for FR2 HST. N can be reduced to 2 and 6 for Scenario A and Scenario B. |
| R4-2118342 | Ericsson | Measurement procedure requirements for HST FR2 Proposal 1: Reuse the Rel-16 FR1 HST scaling factor M2 for FR2 HST. Proposal 2: Reuse the Rel-16 FR1 HST scaling factor K for FR2 HST. |
| R4-2118807 | Huawei, Hisilicon | Discussion on RRM requirements for high speed train scenario in FR2 Proposal 1: DRX upper bound for enhanced RRM HST FR2 requirements in connected mode can be 80ms. Proposal 2: In enhanced FR2 HST requirements, M2 is always equal to 1, as HST FR2 enhanced requirement is applied to SMTC <=40ms. Proposal 3: For PSS/SSS detection delay in FR2 HST - Mpss/sss_sync_w/o_gaps is to be reduced accordingly as per the agreed RX beam number, - The sample number is not supposed to be changed for DRX≤ 80ms. Proposal 4: For measurement period in FR2 HST - M2 is always equal to 1, as HST FR2 enhanced requirement is applied to SMTC <=40ms. - The sample number is not supposed to be changed for DRX≤ 80ms. |
| R4-2119112 | ZTE Corporation | Discussion on Measurement Procedure Requirements for HST FR2 Proposal 1: reusing the Rel-16 FR1 HST scaling factor M2 for FR2 HST or keep the 1.5 factor. Proposal 2: reusing the Rel-16 FR1 HST scaling factor K for FR2 HST L1-RSRP measurement requirement, with the same SMTC periodicity bound of 40ms. |
| R4-2119168 | Nokia, Nokia Shanghai Bell | Discussion on RRM measurement requirements for FR2 HST - RAN4 are encouraged to investigate the potential handover issues in unidirectional scenarios when UE is moving in the direction opposite to the pointing direction of RRH TX beams. Two options for SA intra-frequency cell identification time requirements Option 1: A separate set of requirements for deployment Scenarios A and B For unidirectional Scenario A, the SA intra-frequency cell identification time requirement is enhanced for DRX cycle ≤ [160 ms] for both PSS/SSS detection and L3 measurements in Tables 2 and 3. Table 2: Time period for PSS/SSS detection for Scenario A, (Frequency range FR2) DRX cycle; TPSS/SSS_sync_intra. No DRX; max(600ms, ceil(Mpss/sss_sync_w/o_gaps_A x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra. DRX≤ [160ms]; max(600ms, ceil(1.5 x Mpss/sss_sync_w/o_gaps_A x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra. 160ms< DRX cycle ≤ 320ms; max(600ms, ceil(1.5 x Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra. DRX cycle>320ms; ceil(Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x DRX cycle x CSSFintra. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified. Table 3: Measurement period for intra-frequency measurements for Scenario A, (FR2) DRX cycle; T SSB_measurement_period_intra. No DRX; max(400ms, ceil(Mmeas_period_w/o_gaps_A x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra. DRX≤ [160ms]; max(400ms, ceil(1.5x Mmeas_period_w/o_gaps_A x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra. 160ms< DRX cycle≤ 320ms; max(400ms, ceil(1.5x Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra. DRX cycle>320ms; ceil(Mmeas_period_w/o_gaps xKp x Klayer1_measurement ) x DRX cycle x CSSFintra. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified. Mpss/sss_sync_w/o_gaps_A = Mmeas_period_w/o_gaps_A = 6 For unidirectional and bidirectional Scenario B, the SA intra-frequency cell identification time requirement is enhanced for DRX cycle ≤ [40 ms] for both PSS/SSS detection and L3 measurements in Tables 4 and 5. Table 4: Time period for PSS/SSS detection for unidirectional Scenario B, (Frequency range FR2) DRX cycle; TPSS/SSS_sync_intra. No DRX; max(600ms, ceil(Mpss/sss_sync_w/o_gaps_B x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra. DRX≤ [40ms]; max(600ms, ceil(1.5 x Mpss/sss_sync_w/o_gaps_B x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra. 40ms< DRX cycle ≤ 320ms; max(600ms, ceil(1.5 x Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra. DRX cycle>320ms; ceil(Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x DRX cycle x CSSFintra. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified. Table 5: Measurement period for intra-frequency measurements for unidirectional Scenario B, (FR2) DRX cycle; T SSB_measurement_period_intra. No DRX; max(400ms, ceil(Mmeas_period_w/o_gaps_B x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra. DRX≤ [40ms]; max(400ms, ceil(1.5x Mmeas_period_w/o_gaps_B x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra. 40ms< DRX cycle≤ 320ms; max(400ms, ceil(1.5x Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra. DRX cycle>320ms; ceil(Mmeas_period_w/o_gaps xKp x Klayer1_measurement ) x DRX cycle x CSSFintra. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified. Mpss/sss_sync_w/o_gaps_B = Mmeas_period_w/o_gaps_B = 18 Option 2: One unified/common set of requirements representing all deployment scenarios (Dmin is ranging from 10 m to 150 m) For generic deployment scenarios, the existing Rel-16 SA intra-frequency cell identification time requirement is applied for DRX cycle ≤ [40 ms] for both PSS/SSS detection and L2 measurements in Tables 9 and 10; M2 is adopted from FR1 HST. Table 9: Time period for PSS/SSS detection for all scenarios, (Frequency range FR2) DRX cycle; TPSS/SSS_sync_intra. No DRX; max(600ms, ceil(Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra. DRX cycle≤ 320ms; max(600ms, ceil(M2 x Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra. DRX cycle>320ms; ceil(Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x DRX cycle x CSSFintra. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When TBD is not configured, M2 = 1.5; When TBD is configured, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1. NOTE 3: DRX cycle ≤ [40 ms] is not applicable to FR2 HST scenarios. Table 10: Measurement period for intra-frequency measurements for all scenarios, (FR2) DRX cycle; T SSB_measurement_period_intra. No DRX; max(400ms, ceil(Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra. DRX cycle≤ 320ms; max(400ms, ceil(M2x Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra. DRX cycle>320ms; ceil(Mmeas_period_w/o_gaps xKp x Klayer1_measurement ) x DRX cycle x CSSFintra. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When TBD is not configured, M2 = 1.5; When TBD is configured, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1. NOTE 3: DRX cycle > [40 ms] is not applicable to FR2 HST scenarios.. The pros and cons of the above options are summarized below: Pros and cons. Option 1; The specified requirement is optimized and dedicated to a specific deployment scenario. Consequently, it may not represent a practical deployment scenario. If a deployment scenario differs from Scenarios A or B, then a new set of requirement needs to be defined.. Option 2; One representative requirement for all possible deployment scenarios. This means, one size fits all. Such a requirement may be unfair to specific scenarios.. RAN4 are encouraged to investigate the potential beam switching issues in unidirectional scenarios when UE is moving in the direction opposite to the pointing direction of RRH TX beams. Two options for SSB-based L1 measurement requirements Option 1: Separate sets of requirements for deployment Scenarios A and B For unidirectional Scenario A, the SSB-based L1 measurement requirement is enhanced for DRX cycle ≤ [160 ms] in Table 11. Table 11: Measurement period for TL1-RSRP_Measurement_Period_SSB for FR2 for Scenario A Configuration; TL1-RSRP_Measurement_Period_SSB (ms). non-DRX; max(TReport, ceil(M*P*NA)*TSSB). DRX cycle ≤ [160ms]; max(TReport, ceil(1.5*M*P*NA)*max(TDRX,TSSB)). DRX cycle ≤ 320ms; max(TReport, ceil(1.5*M*P*N)*max(TDRX,TSSB)). DRX cycle > 320ms; ceil(1.5*M*P*N)*TDRX. Note: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting.. NA = 2 For unidirectional and bidirectional Scenario B, the SSB-based L1 measurmeent requirement is enhanced for DRX cycle < [40 ms] in Table 13. Table 13: Measurement period for TL1-RSRP_Measurement_Period_SSB for FR2 for bidirectional Scenario B Configuration; TL1-RSRP_Measurement_Period_SSB (ms). non-DRX; max(TReport, ceil(M*P*NB)*TSSB). DRX cycle < [40ms]; max(TReport, ceil(1.5*M*P*NB)*max(TDRX,TSSB)). DRX cycle ≤ 320ms; max(TReport, ceil(1.5*M*P*N)*max(TDRX,TSSB)). DRX cycle > 320ms; ceil(1.5*M*P*N)*TDRX. Note: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting.. NB = 6 Option 2: One unified/common set of requirements representing all deployment scenarios (Dmin is ranging from 10 m to 150 m) For generic deployment scenarios, the existing Rel-16 SSB-based L1 measurmeent requirement is applied for DRX cycle < [40 ms] in Table 14; K is adopted from FR1 HST. Table 14: Measurement period for TL1-RSRP_Measurement_Period_SSB for FR2 for all scenarios Configuration; TL1-RSRP_Measurement_Period_SSB (ms). non-DRX; max(TReport, ceil(M*P*N)*TSSB). DRX cycle ≤ 320ms; max(TReport, ceil(K*M*P*N)*max(TDRX,TSSB)). DRX cycle > 320ms; ceil(1.5*M*P*N)*TDRX. Note 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. NOTE 2: When TBD is not configured, M2 = 1.5; When TBD is configured, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1. NOTE 3: DRX cycle < [40 ms] is not applicable to FR2 HST scenarios. For Scenarios A and B, reuse the same CSI-RS based L1-RSRP measurement requirement and adopt the FR1 HST methodology for enhancing the scaling factor 1.5 in Table 15. Table 15: Measurement period TL1-RSRP_Measurement_Period_CSI-RS for FR2 for Scenarios A and B Configuration; TL1-RSRP_Measurement_Period_CSI-RS (ms). non-DRX; max(TReport, ceil(M*P*N)*TCSI-RS). DRX cycle ≤ 320ms; max(TReport, ceil(K*M*P*N)*max(TDRX,TCSI-RS)). DRX cycle > 320ms; ceil(M*P*N)*TDRX. Note 1: TCSI-RS is the periodicity of CSI-RS configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: the requirements are applicable provided that the CSI-RS resource configured for L1-RSRP measurement is transmitted with Density = 3. Note 3: K = 1 when TCSI-RS ≤ 40 ms and TBD are configured; otherwise K = 1.5.. For SA cell reselection in idle mode, the scaling factor N1 is 3 with DRX cycle = 320 ms for Scenarios A and B. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 3-1: Enhancement of measurement requirements in CONNECTED state

Sub-topic description:

For reference, the following requirements are defined in TS 38.133:

The UE shall be able to identify a new detectable intra frequency SS block of an already detected cell within Tidentify_intra_without_index. It is assumed that deriveSSB-IndexFromCell is always enabled for FR1 TDD and FR2.

Tidentify_intra_without_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra) ms

Tidentify_intra_with_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra + TSSB_time_index_intra) ms

Table 9.2.5.1-1: Time period for PSS/SSS detection, (Frequency range FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max( 600ms, ceil( 5 x Kp) x SMTC period )Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max( 600ms, ceil(M2 Note 2x 5 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil(5 x Kp) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1. NOTE 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or [intraRAT-MeasurementEnhancement-r16] on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active Scell. |  |

Table 9.2.5.1-2: Time period for PSS/SSS detection, (Frequency range FR2)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max(600ms, ceil(Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max(600ms, ceil(1.5 x Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil(Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 9.2.5.1-3: Time period for time index detection (FR1)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | max(120ms, ceil( 3 x Kp ) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max(120ms, ceil (M2 Note 2 x 3 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | Ceil(3 x Kp) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1 NOTE 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or [intraRAT-MeasurementEnhancement-r16] on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active Scell. |  |

Table 9.2.5.1-2: Time period for PSS/SSS detection, (Frequency range FR2)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max(600ms, ceil(Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max(600ms, ceil(1.5 x Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil(Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 9.2.5.2-1: Measurement period for intra-frequency measurements without gaps(FR1)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max(200ms, ceil( 5 x Kp) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max(200ms, ceil(1.5x 5 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil( 5 x Kp ) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 9.2.5.2-2: Measurement period for intra-frequency measurements without gaps(FR2)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max(400ms, ceil(Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max(400ms, ceil(1.5x Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil(Mmeas_period_w/o_gaps xKp x Klayer1_measurement ) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 9.2.5.2-5: T SSB_measurement_period_intra When highSpeedMeasFlag-r16 is configured (Frequency range FR1

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX Note 2 | max(200ms, ceil( 5 x Kp) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 160ms | max(200ms, ceil(5 x M2 Note 2 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra |
| 160ms < DRX cycle≤ 320ms | ceil(4 x M2 Note 2 x Kp) x max(SMTC period,DRX cycle) x CSSFintra |
| DRX cycle>320ms | ceil( Y Note 3 x Kp ) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1 NOTE 3: Y=3 when SMTC <= 40ms, Y=5 when SMTC > 40ms NOTE 4: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or [intraRAT-MeasurementEnhancement-r16] on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active Scell. |  |

In the previous meeting [R4-2115334], it was agreed for HST FR2 deployments that:

- Apply existing R16 requirements for long DRX cycles, i.e. above the upper bound of DRX cycle.

- HST FR2 enhanced requirement is applied to SMTC <=40ms. SMTC periodicity is not restricted.

Open issues and candidate options before e-meeting:

Presented as background in the corresponding issues.

#### Issue 3-1-1: DRX upper bound for enhanced RRM HST FR2 requirements

- Background

The following options were listed in the previous meeting [R4-2115334]

  - Option 1: Up to [256] ms

  - Option 2: [80] ms

  - Option 3: [60] ms

- Proposals and/or Observations

  - Proposal 1 (CATT): DRX upper bound can be 80ms.

  - Proposal 2 (Ericsson): Support Option2: [80] ms for DRX upper bound.

  - Proposal 3 (Huawei): DRX upper bound for enhanced RRM HST FR2 requirements in connected mode can be 80ms.

- Candidate options:

  - Option 2: [80] ms

- Recommended WF

  - Decide in the first round whether Option 2: [80] ms is agreeable.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Nokia | The DRX upper bound for Scenario A is different from Scenario B due to the different number of UE RX beams for RRM agreed at the last meeting. Further, for unidirectional deployment scenarios, the DRX upper bound is affected by the movement direction of UE. From our simulation analysis, inter-cell mobility performance is worse when UE is moving in the direction that is opposite to the pointing direction of RRH TX beams. Thus, we have made the following observation and proposal in R4-2119166: Observation 2: For unidirectional Scenarios A and B, no handover could be triggered at the switching point Ds_offset when UE is moving in the opposite direction to the pointing direction of RRH TX beams since the condition for handover cannot be satisfied. Proposal 1: RAN4 are encouraged to investigate the potential handover issues in unidirectional scenarios when UE is moving in the direction opposite to the pointing direction of RRH TX beams. As the DRX upper bound, the number of UE RX beams and the M2 factor in Issue 3-1-2 (M2 scaling factor for short DRX) are interrelated, it is recommended to discuss the three scaling factors jointly rather than separately. As such, two options are feasible for defining SA intra-frequency cell identification requirements. Option 1: A separate set of requirements for deployment Scenarios A and B For unidirectional Scenario A, the DRX upper bound = [160 ms] and the 1.5 scaling factor is kept for all SMTC periodicities for both PSS/SSS detection and L3 measurements in Tables below. DRX cycle; TPSS/SSS_sync_intra. No DRX; max(600ms, ceil(Mpss/sss_sync_w/o_gaps_A x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra. DRX≤ [160ms]; max(600ms, ceil(1.5 x Mpss/sss_sync_w/o_gaps_A x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra. 160ms< DRX cycle ≤ 320ms; max(600ms, ceil(1.5 x Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra. DRX cycle>320ms; ceil(Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x DRX cycle x CSSFintra. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified. DRX cycle; T SSB_measurement_period_intra. No DRX; max(400ms, ceil(Mmeas_period_w/o_gaps_A x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra. DRX≤ [160ms]; max(400ms, ceil(1.5x Mmeas_period_w/o_gaps_A x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra. 160ms< DRX cycle≤ 320ms; max(400ms, ceil(1.5x Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra. DRX cycle>320ms; ceil(Mmeas_period_w/o_gaps xKp x Klayer1_measurement ) x DRX cycle x CSSFintra. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified. For unidirectional and bidirectional Scenario B, the DRX upper bound = [40 ms] and the 1.5 scaling factor is kept for all SMTC periodicities for both PSS/SSS detection and L3 measurements in Tables below. DRX cycle; TPSS/SSS_sync_intra. No DRX; max(600ms, ceil(Mpss/sss_sync_w/o_gaps_B x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra. DRX≤ [40ms]; max(600ms, ceil(1.5 x Mpss/sss_sync_w/o_gaps_B x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra. 40ms< DRX cycle ≤ 320ms; max(600ms, ceil(1.5 x Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra. DRX cycle>320ms; ceil(Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x DRX cycle x CSSFintra. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified. DRX cycle; T SSB_measurement_period_intra. No DRX; max(400ms, ceil(Mmeas_period_w/o_gaps_B x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra. DRX≤ [40ms]; max(400ms, ceil(1.5x Mmeas_period_w/o_gaps_B x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra. 40ms< DRX cycle≤ 320ms; max(400ms, ceil(1.5x Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra. DRX cycle>320ms; ceil(Mmeas_period_w/o_gaps xKp x Klayer1_measurement ) x DRX cycle x CSSFintra. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified. Option 2: One unified/common set of requirements representing all deployment scenarios (Dmin is ranging from 10 m to 150 m) For generic deployment scenarios, the DRX upper bound = [40 ms] and M2 = 1.5 if SMTC periodicity > [40] ms, otherwise M2=1 for for both PSS/SSS detection and L2 measurements in Tables below. DRX cycle; TPSS/SSS_sync_intra. No DRX; max(600ms, ceil(Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra. DRX cycle≤ 320ms; max(600ms, ceil(M2 x Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra. DRX cycle>320ms; ceil(Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x DRX cycle x CSSFintra. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When TBD is not configured, M2 = 1.5; When TBD is configured, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1. NOTE 3: DRX cycle ≤ [40 ms] is not applicable to FR2 HST scenarios. DRX cycle; T SSB_measurement_period_intra. No DRX; max(400ms, ceil(Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra. DRX cycle≤ 320ms; max(400ms, ceil(M2x Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra. DRX cycle>320ms; ceil(Mmeas_period_w/o_gaps xKp x Klayer1_measurement ) x DRX cycle x CSSFintra. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When TBD is not configured, M2 = 1.5; When TBD is configured, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1. NOTE 3: DRX cycle > [40 ms] is not applicable to FR2 HST scenarios.. The pros and cons of the above two options are summarized below: Pros and cons. Option 1; The specified requirement is optimized and dedicated to a specific deployment scenario. Consequently, it may not represent a practical deployment scenario. If a deployment scenario differs from Scenarios A or B, then a new set of requirement needs to be defined.. Option 2; One representative requirement for all possible deployment scenarios. This means, one size fits all. Such a requirement may be unfair to specific scenarios.. Comments are appreciated from companies. |
| QC | We support both option 2 in the issue description and option 2 from Nokia’s comment. |
| ZTE | We agree with the option 2, i.e. using 80ms as the DRX upper bound. |
| Huawei | Option 2 is agreeable. |
| Ericsson | We agree with the option 2, i.e. using 80ms as the DRX upper bound. Regarding two options proposed by Nokia, we prefer option1 as we commented in above issues, |
| CATT | 80ms is agreeable for us. |
| Apple | 80ms is agreeable. |
| Intel | Option 2: 80 ms is agreeable |
| Samsung | Option 2: [80] ms is reasonable, especially considering the CPE UE type is under discussion for FR2 HST UE. |

#### Issue 3-1-2: M2 scaling factor for short DRX

- Background

Further consider the possibility of requirement enhancement for SMTC periodicity <= [40] ms

  - Option 1: M2 = 1.5 if SMTC periodicity > [40] ms, otherwise M2=1.

- Proposals and/or Observations

  - Proposal 1 (Apple): M2 = 1.5 if SMTC periodicity > [40] ms, otherwise M2=1.

  - Proposal 2 (CMCC): for the enhancement of short DRX in connected mode, the scaling factor M2 introduced for the enhancement of scaling factor 1.5 specified in Rel-16 NR HST can be reused for FR2 HST: M2 = 1 when SMTC < = 40 ms, otherwise M2 = 1.5.

  - Proposal 3 (Qualcomm): For FR2 HST neighboring cell search enhancement in scenario B, consider 6Rx instead of 8Rx sweeping in the legacy requirement and keep 1.5 relaxation factor and number of samples the same if network can provide assistant information to UE as proposed in section 2.2. The enhancement is applicable only to DRx cycle <= 80ms.

  - Proposal 4 (Ericsson): Support Option1, The way to check the question is: when SMTC >40ms, M2 =1 should be the default value following Rel-16 NR FR1 HST; there is no necessity to relax M2 from 1 to 1.5 in HST FR2 compared to HST FR1.

  - Proposal 5 (ZTE): Reusing Rel-16 FR1 HST enhancement is enough, so we support M2 = 1.5 if SMTC periodicity > [40] ms, otherwise M2=1.

- Candidate options:

  - Option 1: 1.5 if SMTC periodicity > [40] ms, otherwise M2=1.

  - Option 2: Keep the 1.5 factor for all SMTC periodicities.

- Recommended WF

  - TBA

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Nokia | See the comments in Issue 3-1-1 |
| QC | We understand that the majority view is option 1. But that’s from relaxation perspective. To address the possibility of RRHs can be on both side for scenario B, we can also interpret this factor as allowance to search both side and repurpose this factor. |
| ZTE | We support Option 1, which is same as R16 HST FR1 enhancement. |
| Huawei | Support option 1. Fast measurement is expected in FR2 HST scenario. In addition, power consumption is not critical for roof-mounted CPE. |
| Ericsson | Support Option1. To QC, do you expect the factor to be used in all scenarios or only scenario B? |
| CATT | We support option 1. |
| Apple | Support option 1. |
| QC | To Ericsson: our preference is to use this factor to address the uncertainty in real deployment for scenario A and two side RRH possibility for scenario B. But we can compromise to only apply this factor to scenario B since the necessity is more solid in scenario B. |
| Intel | Support Option 1 |
| CMCC | Option 1. |
| Samsung | We support Option 1 |

### Sub-topic 3-2: Connection Re-establishment requirement

Sub-topic description:

For reference, following TS 38.133:

If the UE is not configured with intra-frequency NR carrier for RRC re-establishment then Tidentify_intra_NR=0; otherwise Tidentify_intra_NR shall not exceed the values defined in Table 6.2.1.2.1-1.

In the previous RAN4#100-e meeting the following approach for FR2 HST modification was agreed:

| Serving cell | FR of target NR | Tidentify_intra_NR [ms] |  |
| --- | --- | --- | --- |
| SSB Ês/Iot (dB) | cell | Known NR cell | Unknown NR cell |
| ≥ -8 | FR1 | MAX (200 ms, 5 x TSMTC) | MAX (800 ms, 10 x TSMTC) |
| ≥ -8 | FR2 | MAX (TBD, 5 x N1 x TSMTC) | MAX (1000 ms, 10 x N1 x TSMTC)) |
| < -8 | FR1 | N/A | 800Note1 |
| < -8 | FR2 | N/A | 3520Note1 |
| Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC > 20 ms and serving cell SSB Ês/Iot < -8 dB. |  |  |  |

Open issues and candidate options before e-meeting:

Presented as background in the corresponding issues.

#### Issue 3-2-1: Maximum time for known cell

- Background

In the previous RAN4#100-e meeting the following options were discussed:

- Option 1: MAX (TBD ms, 5 x N1 x TSMTC).

- Option 2: Do not introduce any enhancements.

- Proposals and/or Observations

  - Proposal 1 (Apple): Requirement to identify target NR cell for RRC connection re-establishment to known NR intra-frequency cell MAX (400 ms, 5 x N1 x TSMTC).

  - Proposal 2 (CATT): For time to identify target NR cell for RRC connection re-establishment to NR intra-frequency cell, the requirement can be defined as: MAX (200 ms, 5 x N1 x TSMTC)
N1 can be enhanced by the consensus of number of RX beams for different scenarios.

  - Proposal 3 (Ericsson): Regarding to Connection Re-establishment to known cell, support Option 1: MAX (TBD ms, 5 x N1 x TSMTC). Otherwise, MAX (1000 ms, 80 x TSMTC) for unknown NR intra-frequency cell shall be changed to MAX (1000 ms, 10 x N1 x TSMTC), where N1=RX beam number.

  - Proposal 4 (Huawei): The need of enhancement of RRC connection re-establishment delay for FR2 HST scenario is not clear.

  - Proposal 5 (ZTE): For the enhancement of connection re-establishment, enhanced as the following table: MAX(200ms, 5 x N1 x TSMTC)

- Candidate options:

  - Option 1: 400 ms.

  - Option 2: 200 ms

  - Option 3: Do not introduce any enhancements

- Recommended WF

  - Continue the discussion in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| QC | We support option 3. We don’t see the need for connection re-establishment enhancement in FR2 HST. We don’t consider such enhancement in FR1 even with 500km/h speed. UE is on a fixed trajectory with RRHs serving it along the route, connection re-establishment due to OOC is almost impossible in HST, thus we don’t see the need for this enhancement. |
| ZTE | We support Option 2. |
| Huawei | Actually we’d like to know the justification of defining RRC-Re-establishment requirements for FR2 known cell. In R15, it is agreed that it doesn’t request UE to store the cell/beam information during re-establishment in FR2. Does it mean that additional UE process capability would be required for HST FR2? |
| Ericsson | Regarding to our understanding, we need to enhance ‘MAX (1000 ms, 80 x TSMTC)’ at least in Issue 3-2-2. |
| CATT | We support 400ms now to reuse the same lower bound of meas. |
| Apple | 400ms. 400ms is calculated from 1000ms – 600ms. |

#### Issue 3-2-2: Maximum time for unknown cell

- Background

In the previous RAN4#100-e meeting the following options were discussed:

  - Option 1: Keep current requirements with N1 scaling factor corresponding the number of RX beams:
MAX (1000 ms, 10 x N1 x TSMTC)

  - Option 2: Do not introduce any enhancements.

No new options were added in the contributions for RAN4#101-e.

- Proposals and/or Observations

  - Proposal 1 (Apple): Requirement to identify target NR cell for RRC connection re-establishment to unknown NR intra-frequency cell MAX (1000 ms, 10 x N1 x TSMTC).

  - Proposal 2 (CATT): For time to identify target NR cell for RRC connection re-establishment to NR intra-frequency cell, the requirement can be defined as: MAX (1000 ms, 10 x N1 x TSMTC))

  - Proposal 3 (Ericsson): Regarding to Connection Re-establishment to known cell, support Option 1: MAX (TBD ms, 5 x N1 x TSMTC). Otherwise, MAX (1000 ms, 80 x TSMTC) for unknown NR intra-frequency cell shall be changed to MAX (1000 ms, 10 x N1 x TSMTC), where N1=RX beam number.

  - Proposal 4 (Huawei): The need of enhancement of RRC connection re-establishment delay for FR2 HST scenario is not clear.

  - Proposal 5 (ZTE): For the enhancement of connection re-establishment, enhanced as the following table: MAX (1000 ms, 10 x N1 x TSMTC))

- Recommended WF

  - Confirm in the 1st round that Option 1 is agreeable.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| QC | Same comment as 3-2-1. |
| ZTE | We agree with Option 1. |
| Huawei | As the power saving is not a big for CPE, option 1 is also fine. |
| Ericsson | Ok with Option1. |
| CATT | Option 1 is agreeable. |
| Apple | Support option 1 |

#### Issue 3-2-3: Value of N1

- Background

- Proposals and/or Observations

  - Proposal 1 (CATT): N1 can be enhanced by the consensus of number of RX beams for different scenarios.

  - Proposal 2 (Ericsson): Support opion2, the scaling factor N1 refers to agreed Rx numbers.

  - Proposal 3 (Huawei): The value of N1 refers to agreed RX beam number.

- Candidate options:

  - Option 1: The value of N1 refers to agreed RX beam number

- Recommended WF

  - Confirm in the 1st round that Option 1 is agreeable.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| QC | Same comment as 3-2-1. |
| ZTE | We agree with Option 1. |
| Huawei | Confirm in the 1st round that Option 1 is agreeable. |
| Ericsson | Agree with Option1. |
| CATT | Option 1 is agreeable. |
| Apple | Agree with WF |
| Samsung | Agree with WF by Moderator |

#### Issue 3-2-4: Definition of known cell

- Background:

The definition of known target cell in FR2 is included in TS 38.133, section 6.1.1.4.2.

- Proposals and/or Observations

  - Proposal 1 (Huawei): The existing definition of known cell for RRC connection re-establishment can be reused for FR2 HST.

- Recommended WF

  - Agree on the Proposal 1 in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| ZTE | Agree with Proposal 1. |
| Huawei | Proposal 1. Agreement for handover achieved in last meeting is, Agreement: Criteria of known cell for FR2: The target FR2 cell is known if it has been meeting the relevant cell identification requirement during the last 5 seconds; otherwise, it is unknown. We think the known cell definition for cell re-establishment can be inherited. |
| Ericsson | No strong view. |

### Sub-topic 3-3: Handover

Sub-topic description:

In this sub-topic the HO requirements are discussed.

Open issues and candidate options before e-meeting:

Presented as background in the corresponding issues.

#### Issue 3-3-1: HO to unknow cell

- Proposals and/or Observations

  - Proposal 1 (CATT): Do not define HO requirements for unknown cell in HST FR2 deployments.

  - Proposal 2 (ZTE): Do not need to consider handover to unknown cell.

  - Observation 1 (Ericsson): Unknow cell in HST FR2 deployments may happen where UE is at certain positions relative to RRHs with some configurations due to SNR drop and other practical issues along with railway.

- Candidate options:

  - Option 1: Need to define HO requirements for unknown cell

  - Option 2: Do not need to consider handover to unknown cell

- Recommended WF

  - Discuss in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| QC | We don’t see how HO to unknow cell can be possible for FR2 HST, hence we support option 2. |
| ZTE | Support Option 2. |
| Huawei | Support option 2. In high speed scenario, the common case is that target cell is known, as the railway track is fixed and ANR relation is remained/stored in UE. |
| Ericson | We’d like to ask if the answer of question in fact is ‘unknown cell is possible in FR2 HST or no’, not ‘HO to unknown cell needs enhanced or not’? If yes, we think unknown cell is possible in FR2 HST even it doesn’t occur frequently, e.g. trajectory changes. |
| CATT | Support option 2. |
| Intel | Support Option 2 |
| Samsung | Support Option 2. |

#### Issue 3-3-2: Enhancement of HO requirements

- Background

A recap from TS 38.133, Section 6.1.1.4 NR FR2- NR FR2 Handover:

When the UE receives a RRC message implying handover the UE shall be ready to start the transmission of the new uplink PRACH channel within Dhandover ms from the end of the last TTI containing the RRC command.

Where:

Dhandover equals the applicable RRC procedure delay defined in clause 12 in TS 38.331 [2] plus the interruption time stated in clause 6.1.1.4.2.

When intra-frequency or inter-frequency handover is commanded, the interruption time shall be less than Tinterrupt

Tinterrupt = Tsearch + TIU + Tprocessing + T∆ + Tmargin ms

Tsearch is the time required to search the target cell when the handover command is received by the UE. If the target cell is a known cell, then Tsearch = 0 ms. If the target cell is an unknown intra-frequency cell and the target cell Es/Iot≥-2 dB, then Tsearch = 8* Trs  ms. If the target cell is an unknown inter-frequency cell and the target cell Es/Iot≥-2 dB, then Tsearch = 8*3* Trs  ms. Regardless of whether DRX is in use by the UE, Tsearch shall still be based on non-DRX target cell search times.

T∆ is time for fine time tracking and acquiring full timing information of the target cell. T∆ =  Trs for both known and unknown target cell.

Definition of the known target cell in FR2 agreed at RAN4#100-e:

The target FR2 cell is known if it has been meeting the relevant cell identification requirement during the last 5 seconds; otherwise, it is unknown.

- Proposals and/or Observations

  - Proposal 1 (Apple): Handover requirement can be enhanced depends on the number of Rx beam in general RRM requirement discussion.

  - Proposal 2 (CATT): If only requirements for known cell are defined, the Tsearch for FR2 is zero.

  - Proposal 3 (Ericsson): The scaling factor in HO requirement corresponding the number of RX beam sweep shall follow RX beam number decided.

  - Proposal 4 (Huawei): The existing FR2 handover delay when target cell is known can be applicable in high speed scenario.

  - Proposal 5 (ZTE): For handover to known cell case, the limitation of SMTC periodicity can be relaxed.

- Candidate options:

  - Option 1: The scaling factor in HO requirement corresponds the number of RX beam sweep

  - Option 2: The existing FR2 handover delay when target cell is known can be applicable in high speed scenario

  - Option 3: For handover to known cell case, the limitation of SMTC periodicity can be relaxed.

- Recommended WF

  - Tentative agreement: If the target cell is a known cell, then Tsearch = 0 ms.

  - Discuss candidate options in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| QC | Support option 4. |
| ZTE | We agree with the tentative agreement. Between Option 1, Option 2 and Option 3, we support all of them. |
| Huawei | Option 2. If only known case is considered in FR2 HST, the RX beam number has no impact on requirements. For the known target cell, the handover delay is rough estimated as 102ms (assuming SMTC is 40ms). With 350km/h velocity, the moved distance is about 10m. In realistic network deployment, the handover range can cover such distance. Through the coarse evaluation, the existing FR2 handover delay when target cell is known can be applicable in high speed scenario. |
| Ericsson | Ok with statement ‘If the target cell is a known cell, then Tsearch = 0 ms.’ |
| CATT | Support tentative agreement. Then no need to consider RX beam number. |
| Apple | Agree with the tentative agreement. |
| QC | We support “proposal” 4 and option 2, sorry for the typo in the previous comment. |
| Intel | Ok with tentative agreement |
| Samsung | Agree with the tentative agreement. |

### Sub-topic 3-4: IDEL/INACTIVE state mobility

Sub-topic description:

In the previous RAN4#100-e meeting the following agreements were achieved:

- Use cell selection criterion S and number of DRX cycles as a baseline for cell selection requirement.

- Use table below as a background for further discussion of HST FR2 IDLE mode requirement:

Tdtect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra for UE configured with RRM

enhancements for high speed (Frequency range FR2)

| DRX cycle length [s] | Tdetect,NR_Intra [s] (number of DRX cycles) | Tmeasure,NR_Intra [s] (number of DRX cycles) | Tevaluate,NR_Intra [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 2.56 x N1 x M2 (8 x N1 x M2) | 0.32 x N1 x M3 (1 x N1 x M3) | 0.96 x N1 x M4 (3 x M4) |  |
| 0.64 | 5.12 x N1 (8 x N1) | 0.64 x N1 (1 x N1) | 1.92 x N1 (3 x N1) |  |
| 1.28 | 8.96 x N1 (7 x N1) | 1.28 x N1 (1 x N1) | 3.84 x N1 (3 x N1) |  |
| 2.56 | 58.88 x N1 (23 x N1) | 2.56 x N1 (1 x N1) | 7.68 x N1 (3 x N1) |  |
| Note 1: when SMTC < = 40 ms, M2 = M3 = M4 = 1; and when SMTC > 40 ms, M2 = 1.5, M3 = M4 = 2 |  |  |  |  |

Open issues and candidate options before e-meeting:

Presented as background in the corresponding issues.

#### Issue 3-4-1: The upper bound of DRX cycle with enhanced requirements

- Background

Options from the previous meeting:

  - Option 1: Use 320 ms DRX cycle as baseline for following analysis and requirement definition

- Proposals and/or Observations

  - Proposal 1 (Apple): For idle/inactive mode cell reselection enhancement, Use 320 ms DRX cycle as baseline for following analysis and requirement definition. The scaling factor N1 can reuse the general Rx beam agreement.

  - Proposal 2 (CATT): With considering the maximum RX beam number is 2/ 6, use 320 ms DRX cycle as the feasible upper bound for FR2 HST idle mode enhancement requirement. N1 refers to agreed Rx numbers for different scenarios.

  - Proposal 3 (Ericsson): We have not strong view on the upper bound of DRX cycle with enhanced requirements before N1 issue is fixed, but 320 ms is acceptable. But the table shall be enhanced up to DRX=1.28ms as listed in WF.

  - Proposal 4 (Huawei): Enhance RRM measurements requirements for small DRX (e.g., 320ms) in idle/inactive mode.

  - Proposal 5 (ZTE): Use 320 ms DRX cycle as baseline for following analysis and requirement definition.

  - Proposal 6 (Qualcomm): Restrict idle mode enhancement to DRx cycle within 320ms.

- Candidate options:

  - Option 1: Defined enhanced requirements for DRX 320 ms only

  - Option 2: Defined enhanced requirements for up to DRX=1.28s

- Recommended WF

  - Discuss candidate options in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Nokia | As with the neighbouring cell search in connected mode, the DRX upper bound, the scaling factor N1 and M2 are interrelated, it is recommended to discuss the three factors jointly. For the DRX upper bound > 320 ms and N1 = 3, the distance travelled by UE > the cell ISD = 700 m. For the DRX upper bound = 320 m and N1 refers to the agreed number of RX beams, we can observe that the distance travelled by UE exceeds the cell ISD for Scenario B but not for Scenario A. As a result, we recommended - For SA cell reselection in idle mode, the scaling factor N1 = 3 with DRX cycle = 320 ms for Scenarios A and B. |
| QC | Support option 1. |
| ZTE | We support Option 1. |
| Huawei | Support option 1. |
| Ericsson | We agree with Option 1. The interpretation of option 2 is keeping below numbers in table. |
| CATT | We support option 1. |
| Apple | Support option 1 |
| Intel | Support Option 1 |
| Samsung | Option 1 is okay to us. |

#### Issue 3-4-2: N1 scaling factor

- Background

Options from the previous meeting:

  - Option 1: N1 is not necessarily the agreed Rx numbers for each scenario, N1 = 3

  - Option 2: N1 refers to agreed Rx numbers

- Proposals and/or Observations

  - Proposal 2 (Ericsson): Support opion2, the scaling factor N1 refers to agreed Rx numbers.

  - Proposal 3 (Huawei): For 320ms DRX cycle, the scaling factor N1 need to be changed according to RX beam number

  - Proposal 4 (ZTE): The value of N1 refers to agreed RX beam number.

| DRX cycle length [s] | Tdetect,NR_Intra [s] (number of DRX cycles) | Tmeasure,NR_Intra [s] (number of DRX cycles) | Tevaluate,NR_Intra [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 7.68 x M2 (24 x M2) | 2.56 x M3 (8 x M2) | 7.68 x M4 (24 x M2) |
| Note 1: when SMTC < = 40 ms, M2 = 1; and when SMTC > 40 ms, M2 = 1.5 |  |  |  |

  - Proposal 5 (Qualcomm): Allow 24 DRx cycles detection time when DRx cycle length is 320ms in idle mode enhancement.

- Recommended WF

  - Re-use candidate options from the previous meeting.

  - Continue the discussion in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Nokia | See comments in Issue 3-4-1. |
| QC | If upper bound of 320ms is agreed, we suggest to discuss the number of DRx cycles directly instead of N1. |
| ZTE | We support Option 2. |
| Huawei | Combined with the DRX issue, we suggest only 320ms DRX cycle will be enhanced. N will be scaled. N depends on the agreed RX number. We don’t have strong opinion on whether N shall distinguish [2] or [6] for scenario A/B. DRX cycle length [s]; Tdetect,NR_Intra [s] (number of DRX cycles); Tmeasure,NR_Intra [s] (number of DRX cycles); Tevaluate,NR_Intra [s] (number of DRX cycles). 0.32; 2.56 x N x M2 (8 x N x M2); 0.32 x N x M3 (1 x N x M3); 0.96 x N x M4 (3 x N x M4). 0.64; 5.12 (8); 0.64 (1); 1.92 (3). 1.28; 8.96 (7); 1.28 (1); 3.84 (3). 2.56; 58.88 (23); 2.56 (1); 7.68 (3). Note 1: when SMTC < = 40 ms, M2 = M3 = M4 = 1; and when SMTC > 40 ms, M2 = 1.5, M3 = M4 = 2 Note 2: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or [intraRAT-MeasurementEnhancement-r16].. |
| Ericsson | N1 refers to agreed Rx numbers |
| CATT | Support option 2. |
| Apple | Support option 2 |
| Intel | Support Option 2 |
| Samsung | Support Option 2 |

#### Issue 3-4-3: Enhancement of requirements

- Proposals and/or Observations

  - Proposal 1( CMCC): for the enhancement in idle/inactive mode, M2 = 1 when SMTC < = 40 ms, otherwise M2 = 1.5.

  - Proposal 2 (Huawei): For 320ms DRX cycle, sample number for Tdetect, Tmeasure and Tevaluate can refer to the existing enhancement requirements for R16 FR1 HST, i.e., 8,1,3 respectively.

- Recommended WF

  - Discuss the proposals in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Nokia | See comments in Issue 3-4-1. |
| QC | Same comment as 3-4-2, but these two proposals are acceptable to us for 6Rx beam case. |
| Huawei | Option 2 doesn’t conflict with option 1. We support proposal 2 regarding with sampling number. Regarding M2, reusing FR1 HST principle is reasonable (proposal 1 is fine). |
| CMCC | P1 targets for M2, P2 targets for sample number, both are OK for us. |

### CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| TDoc nubmer | Company A |
|  | Company B |
| NONE |  |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

| Sub-topic | Status summary |
| --- | --- |
| Sub-topic #3-1: Enhancement of measurement requirements in CONNECTED state | Issue 3-1-1: DRX upper bound for enhanced RRM HST FR2 requirements Background: Most of the companies converge to the value of [80]ms. However, one of the companies still sees some issues with the opposite scenario-A, hence tighter requirements can be proposed, i.e. 40ms. It is also mentioned that the DRX upper bound shall be discussed together with other parameters, i.e., M2 and number of RX beams. Tentative agreements: DRX upper bound for enhanced RRM HST FR2 requirements is [80]ms. Candidate options: - Option 1: [80]ms - Option 2: Define different requirements depending on deployment scenarios (number of Rx beams) Recommendations for 2nd round: Discuss tentative agreement and the options. Issue 3-1-2: M2 scaling factor for short DRX Background: Most of the companies do see a need to have enhanced requirements for short DRX. However, one company thinks that the requirements shall not be adjusted to take two-side RRH deployments into account. Tentative agreements: 1.5 if SMTC periodicity > [40] ms, otherwise M2=1 Candidate options: - Option 1: - Option 2: Keep the 1.5 factor for all SMTC periodicities - Option 2a: Keep the 1.5 factor for all SMTC periodicities in scenario-B to consider two-side RRH possibility. Recommendations for 2nd round: Continue the discussion in the second round. |
| Sub-topic #3-2: Connection Re-establishment requirement | Issue 3-2-1: Maximum time for known cell Background: The issues discuss the requirement on time to identify target NR cell for RRC connected re-establishment to NR intra-frequency cell for the known cell. No agreement between the companies was achieved. Tentative agreements: None Candidate options: - Option 1: MAX (TBD ms, 5 x N1 x TSMTC). - Option 1a: TBD is 400 ms - Option 1b: TBD is: 200 ms - Option 2: Do not introduce any enhancements Recommendations for 2nd round: Add the issues to the WF for FFS. Issue 3-2-2: Maximum time for unknown cell Background: The issues discuss the requirement on time to identify target NR cell for RRC connected re-establishment to NR intra-frequency cell for the unknow cell. All companies except one support Option 1, but one company still thinks that no enhancements are needed. Tentative agreements: None Candidate options: - Option 1: MAX (1000 ms, 10 x N1 x TSMTC) - Option 2: Do not introduce any enhancements Recommendations for 2nd round: Add the issues to the WF for FFS. Issue 3-2-3: Value of N1 Background: All companies except one support Option 1, but one company still thinks that no enhancements are needed. Tentative agreements: None Candidate options: - Option 1: The value of N1 refers to agreed RX beam number - Option 2: Do not introduce any enhancements Recommendations for 2nd round: Add the issues to the WF for FFS. Issue 3-2-4: Definition of known cell Background: No active discussion of the issues. Looks like companies do not see a strong need to agree on the issue. Tentative agreements: None Candidate options: - None Recommendations for 2nd round: None |
| Sub-topic #3-3: Handover | Issue 3-3-1: HO to unknow cell Background: All companies expect one thinks that there is no need to consider handover to unknown cell. However, one company thinks that it is still possible in HST FR2 deployments. Tentative agreements: Do not enhance requirements for HO to unknown cell. Candidate options: - None Recommendations for 2nd round: Check and agree on tentative agreement. Issue 3-3-2: Enhancement of HO requirements Background: Tentative agreements: If the target cell is a known cell, then Tsearch = 0 ms. Candidate options: - None Recommendations for 2nd round: Agree on tentative agreement. Keep a need for HO requirement enactments FFS in the WF. |
| Sub-topic #3-4: IDEL/INACTIVE state mobility | Issue 3-4-1: The upper bound of DRX cycle with enhanced requirements Background: Tentative agreements: Defined enhanced requirements for DRX 320 ms only. Requirements for longer DRX cycles are left without changes. Candidate options: - None Recommendations for 2nd round: Agree on tentative agreement Issue 3-4-2: N1 scaling factor Background: DRX cycle length [s]; Tdetect,NR_Intra [s] (number of DRX cycles); Tmeasure,NR_Intra [s] (number of DRX cycles); Tevaluate,NR_Intra [s] (number of DRX cycles). 0.32; [2.56 x N1 x M2 (8 x N1 x M2)]; [0.32 x N1 x M3 (1 x N1 x M3) ]; [0.96 x N1 x M4 (3 x M4) ]. Note 1: when SMTC < = 40 ms, M2 = M3 = M4 = 1; and when SMTC > 40 ms, M2 = 1.5, M3 = M4 = 2 Note 2: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or [intraRAT-MeasurementEnhancement-r16].. Tentative agreements: None Candidate options: - Option 1: N1 = 3 - Option 2: Discuss the number of DRx cycles directly instead of N1 - Option 3: N1 refers to the agreed Rx numbers Recommendations for 2nd round: Add the scaling to the WF FFS. Issue 3-4-3: Enhancement of requirements Background: Tentative agreements: Candidate options: - None Recommendations for 2nd round: Join with the previous issue in the WF, FFS. |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

TBA

### Sub-topic 2-2: TBA

#### Issue 2-2-2: TBA

Agreements from round 1:

TBA

Candidate options:

- Option 1:

- Option 2:

Recommendations for 2nd round:

TBA

Contributor Comments:

| Company | Comments |
| --- | --- |
| XXX |  |
| YYY |  |
| ZZZ |  |

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

# Recommendations for Tdocs

## 1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on … | YYY |  |
| LS on … | ZZZ | To: RAN_X; Cc: RAN_Y |
| WF on WF on FR2 HST RRM (part 1) | Moderator (Nokia, Nokia Shanghai Bell) |  |
| Email discussion summary for [101-e][216] NR_HST_FR2_RRM_1 | Moderator (Nokia, Nokia Shanghai Bell) | Revised TDoc for the second round summary |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-2118340 | CR On HST FR2 RRM | Ericsson | Nor Pursued |  |
| R4-2119167 | LS on network signalling for Rel-17 NR HST RRM | Nokia, Nokia Shanghai Bell | Nor Pursued |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-210xxxx | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-210xxxx | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

# Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Moderator (Nokia, Nokia Shanghai Bell) | Dmitry Petrov | Dmitry.a.petrov@nokia-bell-labs.com |
| Ericsson | Ming Li | Ming.l.li@ericsson.com |
| CATT | Yanze Fu | fuyanze@catt.cn |
| Intel | Ilya Bolotin | Ilya.bolotin@intel.com |
| CMCC | Jingjing Chen | chenjingjing@chinamobile.com |
| Samsung | Jackson Wang | h0809.wang@samsung.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when making comments, i.e., Company A (XX, XX)