3GPP TSG-RAN WG4 Meeting # 101-e 						R4-2120293

Electronic Meeting, 1st – 12th Nov, 2021

Agenda item:			8.9.4

Source:	Moderator (Samsung)

Title:	WF on FR2 HST RRM requirements (part 2)

Document for:	Approval

Introduction

This contribution is to capture the agreements for the email discussion for Rel-17 FR2 HST RRM part 2 in RAN4 #101 meeting

Way-forward

For one shot large uplink timing adjustment

Moderator note: Highlight part is agreed during RAN4 GTW session

  - It is up to network configuration to enable one shot large uplink timing adjustment mechanism

    - RAN4 will further study if additional flag, e.g., unidirectional flag on top of general FR2 HST scenario flag is needed to enable one shot large uplink timing adjustment

    - RAN4 will further study the network configuration means needed to disable one shot large uplink timing adjustment. If it is disabled, existing uplink timing adjustment, i.e., RA based mechanism, and relative existing RAN4 requirements will be applied when needed

  - Introduce a mechanism for one shot large uplink timing adjustment for FR2 HST scenarios with UE allowed to adjust uplink timing beyond Tq

    - FFS for conditions and additional network assistance for UE to apply one shot large uplink timing adjustment.

    - The following options can be considered for triggering condition and network assistance

      - Option 1: No condition except DL timing difference:

        - UE will apply one shot large timing adjustment if UE measurement on DL timing difference is larger than certain threshold.

        - FFS for how to define the threshold

      - Option 2: TCI switching without network assistance:

        - UE will apply one shot large timing adjustment on TCI switching occasion if UE measurement on DL timing difference is larger than certain threshold.

        - FFS for how to define the threshold

      - Option 3: TCI switching with network assistance of indication of inter-RRH and UE large DL timing change detection

        - UE will apply one shot large timing adjustment on TCI switching between RRH occasion if UE measurement on DL timing difference is larger than certain threshold.

        - FFS for how to define the threshold

        - FFS for detailed network indication of inter-RRH. One example could be a flag in MAC-CE command came with TCI state switch command, or could be SSB index and order per RRH.

      - Option 4: TCI switching with network assistance of indication of inter-RRH but without UE large DL timing change detection

        - UE will apply one shot large timing adjustment on TCI switching between RRH occasions

        - FFS for detailed network indication of inter-RRH. One example could be a flag in MAC-CE command came with TCI state switch command, or could be SSB index and order per RRH.

  - Performance degradation and impact to signalling design shall be discussed for above procedures

  - RAN4 will further discuss the accuracy performance and testing issues based on conclusion of above procedures

For RLM/BFD requirements

  - The existing 1280ms duration for known condition is applied for FR2 HST scenario

  - RAN4 will further study the RLM/BFD requirements for DRX <=80ms for FR2 HST scenarios

    - Option 1: following RX beam factor for set-1 and set-2

    - Option 2: The existing RLM/BFD requirements for DRX <=80ms is applied

For CBD requirements

  - No enhancement on CBD requirements for DRX <=80ms

For PSS/SS and intra-freq measurement

  - Reuse the Rel-16 FR1 HST scaling factor M2 for FR2 HST with the same SMTC periodicity bound of 40ms unless technical issues identified

  - FFS if a different scaling factor is needed for scenario-B with two-side RRH

For L1-RSRP measurement

  - Reuse the Rel-16 FR1 HST scaling factor K for FR2 HST L1-RSRP measurement requirement, with the same SMTC periodicity bound of 40ms unless technical issues identified

  - Separate sets of requirements for deployment Scenarios A and B

    - For scenario A

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*NA)*TSSB) |
| DRX cycle ≤ [80ms] | max(TReport, ceil([1.5]*M*P*NA)*max(TDRX,TSSB)) |
| Note: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. |  |

NA = 2

    - For scenario B

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*NB)*TSSB) |
| DRX cycle < [80ms] | max(TReport, ceil([1.5]*M*P*NB)*max(TDRX,TSSB)) |
| Note: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. |  |

NB = 6