3GPP TSG-RAN WG4 Meeting #101-e	R4-2120616

# Electronic Meeting, Nov. 1st – 12th, 2021

Agenda item:			8.12.3

Source:	Moderator (Huawei, HiSilicon)

Title:			Email discussion summary for [101-e][324] NR_perf_enh2_Demod_Part3

Document for:	Information

Introduction

As per the discussion on BS PUSCH demodulation requirements for FR1 256QAM in last RAN4#100-e, all open issues are finalized as per WF R4-2115748, the general simulation assumptions are captured here for information:

| Parameter | Value |
| --- | --- |
| Transform precoding | Disabled |
| CP | Normal CP |
| Number of Tx | 1 |
| Number of Rx | 2, 4, 8 |
| Number of layers | 1 |
| TDD pattern | 15kHz SCS: 3D1S1U, S=10D:2G:2U 30kHz SCS: 7D1S2U, S=6D:4G:4U |
| DM-RS sequence generation | NID=0, nSCID=0 |
| DMRS type | Type 1 with single-symbol DM-RS |
| Number of DMRS symbols | 1+1 |
| Number of DM-RS CDM groups without data | 2 |
| Ratio of PUSCH EPRE to DM-RS EPRE | -3dB |
| symbols length | 14 |
| start symbol index | 0 |
| Time domain resource allocation type | type A and B |
| Frequency domain resource | Full applicable test bandwidth |
| MCS index | 20 |
| Carrier frequency (GHz) | 4 |
| Propagation condition | TDLA30-10 Low |
| SCS and BW | 15kHz: 5MHz, 10MHz; 30kHz: 10MHz, 40MHz, 100MHz |
| SRS, PT-RS | Not configured |
| Phase noise modelling | No explicit PN modelling (Note 1) |
| Tx EVM | No explicit 3.5% Tx EVM modelling (Note 2) |
| Timing offset, Frequency offset | 0, 0 |
| Code block group, Frequency hopping, Limited buffer rate matching | Disabled |
| Number of HARQ transmissions and RV sequence | 4, {0,2,3,1} |
| Testing metric | SNR @70% of maximum throughput |
| Note 1: The phase noise impact can be included in the impairment results, but it is left up to companies Note 2: Consider 3.5% Tx EVM impact in the impaired results, but it is left up to companies. |  |

1st round discussion:

Target to complete the discussion in the 1st round for the following issues:

- Collect companies’ simulation results for alignment

- Reach consensus on how to derive the final performance requirements

- Reach agreement on the proposed FRC by company shared for this meeting

- Agree on the CR work splitting

2nd round discussion: None

Topic #1: Test parameters

Companies’ contributions summary

| T-doc number | Company | Title | Proposals / Observations |
| --- | --- | --- | --- |
| R4-2117391 | CATT | Simulation results for PUSCH 256QAM performance requirement | Both ideal and impairment results |
| R4-2117527 | Nokia, Nokia Shanghai Bell | Simulation results for PUSCH demodulation requirements for FR1 256QAM | Ideal simulation results |
| R4-2117590 | Samsung | Simulation results for PUSCH with 256QAM | Both ideal and impairment results |
| R4-2117685 | Ericsson | Discussion on FRC for PUSCH 256QAM | FRC definition |
| R4-2117686 | Ericsson | Simulation result for PUSCH 256QAM | Both ideal and impairment results |
| R4-2117741 | CMCC | Simulation results for PUSCH 256QAM performance | Both ideal and impairment results |
| R4-2118006 | Intel Corporation | Simulation results for FR1 256QAM PUSCH demodulation requirements | Both ideal and impairment results |
| R4-2118435 | ZTE Corporation | - Simulation results for FR1 UL 256QAM demodulation requirement | Both ideal and impairment results for Type A without 100MHz/30kHz SCS |
| R4-2118868 | China Telecom | Simulation results on PUSCH FR1 256QAM demodulation requirements | Both ideal and impairment results |
| R4-2119027 | Huawei, HiSilicon | Simulation results for PUSCH demodulation requirements for FR1 UL 256QAM | Ideal simulation results |
| R4-2119028 | Huawei, HiSilicon | Summary of simulation results for PUSCH requirements for FR1 UL 256QAM | - Summary of simulation results from all interesting companies |
| R4-2119029 | Huawei, HiSilicon | Considerations for CR splitting for FR1 PUSCH 256QAM performance requirements | - CR work splitting considerations |

Open issues summary

In this section, test parameters for PUSCH 256QAM demodulation performance requirements will be discussed.

Open issues and candidate options before e-meeting:

Simulation results alignment

Issue 2: Principle for alignment and derivation of final performance requirements

- Proposals

  - Option 1: Reuse the agreed rules for derivation of performance requirements for NR Rel-15 in R4-1904713 which are implemented in the summary of R4-2119028

  - Option 2: Other options

- Recommended WF

  - Reuse the rules defined in R4-1904713 for results alignment and final performance requirement derivation

  - Companies are welcome to provide their impairment results during 1st round discussion for overall alignment

  - Companies can update their results for this and next meetings if needed

FRC

The simulation assumptions are agreed as captured in the approved WF R4-2115748:

Issue 1: FRC

- Proposals

  - Option 1: FRC (Ericsson)

- Table A.x-1: FRC parameters for FR1 PUSCH performance requirements, transform precoding disabled, Additional DM-RS position = pos1 and 1 transmission layer (256QAM, R=682/1024)

| Reference channel | G-FR1-Ax-1 | G-FR1-Ax-2 | G-FR1-Ax-3 | G-FR1-Ax-4 | G-FR1-Ax-5 |
| --- | --- | --- | --- | --- | --- |
| Subcarrier spacing [kHz] | 15 | 15 | 30 | 30 | 30 |
| Allocated resource blocks | 25 | 52 | 24 | 106 | 273 |
| CP-OFDM Symbols per slot (Note 1) | 12 | 12 | 12 | 12 | 12 |
| Modulation | 256QAM | 256QAM | 256QAM | 256QAM | 256QAM |
| Code rate (Note 2) | 682/1024 | 682/1024 | 682/1024 | 682/1024 | 682/1024 |
| Payload size (bits) | 18960 | 39456 | 18192 | 80936 | 208976 |
| Transport block CRC (bits) | 24 | 24 | 24 | 24 | 24 |
| Code block CRC size (bits) | 24 | 24 | 24 | 24 | 24 |
| Number of code blocks - C | 3 | 5 | 3 | 10 | 25 |
| Code block size including CRC (bits) (Note 2) | 6352 | 7920 | 6096 | 8120 | 8384 |
| Total number of bits per slot | 7200 | 14976 | 6912 | 30528 | 78624 |
| Total symbols per slot | 3600 | 7488 | 3456 | 15264 | 39312 |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS and the number of DM-RS CDM groups without data is 2, Additional DM-RS position = pos1, l0= 2 and l =11 for PUSCH mapping type A, l0= 0 and l =10 for PUSCH mapping type B as per table 6.4.1.1.3-3 of TS 38.211 [5]. NOTE 2: Code block size including CRC (bits) equals to K' in clause 5.2.2 of TS 38.212 [15]. |  |  |  |  |  |

  - Option 2: Other options

- Recommended WF

  - Companies are welcome to check if the proposed FRC definition is agreeable.

CR work splitting

All open issues are finalized in last RAN4#100-e as captured in the approved WF R4-2115748:

Issue 1: CR work splitting

- Proposals

  - Option 1 Interesting companies are welcome to provide draft CR for next meeting as per the following work splitting.

| Specification | Name | Title | Comments | Company Name |
| --- | --- | --- | --- | --- |
| TS 38.104 V17.3.0 | Perf requirements for conducted | 8.2.1 Requirements for PUSCH with transform precoding disabled | Add the related new performance requirements into the existing corresponding tables | Intel, Nokia (see comment) |
|  | FRC | A.9 Fixed Reference Channels for performance requirements (256QAM, R=682.5/1024) | New section and Table for FRC of 256QAM: - 5/10MHz / 15kHz SCS -10/40/100MHz/30kHz SCS | Samsung |
| TS 38.141-1 V17.3.0 | Manufacture declaration | D.11x FR1 256QAM | New D.11x Declaration of support of FR1 256QAM as specified in Table 5.1.3.1-2 in TS 38.214 [18] | CATT |
|  | Perf requirements | 8.2.1 Requirements for PUSCH with transform precoding disabled | Insert into existing table for different CBW/SCS and PUSCH mapping type | Samsung, Huawei |
|  | FRC | A.6 Fixed Reference Channels for performance requirements (256QAM, R=682.5/1024) | New section and table for FRC 256QAM | Samsung |
|  | MU | Table 4.1.2.4-1: Maximum Test System Uncertainty for performance requirements | Reuse the existing ± 0.3 dB As per R4-2108667, need to add square brackets for TE vendors’ checking | Ericsson, CATT |
|  | TT | Table C.3-1: Derivation of Test Requirements (Performance tests) | Reuse the existing 0.6 dB for 1Tx cases As per R4-2108667, need to add square brackets for TE vendors’ checking |  |
| TS 38.141-2 V17.3.0 | Manufacture declaration | D.115 FR1 256QAM | New D.11x Declaration of support of FR1 256QAM as specified in Table 5.1.3.1-2 in TS 38.214 [21] | CATT |
|  | Perf requirements | 8.2.1.5.1 Requirements for PUSCH with transform precoding disabled for BS type 1-O | Insert into existing table for different CBW/SCS and PUSCH mapping type | Samsung, CMCC |
|  | FRC | A.9 Fixed Reference Channels for performance requirements (256QAM, R=682.5/1024) | New section and table for FRC 256QAM | Samsung |
|  | MU | Table 4.1.2.4-1: Maximum OTA Test System uncertainty for FR1 OTA performance requirements | Reuse the existing ± 0.6 dB As per R4-2108667, need to add square brackets for TE vendors’ checking | Ericsson, CATT |
|  | TT | Table C.3-1: Derivation of Test Requirements (FR1 OTA performance tests) | Reuse the existing 0.6 dB As per R4-2108667, need to add square brackets for TE vendors’ checking |  |
| Draft Big CR | TS 38.104 | Draft Big CR for TS 38.104 | Merge all draft CRs for TS 38.104 after the meeting | Intel, Nokia (see comment) |
|  | TS 38.141-1 | Draft Big CR for TS 38.141-1 | Merge all draft CRs for TS 38.141-1 after the meeting | Huawei |
|  | TS 38.141-2 | Draft Big CR for TS 38.141-2 | Merge all draft CRs for TS 38.141-2 after the meeting | Ericsson |

  - Option 2: Other options

- Recommended WF

  - Companies are welcome to provide comments on the above CR splitting during the 1st round if any contents are missing

  - Total 9 companies submitted contributions and sharing the simulation results for this meeting, moderator suggests the CR work splitting among those interesting companies.

Companies views’ collection for 1st round

Simulation results alignment

| Company | Comments |
| --- | --- |
| Intel | Recommended WF is fine for us, |
| Samsung | We are fine with recommended WF and will check our results during meeting |
| Huawei | We are fine with the recommended WF. |
| Ericsson | We are fine with recommended WF. |
| CATT | Recommended WF is OK with us. |
| China Telecom | Fine with the recommended WF. |
| ZTE | We are fine with the recommended WF. |
| Nokia, Nokia Shanghai Bell | Fine for WF. Follow usual approach. |

FRC

| Company | Comments |
| --- | --- |
| Intel | Based on our calculation TBS for: - 15 kHz, 10 MHz – 39936 - 30 kHz, 20 MHz – 18432 - 30 kHz, 40 MHz – 81976 Please find below the example of our calculation for 15 kHz and 10 MHz: NRE = min(156, 144)*52 = 7488 Ninfo = 7488 * 0.6665 * 8 * 1 = 39926.25 n = floor(log2(39926.25 - 24))- 5 = 10 Ninfo’ = max(3840, 210 * round((39926.25-24)/210)) = 39936 C = ceil((39936+24)/8424) = 5 TBS = 8*5* ceil((39936+24)/(8*5)) – 24 = 39936 Please double check and let us know in case we missed something. |
| Samsung | Based on our calculation, the related FRC table as, company can double check. Reference channel; G-FR1-Ax-1; G-FR1-Ax-2; G-FR1-Ax-3; G-FR1-Ax-4; G-FR1-Ax-5. Subcarrier spacing [kHz]; 15; 15; 30; 30; 30. Allocated resource blocks; 25; 52; 24; 106; 273. CP-OFDM Symbols per slot (Note 1); 12; 12; 12; 12; 12. Modulation; 256QAM; 256QAM; 256QAM; 256QAM; 256QAM. Code rate (Note 2); 682/1024; 682/1024; 682/1024; 682/1024; 682/1024. Payload size (bits); 18960; 39936; 18432; 81976; 208976. Transport block CRC (bits); 24; 24; 24; 24; 24. Code block CRC size (bits); 24; 24; 24; 24; 24. Number of code blocks - C; 3; 5; 3; 10; 25. Code block size including CRC (bits) (Note 2); 6352; 8016; 6176; 8224; 8384. Total number of bits per slot; 28800; 59904; 27648; 122112; 314496. Total symbols per slot; 3600; 7488; 3456; 15264; 39312. NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS and the number of DM-RS CDM groups without data is 2, Additional DM-RS position = pos1, l0= 2 and l =11 for PUSCH mapping type A, l0= 0 and l =10 for PUSCH mapping type B as per table 6.4.1.1.3-3 of TS 38.211 [5]. NOTE 2: Code block size including CRC (bits) equals to K' in clause 5.2.2 of TS 38.212 [15].. |
| Huawei | We have the same calculation for TBS with Intel and Samsung, and same calculation for others with Samsung. |
| Ercisson | Thanks for Intel and Samsung’s comments. We submitted a wrong version of paper. We agree with the Samsung’s FRC table. |
| China Telecom | We have the same FRC calculation with Samsung. |
| ZTE | We have the same FRC calculation with Samsung. |

CR work splitting

| Company | Comments |
| --- | --- |
| Intel | We are happy to take care of Perf requirements for 38.104 (i.e. Company 1) or Draft Big CR 38.104 (i.e. Company 7) |
| Samsung | We are happy to take care of requirements for 141 (either 141-1 (company 4) or 141-2 (company 6)is fine for us), we are also ok for FRC(company2), if companies also want to take requirement for 141 |
| Huawei | We are happy to take perf requirements for 38.141-1 and Draft big CR for 38.141-1. |
| CMCC | We are happy to take care of requirements for 38.141-2(company 6) |
| Ericsson | We are happy to take MU/TT parts for 38.141-1/2 and also draft big CR for 38.141-2. |
| CATT | We are happy to take care of manufacture declaration or MU (Company 3) and TT (Company 5) for 38.141-1/2 |
| Nokia, Nokia Shanghai Bell | We would be happy to take the complement of Intel, i.e., either company 1 or 7, whichever Intel does ultimately not use. |
| Intel2 | If it is fine for Nokia, then we prefer to take Perf requirements for 38.104 (i.e. Company 1) and transfer Draft Big CR 38.104 (i.e. Company 7) to Nokia. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

| Sub-topic# | Status summary |
| --- | --- |
| Sub-topic#2.2.1 Simulation results alignment | Tentative agreements: Based on the feedback, all interesting companies are fine to reuse the rules defined in R4-1904713 for the results alignment and final performance requirement derivation - Reuse the rules defined in R4-1904713 for results alignment and final performance requirement derivation - Companies can update their results for this and next meetings if needed Candidate options: None Recommendations for 2nd round: With the latest ideal results shared by companies during the 1st round, there are only 5 cases with ideal results slightly larger than 2dB - Companies are welcome to provide their updated and/or impairment results during 2nd round discussion for overall alignment |
| Sub-topic#2.2.2 FRC | Tentative agreements: All interesting companies agree with the updated FRC calculation shared by Samsung: Reference channel; G-FR1-Ax-1; G-FR1-Ax-2; G-FR1-Ax-3; G-FR1-Ax-4; G-FR1-Ax-5. Subcarrier spacing [kHz]; 15; 15; 30; 30; 30. Allocated resource blocks; 25; 52; 24; 106; 273. CP-OFDM Symbols per slot (Note 1); 12; 12; 12; 12; 12. Modulation; 256QAM; 256QAM; 256QAM; 256QAM; 256QAM. Code rate (Note 2); 682/1024; 682/1024; 682/1024; 682/1024; 682/1024. Payload size (bits); 18960; 39936; 18432; 81976; 208976. Transport block CRC (bits); 24; 24; 24; 24; 24. Code block CRC size (bits); 24; 24; 24; 24; 24. Number of code blocks - C; 3; 5; 3; 10; 25. Code block size including CRC (bits) (Note 2); 6352; 8016; 6176; 8224; 8384. Total number of bits per slot; 28800; 59904; 27648; 122112; 314496. Total symbols per slot; 3600; 7488; 3456; 15264; 39312. NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS and the number of DM-RS CDM groups without data is 2, Additional DM-RS position = pos1, l0= 2 and l =11 for PUSCH mapping type A, l0= 0 and l =10 for PUSCH mapping type B as per table 6.4.1.1.3-3 of TS 38.211 [5]. NOTE 2: Code block size including CRC (bits) equals to K' in clause 5.2.2 of TS 38.212 [15].. Candidate options: None Recommendations for 2nd round: None |
| Sub-topic#2.2.3 CR work splitting | Tentative agreements: As per companies’ feedback and try to share the workload among companies, moderator suggests the following CR work splitting: Specification; Name; Title; Comments; Company Name. TS 38.104 V17.3.0; Perf requirements for conducted; 8.2.1 Requirements for PUSCH with transform precoding disabled; Add the related new performance requirements into the existing corresponding tables; Intel. FRC; A.9 Fixed Reference Channels for performance requirements (256QAM, R=682.5/1024); New section and Table for FRC of 256QAM: - 5/10MHz / 15kHz SCS -10/40/100MHz/30kHz SCS; Samsung. TS 38.141-1 V17.3.0; Manufacture declaration; D.11x FR1 256QAM; New D.11x Declaration of support of FR1 256QAM as specified in Table 5.1.3.1-2 in TS 38.214 [18]; CATT. Perf requirements; 8.2.1 Requirements for PUSCH with transform precoding disabled; Insert into existing table for different CBW/SCS and PUSCH mapping type; Huawei. FRC; A.6 Fixed Reference Channels for performance requirements (256QAM, R=682.5/1024); New section and table for FRC 256QAM; Samsung. MU; Table 4.1.2.4-1: Maximum Test System Uncertainty for performance requirements; Reuse the existing ± 0.3 dB As per R4-2108667, need to add square brackets for TE vendors’ checking; Ericsson. TT; Table C.3-1: Derivation of Test Requirements (Performance tests); Reuse the existing 0.6 dB for 1Tx cases As per R4-2108667, need to add square brackets for TE vendors’ checking. TS 38.141-2 V17.3.0; Manufacture declaration; D.115 FR1 256QAM; New D.11x Declaration of support of FR1 256QAM as specified in Table 5.1.3.1-2 in TS 38.214 [21]; CATT. Perf requirements; 8.2.1.5.1 Requirements for PUSCH with transform precoding disabled for BS type 1-O; Insert into existing table for different CBW/SCS and PUSCH mapping type; CMCC. FRC; A.9 Fixed Reference Channels for performance requirements (256QAM, R=682.5/1024); New section and table for FRC 256QAM; Samsung. MU; Table 4.1.2.4-1: Maximum OTA Test System uncertainty for FR1 OTA performance requirements; Reuse the existing ± 0.6 dB As per R4-2108667, need to add square brackets for TE vendors’ checking; CATT. TT; Table C.3-1: Derivation of Test Requirements (FR1 OTA performance tests); Reuse the existing 0.6 dB As per R4-2108667, need to add square brackets for TE vendors’ checking. Draft Big CR; TS 38.104; Draft Big CR for TS 38.104; Merge all draft CRs for TS 38.104 after the meeting; Nokia. TS 38.141-1; Draft Big CR for TS 38.141-1; Merge all draft CRs for TS 38.141-1 after the meeting; Huawei. TS 38.141-2; Draft Big CR for TS 38.141-2; Merge all draft CRs for TS 38.141-2 after the meeting; Ericsson. Candidate options: None Recommendations for 2nd round: None |

Discussion on 2nd round

Summary for 2nd round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

| Sub-topic# | Status summary |
| --- | --- |

Recommendations for Tdocs

1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF for FR1 PUSCH with 256QAM performance requirements | Huawei, HiSilicon |  |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2117391 | Simulation results for PUSCH 256QAM performance requirement | CATT | Noted |  |
| R4-2117527 | Simulation results for PUSCH demodulation requirements for FR1 256QAM | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2117590 | Simulation results for PUSCH with 256QAM | Samsung | Noted |  |
| R4-2117685 | Discussion on FRC for PUSCH 256QAM | Ericsson | Noted |  |
| R4-2117686 | Simulation result for PUSCH 256QAM | Ericsson | Noted |  |
| R4-2117741 | Simulation results for PUSCH 256QAM performance | CMCC | Noted |  |
| R4-2118006 | Simulation results for FR1 256QAM PUSCH demodulation requirements | Intel Corporation | Noted |  |
| R4-2118435 | Simulation results for FR1 UL 256QAM demodulation requirement | ZTE Corporation | Noted |  |
| R4-2118868 | Simulation results on PUSCH FR1 256QAM demodulation requirements | China Telecom | Noted |  |
| R4-2119027 | Simulation results for PUSCH demodulation requirements for FR1 UL 256QAM | Huawei, HiSilicon | Revised | Revised to R4-2120627 |
| R4-2119028 | Summary of simulation results for PUSCH requirements for FR1 UL 256QAM | Huawei, HiSilicon | To be Noted | The summary will be uploaded during the 2nd round |
| R4-2119029 | Considerations for CR splitting for FR1 PUSCH 256QAM performance requirements | Huawei, HiSilicon | Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents