3GPP TSG-RAN4 Meeting #102-e 		R4-2206904

E-meeting, 21th Feb. – 3rd Mar., 2022

Title:	Reply LS on NTN UL time and frequency synchronization requirements

Response to:	R1-2102263 LS on NTN UL time and frequency synchronization requirements

Release:	Rel-17

Work Item:	NR_NTN_solutions

Source:	TSG RAN WG4

To:	TSG RAN WG1

Cc:

Contact Person:

- Name:			Xuhua Tao

- E-mail Address:	taoxuhua@xiaomi.com

Send any reply LS to:	3GPP Liaisons Coordinator:  mailto:3GPPLiaison@etsi.org

1. Overall Description:

RAN4 would like to thank RAN1 for the LS on NTN UL time and frequency synchronization requirement for initial access (i.e. PRACH transmission) and UL transmissions in RRC_CONNECTED state (R1-2102263). Based on the last reply LSs (R4-2115347 and R4-2120311), RAN4 would like to provide the further information on UE initial transmit timing error requirement as follows:

Question 1: What are the NTN UL time synchronization requirements?

- For initial access (i.e. PRACH transmission)

- For UL transmissions in RRC Connected State

[RAN4]: The UL timing requirements for NTN UE will be specified in RAN4 are summarized as follows:

- For initial access (i.e. PRACH transmission)

- Initial transmit timing error requirement (Te_NTN)

RAN4 has provided the feedbacks in R4-2120311.

- For UL transmissions in RRC_CONNECTED state

- Initial transmit timing error requirement (Te_NTN)

RAN4 has provided the feedbacks in R4-2120311.

- Gradual timing adjustment requirement

The gradual timing adjustment requirements are specified as follows:

    - When the transmission timing error between the UE and the reference timing exceeds ±Te_NTN then the UE is required to adjust its timing to within ±Te_NTN.

    - The reference timing shall be (NTA+NTA,UE-specific+NTA,common+NTA,offset)×Tc before the downlink timing of the reference cell.

    - All adjustments made to the UE uplink timing shall follow these rules:

      - The maximum amount of the magnitude of the timing change, apart from a change of NTA,UE-specific due to satellite position update and NTA,common between the previous transmission and the current transmission, in one adjustment shall be Tq.

      - The minimum aggregate adjustment rate, apart from a change of NTA,UE-specific due to satellite position update and NTA,common during the last one second, shall be Tp per second.

      - The maximum aggregate adjustment rate, apart from a change of NTA,UE-specific due to satellite position update and NTA,common during the last 200ms, shall be Tq per 200 ms.

      - where the maximum autonomous time adjustment step Tq and the aggregate adjustment rate Tp are specified in Table 1.

Table 1: Tq and Tp for NTN

| Frequency Range | SCS of uplink signals (kHz) | Tq | Tp |
| --- | --- | --- | --- |
| 1 | 15 | [5.5]*64*Tc | [5.5]*64*Tc |
|  | 30 | [5.5]*64*Tc | [5.5]*64*Tc |
|  | 60 | NA | NA |
| NOTE: Tc is the basic timing unit defined in TS 38.211 |  |  |  |

- TA adjustment accuracy requirement

The TA adjustment accuracy requirements are specified as follows:

    - The UE shall adjust the timing of its transmissions, apart from a change of NTA,UE-specific and NTA,common between the preceding uplink transmission and the current transmission, with a relative accuracy better than or equal to the UE Timing Advance adjustment accuracy requirement in Table 2, to the signalled timing advance value compared to the timing of preceding uplink transmission. The timing advance command step is defined in TS 38.213 [3].

Table 2: UE Timing Advance adjustment accuracy

| UL Sub Carrier Spacing(kHz) | 15 | 30 | 60 |
| --- | --- | --- | --- |
| UE Timing Advance adjustment accuracy | ±256 Tc | ±256 Tc | ±128 Tc |

2. Actions:

To TSG RAN WG1

ACTION: 	RAN4 respectfully ask RAN1 to take the above information into account.

3. Date of Next TSG-RAN WG4 Meetings:

TSG-RAN WG4 Meeting #103-e     16 – 27 May 2022		Electronic Meeting

TSG-RAN WG4 Meeting #104-e     22 – 26 Aug. 2022		Toulouse, FR