3GPP TSG-RAN WG4 Meeting #102-e	R4-2203515

Online Meeting, 21 February – 03 March 2022

3GPP TSG-RAN WG2 Meeting #116bis-e	R2-2201935

# E-meeting, January 17 – January 25, 2022

Title:	Reply LS to RAN4 on NCSG

Response to:	LS on NCSG (R4-2120306)

Release:	Rel-17

Work Item:	NR_MG_enh-Core

Source:	RAN2

To:	RAN4

Cc:	-

Contact Person:

Name:	Yuqin Chen

E-mail Address:	yuqin_chen@apple.com

# 1. Overall Description

RAN2 thanks RAN4 for the LS R4-2120306. RAN2 has discussed the technical aspects related to the LS and made following agreements.

From RRC signaling design, RAN2 aim to support joint working among Pre-MG, concurrent gaps, and NCSG For all the 3 objectives in MG enh. WI, RAN2 prioritize the design in NR SA. Re-use the Rel-16 NeedForGap reporting like procedure for NCSG reporting: - - UE indicates capability on NCSG support in UE capability reporting (FFS on UE capability reporting details). - - NW configures the NCSG reporting in RRCReconfiguration and RRCResume message. - - UE reports the NCSG capabilities in RRCReconfigurationComplete and RRCResumeComplete messages. Agree that NCSG can be configured as per UE, (per FR1 and per FR2 patterns is FFS). FFS if NCSG patterns corresponding to legacy patterns #0 and #1 are mandatorily supported if UE supports NCSG. And to further discuss UE capability between reporting an indicator of NCSG feature support and reporting supported NCSG patterns Detailed design Same as Rel-16 NeedForGap, support NCSG reporting for both intra-frequency and inter-frequency. FFS Inter RAT

In addition, RAN2 would like to kindly ask RAN4 following question:

Q: Whether to support simultaneous configurations on the following combinations?

  - NCSG FR1 gap + NCSG FR2 gap

  - Legacy FR1 gap + NCSG FR2 gap

  - Legacy FR2 gap + NCSG FR1 gap

  - One legacy perUE gap + one NCSG perUE gap

  - One legacy perUE gap + NCSG FR1 gap

  - One legacy perUE gap + NCSG FR2 gap

2. Actions:

RAN2 respectfully asks RAN4 to provide answer to the question above.

3. Date of Next RAN2 Meetings:

TSG-RAN WG2 Meeting #117e	      21 Feb - 03 Mar 2022	E-meeting

TSG-RAN WG2 Meeting #118-e	16 - 27 May 2022		E-meeting