3GPP TSG-RAN WG4 Meeting # 102-e	R4-2203651

Electronic Meeting, 21 February – 3 March 2022

Agenda Item:	10.16.4.2

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Proposals on BS receiver requirements for extending current NR operation to 71 GHz

Document for:	Approval

# 1.	Introduction

The work item for extending current NR operation to 71 GHz was approved at TSG RAN#91-e [1]. One of the objectives of this work item is to define BS RF core requirements for the band(s) in the frequency range between 52.6GHz and 71GHz. BS receiver requirements for extending current NR operation to 71 GHz were further discussed at TSG RAN4#101-bis-e and a way forward was approved [2].

This contribution provides further proposals on BS receiver requirements for extending current NR operation to 71 GHz according to the approved way forward and the findings in the corresponding study item as recorded in TR 38.808 [3].

# 2.	Discussion

2.1	FRC

The following point was agreed in [2]:

- Postpone further FRC discussion on EIS FRCs until SU is finalized.

And the following points were agreed in [4] at TSG RAN4#101-e:

- Existing G-FR2-A1-2 can be re-used for 100MHz/120kHz for ICS for 52.6-71GHz.

- For OTA reference sensitivity define a new dedicated FRC (G-FR2-A1-6) for 480 kHz SCS and 400 MHz carrier bandwidth

- For OTA reference sensitivity define a new dedicated FRC (G-FR2-A1-7) for 960 kHz SCS and 400 MHz carrier bandwidth.

We made the following proposals on spectrum utilization in [5]:

-	Apply same spectrum utilization for 120 kHz SCS in FR2-2 as in FR2-1.

-	Consider similar spectrum utilization for scenarios with 800MHz and 1600MHz as 120 kHz SCS in FR2-1.

-	Support reduced spectrum utilization for 960 kHz SCS & 2 GHz CBW.

It was endorsed in the draft CR in [6] to apply the same spectrum utilization for 120 kHz SCS in FR2-2 as in FR2-1. If similar spectrum utilization for 120 kHz SCS with 400 MHz carrier bandwidth in FR2-2 is applied to 480 kHz SCS and 960 kHz SCS with 400 MHz carrier bandwidth in FR2-2, then there will be 66 RBs for 480 kHz SCS and 32 RBs for 960 kHz SCS with 400 MHz carrier bandwidth. Assuming the same 1/3 coding rate is targeted in the new FRCs, then the new FRCs (G-FR2-A1-6 and G-FR2-A1-7 for reference sensitivity, G-FR2-A1-8 and G-FR2-A1-9 for in-channel selectivity) can be defined in table A.1-2 of TS 38.104 [7] as follows (updates in yellow highlight):

Table A.1-2: FRC parameters for FR2 OTA reference sensitivity level, OTA ACS, OTA in-band blocking, OTA out-of-band blocking, OTA receiver intermodulation and OTA in-channel selectivity

| Reference channel | G-FR2-A1-1 | G-FR2-A1-2 | G-FR2-A1-3 | G-FR2-A1-4 | G-FR2-A1-5 | G-FR2-A1-6 | G-FR2-A1-7 | G-FR2-A1-8 | G-FR2-A1-9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Subcarrier spacing (kHz) | 60 | 120 | 120 | 60 | 120 | 480 | 960 | 480 | 960 |
| Allocated resource blocks | 66 | 32 | 66 | 33 | 16 | 66 | 32 | 33 | 16 |
| CP-OFDM Symbols per slot (Note 1) | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Modulation | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
| Code rate (Note 2) | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 |
| Payload size (bits) | 5632 | 2792 | 5632 | 2856 | 1416 | 5632 | 2792 | 2856 | 1416 |
| Transport block CRC (bits) | 24 | 16 | 24 | 16 | 16 | 24 | 16 | 16 | 16 |
| Code block CRC size (bits) | - | - | - | - | - | - | - | - | - |
| Number of code blocks - C | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Code block size including CRC (bits) (Note 3) | 5656 | 2808 | 5656 | 2872 | 1432 | 5656 | 2808 | 2872 | 1432 |
| Total number of bits per slot | 19008 | 9216 | 19008 | 9504 | 4608 | 19008 | 9216 | 9504 | 4608 |
| Total symbols per slot | 9504 | 4608 | 9504 | 4752 | 2304 | 9504 | 4608 | 4752 | 2304 |
| - NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS, additional DM-RS position = pos1 with l0 = 2, l = 11 as per table 6.4.1.1.3-3 of TS 38.211 [9]. - NOTE 2: MCS index 4 and target coding rate = 308/1024 are adopted to calculate payload size. - NOTE 3: Code block size including CRC (bits) equals to K' in sub-clause 5.2.2 of TS 38.212 [15]. |  |  |  |  |  |  |  |  |  |

Proposal 1: Consider the FRC parameters for reference sensitivity and in-channel selectivity in updated table A.1-2 above.

2.2	ACS

The following point was agreed in [2]:

- For ACS define interfering signal type as 100 MHz DFT-s-OFDM NR signal, 120 kHz SCS, 32 RBs. Confirm the PRB value once SU is agreed.

And the BS ACS was agreed to be 22 dB in [8].

It was endorsed in the draft CR in [6] to apply the same spectrum utilization for 120 kHz SCS in FR2-2 as in FR2-1. Hence there will be 66 RBs for 120 kHz SCS with 100 MHz carrier bandwidth. Therefore, the ACS interfering signal should be defined with 64 RBs instead of 32 RBs.

Proposal 2: Define the ACS interfering signal type as 100 MHz DFT-s-OFDM NR signal, 120 kHz SCS, 64 RBs.

2.3	In-band blocking

The following point was agreed in [2]:

- For in-band blocking set the interferer level 33 dB higher than the OTA REFSENS level.

The following points were agreed in [4] at TSG RAN4#101-e:

- Use 100 MHz DFT-s-OFDM interferer, SU details FFS.

- Add new rows for FR2-2 in relevant IBB tables.

Similar to the discussion on ACS interfering signal, the in-band blocking interfering signal should be defined with 64 RBs.

Proposal 3: Define the in-band blocking interfering signal type as 100 MHz DFT-s-OFDM NR signal, 120 kHz SCS, 64 RBs.

2.4	Receiver spurious emissions

The following points were agreed in [2]:

- For FR2-2 set fOOB to 3.5 GHz.

- For FR2-2 set Fstep,x according to Table 2-2.

Table 2-2: Step frequencies for defining the radiated Rx spurious emission limits for BS type 2-O

| Operating band | Fstep,1 (GHz) | Fstep,2 (GHz) | Fstep,3 (GHz) | Fstep,4 (GHz) | Fstep,5 (GHz) | Fstep,6 (GHz) |
| --- | --- | --- | --- | --- | --- | --- |
| n257 | 18 | 23.5 | 25 | 31 | 32.5 | 41.5 |
| n258 | 18 | 21 | 22.75 | 29 | 30.75 | 40.5 |
| n259 | 23.5 | 35.5 | 38 | 45 | 47.5 | 59.5 |
| n260 | 25 | 34 | 35.5 | 41.5 | 43 | 52 |
| n261 | 18 | 25.5 | 26.0 | 29.85 | 30.35 | 38.35 |
| n262 | 37.2 | 45.2 | 45.7 | 49.7 | 50.2 | 58.2 |
| [Licensed band] | [46] | [61] | [62.5] | [74.5] | [76] | [91] |
| n263 | 18 | 43 | 53.5 | 74.5 | 85 | 127 |

The upper bound of FDL,high – FDL,low for ΔfOBUE = 3500 MHz is FFS in the approved way forward in [9]. We consider it technically reasonable that the same upper bound of FDL,high – FDL,low should also apply to ΔfOOB when the upper bound of FDL,high – FDL,low for ΔfOBUE is agreed.

Proposal 4: Apply the same upper bound of FDL,high – FDL,low to ΔfOOB when the upper bound of FDL,high – FDL,low for ΔfOBUE is agreed.

2.5	Receiver intermodulation

The following point was agreed in [2]:

- For in-band blocking set the interferer level 33 dB higher than the OTA REFSENS level.

The following points were agreed in [4] at TSG RAN4#101-e:

- Use 100 MHz DFT-s-OFDM signal with [8] dB offset.

- SU agreement is needed before frequency offset and RB number details can be agreed.

Similar to the discussion on ACS interfering signal, the modulated interfering signal for receiver intermodulation requirement should be defined with 64 RBs.

Proposal 5: Define the modulated interfering signal type for receiver intermodulation requirement as 100 MHz DFT-s-OFDM NR signal, 120 kHz SCS, 64 RBs.

# 3.	Conclusion

This contribution has provided further proposals on BS receiver requirements for extending current NR operation to 71 GHz according to the approved WF and the findings in the corresponding study item as recorded in TR 38.808. They are summarized as follows.

Proposal 1: Consider the FRC parameters for reference sensitivity and in-channel selectivity in updated table A.1-2 above.

Proposal 2: Define the ACS interfering signal type as 100 MHz DFT-s-OFDM NR signal, 120 kHz SCS, 64 RBs.

Proposal 3: Define the in-band blocking interfering signal type as 100 MHz DFT-s-OFDM NR signal, 120 kHz SCS, 64 RBs.

Proposal 4: Apply the same upper bound of FDL,high – FDL,low to ΔfOOB when the upper bound of FDL,high – FDL,low for ΔfOBUE is agreed.

Proposal 5: Define the modulated interfering signal type for receiver intermodulation requirement as 100 MHz DFT-s-OFDM NR signal, 120 kHz SCS, 64 RBs.

# References

[1]	RP-210862, “Revised WID:  Extending current NR operation to 71 GHz”, CMCC.

[2]	R4-2203017, “WF on BS RF Rx requirements”, Ericsson.

[3]	3GPP TR 38.808 v17.0.0: “Study on supporting NR from 52.6 GHz to 71 GHz”.

[4]	R4-2120678, “WF on BS RF Rx requirements and CR work split”, Ericsson.

[5]	R4-2201598, “System parameters for a NR band in the range 52.6GHz – 71GHz”, Nokia, Nokia Shanghai Bell.

[6]	R4-2202364, “Draft CR for TS 38.101-2: Introduction of system parameters for FR2-2”, Vivo.

[7]	3GPP TS 38.104 v17.4.0: “NR; Base Station (BS) radio transmission and reception”.

[8]	R4-2202367, “WF on the decision of DL/UL ACIR and BS/UE ACLR/ACS for FR2-2”, CATT, Qualcomm, Nokia, ZTE, Ericsson, vivo, Korea Testing Laboratory, Huawei.

[9]	R4-2203016, “WF on BS RF Tx requirements”, Nokia, Nokia Shanghai Bell.