3GPP RAN WG4 Meeting #102-e		R4-2203692

Online, Feb 21st – Mar 3rd, 2022

Agenda item:	10.20.2.1.2

Source:	Apple

Title:	RedCap UE HD-FDD REFSENS requirements

WI/SI:	NR_redcap-Core

Release:	Rel-17

Document for:	Approval

# 1	Introduction

The RedCap UE REFSENS requirements have been discussed in last three RAN4 meetings [1-3]. For HD-FDD versus FD-FDD REFSENS requirement, it was agreed that the tightening to be based on Option 1a which was proposed in [4] except for bands n91, n92, n93, and n94. In the contribution, we propose no REFSENS tightening for bands n91, n92, n93, and n94 in conjunction with the agreement on Option 1a to consolidate the HD-FDD REFSENS requirements. We also propose to explicitly tabulate the 2Rx HD-FDD REFSENS power levels in the specifications.

# Discussion

The RedCap UE HD-FDD versus FD-FDD REFSENS requirement tightening has been agreed to be based on the framework of Option 1a which was proposed in [4] and restated as below:

Proposal 1: The HD-FDD 5MHz REFSENS tightening from FD-FDD is proposed as in Table 2-1.

| FD-FDD 5MHz REFSENS | HD-FDD REFSENS Tightening | Bands |
| --- | --- | --- |
| ≤ -100 dBm | 0 dB | n1, n18, n24, n70 |
| > -100 dBm and ≤ -99 dBm | 0.5 dB | n30, n65, n66, n74 |
| > -99 dBm | 0.8 dB | n2, n3, n5, n7, n8, n12, n13, n14, n20, n25, n26, n28, n71, n85 |

Table 2-1 HD-FDD 5MHz REFSENS tightening for different NR bands

Proposal 2: HD-FDD REFSENS for channel BW wider than 5 MHz can be calculated by REFSENS(5MHz) + 10log10(n x NRB/25), where NRB is the maximum transmission bandwidth configuration with n=1 for 15kHz SCS, n=2 for 30kHz SCS, and n=4 for 60kHz SCS.

However, for bands n91, n92, n93, and n94, the HD-FDD REFSENS tightening has not been settled. Since the DL bands for n91, n92, n93, and n94 are SDL bands for which the REFSENS requirements were not derived based on higher insertion loss duplexer implementation, therefore, there should not be any tightening from FD-FDD to HD-FDD. This is also aligned with the principle in Proposal 1.

Proposal 3: For bands n91, n92, n93, and n94, no REFSENS tightening is needed from FD-HDD to HD-FDD.

On the other hand, for half-duplex operation, there is no need to impose UL RB number restriction. Therefore, full allocation is proposed as the UL configuration for HD-FDD REFSENS requirements.

Proposal 4: UL configuration for HD-FDD REFSENS requirements is specified with full allocation.

Since not all FDD bands REFSENS with channel bandwidth up to 20 MHz were specified without Tx noise, the HD-FDD REFSENS requirements cannot be directly referenced to FDD bands REFSENS requirements for non-RedCap UE, it is proposed to explicitly tabulate the RedCap UE HD-FDD REFSENS power levels in the specifications.

Proposal 5: RedCap UE 2Rx HD-FDD REFSENS power levels are explicitly tabulated in the specifications.

# 3	Conclusion

In the contribution, we propose no REFSENS tightening for bands n91, n92, n93, and n94 in conjunction with the agreement on Option 1a to consolidate the HD-FDD REFSENS requirements. We also propose to explicitly tabulate the 2Rx HD-FDD REFSENS power levels in the specifications.

Proposal 1: The HD-FDD 5MHz REFSENS tightening from FD-FDD is proposed as in the table below.

| FD-FDD 5MHz REFSENS | HD-FDD REFSENS Tightening | Bands |
| --- | --- | --- |
| ≤ -100 dBm | 0 dB | n1, n18, n24, n70 |
| > -100 dBm and ≤ -99 dBm | 0.5 dB | n30, n65, n66, n74 |
| > -99 dBm | 0.8 dB | n2, n3, n5, n7, n8, n12, n13, n14, n20, n25, n26, n28, n71, n85 |

Proposal 2: HD-FDD REFSENS for channel BW wider than 5 MHz can be calculated by REFSENS(5MHz) + 10log10(n x NRB/25), where NRB is the maximum transmission bandwidth configuration with n=1 for 15kHz SCS, n=2 for 30kHz SCS, and n=4 for 60kHz SCS.

Proposal 3: For bands n91, n92, n93, and n94, no REFSENS tightening is needed from FD-HDD to HD-FDD.

Proposal 4: UL configuration for HD-FDD REFSENS requirements is specified with full allocation.

Proposal 5: RedCap UE 2Rx HD-FDD REFSENS power levels are explicitly tabulated in the specifications.

Based on the proposals 1 – 4 above, the RedCap UE REFSENS requirements for HD-FDD with 2Rx can be concluded as shown in Table 3-1 and Table 3-2.

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| n1 | 15 | -100.0 | -96.8 | -95.0 | -93.7 |
|  | 30 |  | -97.2 | -95.2 | -93.9 |
|  | 60 |  | -97.5 | -95.4 | -94.2 |
| n2 | 15 | -98.8 | -95.6 | -93.8 | -92.5 |
|  | 30 |  | -96.0 | -94.0 | -92.7 |
|  | 60 |  | -96.3 | -94.2 | -93.0 |
| n3 | 15 | -97.8 | -94.6 | -92.8 | -91.5 |
|  | 30 |  | -95.0 | -93.0 | -91.7 |
|  | 60 |  | -95.3 | -93.2 | -92.0 |
| n5 | 15 | -98.8 | -95.6 | -93.8 | -92.5 |
|  | 30 |  | -96.0 | -94.0 | -92.7 |
| n7 | 15 | -98.8 | -95.6 | -93.8 | -92.5 |
|  | 30 |  | -96.0 | -94.0 | -92.7 |
|  | 60 |  | -96.3 | -94.2 | -93.0 |
| n8 | 15 | -97.8 | -94.6 | -92.8 | -91.5 |
|  | 30 |  | -95.0 | -93.0 | -91.7 |
| n12 | 15 | -97.8 | -94.6 | -92.8 |  |
|  | 30 |  | -95.0 | -93.0 |  |
| n13 | 15 | -97.8 | -94.6 |  |  |
|  | 30 |  | -95.0 |  |  |
| n14 | 15 | -97.8 | -94.6 |  |  |
|  | 30 |  | -95.0 |  |  |
| n18 | 15 | -100.0 | -96.8 | -95.0 |  |
|  | 30 |  | -97.2 | -95.2 |  |
| n20 | 15 | -97.8 | -94.6 | -92.8 | -91.5 |
|  | 30 |  | -95.0 | -93.0 | -91.7 |
| n24 | 15 | -100.0 | -96.8 |  |  |
|  | 30 |  | -97.2 |  |  |
|  | 60 |  | -97.5 |  |  |
| n25 | 15 | -97.3 | -94.1 | -92.3 | -91.0 |
|  | 30 |  | -94.5 | -92.5 | -91.2 |
|  | 60 |  | -94.8 | -92.7 | -91.5 |
| n26 | 15 | -98.3 | -95.1 | -93.3 | -92.0 |
|  | 30 |  | -95.5 | -93.5 | -92.2 |
| n28 | 15 | -99.3 | -96.1 | -94.3 | -93.0 |
|  | 30 |  | -96.5 | -94.5 | -93.2 |
| n30 | 15 | -99.5 | -96.3 |  |  |
|  | 30 |  | -96.7 |  |  |
| n65 | 15 | -100.0 | -96.8 | -95.0 | -93.7 |
|  | 30 |  | -97.2 | -95.2 | -93.9 |
|  | 60 |  | -97.5 | -95.4 | -94.2 |
| n66 | 15 | -100.0 | -96.8 | -95.0 | -93.7 |
|  | 30 |  | -97.2 | -95.2 | -93.9 |
|  | 60 |  | -97.5 | -95.4 | -94.2 |
| n70 | 15 | -100.0 | -96.8 | -95.0 | -93.7 |
|  | 30 |  | -97.2 | -95.2 | -93.9 |
|  | 60 |  | -97.5 | -95.4 | -94.2 |
| n71 | 15 | -98.0 | -94.8 | -93.0 | -91.7 |
|  | 30 |  | -95.2 | -93.2 | -91.9 |
| n74 | 15 | -100.0 | -96.8 | -95.0 | -93.7 |
|  | 30 |  | -97.2 | -95.2 | -93.9 |
|  | 60 |  | -97.5 | -95.4 | -94.2 |
| n85 | 15 | -97.8 | -94.6 | -92.8 |  |
|  | 30 |  | -95.0 | -93.0 |  |
| n91 | 15 | -100.0 |  |  |  |
| n92 | 15 | -100.0 | -96.8 | -95.0 | -93.7 |
|  | 30 |  | -97.2 | -95.2 | -93.9 |
| n93 | 15 | -100.0 |  |  |  |
| n94 | 15 | -100.0 | -96.8 | -95.0 | -93.7 |
|  | 30 |  | -97.2 | -95.2 | -93.9 |

Table 3-1 Proposed RedCap UE REFSENS requirements for HD-FDD with 2Rx

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| n1 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
|  | 60 |  | 10 | 18 | 24 |
| n2 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
|  | 60 |  | 10 | 18 | 24 |
| n3 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
|  | 60 |  | 10 | 18 | 24 |
| n5 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
| n7 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
|  |  |  | 10 | 18 | 24 |
| n8 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
| n12 | 15 | 25 | 50 | 75 |  |
|  | 30 |  | 24 | 36 |  |
| n13 | 15 | 25 | 50 |  |  |
|  | 30 |  | 24 |  |  |
| n14 | 15 | 25 | 50 |  |  |
|  | 30 |  | 24 |  |  |
| n18 | 15 | 25 | 50 | 75 |  |
|  | 30 |  | 24 | 36 |  |
| n20 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
| n24 | 15 | 25 | 50 |  |  |
|  | 30 |  | 24 |  |  |
|  |  |  | 10 |  |  |
| n25 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
|  | 60 |  | 10 | 18 | 24 |
| n26 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
| n28 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
| n30 | 15 | 25 | 50 |  |  |
|  | 30 |  | 24 |  |  |
| n65 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
|  |  |  | 10 | 18 | 24 |
| n66 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
|  |  |  | 10 | 18 | 24 |
| n70 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
|  |  |  | 10 | 18 | 24 |
| n71 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
| n74 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
|  |  |  | 10 | 18 | 24 |
| n85 | 15 | 25 | 50 | 75 |  |
|  | 30 |  | 24 | 36 |  |
| n91 | 15 | 25 |  |  |  |
| n92 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
| n93 | 15 | 25 |  |  |  |
| n94 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |

Table 3-2 Proposed RedCap UE REFSENS UL configurations for HD-FDD

# 4	References

R4-2114995 “WF on RedCap REFSENS in FR1”, Sony, 3GPP TSG-RAN WG4 Meeting #100-e, August 16th – 27th, 2021

R4-2120067 “WF on the RedCap RF”, Ericsson, 3GPP TSG-RAN WG4 Meeting #101-e, November 1st – 12th, 2021

R4-2202371 “WF on the RedCap RF”, Ericsson, 3GPP TSG-RAN WG4 Meeting #101bis-e, January 17th – 25th, 2022

R4-2200442 “RedCap UE HD-FDD REFSENS requirements”, Apple, 3GPP TSG-RAN WG4 Meeting #101bis-e, January 17th – 25th, 2022