3GPP TSG-RAN4 Meeting #102-e	R4-2203741

# Online, 21 February – 3 March, 2022

Agenda item:	10.8.2.3

Source:	Apple

Title:	On remaining issues for R17 FR1 HST

Document for:	Discussion

# Introduction

In RAN4#101e-bis RAN4 requirements for R17 FR1 HST was widely discussed. Corresponding agreement and open items are captured in the approved WF [1]. In this contribution, we continue discussing the open issues listed in [1].

# Discussion

Issue 3-1-1: how to specify the L1-SINR requirements for high speed scenario

- Option 1: Existing L1-SINR measurement requirements are reused for HST, and the upper bound of the side condition is 5dB

- Option 2: Existing L1-SINR measurement requirements are reused for HST, no need to add any upper bound to the side condition of L1-SINR requirements under FR1 HST scenario

Option 1 is supported. First, 5dB cap comes from high Doppler shift which has been widely discussed since LTE HST speed train WI. Second, even though it is up to NW whether to configure L1-SINR, we don’t think there is much room for NW Tx beam determination based on L1-SINR on top of L1-RSRP in high-speed train scenario, due to the specific UE trajectory and linear deployment of RRH.

Proposal 1: Existing L1-SINR measurement requirements are reused for HST, and the upper bound of the side condition is 5dB.

# Conclusion

In this contribution, we further discuss the open issues according to [1]. After discussion, the following conclusions are provided:

Proposal 1: Existing L1-SINR measurement requirements are reused for HST, and the upper bound of the side condition is 5dB.

# References

- R4-2202590, WF on RRM for FR1 HST, CMCC

- R4-2120285, WF on RRM for FR1 HST, CMCC