3GPP TSG-RAN4 Meeting #102-e	R4-2203744

# Online, 21 February – 3 March, 2022

Agenda item:	10.22.2.1

Source:	Apple

Title:	On efficient activation/de-activation mechanism for SCells

Document for:	Discussion

# Introduction

In RAN4#101e-bis RAN4 requirements for fast SCell activation was widely discussed. Corresponding agreement and open items are captured in the approved WF [1]. In this contribution, we continue discussing the open issues listed in [1].

# Discussion

Issue 1-2-2: Tuncertainty_MAC for scenario #3

Scenario #3: SCell to be activated belongs to FR2, if there is no active serving cell on that FR2 band, and target SCell is known to UE.

  - Option 1(MTK, QC, Apple, OPPO): Assuming PDCCH TCI and PDSCH TCI (when applicable) shall be associated with the triggered temporary RS burst:

          - if semi-persistent CSI-RS is used for CSI reporting, Tactivation_time is 3ms + max (Ttemp_RS+ 2ms, Tuncertainty_SP)

          - if periodic CSI-RS is used for CSI reporting, Tactivation_time is max (Ttemp_RS + 5ms, Tuncertainty_RRC + TRRC_delay-THARQ)

under the condition that

          - One of the candidate TCI states configured in TCI-StatesPDCCH-ToAddList has the same QCL source of the triggered A-TRS,

          - The QCL source of CSI-RS for CQI reporting is the same as the triggered A-TRS,

          - The TCI state for PDCCH/PDSCH that is the same as A-TRS is assumed during SCell activation until changed by network after SCell activation.

  - Option 2 (Huawei, Ericsson, Nokia): Not assuming PDCCH TCI and PDSCH TCI (when applicable) is associated with the triggered temporary RS burst.

    - Option 2a (Huawei):

          - if semi-persistent CSI-RS is used for CSI reporting, Tactivation_time is 3ms + max(Ttemp_RS+ 2ms, Tuncertainty_MAC, Tuncertainty_SP)

          - if periodic CSI-RS is used for CSI reporting, Tactivation_time is max(Ttemp_RS + 5ms, Tuncertainty_MAC+3ms,  Tuncertainty_RRC + TRRC_delay-THARQ)

    - Option 2b (Ericsson):

          - if semi-persistent CSI-RS is used for CSI reporting, Tactivation_time is 3ms + max(TATRS + 2ms, Tuncertainty_MAC + 2ms, Tuncertainty_SP)

          - if periodic CSI-RS is used for CSI reporting, Tactivation_time is max(TATRS + 5ms, Tuncertainty_MAC + 5ms, Tuncertainty_RRC + TRRC_delay-THARQ)

Option 1 is supported. It is reasonable and typical for NW to associate PDCCH TCI and PDSCH TCI with the triggered temporary RS burst during scell activation. If NW would like to use different TCI state for PDCCH/PDSCH, e.g. due to updated L1 measurement reporting after activation procedure, NW can change the TCI state after SCell activation.

Proposal 1: (option 1)

- Tuncertainty_MAC for scenario #3: SCell to be activated belongs to FR2, if there is no active serving cell on that FR2 band, and target SCell is known to UE.

  - Assuming PDCCH TCI and PDSCH TCI (when applicable) shall be associated with the triggered temporary RS burst:

          - if semi-persistent CSI-RS is used for CSI reporting, Tactivation_time is 3ms + max (Ttemp_RS+ 2ms, Tuncertainty_SP)

          - if periodic CSI-RS is used for CSI reporting, Tactivation_time is max (Ttemp_RS + 5ms, Tuncertainty_RRC + TRRC_delay-THARQ)

under the condition that

          - One of the candidate TCI states configured in TCI-StatesPDCCH-ToAddList has the same QCL source of the triggered A-TRS,

          - The QCL source of CSI-RS for CQI reporting is the same as the triggered A-TRS,

          - The TCI state for PDCCH/PDSCH that is the same as A-TRS is assumed during SCell activation until changed by network after SCell activation.

# Conclusion

In this contribution, we further discuss the open issues according to [1]. After discussion, the following conclusions are provided:

Proposal 1: (option 1)

- Tuncertainty_MAC for scenario #3: SCell to be activated belongs to FR2, if there is no active serving cell on that FR2 band, and target SCell is known to UE.

  - Assuming PDCCH TCI and PDSCH TCI (when applicable) shall be associated with the triggered temporary RS burst:

          - if semi-persistent CSI-RS is used for CSI reporting, Tactivation_time is 3ms + max (Ttemp_RS+ 2ms, Tuncertainty_SP)

          - if periodic CSI-RS is used for CSI reporting, Tactivation_time is max (Ttemp_RS + 5ms, Tuncertainty_RRC + TRRC_delay-THARQ)

under the condition that

          - One of the candidate TCI states configured in TCI-StatesPDCCH-ToAddList has the same QCL source of the triggered A-TRS,

          - The QCL source of CSI-RS for CQI reporting is the same as the triggered A-TRS,

          - The TCI state for PDCCH/PDSCH that is the same as A-TRS is assumed during SCell activation until changed by network after SCell activation.

# References

- R4-2202688, WF on R17 further Multi-RAT Dual-Connectivity enhancements, Huawei

- R4-2120334, WF on R17 further Multi-RAT Dual-Connectivity enhancements, Huawei