3GPP TSG-RAN WG4 Meeting # 102-e	R4-2203767

# Electronic Meeting, Feb 21- Mar 3, 2022

Agenda item:	10.12.2.2

Source:	Apple Inc.

Title:	Discussion on PDSCH requirements in MU-MIMO scenarios

Release:	Rel-17

Document for:	Discussion

# 1. Introduction

In RAN4#101-bis-e PDSCH demodulation requirements with MMSE-IRC for intra-cell inter-user interference was discussed and way forward [1] was agreed. In this contribution we present our views on the open issues, simulation assumptions for requirements definition for requirements in MU-MIMO.

# 2. Discussion

## Inter-user Interference modeling

The agreements in [1] related to inter-user interference modeling:

PMI matrix selection for Co-scheduled UE for 2TX and 4TX - Agree with option 3.i.e. Use random PMI selection for rank 1+1 and orthogonal PMI selection for rank 2+2 DMRS ports for case with rank 1+1 - DMRS port 0 for target UE, DMRS port 1 for the interference UE, i.e., same CDM group Number of CDM groups without data configuration for case with rank 1+1 - Set CDM groups without data to 1 for paired UEs for case with rank 1+1

Open Issues for inter-user interference modeling are:

- DMRS scrambling ID for target UE and co-scheduled UE

DMRS Scrambling ID

For DMRS scrambling ID for target UE and co-scheduled UE, the following options were discussed:

- Option 1: Same scrambling ID when paired UEs are in the same CDM group. Different scrambling ID when paired UEs are in different CDM groups.

- Option 2: Same scrambling ID for all cases

- Option 3: Configure variable scrambling ID during the test.

Firstly, Option 3 is very impractical, and we don’t support using such configuration for requirements definition. We don’t expect to see performance delta whether same of different scrambling IDs are used. To reduce test set up complexity we prefer to use same scrambling ID for all cases. Also, in Rel-18 if we explore E-MMSE-IRC receiver for joint decoding, then we would use same scrambling ID and it would be useful to compare against performance with MMSE-IRC receiver.

Proposal #1: For requirements with MU-MIMO configure same DMRS scrambling ID for target and co-scheduled UEs.

## Simulation Assumptions

For the simulation assumptions agreed in [1], we present simulation results below.

Table 1: Simulation results for PDSCH demod in MU-MIMO

| Duplex | Ant Config | Prop. Channel | MCS | Co-scheduled UE precoder | SNR @ 70% TP |
| --- | --- | --- | --- | --- | --- |
| FDD | 2x2 | TDLC300 | MCS13 | Random | 14.2 |
|  | 2x4 | TDLC300 | MCS13 | Random | 9.7 |
|  | 4x4 | TDLA30 | MCS13 | Orthogonal | 14.5 |
| TDD | 2x2 | TDLC300 | MCS13 | Random | 14.0 |
|  | 2x4 | TDLC300 | MCS13 | Random | 9.8 |
|  | 4x4 | TDLA30 | MCS13 | Orthogonal | 14.5 |

# 3. Conclusion

In this paper, we present our views on the open issues, simulation results for requirements definition for requirements in MU-MIMO. Our proposals are captured below:

Proposal #1: For requirements with MU-MIMO configure same DMRS scrambling ID for target and co-scheduled UEs.

# Reference

- R4-2203011, “Way Forward on MMSE-IRC for intra-cell inter-user interference”, Huawei, HiSilicon.