3GPP TSG-RAN WG4 Meeting # 102-e	R4-2203771

# Electronic Meeting, Feb 21- Mar 3, 2022

Agenda item:	10.12.2.3.3

Source:	Apple Inc.

Title:	Discussion on Test Setup for CRS-IM Requirements

Release:	Rel-17

Document for:	Discussion

# 1. Introduction

In RAN4#101-e CRS interference mitigation in NR was discussed and way forwards [1] and [2] were agreed. In this contribution we present our views on test setup for CRS-IM requirements with 15KHz SCS.

# 2. Discussion

The open issues in [1] related to test setup with 15KHz SCS:

- PDSCH loading level

- Interference power level

- TX antenna and number of LTE CRS ports

PDSCH Loading level and interference levels

The options discussed in [1] for PDSCH loading of LTE cell:

- Option 1: Add one of the combinations below that achieves about 1dB performance gain over the reference scheme

  - Loading 30 %, INR 1 = 9.69 dB, INR 2 = 3.7 dB

  - Loading 40 %, INR 1 = 8.79 dB, INR 2 = 2.7 dB

  - Loading 50 %, INR 1 = 8.36 dB, INR 2 = 1.7 dB

- Option 2: Only consider 20% PDSCH loading level

- Option 3: Consider 30% interference PDSCH loading for Scenario 1 and 20% interference PDSCH loading for Scenario 2

- Option 4: Also include 80% loading

- Option 5: Further check 30% loading level

- Option 6: Either 30% or 20%

The options discussed in [1] for interference power levels:

- Option 1: Add one of the combinations below that achieves about 1dB performance gain over the reference scheme

  - Loading 30 %, INR 1 = 9.69 dB, INR 2 = 3.7 dB

  - Loading 40 %, INR 1 = 8.79 dB, INR 2 = 2.7 dB

  - Loading 50 %, INR 1 = 8.36 dB, INR 2 = 1.7 dB

- Option 2: Only consider INR1 = 10.45 dB and INR2 = 4.6 dB

- Option 3: Only consider INR1 = 10.45 dB and INR2 = 4.6 dB in case of 20% interference PDSCH loading

- Option 4: 30% PDSCH loading and INR 1 = 9.69 dB, INR 2 = 3.7 dB for Scenario 1 and 20% interference PDSCH loading and INR1 = 10.45 dB and INR2 = 4.6 dB for Scenario 2

We have evaluated performance only for 20% loading. Higher loading and lower interference levels would reduce the gain of CRS-IM. We don’t support introducing additional interference power or loading levels for CRS-IM requirements.

Proposal #1: Only consider 20% PDSCH loading with INR1 = 10.45 dB and INR2 = 4.6 dB interference levels.

TX antenna

The options in [1] for number of TX antenna and CRS ports:

- Option 1: Only cover 4 CRS ports

- Option 2: Cover 2 and 4 for CRS ports

  - Option 2A: Consider 2 CRS ports for Scenario 1 and 4 CRS ports for Scenario 2

  - Option 2B: Use different CRS port number in the tests with different INR levels

- Option 3: Only cover 2 CRS ports (Nokia)

With 4 CRS ports maximum impact of CRS interference on NR is observed. We don’t see the need to use 2 CRS ports for requirements as it might need another study for suitable interference levels and loading. We prefer to only define requirements with 4 ports for LTE CRS and 4TX antenna for NR.

Proposal #2: Only cover 4 CRS ports and 4 TX antenna for all requirements for CRS-IM.

# 3. Conclusion

In this paper, we provide our views on test setup for CRS-IM requirements with 15KHz SCS. Our observations and proposals are captured below:

Proposal #1: Only consider 20% PDSCH loading with INR1 = 10.45 dB and INR2 = 4.6 dB interference levels.

Proposal #2: Only cover 4 CRS ports and 4 TX antenna for all requirements for CRS-IM.

# Reference

- R4-2203131, “WF on general part and 15kHz NR SCS scenario for CRS-IM receiver”, China Telcom.

- R4-2203029, “WF on 30 kHz NR SCS scenario for CRS-IM receiver”, CMCC.