3GPP TSG-RAN WG4 Meeting # 102-e	R4-2203782

# Electronic Meeting, Feb 21- Mar 3, 2022

Agenda item:	10.20.4.3

Source:	Apple Inc.

Title:	On CSI Reporting Requirements for Reduced Capability Devices in NR

Release:	Rel-17

Document for:	Discussion

# 1. Introduction

In RAN4#101-bis-e UE demodulation and CSI requirements for RedCap devices was discussed and way forward [1] was agreed. In this contribution we present our views on PDCCH and PBCH demod requirements for RedCap UE.

# 2. Discussion

## CQI Reporting Requirements

For CQI reporting requirements in [1] the following agreements were made:

- Define CQI reporting tests for both static condition and fading condition for RedCap 1Rx/2Rx UE. - For the fading condition define wideband CQI reporting test only. - Use CQI table 1 for RedCap UE CQI reporting requirements. - For 2Rx tests for FR1 FDD, - Define new 2Rx tests for FR1 FDD with CQI table 1 - FFS whether to define test with CQI table 2 for Redcap UE supporting 256 QAM. If agreed, existing test requirements can be reused

Supporting 256QAM is optional for RedCap UEs. We don’t need to introduce requirements for 256QAM for RedCap.

Proposal #1: Do not introduce CQI reporting requirements with 256QAM CQI table for RedCap.

## PMI/RI Reporting Requirements

The PMI and RI reporting requirements for RedCap are still FFS.

From the options discussed in [1]:

- RAN4 discuss further whether to define PMI reporting tests for RedCap UE.

  - Option 1: Define for both 1Rx UE and 2Rx UE

  - Option 2: Not define for both 1Rx UE and 2Rx UE

  - Option 3 Define only for 2Rx UE

- If PMI reporting test is defined, discuss further applied test cases

Firstly, we think we should have the same set of test cases for 2RX and 1RX for RedCap in order to reduce testing efforts and introduce applicability rules between 1RX and 2RX requirements. We already have agreed to this in [1] for TDD.

- Define 2Rx FR1 TDD demodulation (e.g., PBCH, PDCCH and PDSCH) and CSI requirements (e.g., CQI, PMI and RI) with channel bandwidth equal to 20MHz. - Keep the requirements aligned between 2Rx and 1Rx.

Observation #1: We have agreement to define the same requirements for 1RX and 2RX for RedCap for TDD.

In order to reduce the number of requirements for RedCap UE we propose not to introduce PMI reporting requirements.

Proposal #2: Do not define PMI reporting requirements for RedCap UE for either 1RX or 2RX.

RI reporting is also not needed for RedCap UE to reduce the UE implementation complexity and testing burden. Moreover, RI reporting can only be for 2RX and 2 layers which is subject to UE capability. Hence, we don’t see the necessity to introduce RI reporting requirements for 2RX for RedCap.

Proposal #3: Do not define RI reporting requirements for RedCap.

# 3. Conclusion

In this paper, we provide our views on CSI reporting requirements for RedCap UE. Our observations and proposals are captured below:

Proposal #1: Do not introduce CQI reporting requirements with 256QAM CQI table for RedCap.

Observation #1: We have agreement to define the same requirements for 1RX and 2RX for RedCap for TDD.

Proposal #2: Do not define PMI reporting requirements for RedCap UE for either 1RX or 2RX.

Proposal #3: Do not define RI reporting requirements for RedCap.

# Reference

- R4-2203032, “WF on RedCap demodulation and CQI reporting requirements”, Ericsson.