3GPP TSG-RAN WG4 Meeting #102-e                                                    R4-2204009

# Electronic Meeting, February 21 – March 3, 2022

Source:	ZTE Corporation

Title:	TP to TR 38.862 on simplification for EN-DC and NE-DC configuration tables

Agenda Item:	11.3.3.2

Document for:	Approval

- 1	Introduction

A discussion paper on simplification for DC configuration table in Rel-18 is proposed in [1]. It suggests to use the field delimiter “/” for the notation of FR2 CA BW classes in EN-DC and NE-DC configurations having the common FR1 part.  In this paper, A TP to TR38.862 to reflect the usage of the field delimiter “/” is provided.

Proposal 1:	 It is proposed to approve the text proposal provided in this contribution.

- 2	References

[1] R4-2204005, Further discussion on simplification for DC configuration table in Rel-18, ZTE.

- Text Proposal

<< begin text proposal >>

5.3	Rules to be used for the notation of CA or DC configurations

The following are the rules for generating the configuration notations:

-	Each configuration needs to start with “CA_”, “DC_”, “SUL_” or “V2X_”.

-	DC combinations include a list of LTE carriers first, followed by the list of NR carriers.

-	Entries within a list of either LTE carriers or NR carriers need to be separated by “-“, not “_”.

-	The list of LTE carriers and the list of NR carriers within an EN_DC combination need to be separated by “_”, for contiguous intra-band EN-DC the two lists are connected with the (n)xxAA like notation, not “_” (xx is the band number) , for contiguous intra-band NE-DC the two lists are connected with the xx(n)AA like notation, not “_” (xx is the band number). In specific cases “_SUL_” connects the two lists.

-	Contiguous LTE+NR intra-band carriers within a DC combination are using the notation (n)xxAA (xx is the band number) , Contiguous NR+LTE intra-band carriers within a DC combination are using the notation xx(n)AA (xx is the band number).

-	No other characters than “A” to “Z”, “0” to “9”, “(“, “)”, “-“ “_” and “n” are allowed within the notation, especially no spaces “ “, “/”, “.”, “.”, LineFeed, CR, other special characters. For character “/”, more usage in Rel-18 can be referred to clause 8.3.x.2.

-	Entries within the list of carriers need to be sorted in numerical order, i.e. first band n1, then n2, then n3, then n260, i.e. CA_1A-2A, not CA_2A-1A, but LTE and NR combinations are separately sorted, i.e. DC_2A_n1A, entries with (n) are always between the LTE and NR lists.

-	Bandwidth notations are either a single character according to the BW class lists of contiguously aggregated carriers, two of these characters in case of  combinations with (n) or for NR non-contiguous intra-band combinations specific expressions listing multiple carriers within “()”.

-	Within the “()” of non-contiguous NR combinations there will only be BW class letters for the BW class of contiguous sub-blocks preceded by a number indicating the number of sub-blocks of this BW class, if there are multiple different BW classes they are listed in ascending BW class order separated by “-“.

Examples of correct notations are as follows:

-	DC_1A-2A_n260(A-M)

-	DC_1A-2A-2A-2A_n3(3A)

-	DC_1A-(n)2AA-n3A

-	DC_1A-2A-3A-4A-5A_n6A-n260(2A-3G-Q)

-	Some incorrect examples we have seen: DC_1A-2A_n3A(3A) (no “A” before the bracket); DC_2A-1A_n3(3A) (wrong sort order of LTE bands); DC_1A- 2A_n260(A-M) (a “ “ (space) between the “-“ and the “2”.

Currently this notation for the CA/DC configurations is used as specified in 36.101 for LTE and 38.101 for NR. However, multiple errors within the specifications have been observed, which motivate the need to define the following rules how to handle the CA/DC configurations in the -101 specs. Below are the general rules how to implement these band combinations within the CA/DC configurations within the tables in clause 5.5:

-	Each cell of the configuration table should contain only one combination of bands in the first column with the exception that combinations having the same bands but different intra-band contiguous BW classes can be listed in the same cell. Also all non-contiguous combinations can be listed in one cell, but separated from the contiguous combinations in another cell.

-	In the UL column there shall only be UL configurations that belong to the configurations in the first column. Unfortunately this means that in the UL column there can be higher order configurations than some of the configurations in the first column, however, they cannot be used with such a lower order combination. This was agreed some time ago as a “table simplification”, but creates some hassle as there are UL combinations listed that cannot be supported with the DLs.

-	Multiple configurations with different bandwidth classes shall be separated by pressing the return key, you will see the  sign at the end of the line if you activated the button to view these special characters, no other special characters to separate configuration shall be used. Some exceptions for the configurations of inter-band EN-DC/NE-DC including FR2, inter-band EN-DC including FR1 and FR2 in Rel-18 can be found in clause 8.3.x.2.

-	There shall be no special characters not belonging to the combinations in any configuration cell, no spaces “ “, “/”, “.”, “.”, or any other special characters. Character “/” will be used in Rel-18 to separate multiple configurations for EN-DC/NE-DC including FR2 bands.

-	If there are notes for a specific configuration, the note shall be using superscript font and added at the end of the configuration list within a cell, not anywhere within the configuration or separated with any other characters, multiple notes shall be separated just by a comma, all in superscript.

- << Unchanged sections omitted >>

- 8.3.x	Further simplification on configuration tables for DC configurations including FR2 bands in Rel-18

- 8.3.x.1	Necessity for further simplification

In current RAN4 specs, with the introduction of more new CA BW classes, the situation of redundancy for CA/DC combinations has become much more serious than before, especially for the configurations having FR2 band part. Also it is noted that the redundancy for FR2 bands is more serious than FR1 bands. The main reason is that for NR FR1 intra-band contiguous CA bandwidth class, only a small part of the classes, i.e. A to E, in the whole FR1 CA BW class table are mostly used. It can be predicted that the DC combinations including FR2 will become more redundant in the future.

- 8.3.x.2	Proposed approaches

In order to solve the redundancy of the DC configuration tables having FR2 band, the following rules are applied to further optimize the EN-DC and NE-DC configuration tables.

(1)  Merge different intra-band contiguous CA BW classes with field delimiter “/” for NR FR2 band in the configurations having the common FR1 part.

-	For EN-DC configurations, only CA BW classes for the last NR FR2 band can be merged.

-	For NE-DC configurations, only CA BW classes for the first NR FR2 band can be merged.

(2)  The first configuration with a common FR1 part should be in a separate row in the DC configuration table.

To further illustrate the above usage of delimiter “/” as shown in Proposal 1, we take the following configurations as an example.

●  Inter-band EN-DC configurations including FR2

Table 5.5B.5.1-1: Inter-band EN-DC configurations including FR2 (two bands) in TS 38.101-3

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A_n257A DC_1A_n257D DC_1A_n257E DC_1A_n257F DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_1A_n257J DC_1A_n257K DC_1A_n257L DC_1A_n257M | DC_1A_n257A DC_1A_n257D DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_1A_n257J DC_1A_n257K DC_1A_n257L DC_1A_n257M |
| DC_1A_n258A DC_1A_n258B DC_1A_n258C DC_1A_n258D DC_1A_n258E DC_1A_n258F DC_1A_n258G DC_1A_n258H DC_1A_n258I DC_1A_n258J DC_1A_n258K DC_1A_n258L DC_1A_n258M | DC_1A_n258A DC_1A_n258D DC_1A_n258G |

Table 8.3.x.2-1: Optimized table for inter-band EN-DC configurations including FR2 (two bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A_n257A DC_1A_n257D/E/F/G/H/I/J/K/L/M | DC_1A_n257A DC_1A_n257D/G/H/I/J/K/L/M |
| DC_1A_n258A DC_1A_n258B/C/D/E/F/G/H/I/J/K/L/M | DC_1A_n258A DC_1A_n258D/G |

●   Inter-band NE-DC configurations including FR2

Table 5.5B.5a.2-1: Inter-band NE-DC configurations including FR2 (three bands) in TS 38.101-3

| NE-DC configuration | Uplink NE-DC configuration (NOTE 1) |
| --- | --- |
| DC_n257A_1A-3A DC_n257G_1A-3A DC_n257H_1A-3A DC_n257I_1A-3A DC_n257J_1A-3A DC_n257K_1A-3A DC_n257L_1A-3A DC_n257M_1A-3A DC_n257A_1A-3C DC_n257G_1A-3C DC_n257H_1A-3C DC_n257I_1A-3C DC_n257J_1A-3C DC_n257K_1A-3C DC_n257L_1A-3C DC_n257M_1A-3C | DC_n257A_1A DC_n257A_3A DC_n257G_3A DC_n257H_3A DC_n257I_3A DC_n257J_3A DC_n257K_3A DC_n257L_3A DC_n257M_3A |
| DC_n257A_1A-5A DC_n257G_1A-5A DC_n257H_1A-5A DC_n257I_1A-5A DC_n257J_1A-5A DC_n257K_1A-5A DC_n257L_1A-5A DC_n257M_1A-5A | DC_n257A_1A DC_n257A_5A |

Table 8.3.x.2-2: Optimized table for inter-band NE-DC configurations including FR2 (three bands)

| NE-DC configuration | Uplink NE-DC configuration (NOTE 1) |
| --- | --- |
| DC_n257A_1A-3A DC_n257G/H/I/J/K/L/M_1A-3A DC_n257A_1A-3C DC_n257G/H/I/J/K/L/M_1A-3C | DC_n257A_1A DC_n257A_3A DC_n257G/H/I/J/K/L/M_3A |
| DC_n257A_1A-5A DC_n257G/H/I/J/K/L/M_1A-5A | DC_n257A_1A DC_n257A_5A |

●   Inter-band EN-DC configurations including FR1 and FR2

Table 5.5B.6.2-1: Inter-band EN-DC configurations including FR1 and FR2 (three bands) in TS 38.101-3

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A_n3A-n257A2 DC_1A_n3A-n257G2 DC_1A_n3A-n257H2 DC_1A_n3A-n257I2 | DC_1A_n3A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I |
| DC_1A_n28A-n257A2 DC_1A_n28A-n257G2 DC_1A_n28A-n257H2 DC_1A_n28A-n257I2 | DC_1A_n28A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I |
| DC_1A_n77A-n257A2 DC_1A_n77A-n257D2 DC_1A_n77A-n257E2 DC_1A_n77A-n257F2 DC_1A_n77A-n257G2 DC_1A_n77A-n257H2 DC_1A_n77A-n257I2 DC_1A_n77C-n257A2 DC_1A_n77C-n257D2 DC_1A_n77C-n257E2 DC_1A_n77C-n257F2 | DC_1A_n77A DC_1A_n257A DC_1A_n257D DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_1A_n77A-n257A DC_1A_n77A-n257G DC_1A_n77A-n257H DC_1A_n77A-n257I |
| DC_1A_n77(2A)-n257A2 DC_1A_n77(2A)-n257D2 DC_1A_n77(2A)-n257G2 DC_1A_n77(2A)-n257H2 DC_1A_n77(2A)-n257I2 | DC_1A_n77A DC_1A_n257A DC_1A_n257D DC_1A_n257G DC_1A_n257H DC_1A_n257I |

Table 8.3.x.2-3: Optimized table for inter-band EN-DC configurations including FR1 and FR2 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A_n3A-n257A2 DC_1A_n3A-n257G2/H2/I2 | DC_1A_n3A DC_1A_n257A DC_1A_n257G/H/I |
| DC_1A_n28A-n257A2 DC_1A_n28A-n257G2/H2/I2 | DC_1A_n28A DC_1A_n257A DC_1A_n257G/H/I |
| DC_1A_n77A-n257A2 DC_1A_n77A-n257D2/E2/F2/G2/H2/I2 DC_1A_n77C-n257A2 DC_1A_n77C-n257D2/E2/F2 | DC_1A_n77A DC_1A_n257A DC_1A_n257D/G/H/I DC_1A_n77A-n257A DC_1A_n77A-n257G/H/I |
| DC_1A_n77(2A)-n257A2 DC_1A_n77(2A)-n257D2/G2/H2/I2 | DC_1A_n77A DC_1A_n257A DC_1A_n257D/G/H/I |

With regards to the issue of searching the inter-band DC configuration including FR2 band in the spec, there are two options which can be used.

-	(Option a) Use the first configuration with the common FR1 part to locate the higher order DC configurations. For example, if we want to search DC_n257H_1A-3A, we can first locate DC_n257A_1A-3A and then find DC_n257H_1A-3A in the following configurations.

-	(Option b) Use the DC configuration by removing the higher order of CA BW class to locate the higher order DC configurations. For example, if we want to search DC_1A_n257G, we can use the phrase “DC_1A_n257” as the first step, and then we will find DC_1A_n257G in the following configurations.

-	For searching the EN-DC configuration including FR2 band or including FR1 and FR2 band, either (Option a) or (Option b) can be used.

-	For searching the NE-DC configuration including FR2 band, only (Option a) can be used.

- 8.3.x.3	When to implement

Considering that the completion time for Rel-17 basket WID is Mar. 2022, and after that a Rel-18 spec will be created. The sooner simplifications are made in Rel-18, the less the RAN4 specifications will be affected. It is suggested that the new format should be used in the combination requests in the EXCEL file for inter-band EN-DC/NE-DC including FR2, inter-band EN-DC including FR1 and FR2. Also the new formats should be used in proponent’s TP and rapporteurs TR in Rel-18.

<< end text proposal >>