3GPP TSG-RAN WG4 Meeting #102-e	R4-2204022

Electronic Meeting, 21st February – 3rd March 2022

Source:	AT&T

Title:	TP for TR 38.842 Addition of CA_n29-n30-n77

Agenda item:			9.32.2

Document for:	Approval

# 1. Introduction

A text proposal for TR 38.842 to add CA_n29-n30-n77.

# 2. Text Proposal

---Start of changes---

## 5.X	CA_n29-n30-n77

### 5.X.1	Configurations

Table 5.X.1-1: NR CA configurations and bandwidth combinations sets for supporting power class 2

| NR CA configuration | Uplink CA configuration or single uplink carrier6 | NR Band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n29A-n30A-n77A | n777 | n29 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n30 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n29A-n30A-n77(2A) | n777 | n29 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n30 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 6: Only single uplink carriers with power class other than PC3 are listed. NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 5.X.2	Maximum output power

The combination CA_n29_n30-n77 does not have an uplink CA configuration containing n29 since n29 is a SDL operating band.

Table 5.X.2-1 UE Power Class 2 for One UL Carrier

| CA power class | Carrier n29 power class | Carrier n30 power class | Carrier n77 power class |
| --- | --- | --- | --- |
| 26dBm | N/A | 23dBm | 26dBm |

### 5.X.3	REFSENS requirements

MSD requirements for single uplink PC2 are captured in lower order combinations.

---Skipping to Dual Uplink Carrier---

## 6.X	CA_n29-n30-n77

### 6.X.1	Configurations

Table 6.x.1-1: NR CA configurations and bandwidth combinations sets for supporting power class 2

| NR CA configuration | Uplink CA configuration or single uplink carrier6 | NR Band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n29A-n30A-n77A | CA_n30A-n77A7 | n29 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n30 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n29A-n30A-n77(2A) | CA_n30A-n77A7 | n29 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n30 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 6: Only single uplink carriers with power class other than PC3 are listed. NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.X.2	Maximum output power

Table 6.x.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_n30-n77 | CA power class | Carrier n30 power class | Carrier n77 power class |
| --- | --- | --- | --- | --- |
| CA_n30A-n77A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

The tolerance +2/-3 dB is applied.

### 6.X.3	REFSENS requirements

The PC2 dual-uplink carrier co-existence analysis for CA_n29-n30-n77 can be re-used from DC_29-30_n77 in TR 37.827. Based on this, it can be seen that:

-	IMD3 and IMD5 products are produced by Band n30 and n77 that might fall in Rx of band n29.

#### 6.X.3.1	Power class 2 Case A

The additional MSD due to intermodulation for PC2 CA_n29-n30-n77 can be re-used from PC2 DC_29-30_n77 in TR 37.827 and are defined in Table 6.X.3.1-1.

Table 6.X.3.1-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n29-n30-n77 | n29 | N/A | 5 | N/A | 722 | 23.5 | FDD | IMD31 |
|  | n30 | 2310 | 5 | 25 | 2355 | N/A | FDD | N/A |
|  | n77 | 3898 | 10 | 50 | 3898 | N/A | TDD | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |

#### 6.X.3.2	Power class 2 Case B

The additional MSD due to intermodulation for PC2 Case B CA_n29-n30-n77 are same as the Case A defined in Table 6.X.3.1-1.

---End of changes---

# References