| 3GPP TR 37.717-11-11 V1.1.0 (2022-03) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Networks; Rel-17 Dual Connectivity (DC) of 1 LTE band (1DL/1UL) and 1 NR band (1DL/1UL) (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	10

3	Definitions of terms, symbols and abbreviations	11

3.1	Terms	11

3.2	Symbols	11

3.3	Abbreviations	11

4	Background	12

4.1	TR Maintenance	12

5	DC of 1 LTE band (1DL/1UL) and 1 NR band (1DL/1UL): General Part	12

5.1	Maximum Sensitivity Degradation (MSD) analysis	12

5.1.1	Inter-band EN-DC/NE-DC within FR1	12

5.1.2	Inter-band EN-DC/NE-DC including FR2	13

5.2	Interference analysis for other system	13

5.3	Spurious emission band UE co-existence for EN-DC/NE-DC	13

5.4	Handling of Band specific requirements for inter-band EN-DC including FR2	14

6	DC of 1 LTE band (1DL/1UL) and 1 NR band: Specific Band Combination Part	15

6.1	Inter-band DC within FR1	15

6.1.1	DC_8_n7	15

6.1.2	DC_66_n77, DC_66-66_n77 and DC_66-66-66_n77	17

6.1.3	DC_13_n77	21

6.1.4	DC_5_n77	25

6.1.5	DC_2_n77 and DC_2-2_n77	28

6.1.6	DC_19_n1	32

6.1.7	DC_21_n1	34

6.1.8	DC_42_n1	36

6.1.9	DC_48_n25	38

6.1.10	DC_28_n2 and DC_2_n28	41

6.1.11	DC_4_n2	44

6.1.12	DC_4_n5	45

6.1.13	DC_4_n7	47

6.1.14	DC_28_n66	49

6.1.15	DC_4_n28	52

6.1.16	DC_66_n28	54

6.1.17	DC_28_n1	56

6.1.18	DC_42_n3	59

6.1.19	DC_8_n2	63

6.1.20	DC_18_n28	65

6.1.21	DC_18_n41	68

6.1.22	DC_7_n2	70

6.1.23	DC_25_n77	72

6.1.24	DC_25_n78	75

6.1.25	DC_21_n28	77

6.1.26	DC_12_n71	80

6.1.27	DC_71_n41	82

6.1.28	DC_71_n2	84

6.1.29	DC_2_n30	87

6.1.30	DC_5_n30	89

6.1.31	DC_12_n30	91

6.1.32	DC_66_n30	93

6.1.33	DC_11_n41	94

6.1.34	DC_38_n28	96

6.1.35	DC_12_n77	98

6.1.36	DC_14_n77	101

6.1.37	DC_30_n77	103

6.1.38	DC_14_n30	105

6.1.39	DC_7_n25	107

6.1.40	DC_13_n25	109

6.1.41	DC_3-3_n8	111

6.1.42	DC_7-7_n8	112

6.1.43	DC_48_n77	112

6.1.44	DC_n28_3	114

6.1.45	DC_n28_8	114

6.1.46	DC_n28_40	115

6.1.47	DC_n41_3	116

6.1.48	DC_n41_8	117

6.1.49	DC_n41_34	118

6.1.50	DC_n41_39	119

6.1.51	DC_n41_40	120

6.1.52	DC_38_n3	121

6.1.53	DC_38_n1	122

6.1.54	DC_38_n8	124

6.1.55	DC_n3_1	127

6.1.56	DC_n8_1	128

6.1.57	DC_n77_1, DC_n77(*)_1	129

6.1.58	DC_n77_3, DC_n77(*)_3	130

6.1.59	DC_n3_8	131

6.1.60	DC_n8_3	132

6.1.61	DC_n77_8, DC_n77(*)_8	133

6.1.62	DC_n28_34	134

6.1.63	DC_n28_39	137

6.1.64	DC_14_n5	139

6.1.65	DC_11_n1	141

6.1.66	DC_41_n1	144

6.1.67	DC_7_n79	147

6.1.68	DC_38_n79	149

6.1.69	DC_5_n1	151

6.1.70	DC_5_n3	153

6.1.71	DC_2_n25	155

6.2	Inter-band DC including FR2	157

6.2.1	DC_66_n261	157

6.2.2	DC_48_n261	157

6.2.3	DC_13_n261	158

6.2.4	DC_5_n261	158

6.2.5	DC_2_n261	159

6.2.6	DC_20_n257	159

6.2.7	DC_3-3_n257	160

6.2.8	DC_7_n257, DC_7-7_n257	161

6.3	Intra-band contiguous DC	162

6.3.1	DC_(n)66	162

6.3.2	DC_(n)3	162

6.4	Intra-band non-contiguous DC	164

6.4.1	DC_71_n71	164

6.4.2	DC_66_n66	166

Annex A (informative): Change history	168

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

1	Scope

The present document is a technical report for Dual Connectivity (DC) of 1 LTE band (1DL/1UL) and 1 NR band (1DL/1UL) under Rel-17 time frame.

The purpose is to gather the relevant background information and studies in order to address Dual Connectivity (DC) of 1 LTE band (1DL/1UL) and 1 NR band (1DL/1UL) for the Rel-17 band combinations in Table 1-1, 1-2, 1-3 and 1-4, including EN-DC and NE-DC combinations. The actual requirements are added to the corresponding technical specifications.

Table 1-1: Release 17 EN-DC of 1 LTE band (1DL/1UL) and 1 NR band (1DL/1UL) within FR1

| DC combination | Uplink EN-DC configuration |
| --- | --- |
| DC_8A_n7A | DC_8A_n7A |
| DC_66A_n77A DC_66A-66A_n77A DC_66A-66A-66A_n77A | DC_66A_n77A |
| DC_13A_n77A | DC_13A_n77A |
| DC_5A_n77A | DC_5A_n77A |
| DC_2A_n77A DC_2A-2A_n77A | DC_2A_n77A |
| DC_19A_n1A | DC_19A_n1A |
| DC_21A_n1A | DC_21A_n1A |
| DC_42A_n1A DC_42C_n1A | DC_42A_n1A |
| DC_48A_n25A DC_48C_n25A DC_48D_n25A | DC_48A_n25A |
| DC_28A_n2A | DC_28A_n2A |
| DC_2A_n28A | DC_2A_n28A |
| DC_4A_n2A | DC_4A_n2A |
| DC_4A_n5A | DC_4A_n5A |
| DC_4A_n7A | DC_4A_n7A |
| DC_28A_n66A | DC_28A_n66A |
| DC_4A_n28A | DC_4A_n28A |
| DC_66A_n28A | DC_66A_n28A |
| DC_28A_n1A | DC_28A_n1A |
| DC_42A_n3A | DC_42A_n3A |
| DC_42C_n3A | DC_42A_n3A DC_42C_n3A |
| DC_8A_n2A | DC_8A_n2A |
| DC_18A_n28A | DC_18A_n28A |
| DC_18A_n41A | DC_18A_n41A |
| DC_7A_n2A DC_7C_n2A | DC_7A_n2A |
| DC_71A_n71A | DC_71A_n71A1 |
| DC_25A_n77A | DC_25A_n77A |
| DC_25A-25A_n77A | DC_25A_n77A |
| DC_25A_n78A | DC_25A_n78A |
| DC_25A-25A_n78A | DC_25A_n78A |
| DC_21A_n28A2 | DC_21A_n28A |
| DC_12A_n71A | DC_12A_n71A3,4 |
| DC_71A_n41A | DC_71A_n41A |
| DC_71A_n2A | DC_71A_n2A |
| DC_2A_n30A | DC_2A_n30A |
| DC_5A_n30A | DC_5A_n30A |
| DC_12A_n30A | DC_12A_n30A |
| DC_66A_n30A | DC_66A_n30A |
| DC_11A_n41A | DC_11A_n41A |
| DC_66A_n66A | DC_66A_n66A1 |
| DC_38A_n28A | DC_38A_n28A |
| DC_12A_n77A | DC_12A_n77A |
| DC_14A_n77A | DC_14A_n77A |
| DC_30A_n77A | DC_30A_n77A |
| DC_14A_n30A | DC_14A_n30A |
| DC_7A_n25A | DC_7A_n25A |
| DC_7A-7A_n25A | DC_7A_n25A |
| DC_7C_n25A | DC_7A_n25A |
| DC_13A_n25A | DC_13A_n25A |
| DC_3A-3A_n8A | DC_3A_n8A |
| DC_7A-7A_n8A | DC_7A_n8A |
| DC_48A_n77A5. 6. 7, 8 | N/A |
| DC_38A_n3A | DC_38A_n3A |
| DC_(n)66AA | DC_(n)66AA1 |
| DC_38A_n1A | DC_38A_n1A |
| DC_38A_n8A | DC_38A_n8A |
| DC_(n)3AA | DC_(n)3AA1 |
| DC_14A_n5A | DC_14A_n5A |
| DC_11A_n1A | DC_11A_n1A |
| DC_41A_n1A DC_41C_n1A | DC_41A_n1A DC_41C_n1A |
| DC_7A_n79A DC_7A_n79C | DC_7A_n79A |
| DC_38A_n79A DC_38A_n79C | DC_38A_n79A |
| DC_5A_n1A | DC_5A_n1A |
| DC_5A_n3A | DC_5A_n3A |
| DC_2A_n25A9. 10. 11 | n25A |
| NOTE 1: Only single switched UL is supported. NOTE 2: The frequency range in band n28 is restricted for this band combination to 728 - 738 MHz for the UL and 783 - 793 MHz for the DL. This restriction applies also for these band combinations when applicable EN-DC configuration is part of a higher order EN-DC configuration. NOTE 3: Only single switched UL is supported. NOTE 4: The implementation with 4 antennas is targeted for FWA form factor for this band combination. NOTE 5: The minimum requirements apply only when there is non-simultaneous Tx/Rx operation between E-UTRA and NR carriers. This restriction applies also for these carriers when applicable EN-DC configuration is part of a higher order EN-DC configuration. NOTE 6: The minimum requirements for intra-band non-contiguous EN-DC apply. When UE capability interBandContiguousMRDC is indicated, the minimum requirements for intra-band-contiguous EN-DC also should be met in addtion to intra-band non-contiguous EN-DC. The intra-band requirements also apply for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. NOTE 7: The combination is not used alone as fall back mode of other band combinations in which UL in Band 42 or Band 48 is not used. NOTE 8: The minimum requirements for inter-band EN-DC apply when the maximum power spectral density imbalance between downlink carriers is within 6 dB. The power spectral density imbalance condition also applies for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. NOTE 9: The combination is not used alone as fallback mode of other band combinations in which UL in Band 2 is not used. NOTE 10: The minimum requirements for inter-band EN-DC apply when the maximum power spectral density imbalance between downlink carriers is within 6 dB. The power spectral density imbalance condition also applies for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. NOTE 11: The minimum requirements apply for synchronized DL carriers with a maximum receive time difference ≤ 3 usec. The requirements also apply for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. |  |

Table 1-2: Release 17 EN-DC of 1 LTE band (1DL/1UL) and 1 NR band (1DL/1UL) including FR2

| DC combination | Uplink EN-DC configuration |
| --- | --- |
| DC_66A_n261(A-L) | DC_66A_n261A DC_66A_n261G DC_66A_n261H DC_66A_n261I |
| DC_66A_66A_n261(A-L) | DC_66A_n261A DC_66A_n261G DC_66A_n261H DC_66A_n261I |
| DC_48A_n261(A-L) | DC_48A_n261A DC_48A_n261G DC_48A_n261H DC_48A_n261I |
| DC_13A_n261(A-L) | DC_13A_n261A DC_13A_n261G DC_13A_n261H DC_13A_n261I |
| DC_5A_n261(A-J) | DC_5A_n261A DC_5A_n261G DC_5A_n261H DC_5A_n261I |
| DC_5A_n261(A-L) | DC_5A_n261A DC_5A_n261G DC_5A_n261H DC_5A_n261I |
| DC_2A_n261(A-L) | DC_2A_n261A DC_2A_n261G DC_2A_n261H DC_2A_n261I |
| DC_2A-2A_n261A | DC_2A_n261A |
| DC_2A-2A_n261I | DC_2A_n261A DC_2A_n261G DC_2A_n261H DC_2A_n261I |
| DC_2A-2A_n261M | DC_2A_n261A DC_2A_n261G DC_2A_n261H DC_2A_n261I |
| DC_20A_n257A DC_20A_n257B DC_20A_n257C DC_20A_n257D DC_20A_n257E DC_20A_n257F DC_20A_n257G DC_20A_n257H DC_20A_n257I DC_20A_n257J DC_20A_n257K DC_20A_n257L DC_20A_n257M | DC_20A_n257A |
| DC_3A-3A_n257G DC_3A-3A_n257H DC_3A-3A_n257I DC_3A-3A_n257J DC_3A-3A_n257K DC_3A-3A_n257L DC_3A-3A_n257M | DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K |
| DC_7A-7A_n257J DC_7A-7A_n257K DC_7A-7A_n257L DC_7A-7A_n257M | DC_7A_n257J DC_7A_n257K |
| DC_7A_n257J DC_7A_n257K DC_7A_n257L DC_7A_n257M | DC_7A_n257J DC_7A_n257K |

Table 1-3: Release 17 NE-DC of 1 LTE band (1DL/1UL) and 1 NR band (1DL/1UL) within FR1

| DC combination | Uplink NE-DC configuration |
| --- | --- |
| DC_n28A_3A DC_n28A_3C | DC_n28A_3A |
| DC_n28A_8A DC_n28A_8B | DC_n28A_8A |
| DC_n28A_40A DC_n28A_40C | DC_n28A_40A |
| DC_n41A_3A DC_n41A_3C DC_n41C_3A | DC_n41A_3A |
| DC_n41A_8A DC_n41A_8B DC_n41C_8A | DC_n41A_8A |
| DC_n41A_34A1 | DC_n41A_34A |
| DC_n41A_39A1 DC_n41A_39C1 DC_n41C_39A1 | DC_n41A_39A |
| DC_n41A_40A1 DC_n41A_40C1 DC_n41C_40A1 | DC_n41A_40A |
| DC_n3A_1A | DC_n3A_1A |
| DC_n8A_1A | DC_n8A_1A |
| DC_n77A_1A DC_n77(2A)_1A | DC_n77A_1A |
| DC_n77A_3A DC_n77(2A)_3A | DC_n77A_3A |
| DC_n3A_8A | DC_n3A_8A |
| DC_n8A_3A | DC_n8A_3A |
| DC_n77A_8A DC_n77(2A)_8A | DC_n77A_8A |
| DC_n28A_34A | DC_n28A_34A |
| DC_n28A_39A DC_n28A_39C | DC_n28A_39A |
| NOTE 1: The minimum requirements apply only when there is non-simultaneous Tx/Rx operation between E-UTRA and NR carriers. This restriction applies also for these carriers when applicable EN-DC configuration is part of a higher order EN-DC configuration. |  |

Table 1-4: Release 17 NE-DC of 1 LTE band (1DL/1UL) and 1 NR band (1DL/1UL) including FR2

| DC combination | Uplink NE-DC configuration |
| --- | --- |

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.101-1: “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”

[3]	3GPP TS 38.101-2: “NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone”

[4]	3GPP TS 36.101: “Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception”

[5]	3GPP TS 38.101-3: “NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios”

[6]	3GPP TR 37.863-01-01: “E-UTRA (Evolved Universal Terrestrial Radio Access) - NR Dual Connectivity (EN-DC) of LTE 1 Down Link (DL) / 1 Up Link (UL) and 1 NR band (Release 15)”

[7]	3GPP TR 37.716-11-11: “”E-UTRA (Evolved Universal Terrestrial Radio Access) - NR Dual Connectivity (EN-DC) of 1 band LTE (1 Down Link (DL) / 1 Up Link (UL)) and 1 NR band (1 DL / 1 UL)(1DL/1UL)  (Release 16)”

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Inter-band dual connectivity: Dual connectivity of component carriers in different operating bands.

Intra-band contiguous dual connectivity: Contiguous carriers of dual connectivity aggregated in the same operating band.

Intra-band non-contiguous dual connectivity: Non-contiguous carriers of dual connectivity aggregated in the same operating band.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

ΔRIB,c	Allowed reference sensitivity relaxation due to support for inter-band CA or DC operation, for serving cell c.

∆RIBNC	Allowed reference sensitivity relaxation due to support for intra-band non-contiguous CA or DC operation

ΔTIB,c	Allowed maximum configured output power relaxation due to support for inter-band CA or DC operation, for serving cell c

FDL_low	The lowest frequency of the downlink operating band

FDL_high	The highest frequency of the downlink operating band

FUL_low	The lowest frequency of the uplink operating band

FUL_high	The highest frequency of the uplink operating band

RBstart	Indicates the lowest RB index of transmitted resource blocks

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

A-MPR	Additional Maximum Power Reduction

BCS	Bandwidth combination set

CBW	Channel bandwidth

DL	DownLink

EN-DC	E-UTRA-NR Dual Connectivity

FDD	Frequency Division Duplex

FR	Frequency Range

FWA	Fixed Wireless Access

IMD	Inter-modulation

MBW	Measurement bandwidth defined for the protected band

MPR	Allowed maximum power reduction

MSD	Maximum Sensitivity Degradation

NE-DC	NR-E-UTRA Dual Connectivity

NR	New radio

OOB	Out-of-band

REFSENS	Reference Sensitivity

SCS	Subcarrier spacing

TDD	Time Division Duplex

UE	User Equipment

UL	UpLink

NR	New RAT

4	Background

The present document is a technical report for Dual Connectivity (DC) of 1 LTE band  and 1 NR band under Rel-17 timeframe. The document covers each band combination specific issues (i.e. one sub-clause defined per band combination)

4.1	TR Maintenance

A single company is responsible for introducing all approved TPs in the current TR, i.e. TR editor. However, it is the responsibility of the contact person of each band combination to ensure that the TPs related to the band combination have been implemented.

5	DC of 1 LTE band (1DL/1UL) and 1 NR band (1DL/1UL): General Part

5.1	Maximum Sensitivity Degradation (MSD) analysis

5.1.1	Inter-band EN-DC/NE-DC within FR1

For each inter-band EN-DC/NE-DC band combination, sensitivity degradation is allowed for a band if it is impacted by UL harmonic interference, harmonic mixing, cross band isolation, and intermodulation interference from another band part of the same DC configuration. For 2UL inter-band DC, the harmonic and IMD products can be calculated by the formula shown in Table 5.1.1-1.

Table 5.1.1-1: Formulation for Harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |

Note that, for each DC band combination, MSD table shall be provided for interference that requires sensivity degradation for an Rx band according to the same manner as specified in section 7.3B in TS 38.101-3.

5.1.2	Inter-band EN-DC/NE-DC including FR2

For each inter-band EN-DC/NE-DC band combination, sensitivity degradation is allowed for a band if it is impacted by UL harmonic interference from another band part of the same DC configuration. Note that for each DC band combination in section 6, MSD table shall be provided for interference that requires sensivity degradation for a Rx band according to the same mannter as specified in section 7.3B in TS 38.101-3.

5.2	Interference analysis for other system

When 2UL inter-band EN-DC/NE-DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems shown in Table 5.2-1.

Table 5.2-1: Harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Harmonic/IMD |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 |  |  |  |
| Galileo | 1559 | - | 1591 |  |  |  |
| GLONASS | 1591 | - | 1610 |  |  |  |
| GPS | 1563 | - | 1587 |  |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 |  | US/Europe |  |
|  | 2400 | - | 2494 |  | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 |  | US |  |
|  | 5150 | - | 5350 |  | Europe |  |
|  | 5470 | - | 5725 |  |  |  |
|  | 5150 | - | 5825 |  | Asia |  |

To mitigate the impact of interference to other system is implementation dependent, thus it is optionl in section 6 for each DC band combination.

5.3	Spurious emission band UE co-existence for EN-DC/NE-DC

Table 5.3-1 specifies the requirements for the specified EN-DC configurations for coexistence with protected bands.

Table 5.3-1: Spurious emissions for uplink inter-band EN-DC (two bands)

| EN-DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_XA_nYA | E-UTRA Band xx, yy, zz NR band aa, bb, cc | FDL_low | - | FDL_high | -50 | 1 |  |
| NOTE1: The protected E-UTRA/NR bands are the common set of the protected bands for the constituted bands. |  |  |  |  |  |  |  |

Note that for the NE-DC configurations that have a corresponding specified EN-DC configuration, the requirements in the Table 6.5B.3.3.2-1 of TS 38.101-3 apply on each component carrier with all component carriers are active.

5.4	Handling of Band specific requirements for inter-band EN-DC including FR2

Unless otherwise stated, for inter-band EN-DC configurations including FR2, there are no additional requirements of spurious emission band UE co-existence, MSD analysis, and ∆TIB and ∆RIB values.

-	For spurious emission band UE co-existence, no requirements for FR2 NR bands to protect E-UTRA and FR1 NR bands are applied to the constituent FR2 NR bands. Spurious emission band UE co-existence requirements specified in TS 36.101 are applied to the constituent E-UTRA bands for the EN-DC configuration.

-	For spurious emission band UE co-existence table, although the requirements are specified, it seems that they are not testable under the condition RAN4 originally expected. In addition, the impact of the harmonics and IMD issues between bands below 6GHz and bands above 24.25GHz inside UEs had been ignored. Hence at least the co-existence between bands below 6GHz and bands above 24.25GHz should not have any issues if the assumption that no impact of harmonic and IMD on co-existence between FR1 and FR2 is right.

-	MSD for E-UTRA and FR2 NR bands of inter-band DC combinations is set to N/A.

-	ΔTIB,c and ΔRIB,c for E-UTRA and FR2 NR bands of inter-band EN-DC combinations is set to zero.

6	DC of 1 LTE band (1DL/1UL) and 1 NR band: Specific Band Combination Part

6.1	Inter-band DC within FR1

6.1.1	DC_8_n7

6.1.1.1	Operating bands for DC

Table 6.1.1.1-1: DC band combination of 1 LTE band + 1 NR band

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_8_n7 | 8 | n7 | No |

6.1.1.2	Configuration for DC

Table 6.1.1.2-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_8A_n7A | DC_8A_n7A | 8A | n7A |

6.1.1.3	Maximum output power for DC

Table 6.1.1.3-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_8A_n7A | 23 | +2/-3 |

6.1.1.4	Spurious emission band UE co-existence for DC

Table 6.1.1.4-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_8A_n7A | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 10, 12, 13, 14, 17, 20, 22, 26, 27, 28, 29, 30, 31, 32, 33, 34, 40, 42, 43, 45, 50, 51, 52, 65, 66, 67, 68, 69, 72, 73, 74, 75, 76, 85, | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3, 7, 22, 41, 42, 43, 52 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 8 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 12 |
|  | Frequency range | 860 | - | 890 | -40 | 1 | 5, 12 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 12 |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180 kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 6: This requirement is applicable for any channel bandwidths within the range 2500 - 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 12: This requirement is applicable only for the following cases: A: for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 902.5 MHz ≤ Fc < 907.5 MHz with an uplink transmission bandwidth less than or equal to 20 RB; B: for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 907.5 MHz ≤ Fc ≤ 912.5 MHz without any restriction on uplink transmission bandwidth; C: for carriers of 10 MHz channel bandwidth when carrier centre frequency (Fc) is Fc = 910 MHz with an uplink transmission bandwidth less than or equal to 32 RB with RBstart > 3. |  |  |  |  |  |  |  |

6.1.1.5	MSD analysis for DC

For 2UL/2DL UE coexistence study 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.1.1.5-1.

Table 6.1.1.5-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2500 | 2570 | 880 | 915 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 5000 | 5140 | 1760 | 1830 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7500 | 7710 | 2640 | 2745 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 10000 | 10280 | 3520 | 3660 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 12500 | 12850 | 4400 | 4575 |
| Two tone 2nd order IMD products | ¦fy_low - fx_high¦ | ¦fy_high - fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1690 | 1585 | 3380 | 3485 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 4085 | 4260 | 810 | 670 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5880 | 6055 | 4260 | 4400 |
| Two-tone 4th order IMD products | ¦3*fx_low - fy_high¦ | ¦3*fx_high - fy_low¦ | ¦3*fy_low - fx_high¦ | ¦3*fy_high - fx_low¦ |
| IMD frequency limits (MHz) | 6585 | 6830 | 70 | 245 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8380 | 8625 | 5140 | 5315 |
| Two-tone 4th order IMD products | ¦2*fx_low - 2*fy_high¦ | ¦2*fx_high - 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3170 | 3380 | 6760 | 6970 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1160 | 950 | 9400 | 9085 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 6020 | 6230 | 10880 | 11195 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2255 | 2500 | 5950 | 5670 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7640 | 7885 | 9260 | 9540 |

Based on above table,

-	the 3rd order harmonic of Band 8 will fall into Band n7.

-	5th order IMD may fall into Rx frequencies of band n8

-	Regarding the MSD due to 5th order IMD, if n7 uplink is at it highest position (2565 MHz) and band 8 uplink at its lowest position (882.5 MHz) then center of IMD is at 965 MHz which is out of band. Naturally some part of the IMD hits also the downlink band of band 8 but given this is IMD5 we propose not to specify IMD at all.

Regarding the MSD due to harmonics of Band 8 fall into n7, due to similarity with band 7+ n8 it is proposed to re-use the MSD value due to 3rd harmonic.

Reference sensitivity exceptions for DC_8_n7 are proposed in Table 6.1.1.5-2 with uplink configuration specified in Table 6.1.1.5-3.

Table 6.1.1.5-2: MSD due to UL harmonic for EN-DC in NR FR1

| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | n78,9 | 10 | 7.6 | 6.2 | 5.3 | 4.3 | 3.2 | 2.3 | 1.3 |  |  |  |  |
| NOTE 8: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 9: The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.1.1.5-3: Uplink configuration for reference sensitivity exceptions due to UL harmonic interference for EN-DC in NR FR1

|  | E-UTRA or NR Band / Channel bandwidth of the high band |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| 8 | n7 | 8 | 16 | 25 | 25 | 25 | 25 | 25 | 25 |  |  |  |  |

6.1.1.6	∆TIB and ∆RIB values

For DC_8_n7, the TIB,c and RIB,c values are given in the tables below and are re-used from E-UTRA CA_7-8.

Table 6.1.1.6-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8_n7 | 8 | 0.6 |
|  | n7 | 0.3 |

Table 6.1.1.6-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8_n7 | 8 | 0.2 |
|  | n7 | 0 |

6.1.2	DC_66_n77, DC_66-66_n77 and DC_66-66-66_n77

6.1.2.1	Configuration for DC

Table 6.1.2.1-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_66A_n77A | DC_66A_n77A | YES |
| DC_66A-66A_n77A | DC_66A_n77A | YES |
| DC_66A-66A-66A_n77A | DC_66A_n77A | YES |

6.1.2.2	Maximum output power for DC

Table 6.1.2.2-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| EN-DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_66A_n77A | 23 | +2/-3 |

6.1.2.3	Spurious emission band UE co-existence for DC

Table 6.1.2.3-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| EN-DC CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_66A_n77A | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 26, 29, 30, 41, 65, 66, 70, 71 | FDL_low | - | FDL_high | -50 | 1 |  |

6.1.2.4	MSD analysis for DC

For study of 2UL/2DL, Table 6.1.2.4-1 lists up to 5th order harmonics and intermodulation products for DC_66A-n77A

Table 6.1.2.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1780 | 3300 | 4200 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3560 | 6600 | 8400 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5340 | 9900 | 12600 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4*fy_low | 4*fy_high |
| 4th harmonics frequency limits (MHz) | 6840 | 7120 | 13200 | 16800 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 8550 | 8900 | 16500 | 21000 |
| Two tone 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |
| IMD frequency limits (MHz) | 1520 | 2490 | 5010 | 5980 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | 2*fy_low – fx_high | 2*fy_high – fx_low |
| IMD frequency limits (MHz) | 780 | 260 | 4820 | 6690 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 6720 | 7760 | 8310 | 10180 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | 3*fy_low – fx_high | 3*fy_high – fx_low |
| IMD frequency limits (MHz) | 930 | 2040 | 8120 | 10890 |
| Two-tone 4th order IMD products | 3*fx_low + fy_low | 3*fx_high + fy_high | 3*fy_low + fx_low | 3*fy_high + fx_high |
| IMD frequency limits (MHz) | 8430 | 9540 | 11610 | 14380 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | 2*fx_low + 2*fy_low | 2*fx_high + 2*fy_high |
| IMD frequency limits (MHz) | 4980 | 3040 | 10020 | 11960 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 15090 | 11420 | 3820 | 2640 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14910 | 18580 | 10140 | 11320 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 9180 | 6340 | 1260 | 3270 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13320 | 16160 | 11730 | 13740 |

Table 6.1.2.4-2 lists up to 5th order harmonics mixing products for DC_66A_n77A

Table 6.1.2.4-2: Band 66 and Band n77 UL harmonic mixing products

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| 66 | 1710 | 1780 | 2110 | 2200 | 4220 | 4400 | 6330 | 6600 | 8440 | 8800 | 10550 | 8900 |
| n77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 | 16500 | 21000 |

In summary of study, it could be seen,

-	The 2nd harmonic from band 66 UL may fall into band n77 DL frequency range (Note 1)

-	No harmonic mixing products will fall into band n77 UL frequency range

-	The 2nd and 5th order IMD products generated from dual uplinks of band 66 and n77 may fall into band 66 Rx frequency range

-	The 4th and 5th order IMD products generated from dual uplink of band 66 and n77 may fail into band n77 frequency range, but there is no impact to TDD band.

The MSD should be considered to mitigate the impact of the interference for this combination.

Note-1:	There is no a 2nd harmonic issue in the DC_66A_n77A band combination once the U.S. C-band (3700-3980MHz) is applied.

A study of sensitivity degradation for CA_n66-n77 has been captured in [2], and the same degradation is allowed for UL harmonic product to DL. The reference sensitivity is specified in Table 6.1.2.4-3 with uplink configuration specified in Table 6.1.2.4-4.

Table 6.1.2.4-3: MSD due to UL harmonic issue for DC_66_n77 (Note 1)

|  | NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| 66 | n772, 4 |  | 23.9 | 22.1 | 20.9 | 19.8 | 19.0 | 17.9 | 16.8 | 16.0 | 15.3 | 14.8 | 14.3 | 13.8 |
|  | n773 |  | 1.1 | 0.8 | 0.3 | 0.1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| NOTE 2: The requirements should be verified for UL EARFCN or NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 3: The requirements are only applicable to channel bandwidths no larger than 20 MHz and with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 4: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.1.2.4-4 Uplink configuration due to UL harmonic interference

| NR Band / Channel bandwidth of the affected DL band / UL RB allocation of the aggressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 30 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 70 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| 66 | n77 |  | 25 | 36 | 50 | 64 | 80 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |

Sensitivity degradation is allowed for the identified IMDs in the configuration of DC_66A_n77A. The study of reference sensitivity exceptions captured in [2] can be applied in Table 6.1.2.4-5 for the dual connectivity configuration.

Table 6.1.2.4-5: MSD test points for PCell due to uplink operation for EN-DC in NR FR1

| Operating Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN CA CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_66A_n77A DC_66-66_n77A DC_66-66-66_n77A | 66 | 1775 | 5 | 25 | 2175 | 31.0 | FDD | IMD2 |
|  | n77 | 3950 | 10 | 50 | 3950 | N/A | TDD | N/A |
|  | 66 | 1730 | 5 | 25 | 2130 | 5.0 | FDD | IMD5 |
|  | n77 | 3660 | 10 | 50 | 3660 | N/A | TDD | N/A |

6.1.2.5	∆TIB and ∆RIB values

For DC_66_n77, the ΔTIB,c and ΔRIB,c values are given in the tables below.

Table 6.1.2.5-1: ΔTIB,c

| Inter-band DA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66_n77 DC_66-66_n77 DC_66-66-66_n77 | 66 | 0.6 |
|  | n77 | 0.8 |

Table 6.1.2.5-2: ΔRIB,c

| Inter-band DC Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_66_n77 DC_66-66_n77 DC_66-66-66_n77 | 66 | 0.2 |
|  | n77 | 0.5 |

6.1.2.6	Self-interference analysis

This section provides the formulation to determine whether the DL carriers would be impacted by UL harmonics, 2UL IMD products, or DL harmonic mixing with UL interferer based on UL and DL channel combinations. The potential self-interference mechanism has been identified in section 6.1.2.4 except UL harmonic order higher than 5 which are used to generate an interference mixing coefficient table for this DC combination, as shown in Table 6.1.2.6-1.

Table 6.1.2.6-1: Band 66 and Band n77 interference mixing coefficients

| EUTRA-NR DC Configuration | EUTRA/NR Band | UL Coefficient |  | DL Coefficient |  | Harmonic/IMD Order | Victim Band | Interference Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_66A_n77A | 66 | a | -1 | b | 1 | 2 | 66 | IMD |
|  | n77 | c | 1 | d | 0 |  |  |  |
|  | 66 | a | -3 | b | 1 | 5 | n77 | IMD |
|  | n77 | c | 2 | d |  |  |  |  |

Equations (1) and (2) below are used to calculate the interference centre frequency (fINT) and its effective bandwidth (BWINT) where coefficients a, b, c, and d are defined in Table 6.1.44.8-1 and CBW stands for channel bandwidth.

(1)

(2)

Formula (3-1) and (3-2) below are then used to indicate when the interference is overlapping with RX1 and RX2, respectively.

(3-1)

(3-2)

6.1.2.7	OOB blocking exception requirements

For the band combination CA_66-n77, the blocking at 5010MHz in band n77 filter is identified, and the existing filter cannot reach to 30dB rejection. A further study is needed.

6.1.3	DC_13_n77

6.1.3.1	Configuration for DC

Table 6.1.3.1-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration | Single UL allowed |
| --- | --- | --- |
| DC_13A_n77A | DC_13A_n77A | No |

6.1.3.2	Maximum output power for DC

Table 6.1.3.2-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| EN-DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_13A_n77A | 23 | +2/-3 |

6.1.3.3	Spurious emission band UE co-existence for DC

Table 6.1.3.3-1 lists the protected bands required for the 2UL bands DC configuration.

Table 6.1.3.3-1: Protected bands for the 2UL bands EN-DC configuration

| EN-DC CA combination | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected Band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_13_n77 | E-UTRA Band 2, 4, 5, 10, 12, 13, 17, 25, 26, 29, 41, 66, 70, 71 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 14 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 24, 30 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 | 5 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 5, 21 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180 kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 21: Whether the applicable frequency range should be 793 – 805 MHz instead of 799 – 805 MHz is TBD |  |  |  |  |  |  |  |

6.1.3.4	MSD analysis for DC

For study of 2UL/2DL, Table 6.1.3.4-1 lists up to 5th order harmonics and intermodulation products for DC_13A_n77A.

Table 6.1.3.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 777 | 787 | 3300 | 4200 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1554 | 1574 | 6600 | 8400 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2331 | 2361 | 9900 | 12600 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4*fy_low | 4*fy_high |
| 4th harmonics frequency limits (MHz) | 3108 | 3148 | 13200 | 16800 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3885 | 3935 | 16500 | 21000 |
| Two tone 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |
| IMD frequency limits (MHz) | 2513 | 3423 | 4077 | 4987 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | 2*fy_low – fx_high | 2*fy_high – fx_low |
| IMD frequency limits (MHz) | 2646 | 1726 | 5813 | 7623 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 4854 | 5774 | 7377 | 9187 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | 3*fy_low – fx_high | 3*fy_high – fx_low |
| IMD frequency limits (MHz) | 1869 | 939 | 9113 | 11823 |
| Two-tone 4th order IMD products | 3*fx_low + fy_low | 3*fx_high + fy_high | 3*fy_low + fx_low | 3*fy_high + fx_high |
| IMD frequency limits (MHz) | 5631 | 6561 | 10677 | 13387 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | 2*fx_low + 2*fy_low | 2*fx_high + 2*fy_high |
| IMD frequency limits (MHz) | 6846 | 5026 | 8154 | 9974 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 16023 | 12413 | 152 | 1092 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 13977 | 17587 | 6408 | 7348 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 11046 | 8326 | 4239 | 6069 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11454 | 14174 | 8931 | 10761 |

Table 6.1.3.4-2 lists up to 5th order harmonics mixing products for DC_13A_n77A

Table 6.1.3.4-2: Band 13 and Band n77 UL harmonic mixing products

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 13 | 777 | 787 | 746 | 756 | 1492 | 1512 | 2238 | 2268 | 2984 | 3024 | 3730 | 3935 |
| n77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 | 16500 | 21000 |

In summary of study, it could be seen,

-	The 5th harmonic interferences from band 13 UL may fall into band n77 DL frequency range

-	The 5th harmonic mixing products from band 13 may fall into band n77 UL frequency range

-	The 5th IMD products generated from dual uplinks of band 13 and n77 UL may fall into band 13 Rx frequency range

The MSD should be considered to mitigate the impact of the interference for this combination.

Sensitivity degradation is allowed for UL harmonic product interference to DL. The MSD range are specified in Table 6.1.3.4-3 with uplink configuration specified in Table 6.1.3.4-4.

Table 6.1.3.4-3: MSD due to UL harmonic issue for DC_13_n77

|  | NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| 13 | n774, 5 |  | 10.4 | 8.9 | 7.8 | 6.7 | 5.7 | 4.7 | 3.7 | 3 | 2.3 | 1.7 | 1.2 | 0.7 |
| NOTE 4: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 5th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 5: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.1.3.4-4 Uplink configuration due to UL harmonic interference

|  |  | EN-DC or NR Band / Channel bandwidth of the affected DL band / UL RB allocation of the aggressor band |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 30 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 70 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| 13 | n77 |  | 15 | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 |

Sensitivity degradation is allowed for the mixing harmonics in UL to the EN-DC configuration. The reference sensitivity exceptions are captured in Table 6.1.3.1.4-5 with the uplink configuration specified in Table 6.1.3.4-6.

Table 6.1.3.4-5:  MSD due to receiver harmonic mixing for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 100 MHz |
|  |  | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB |
| n77 | 132 | 31 | 28 |  |  |  |  |  |  |  |  |  |
| NOTE 2: The requirements should be verified for UL NR-ARFCN of the aggressor (high) band (superscript HB) such that in MHz and with carrier frequency in the victim (lower) band in MHz and the channel bandwidth configured in the higher band. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.1.3.4-6: Uplink configuration due to receiver harmonic mixing for EN-DC in NR FR1

| NR Band / Channel bandwidth of the affected DL band /UL RB allocation of the aggressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| n77 | 13 | 15 | 25 | 50 |  |  |  |  |  |  |  |  |  |

Sensitivity degradation is allowed for the identified IMDs in the configuration of DC_13A_n77A. The reference sensitivity exceptions are specified Table 6.1.3.4-7 for the dual connectivity configuration.

Table 6.1.3.4-7: MSD test points for PCell due to uplink operation for EN-DC in NR FR1

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC CA Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_13A_n77A | 13 | 784.5 | 5 | 20 | 753.5 | 5.5 | IMD5 |
|  | n77 | 3891.5 | 10 | 50 | 3891.5 | N/A | N/A |
| NOTE 4: Applicable only if operation with 4 antenna ports is supported in the band with EN-DC configured. |  |  |  |  |  |  |  |

6.1.3.5	∆TIB and ∆RIB values

For DC_13_n77, the ΔTIB,c and ΔRIB,c values are given in the tables below.

Table 6.1.3.5-1: ΔTIB,c

| Inter-band DA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_13_n77 | 13 | 0.5 |
|  | n77 | 0.8 |

Table 6.1.3.5-2: ΔRIB,c

| Inter-band DC Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_13_n77 | 13 | 0.2 |
|  | n77 | 0.5 |

6.1.3.6	Self-interference analysis

This section provides the formulation to determine whether the DL carriers would be impacted by UL harmonics, 2UL IMD products, or DL harmonic mixing with UL interferer based on UL and DL channel combinations. The potential self-interference mechanism has been identified in section 6.1.3.4 except UL harmonic order higher than 5 which are used to generate an interference mixing coefficient table for this DC combination, as shown in Table 6.1.3.6-1.

Table 6.1.3.6-1: Band 13 and Band n77 interference mixing coefficients

| EUTRA-NR DC Configuration | EUTRA/NR Band | UL Coefficient |  | DL Coefficient |  | Harmonic/IMD Order | Victim Band | Interference Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_13A_n77A | 13 | a | -4 | b | 1 | 5 | 13 | IMD |
|  | n77 | c | 1 | d | 0 |  |  |  |

Equations (1) and (2) below are used to calculate the interference centre frequency (fINT) and its effective bandwidth (BWINT) where coefficients a, b, c, and d are defined in Table 6.1.3.8-1 and CBW stands for channel bandwidth.

(1)

(2)

Formula (3-1) and (3-2) below are then used to indicate when the interference is overlapping with RX1 and RX2, respectively.

(3-1)

(3-2)

6.1.3.7	OOB blocking exception requirements

For the EN-DC band combination DC_13_n77, the lower frequency band UL carrier and the CW interfering signal may partially overlaps with the higher frequency band DL carrier. An exception requirement should be allowed.

Table 6.1.3.7-1: EN-DC band combination with exceptions allowed

EN-DC band combination

DC_13_n77

6.1.4	DC_5_n77

6.1.4.1	Configuration for DC

Table 6.1.4.1-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration | Single UL allowed |
| --- | --- | --- |
| DC_5A_n77A | DC_5A_n77A | No |

6.1.4.2	Maximum output power for DC

Table 6.1.4.2-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| EN-DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_5A_n77A | 23 | +2/-3 |

6.1.4.3	Spurious emission band UE co-existence for DC

Table 6.1.4.3-1 lists the protected bands required for the 2UL bands DC configuration.

Table 6.1.4.3-1: Protected bands for the 2UL bands EN-DC configuration

| EN-DC CA combination | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected Band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_5_n77 | E-UTRA Band 2, 4, 12, 13, 14, 17, 25, 26, 28, 29, 30, 65, 66, 70, 71 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180 kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz |  |  |  |  |  |  |  |

6.1.4.4	MSD analysis for DC

Note-1:	The results of MSD study in this section will be NOT applied to the DC_5A_n77A band combination once the U.S. C-band (3700-3980MHz) is applied. All of the identified harmonic and IMD products are out of the frequency ranges of this band combination restricted by the U.S. regulatory frequency range.

For study of 2UL/2DL, Table 6.1.4.4-1 lists up to 5th order harmonics and intermodulation products for DC_5A_n77A

Table 6.1.4.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 3300 | 4200 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 | 1698 | 6600 | 8400 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 | 2547 | 9900 | 12600 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4*fy_low | 4*fy_high |
| 4th harmonics frequency limits (MHz) | 3296 | 3396 | 13200 | 16800 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 4120 | 4245 | 16500 | 21000 |
| Two tone 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |
| IMD frequency limits (MHz) | 2451 | 3376 | 4124 | 5049 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | 2*fy_low – fx_high | 2*fy_high – fx_low |
| IMD frequency limits (MHz) | 2552 | 1602 | 5751 | 7576 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 4948 | 5898 | 7424 | 9249 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | 3*fy_low – fx_high | 3*fy_high – fx_low |
| IMD frequency limits (MHz) | 1728 | 753 | 9051 | 11776 |
| Two-tone 4th order IMD products | 3*fx_low + fy_low | 3*fx_high + fy_high | 3*fy_low + fx_low | 3*fy_high + fx_high |
| IMD frequency limits (MHz) | 5772 | 6747 | 10724 | 13449 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | 2*fx_low + 2*fy_low | 2*fx_high + 2*fy_high |
| IMD frequency limits (MHz) | 6752 | 4902 | 8248 | 10098 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 15976 | 12351 | 96 | 904 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14024 | 17649 | 6596 | 7596 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 10952 | 8202 | 4053 | 5928 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11548 | 14298 | 9072 | 10947 |

In summary of study, it could be seen,

-	The 4th and 5th harmonic interferences from dual uplink of band 5 and n77 may fall into band n77 Rx frequency range, and there is no impact to this TDD band.

-	The 2nd and 5th IMD products from dual uplink of band 5 and n77 may fall into n77 Rx frequency range, and there is no impact to this TDD band

-	The 4th and 5th IMD products generated from dual uplink from both band 5 and n77 may fall into band 5 Rx frequency range

-	No harmonic mixing product issue found

The MSD should be considered to mitigate the impact of the interference for this combination.

A study of sensitivity degradation for CA_n5-n77 has been captured in TR 38.716-02-00, and the same degradation is allowed for band 5 UL harmonic product for this DC configuration. The reference sensitivity is specified in Table 6.1.4.4-3 with the uplink configuration in Table 6.1.4.4-4.

Table 6.1.4.4-3: MSD due to UL harmonic issue for DC_5_n77 (Note 1)

|  | NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| 5 | n774, 5 |  | 10.5 | 8.9 | 7.8 | 7.2 | 6.5 | 5.1 | 4.2 | 3.5 | 2.8 | 2.3 | 2.1 | 1.4 |
|  | n776,7 |  | 10.4 | 8.9 | 7.8 | 7.4 | 6.5 | 4.7 | 3.7 | 3 | 2.35 | 1.7 | 1.2 | 0.7 |
| NOTE 4: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 5th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 5: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 6: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 4th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 7: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.1.4.4-4 Uplink configuration due to UL harmonic interference

|  |  | EN-DC or NR Band / Channel bandwidth of the affected DL band / UL RB allocation of the aggressor band |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 30 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 70 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| 5 | n77 | - | 16 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 |

Sensitivity degradation is allowed for the identified IMDs in the configuration of DC_5A_n77A. The values of reference sensitivity captured in TR 38.716-02-00 are applied in the Table 6.1.4.4-5 for the dual connectivity configuration.

Table 6.1.4.4-5: MSD test points for PCell due to uplink operation for EN-DC in NR FR1 (Note 1)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC CA Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) |  | UL LCRB |  | DL Fc (MHz) |  | MSD (dB) |  | IMD order |
| DC_5A_n77A | 5 | 844 |  | 5 |  | 25 |  | 889 |  | 8.3 | IMD4 |
|  | n77 | 3421 |  | 10 |  | 50 |  | 3421 |  | N/A | N/A |
|  | 5 | 826.5 | 5 |  | 25 |  | 871.5 |  | 5.5 |  | IMD5 |
|  | n77 | 4177.5 | 10 |  | 50 |  | 4177.5 |  | N/A |  | N/A |
| NOTE 4: Applicable only if operation with 4 antenna ports is supported in the band with EN-DC configured. |  |  |  |  |  |  |  |  |  |  |  |

6.1.4.5	∆TIB and ∆RIB values

For DC_5_n77, the ΔTIB,c and ΔRIB,c values are given in the tables below.

Table 6.1.4.5-1: ΔTIB,c

| Inter-band DA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5_n77 | 5 | 0.6 |
|  | n77 | 0.8 |

Table 6.1.4.5-2: ΔRIB,c

| Inter-band DC Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_5_n77 | 5 | 0.2 |
|  | n77 | 0.5 |

6.1.4.6	Self-interference analysis

This section provides the formulation to determine whether the DL carriers would be impacted by UL harmonics, 2UL IMD products, or DL harmonic mixing with UL interferer based on UL and DL channel combinations. The potential self-interference mechanism has been identified in section 6.1.4.4 except UL harmonic order higher than 5 which are used to generate an interference mixing coefficient table for this DC combination, as shown in Table 6.1.4.6-1.

Table 6.1.4.6-1: Band 5 and Band n77 interference mixing coefficients

| EUTRA-NR DC Configuration | EUTRA/NR Band | UL Coefficient |  | DL Coefficient |  | Harmonic/IMD Order | Victim Band | Interference Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_5A_n77A | 5 | a | 3 | b | -1 | 4 | 5 | IMD |
|  | n77 | c | -1 | d | 0 |  |  |  |
| DC_5A_n77A | 5 | a | -4 | b | 1 | 5 | 5 | IMD |
|  | n77 | c | 1 | d | 0 |  |  |  |

Equations (1) and (2) below are used to calculate the interference centre frequency (fINT) and its effective bandwidth (BWINT) where coefficients a, b, c, and d are defined in Table 6.1.4.8-1 and CBW stands for channel bandwidth.

(1)

(2)

Formula (3-1) and (3-2) below are then used to indicate when the interference is overlapping with RX1 and RX2, respectively.

(3-1)

(3-2)

6.1.4.7	OOB blocking exception requirements

For the EN-DC band combination DC_5_n77, the lower frequency band UL carrier and the CW interfering signal may partially overlaps with the higher frequency band DL carrier. An exception requirement should be allowed.

Table 6.1.4.7-1: EN-DC CA band combination with exceptions allowed

CA band combination

DC_5_n77

6.1.5	DC_2_n77 and DC_2-2_n77

6.1.5.1	Configuration for DC

Table 6.1.5.1-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration | Single UL allowed |
| --- | --- | --- |
| DC_2A_n77A | DC_2A_n77A | YES |
| DC_2A-2A_n77A | DC_2A_n77A | YES |

6.1.5.2	Maximum output power for DC

Table 6.1.5.2-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| EN-DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_2A_n77A | 23 | +2/-3 |

6.1.5.3	Spurious emission band UE co-existence for DC

Table 6.1.5.3-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| EN-DC CA combination | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected Band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_2A_n77A | E-UTRA Band 4, 5, 12, 13, 14, 17, 26, 29, 30, 41, 65, 66, 70, 71 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180 kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval |  |  |  |  |  |  |  |

6.1.5.4	MSD analysis for DC

For study of 2UL/2DL, Table 6.1.5.4-1 lists up to 5th order harmonics and intermodulation products for DC_2A_n77A.

Table 6.1.5.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 3300 | 4200 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3700 | 3820 | 6600 | 8400 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5550 | 5730 | 9900 | 12600 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4*fy_low | 4*fy_high |
| 4th harmonics frequency limits (MHz) | 7400 | 7640 | 13200 | 16800 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 9250 | 9550 | 16500 | 21000 |
| Two tone 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |
| IMD frequency limits (MHz) | 1390 | 2350 | 5150 | 6110 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | 2*fy_low – fx_high | 2*fy_high – fx_low |
| IMD frequency limits (MHz) | 500 | 520 | 4690 | 6550 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 7000 | 8020 | 8450 | 10310 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | 3*fy_low – fx_high | 3*fy_high – fx_low |
| IMD frequency limits (MHz) | 1350 | 2430 | 7990 | 10750 |
| Two-tone 4th order IMD products | 3*fx_low + fy_low | 3*fx_high + fy_high | 3*fy_low + fx_low | 3*fy_high + fx_high |
| IMD frequency limits (MHz) | 8850 | 9930 | 11750 | 14510 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | 2*fx_low + 2*fy_low | 2*fx_high + 2*fy_high |
| IMD frequency limits (MHz) | 4700 | 2780 | 10300 | 12220 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14950 | 11290 | 4340 | 3200 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15050 | 18710 | 10700 | 11840 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 8900 | 6080 | 870 | 2850 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13600 | 16420 | 12150 | 14130 |

Table 6.1.5.4-2 lists up to 5th order harmonics mixing products for DC_2A_n77A

Table 6.1.5.4-2: Band 2 and Band n77 UL harmonic mixing products

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| 2 | 1850 | 1910 | 1930 | 1990 | 3860 | 3980 | 5790 | 5970 | 7720 | 7960 | 9650 | 9550 |
| n77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 | 16500 | 21000 |

In summary of study, it could be seen,

-	The 2nd harmonic interference from band 2 UL may fall into band n77 DL frequency range.

-	The 2nd harmonic mixing products from band 2 may fall into band n77 UL frequency range.

-	The 2nd, 4th and 5th order IMD products generated by dual uplink from both band 2 and band n77 may fall into band 2 Rx frequency range, but there is no IMD impact to n77

The MSD should be considered to mitigate the impact of the interference for this combination.

A study of sensitivity degradation for CA_n2-n77 has been captured in TR 38.716-02-00, and the same degradation is allowed for the 2nd harmonic products in UL to DL. The reference sensitivity is specified in Table 6.1.5.4-3 with the uplink configuration in Table 6.1.5.4-4.

Table 6.1.5.4-3: MSD due to harmonic issue for DC_2A_n77A

| MSD due to harmonic exception for the DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) |  | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| 2 | n772, 4 |  | 23.9 | 22.1 | 20.9 | 19.8 | 19.0 | 17.9 | 16.8 | 16.0 | 15.5 | 14.8 |  | 14.3 | 13.8 |
|  | n773 |  | 1.1 | 0.8 | 0.3 | 0.1 | 0 | 0 | 0 | 0 | 0 | 0 |  | 0 | 0 |
| NOTE 2: The requirements should be verified for UL EARFCN or NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 3: The requirements are only applicable to channel bandwidths no larger than 20 MHz and with a carrier frequency at MHz offset from in the victim (higher band) with , where andare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 4: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.1.5.4-4 Uplink configuration due to UL harmonic interference

|  |  | NR Band / Channel bandwidth of the affected DL band / UL RB allocation of the aggressor band |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 30 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 70 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| 2 | n77 |  | 25 | 36 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 |

Sensitivity degradation is allowed for the mixing harmonic products in UL to the DC configuration. The study of reference sensitivity exceptions captured in TR 38.716-02-00 can be applied in Table 6.1.5.4-5 with the uplink configuration specified in Table 6.1.5.4-6 for the harmonic mixing of band 2 in the combo DC_2A_n77A.

Table 6.1.5.4-5: MSD due to receiver harmonic mixing for EN-DC in NR FR1

| NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n77 | 2 | 6.1 | 5.0 | 4.0 | 3.7 |  |  |  |  |  |  |  |  |

Table 6.1.5.4-6: Uplink configuration due to receiver harmonic mixing for EN-DC in NR FR1

| NR Band / SCS / Channel bandwidth of the affected DL band / UL RB allocation of the aggressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 70 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| n77 | 2 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |  |

Sensitivity degradation is allowed for the identified IMDs in the configuration of DC_2A_n77A. The values for reference sensitivity exceptions captured in TR 38.716-02-00 are applied in the Table 6.1.5.4-7 for the dual connectivity configuration

Table 6.1.5.4-7: MSD due to IMD issue

|  | Operating band/ Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC CA Configuration | Operating band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| DC_2A_n77A DC_2A-2A_n77A | 2 | 1855 | 5 | 25 | 1935 | 26 | FDD | IMD2 |
|  |  |  |  |  |  | 28.75 |  |  |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
|  | 2 | 1885 | 5 | 25 | 1965 | 8.0 | FDD | IMD4 |
|  |  |  |  |  |  | 10.75 |  |  |
|  | n77 | 3690 | 10 | 50 | 3690 | N/A | TDD | N/A |
|  | 2 | 1885 | 5 | 25 | 1965 | 5 | FDD | IMD5 |
|  |  |  |  |  |  | 7.75 |  |  |
|  | n77 | 3810 | 10 | 50 | 3810 | N/A | TDD | N/A |
| NOTE 5: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |

6.1.5.5	∆TIB and ∆RIB values

For DC_2_n77, the ΔTIB,c and ΔRIB,c values are given in the tables below.

Table 6.1.5.5-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2_n77 DC_2-2_n77 | 2 | 0.6 |
|  | n77 | 0.8 |

Table 6.1.5.5-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2_n77 DC_2-2_n77 | 2 | 0.2 |
|  | n77 | 0.5 |

6.1.5.6	Self-interference analysis

This section provides the formulation to determine whether the DL carriers would be impacted by UL harmonics, 2UL IMD products, or DL harmonic mixing with UL interferer based on UL and DL channel combinations. The potential self-interference mechanism has been identified in section 6.1.5.4 except UL harmonic order higher than 5 which are used to generate an interference mixing coefficient table for this DC combination, as shown in Table 6.1.5.6-1.

Table 6.1.5.6-1: Band 2 and Band n77 interference mixing coefficients

| EUTRA-NR DC Configuration | EUTRA/NR Band | UL Coefficient |  | DL Coefficient |  | Harmonic/IMD Order | Victim Band | Interference Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_2A_n77A | 2 | a | -1 | b | 1 | 2 | 2 | IMD |
|  | n77 | c | 1 | d | 0 |  |  |  |
| DC_2A_n77A | 2 | a | 3 | b | 1 | 4 | 2 | IMD |
|  | n77 | c | -1 | d | 0 |  |  |  |
| DC_2A_n77A | 2 | a | -3 | b | 1 | 5 | 2 | IMD |
|  | n77 | c | 2 | d | 0 |  |  |  |

Equations (1) and (2) below are used to calculate the interference centre frequency (fINT) and its effective bandwidth (BWINT) where coefficients a, b, c, and d are defined in Table 6.1.44.8-1 and CBW stands for channel bandwidth.

(1)

(2)

Formula (3-1) and (3-2) below are then used to indicate when the interference is overlapping with RX1 and RX2, respectively.

(3-1)

(3-2)

6.1.5.7	OOB blocking exception requirements

For the band combination CA_2-n77, the blocking at 5150MHz in the band n77 filter is identified, and the existing filter cannot reach to 30dB rejection. A further study is needed.

6.1.6	DC_19_n1

6.1.6.1	Configuration for DC

Table 6.1.6.1-1: Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration | Single UL allowed |
| --- | --- | --- |
| DC_19A_n1A | DC_19A_n1A | No |

6.1.6.2	Maximum output power for DC

Table 6.1.6.2-1: Maximum output power for inter-band EN-DC (two bands)

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_19A_n1A | 23 | +2/-3 |

6.1.6.3	Spurious emission band UE co-existence for DC

Table 6.1.6.3-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_19_n1 | E-UTRA Band 1, 11, 21, 28, 42, 65, 74 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 5, 16 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 16 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 16 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180 kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 16: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. |  |  |  |  |  |  |  |

6.1.6.4	MSD analysis for DC

For 2UL/2DL UE coexistence study 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.1.6.4-1.

Table 6.1.6.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 830 | 845 | 1920 | 1980 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1660 | 1690 | 3840 | 3960 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2490 | 2535 | 5760 | 5940 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3320 | 3380 | 7680 | 7920 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 4150 | 4225 | 9600 | 9900 |
| 6th harmonics frequency limits | 6*fx_low | 6*fx_high | 6* fy_low | 6* fy_high |
| 6th harmonics frequency limits (MHz) | 4980 | 5070 | 11520 | 11880 |
| 7th harmonics frequency limits | 7*fx_low | 7*fx_high | 7* fy_low | 7* fy_high |
| 7th harmonics frequency limits (MHz) | 5810 | 5915 | 13440 | 13860 |
| Two tone 2nd order IMD products | ¦fx_low – fy_high¦ | ¦fx_high – fy_low¦ | ¦fx_low + fy_low¦ | ¦fx_high + fy_high¦ |
| IMD frequency limits (MHz) | 1150 | 1075 | 2750 | 2825 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 320 | 230 | 2995 | 3130 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3580 | 3670 | 4670 | 4805 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 510 | 615 | 4915 | 5110 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 2300 | 2150 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4410 | 4515 | 6590 | 6785 |
| Two-tone 4th order IMD products | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |  |  |
| IMD frequency limits (MHz) | 5500 | 5650 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7090 | 6835 | 1460 | 1340 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4280 | 4070 | 1305 | 1470 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8510 | 8765 | 5240 | 5360 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7420 | 7630 | 6330 | 6495 |

Based on Table 6.1.6.4-1, it can be seen that

-	2nd order harmonic may fall into Rx frequencies of band n77.

-	3rd order harmonic may fall into Rx frequencies of band 41, 46, 47, 53 and n90.

-	4rh order harmonic may fall into Rx frequencies of band 52, n77 and n78.

-	5rh order harmonic may fall into Rx frequencies of band n77.

-	3rd order IMD products may fall into Rx frequencies of bands 22, 42, 43, 48, 49, n77, n78 and n79.

-	4rh order IMD products may fall into Rx frequencies of bands 1, 4, 10, 23, 46, 65, 66 and n79.

-	5rh order IMD products may fall into Rx frequencies of bands 32, 45, 46, 50, 51, 75, 76, n77, n91, n92, n93 and n94.

Based on above,

-	the 4th order IMD may fall into Rx frequencies of band n1.

Based on the similarities with the existing LTE combination CA_1A-19A, there is no need to have MSD added.

6.1.6.5	∆TIB and ∆RIB values

For DC_19_n1, the TIB,c and RIB,c values are reused from the LTE combination CA_1-19, and are given in the tables below.

Table 6.1.6.5-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_19_n1 | 19 | 0.3 |
|  | n1 | 0.3 |

Table 6.1.6.5-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_19_n1 | 19 | 0 |
|  | n1 | 0 |

6.1.6.6		Self-interference analysis

This section is omitted because there is no need to have MSD added.

6.1.7	DC_21_n1

6.1.7.1	Configuration for DC

Table 6.1.7.1-1: Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration | Single UL allowed |
| --- | --- | --- |
| DC_21A_n1A | DC_21A_n1A | No |

6.1.7.2	Maximum output power for DC

Table 6.1.7.2-1: Maximum output power for inter-band EN-DC (two bands)

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_21A_n1A | 23 | +2/-3 |

6.1.7.3	Spurious emission band UE co-existence for DC

Table 6.1.7.3-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_21_n1 | E-UTRA Band 1, 18, 19, 28, 42, 65 NR Band n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 5, 16 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 16 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 16 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180 kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 16: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. |  |  |  |  |  |  |  |

6.1.7.4	MSD analysis for DC

For 2UL/2DL UE coexistence study 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.1.7.4-1.

Table 6.1.7.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1447.9 | 1462.9 | 1920 | 1980 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 2895.8 | 2925.8 | 3840 | 3960 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 4343.7 | 4388.7 | 5760 | 5940 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 5791.6 | 5851.6 | 7680 | 7920 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 7239.5 | 7314.5 | 9600 | 9900 |
| 6th harmonics frequency limits | 6*fx_low | 6*fx_high | 6* fy_low | 6* fy_high |
| 6th harmonics frequency limits (MHz) | 8687.4 | 8777.4 | 11520 | 11880 |
| 7th harmonics frequency limits | 7*fx_low | 7*fx_high | 7* fy_low | 7* fy_high |
| 7th harmonics frequency limits (MHz) | 10135.3 | 10240.3 | 13440 | 13860 |
| Two tone 2nd order IMD products | ¦fx_low – fy_high¦ | ¦fx_high – fy_low¦ | ¦fx_low + fy_low¦ | ¦fx_high + fy_high¦ |
| IMD frequency limits (MHz) | 532.1 | 457.1 | 3367.9 | 3442.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 915.8 | 1005.8 | 2377.1 | 2512.1 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4815.8 | 4905.8 | 5287.9 | 5422.9 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2363.7 | 2468.7 | 4297.1 | 4492.1 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 1064.2 | 914.2 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6263.7 | 6368.7 | 7207.9 | 7402.9 |
| Two-tone 4th order IMD products | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |  |  |
| IMD frequency limits (MHz) | 6735.8 | 6885.8 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6472.1 | 6217.1 | 3931.6 | 3811.6 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3044.2 | 2834.2 | 548.7 | 383.7 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9127.9 | 9382.9 | 7711.6 | 7831.6 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8655.8 | 8865.8 | 8183.7 | 8348.7 |

Based on Table 6.1.7.4-1, it can be seen that

-	2nd order harmonic may fall into Rx frequencies of band n77.

-	3rd order harmonic may fall into Rx frequencies of band 46 and 47.

-	4rh order harmonic may fall into Rx frequencies of band 46.

-	2nd order IMD products may fall into Rx frequencies of bands 31, 42, 52, 72, 73, n77 and n78.

-	3rd order IMD products may fall into Rx frequencies of bands 8, 40, 41, 46, 53, n79 and n90.

-	4rh order IMD products may fall into Rx frequencies of bands 8, 40 and n79.

-	5rh order IMD products may fall into Rx frequencies of bands 31, 72, 73, 87, 88 and n77.

Based on Table 6.1.7.4-1, these is no IMD issue for this configuration.

6.1.7.5	∆TIB and ∆RIB values

For DC_21_n1, the TIB,c and RIB,c values are reused from the LTE combination CA_1-21, and are given in the tables below.

Table 6.1.7.5-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_21_n1 | 21 | 0.3 |
|  | n1 | 0.3 |

Table 6.1.7.5-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_21_n1 | 21 | 0 |
|  | n1 | 0 |

6.1.7.6	Self-interference analysis

This section is omitted because there is no need to have MSD added.

6.1.8	DC_42_n1

6.1.8.1	Configuration for DC

Table 6.1.8.1-1: Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration | Single UL allowed |
| --- | --- | --- |
| DC_42A_n1A DC_42C_n1A | DC_42A_n1A | No |

6.1.8.2	Maximum output power for DC

Table 6.1.8.2-1: Maximum output power for inter-band EN-DC (two bands)

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_42A_n1A | 23 | +2/-3 |

6.1.8.3	Spurious emission band UE co-existence for DC

Table 6.1.8.3-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_42_n1 | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 38, 40, 41, 44, 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76, NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 5, 16 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 16 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 16 |
| NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 16: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. |  |  |  |  |  |  |  |

6.1.8.4	MSD analysis for DC

For 2UL/2DL UE coexistence study 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.1.8.4-1.

Table 6.1.8.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 3400 | 3600 | 1920 | 1980 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 6800 | 7200 | 3840 | 3960 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 10200 | 10800 | 5760 | 5940 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 13600 | 14400 | 7680 | 7920 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 17000 | 18000 | 9600 | 9900 |
| 6th harmonics frequency limits | 6*fx_low | 6*fx_high | 6* fy_low | 6* fy_high |
| 6th harmonics frequency limits (MHz) | 20400 | 21600 | 11520 | 11880 |
| 7th harmonics frequency limits | 7*fx_low | 7*fx_high | 7* fy_low | 7* fy_high |
| 7th harmonics frequency limits (MHz) | 23800 | 25200 | 13440 | 13860 |
| Two tone 2nd order IMD products | ¦fx_low – fy_high¦ | ¦fx_high – fy_low¦ | ¦fx_low + fy_low¦ | ¦fx_high + fy_high¦ |
| IMD frequency limits (MHz) | 1420 | 1680 | 5320 | 5580 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 4820 | 5280 | 240 | 560 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8720 | 9180 | 7240 | 7560 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 8220 | 8880 | 2160 | 2540 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 2840 | 3360 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 12120 | 12780 | 9160 | 9540 |
| Two-tone 4th order IMD products | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |  |  |
| IMD frequency limits (MHz) | 10640 | 11160 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 4520 | 4080 | 12480 | 11620 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 860 | 1440 | 6960 | 6240 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11080 | 11520 | 15520 | 16380 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 12560 | 13140 | 14040 | 14760 |

Based on Table 6.1.8.4-1, it can be seen that

-	2nd order harmonic may fall into Rx frequencies of band n77.

-	3rd order harmonic may fall into Rx frequencies of band 46 and 47.

-	2nd order IMD products may fall into Rx frequencies of bands 11, 21, 24, 32, 45, 46, 50, 51, 74, 75, 76, n91, n92, n93 and n94.

-	3rd order IMD products may fall into Rx frequencies of bands 31, 46, 72, 73, 87, 88 and n79.

-	4rh order IMD products may fall into Rx frequencies of bands 1, 10, 23, 30, 40, 41, 52, 53, 65, 66, n77, n78 and n90.

-	5rh order IMD products may fall into Rx frequencies of bands 5, 6, 8, 18, 19, 26, 27, 50, 51, 75, 76, n77, n79, n91, n92, n93 and n94.

Based on above,

-	the 4th order IMD may fall into Rx frequencies of band n1.

Based on the similarities with the existing LTE combination CA_1A-42A, there is no need to have MSD added.

6.1.8.5	∆TIB and ∆RIB values

For DC_42_n1, the TIB,c and RIB,c values are reused from the LTE combination CA_1-42, and are given in the tables below.

Table 6.1.8.5-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_42_n1 | 42 | 0.8 |
|  | n1 | 0.3 |

Table 6.1.8.5-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_42_n1 | 42 | 0.5 |
|  | n1 | 0 |

6.1.8.6	Self-interference analysis

This section is omitted because there is no need to have MSD added.

6.1.9	DC_48_n25

6.1.9.1	Configuration for DC

Table 6.1.9.1-1:  Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration | Single UL allowed |
| --- | --- | --- |
| DC_48A_n25A DC_48C_n25A DC_48D_n25A | DC_48A_n25A | No |

6.1.9.2	Maximum output power for DC

The maximum output power for the uplink EN-DC configurations is given as.

Table 6.1.9.2-1: Maximum output power for inter-band EN-DC (two bands)

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_48A_n25A | 23 | +2/-3 |

6.1.9.3	Spurious emission band UE co-existence for DC

The requirements for the EN-DC configuration for DC_48_n25, to be specified in TS 38.101-3 Table 6.5B.3.3.2-1 is given below.

Table 6.1.9.3-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_48_n25 | E-UTRA Band 4, 5, 12, 13 , 14, 17, 24, 26, 29, 30, 41, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 15 |
|  | Frequency range | 1880 |  | 1895 | -40 | 1 | 5, 16 |
|  | Frequency range | 1895 |  | 1915 | -15.5 | 5 | 5, 7, 16 |
|  | Frequency range | 1915 |  | 1920 | +1.6 | 5 | 5, 7, 16 |
| NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 15: Applicable when NS_05 in clause 6.6.3.3.1 is signalled by the network. NOTE 16: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. |  |  |  |  |  |  |  |

6.1.9.4	MSD analysis for DC

For 2UL/2DL UE coexistence study 2nd, 3rd, 4th, 5th, 6th and 7th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented below.

Table 6.1.9.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 3550 | 3700 | 1850 | 1915 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 7100 | 7400 | 3700 | 3830 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 10650 | 11100 | 5550 | 5745 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 14200 | 14800 | 7400 | 7660 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 17750 | 18500 | 9250 | 9575 |
| 6th harmonics frequency limits | 6*fx_low | 6*fx_high | 6* fy_low | 6* fy_high |
| 6th harmonics frequency limits (MHz) | 21300 | 22200 | 11100 | 11490 |
| 7th harmonics frequency limits | 7*fx_low | 7*fx_high | 7* fy_low | 7* fy_high |
| 7th harmonics frequency limits (MHz) | 24850 | 25900 | 12950 | 13405 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1850 | 1635 | 5400 | 5615 |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 5185 | 5550 | 0 | 280 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8950 | 9315 | 7250 | 7530 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 8735 | 9250 | 1850 | 2195 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3270 | 3700 | 10800 | 11230 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 12500 | 13015 | 9100 | 9445 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 4110 | 3700 | 12950 | 12285 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1355 | 1850 | 7400 | 6820 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10950 | 11360 | 16050 | 16715 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 12650 | 13145 | 14350 | 14930 |

Based on Table 6.1.9.4-1, it can be seen that

-	2nd harmonics might fall into Rx frequencies of band 43.

-	3rd harmonics might fall into Rx frequencies of band 46.

-	2nd order IMD products may fall into Rx frequencies of band 3 and 9.

-	3rd order IMD products may fall into Rx frequencies of band 46.

-	4th order IMD products may fall into Rx frequencies of band 1, 2, 3, 4, 9, 10, 22, 23, 25, 33, 34, 35, 36, 37, 39, 42, 43, 48, 49, 52, 70, n77 and n78.

-	5th order IMD products may fall into Rx frequencies of bands 3, 9, 11, 21, 24, 32, 43, 45, 50, 51, 74, 75, 76, n77, n78, n91, n92 and n93.

From above it can be concluded that 4th order IMD can fall in the Rx frequency of both band 48 and n25. However, band 48 is a TDD band and there is no simultaneous Tx/Rx transmission. Therefore, there should not be any impact from dual uplink transmission to band 48. For band 48 it can be noted that the 2nd HAM falls at 3700 which is the upper limit of band 48. This means MSD is defined similar as for DC_2A_n48A in 38.101-3.

When 2UL inter-band EN-DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. A summary of this is given below.

Table 6.1.9.4-2: Harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | YES | US | 3rd IMD/HAM |
|  | 5150 | - | 5350 | YES | Europe | 3rd IMD/HAM |
|  | 5470 | - | 5725 | YES |  | 3rd IMD/HAM |
|  | 5150 | - | 5825 | YES | Asia | 3rd IMD/HAM |
| 45GHz Unlicensed Bands | 42300 | - | 47000 | No | China |  |
|  | 47200 | - | 48400 | No | China |  |
| 60GHz Unlicensed Bands | 57000 | - | 66000 | No | Europe |  |
|  | 57050 | - | 64000 | No | USA Canada |  |
|  | 57000 | - | 64000 | No | South Korea |  |
|  | 59000 | - | 66000 | No | Japan |  |
|  | 59000 | - | 64000 | No | China |  |
|  | 59400 | - | 62900 | No | Australia |  |

6.1.9.5	∆TIB and ∆RIB values

For DC_48_n25, the TIB,c and RIB,c values are reused from CA_2A-48A as specified in TS 36.101 due to similarity in frequency range.

Table 6.1.9.5-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_48_n25 | 48 | 0.8 |
|  | n25 | 0.6 |

Table 6.1.9.5-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_48_n25 | 48 | 0.5 |
|  | n25 | 0.2 |

6.1.9.6	Self-interference analysis

Based on the co-existence studies for DC_48_n25 it is found that MSD it needed defined due to 4th order IMD that can fall in the Rx frequency of band n25. MSD have therefor been defined based on DC_2A_n48A, already defined in 38.101-3, due to similarity in frequency range.

Table 6.1.9.6-1: MSD test points for PCell due to dual uplink operation for EN-DC in NR FR1 (two bands)

| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC_48A_n25A DC_48C_n25A DC_48D_n25A | 48 | 3625 | 20 | 100 | 3625 | N/A | N/A |
|  | n25 | 1852.5 | 5 | 25 | 1932.5 | 12 | IMD4 |

Similar Reference sensitivity exceptions (MSD) due to UL harmonic for EN-DC in NR FR1 is defined based on DC_2A_n48A, already defined in 38.101-3.

Table 6.1.9.6-2: Reference sensitivity exceptions (MSD) due to UL harmonic for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n25 | 482,13 | 27.3 | 24.4 | 22.4 | 21.2 |  |  |  |  |  |  |  |  |
|  | 483 | 1.9 | 1.4 | 0.9 | 0.4 |  |  |  |  |  |  |  |  |
| NOTE 2: The requirements should be verified for UL EARFCN or NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 3: The requirements are only applicable to channel bandwidths no larger than 20 MHz and with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 13: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range ∆FHD above and below the edge of this downlink transmission bandwidth. The value ∆FHD depends on the EN-DC band combination: ∆FHD = 10 MHz for DC_1_n77, DC_2_n48, DC_2_n77, DC_48_n25, DC_48_n66, DC_66_n48, DC_66_n77, DC_3_n77, DC_3_n78, DC_11_n28 and DC_28_n50 |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.1.10	DC_28_n2 and DC_2_n28

6.1.10.1	Configuration for DC

Table 6.1.10.1-1:  Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration | Single UL allowed |
| --- | --- | --- |
| DC_28A_n2A | DC_28A_n2A | No |
| DC_2A_n28A | DC_2A_n28A | No |

6.1.10.2	Maximum output power for DC

The maximum output power for the uplink EN-DC configurations is given as.

Table 6.1.10.2-1: Maximum output power for inter-band EN-DC (two bands)

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_28A_n2A | 23 | +2/-3 |
| DC_2A_n28A | 23 | +2/-3 |

6.1.10.3	Spurious emission band UE co-existence for DC

The requirements to be specified in TS 38.101-3 Table 6.5B.3.3.2-1 is given below.

Table 6.1.10.3-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_2_n28 DC_28_n2 | E-UTRA Band 5, 26, 27, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 10, 42, 43, 50, 51, 66, 74 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |
|  | E-UTRA Band 1, 65 | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 14 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 2570 | - | 2575 | 1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180 kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 6: This requirement is applicable for any channel bandwidths within the range 2500 - 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 9: Applicable when the assigned E-UTRA carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 10: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 11: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 14: This requirement is applicable for 5 and 10 MHz E-UTRA channel bandwidth allocated within 718-728MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and RBstart < 48. NOTE 15: Applicable when NS_05 in clause 6.6.3.3.1 is signalled by the network. |  |  |  |  |  |  |  |

6.1.10.4	MSD analysis for DC

For 2UL/2DL UE coexistence study 2nd, 3rd, 4th, 5th, 6th and 7th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented below.

Table 6.1.10.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 1850 | 1910 | 703 | 748 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3700 | 3820 | 1406 | 1496 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5550 | 5730 | 2109 | 2244 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 7400 | 7640 | 2812 | 2992 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 9250 | 9550 | 3515 | 3740 |
| 6th harmonics frequency limits | 6*fx_low | 6*fx_high | 6* fy_low | 6* fy_high |
| 6th harmonics frequency limits (MHz) | 11100 | 11460 | 4218 | 4488 |
| 7th harmonics frequency limits | 7*fx_low | 7*fx_high | 7* fy_low | 7* fy_high |
| 7th harmonics frequency limits (MHz) | 12950 | 13370 | 4921 | 5236 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1207 | 1102 | 2553 | 2658 |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2952 | 3117 | 504 | 354 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4403 | 4568 | 3256 | 3406 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4802 | 5027 | 199 | 394 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2204 | 2414 | 5106 | 5316 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6253 | 6478 | 3959 | 4154 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1142 | 902 | 6937 | 6652 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1456 | 1711 | 4324 | 4054 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4662 | 4902 | 8103 | 8388 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5809 | 6064 | 6956 | 7226 |

Based on Table 6.1.10.4-1, it can be seen that:

-	there are no harmonics or IMD products fall into own Rx frequencies

-	2rd order harmonic may fall into Rx frequencies of bands 6, 11, 21, 32, 43, 45, n74, n77 and n78

-	3rd order harmonic may fall into Rx frequencies of bands 1, 4, 10, 23, 46, 65 and 66

-	2nd order IMD products may fall into Rx frequencies of bands 7, 38, 41 and 69

-	3rd order IMD products may fall into Rx frequencies of bands 6, 12, 13, 14, 17, 20, 28, 29. 31, 44, 67, 68, 71, n77 and n78

-	4th order IMD products may fall into Rx frequencies of bands 6, 30, 40, 42 and 46

-	5th order IMD products may fall into Rx frequencies of band 6, 8, 11, 21, 24, 32, 42, 45, n74, n75, n77 and n79.

When 2UL inter-band EN-DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. A summary of this is given below.

Table 6.1.10.4-2: Harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | YES |  | 5th IMD |
| Galileo | 1559 | - | 1591 | YES |  | 5th IMD |
| GLONASS | 1591 | - | 1610 | YES |  | 5th IMD |
| GPS | 1563 | - | 1587 | YES |  | 5th IMD |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | YES | US/Europe | 4th IMD |
|  | 2400 | - | 2494 | YES | Asia | 4th IMD |
| ISM band (5GHz) | 5150 | - | 5925 | YES | US | 4th IMD |
|  | 5150 | - | 5350 | YES | Europe | 4th IMD |
|  | 5470 | - | 5725 | NO |  |  |
|  | 5150 | - | 5825 | YES | Asia | 4th IMD |
| 45GHz Unlicensed Bands | 42300 | - | 47000 | No | China |  |
|  | 47200 | - | 48400 | No | China |  |
| 60GHz Unlicensed Bands | 57000 | - | 66000 | No | Europe |  |
|  | 57050 | - | 64000 | No | USA Canada |  |
|  | 57000 | - | 64000 | No | South Korea |  |
|  | 59000 | - | 66000 | No | Japan |  |
|  | 59000 | - | 64000 | No | China |  |
|  | 59400 | - | 62900 | No | Australia |  |

6.1.10.5	∆TIB and ∆RIB values

The TIB,c and RIB,c values are reused from CA_2A-28A as specified in TS 36.101.

Table 6.1.10.5-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2_n28 | 2 | 0.3 |
|  | n28 | 0.3 |
| DC_28_n2 | 28 | 0.3 |
|  | n2 | 0.3 |

Table 6.1.10.5-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2_n28 | 2 | 0 |
|  | n28 | 0 |
| DC_28_n2 | 28 | 0 |
|  | n2 | 0 |

6.1.10.6	Self-interference analysis

Based on the co-existence studies no further MSD is needed.

6.1.11	DC_4_n2

6.1.11.1	Configuration for DC

Table 6.1.11.1-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_4A_n2A | DC_4A_n2A | No |

6.1.11.2	Maximum output power for DC

Table 6.1.11.2-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_4A_n2A | 23 | +2/-3 |

6.1.11.3	Spurious emission band UE co-existence for DC

Table 6.1.11.3-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_4_n2 | E-UTRA Band 4, 5, 10, 12, 13, 14, 17, 22, 24, 26, 27, 28, 29, 30, 41, 50, 51, 53, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 42, 43, NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180 kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

6.1.11.4	MSD analysis for DC

For DC_4A_n2A, it shares the same MSD issues as CA_2A-4A. The MSD value of CA_2A-4A is reused for DC_4A_n2A.

Table 6.1.11.4-1: MSD due to dual uplink operation for EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Source of IMD |
| DC_4A_n2A | 2 | 1860 | 20 | 502 | 1940 | 5 | IMD3 |
|  | 4 | 1752.5 | 5 | 25 | 2152.5 | N/A | N/A |
| DC_4A_n2A | 2 | 1868.3 | 5 | 25 | 1948.3 | N/A | N/A |
|  | 4 | 1735 | 5 | 25 | 2135 | 5 | IMD5 |
| NOTE 2: RBstart = 0 |  |  |  |  |  |  |  |

6.1.11.5	∆TIB and ∆RIB values

For DC_2_n4, the ΔTIB,c and ΔRIB,c values are given in the tables below.

Table 6.1.11.5-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_4_n2 | 4 | 0.5 |
|  | n2 | 0.5 |

Table 6.1.11.5-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_4_n2 | 4 | 0.3 |
|  | n2 | 0.3 |

6.1.12	DC_4_n5

6.1.12.1	Operating bands for DC

Table 6.1.12.1-1: DC band combination of 1 LTE band + 1 NR band

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_4_n5 | 4 | n5 | DC_4_n5 |

6.1.12.2	Configuration for DC

Table 6.1.12.2-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_4A_n5A | DC_4A_n5A | 4A | n5A |

6.1.12.3	Maximum output power for DC

Table 6.1.12.3-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_4A_n5A | 23 | +2/-3 |

6.1.12.4	Spurious emission band UE co-existence for DC

Table 6.1.12.4-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |  |
| DC_4A_n5A | Bands 1, 2, 3, 4, 5, 6, 7, 8, 10, 12, 13, 14, 17, 24, 25, 28, 29, 30, 34, 38, 40, 43, 45, 50, 51, 65, 66, 70, 71, n71, 85, n257 | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  | E-UTRA Band 26 | 859 | - | 869 | -27 | 1 |  |  |  |
|  | Bands 41, 42, 48, 52 | FDL_low | - | FDL_high | -50 | 1 | 3 |  |  |
|  | E-UTRA Band 18, 19 | FDL_low | - | FDL_high | -40 | 1 | 5 |  |  |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 4, 5 |  |  |
| NOTE 3: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 of TS 36.101 [2] are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 4: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz. NOTE 5: Applicable only when the assigned E-UTRA carrier is confined within 824 MHz and 849 MHz for UE category M1, M2 and UE category NB1 and NB2 |  |  |  |  |  |  |  |  |

6.1.12.5	MSD analysis for DC

For 2UL/2DL UE coexistence study 2nd, 3rd, 4th, 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.1.12.5-1

Table 6.1.12.5-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 1710 | 1755 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high | 2*fx_low | 2*fx_high |
| 2nd harmonics frequency limits (MHz) | 1648 | 1698 | 3420 | 3510 |
| 3rd harmonics frequency limits | 3* fy_low | 3* fy_high | 3*fx_low | 3*fx_high |
| 3rd harmonics frequency limits (MHz) | 2472 | 2547 | 5130 | 5265 |
| 4th harmonics frequency limits | 4* fy_low | 4* fy_high | 4*fx_low | 4*fx_high |
| 4th harmonics frequency limits (MHz) | 3296 | 3396 | 6840 | 7020 |
| 5th harmonics frequency limits | 5* fy_low | 5* fy_high | 5*fx_low | 5*fx_high |
| 5th harmonics frequency limits (MHz) | 4120 | 4245 | 8550 | 8775 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 861 | 931 | 2534 | 2604 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 12 | 107 | 2571 | 2686 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3358 | 3453 | 4244 | 4359 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1722 | 1862 | 5068 | 5208 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 717 | 837 | 4281 | 4441 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4182 | 4302 | 5954 | 6114 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6196 | 5991 | 1686 | 1541 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7664 | 7869 | 5006 | 5151 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3617 | 3432 | 873 | 1038 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6778 | 6963 | 5892 | 6057 |

Based on Table 6.1.12.5-1, it can be seen that

-	There are 2nd IMD product falling into the Rx frequencies of band n5.

6.1.12.6	∆TIB and ∆RIB values

For DC_4_n5, the TIB,c and RIB,c values are the same with DC_66A_n5A which are already specified in TS 38.101-3.

Table 6.1.12.6-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_4_n5 | 4 | 0.3 |
|  | n5 | 0.3 |

Table 6.1.12.6-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_4_n5 | 4 | 0 |
|  | n5 | 0 |

6.1.12.7	MSD

Due to 2nd order IMD, MSD is defined in table 6.1.12.7-1. Values are reused from existing EN-DC combination DC_66A_n5A.

Table 6.1.12.7-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_4A_n5A | n5 | 838 | 5 | 25 | 883 | 30 | IMD23 |
|  | 4 | 1721 | 5 | 25 | 2121 | N/A | N/A |

6.1.13	DC_4_n7

6.1.13.1	Operating bands for DC

Table 6.1.13.1-1: DC band combination of 1 LTE band + 1 NR band

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_4_n7 | 4 | n7 | No |

6.1.13.2	Configuration for DC

Table 6.1.13.2-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_4A_n7A | DC_4A_n7A | 4A | n7A |

6.1.13.3	Maximum output power for DC

Table 6.1.13.3-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_4A_n7A | 23 | +2/-3 |

6.1.13.4	Spurious emission band UE co-existence for DC

Table 6.1.13.4-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |  |
| DC_4A_n7A | E-UTRA Band 2, 4, 5, 7, 10, 12, 13, 14, 17, 26, 27, 28, 29, 30, 43, 50, 51, 66, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |  |  |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |  |  |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180 kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 6: This requirement is applicable for any channel bandwidths within the range 2500 - 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. |  |  |  |  |  |  |  |  |

6.1.13.5	MSD analysis for DC

For 2UL/2DL UE coexistence study 2nd, 3rd, 4th, 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.1.13.5-1

Table 6.1.13.5-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1755 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high | 2*fx_low | 2*fx_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3510 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3* fy_low | 3* fy_high | 3*fx_low | 3*fx_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5265 | 7500 | 7710 |
| 4th harmonics frequency limits | 4* fy_low | 4* fy_high | 4*fx_low | 4*fx_high |
| 4th harmonics frequency limits (MHz) | 6840 | 7020 | 10000 | 10280 |
| 5th harmonics frequency limits | 5* fy_low | 5* fy_high | 5*fx_low | 5*fx_high |
| 5th harmonics frequency limits (MHz) | 8550 | 8775 | 12500 | 12850 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 860 | 745 | 4210 | 4325 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 850 | 1010 | 3245 | 3430 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5920 | 6080 | 6710 | 6895 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1720 | 1490 | 8420 | 8650 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2560 | 2765 | 5745 | 6000 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7630 | 7835 | 9210 | 9465 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8570 | 8245 | 4520 | 4270 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11710 | 12035 | 9340 | 9590 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4290 | 3990 | 265 | 10 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10920 | 11220 | 10130 | 10405 |

Based on Table 6.1.13.5-1, it can be seen that

-	There is 4th IMD products falling into the Rx frequencies of band n7.

6.1.13.6	∆TIB and ∆RIB values

For DC_4_n7, the TIB,c and RIB,c values are the same with DC_66A_n7A which are already specified in TS 38.101-3.

Table 6.1.13.6-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_4_n7 | 4 | 0.5 |
|  | n7 | 0.5 |

Table 6.1.13.6-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_4_n7 | 4 | 0 |
|  | n7 | 0 |

6.1.13.7	MSD

Due to 4th order IMD, MSD is defined in table 6.1.13.7-1. Values are reused from existing EN-DC combination DC_66A_n7A.

Table 6.1.13.7-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_4A_n7A | 4 | 1730 | 5 | 25 | 2130 | N/A | N/A |
|  | n7 | 2535 | 10 | 50 | 2655 | 15 | IMD4 |

6.1.14	DC_28_n66

6.1.14.1	Configuration for DC

Table 6.1.14.1-1:  Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_28A_n66A | DC_28A_n66A | No |

6.1.14.2	Maximum output power for DC

Table 6.1.14.2-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_28A_n66A | 23 | +2/-3 |

6.1.14.3	Spurious emission band UE co-existence for DC

Table 6.1.14.3-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_28_n66 | E-UTRA Band 2, 5, 7, 25, 26, 27, 38, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 10, 42, 43, 50, 51, 66, 74, NR band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 5, 17 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 14 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 5 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180 kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 14: This requirement is applicable for 5 and 10 MHz E-UTRA channel bandwidth allocated within 718-728MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and RBstart < 48. NOTE 17: This requirement is applicable in the case of a 10 MHz E-UTRA carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. |  |  |  |  |  |  |  |

6.1.14.4	MSD analysis for DC

For 2UL/2DL UE coexistence study 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.1.14.4-1.

Table 6.1.14.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 | 1710 | 1780 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1406 | 1496 | 3420 | 3560 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2109 | 2244 | 5130 | 5340 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2812 | 2992 | 6840 | 7120 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3515 | 3740 | 8550 | 8900 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 962 | 1077 | 2413 | 2528 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 374 | 214 | 2672 | 2857 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3116 | 3276 | 4123 | 4308 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 329 | 534 | 4382 | 4637 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3819 | 4024 | 5833 | 6088 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2154 | 1924 | 4826 | 5056 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6417 | 6092 | 1282 | 1032 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 3934 | 3634 | 1176 | 1451 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7543 | 7868 | 4522 | 4772 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6536 | 6836 | 5529 | 5804 |

3rd harmonic interference may fall into Rx of band n66 for DC_28_n66.

IMD4 may fall into Rx of band n66 for DC_28_n66.

6.1.14.5	∆TIB and ∆RIB values

For DC_28_n66, TIB,c and RIB,c values will be the same as for DC_1_n28 in 38.101-3. The TIB,c and RIB,c values are given in the tables below.

Table 6.1.14.5-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_28_n66 | 28 | 0.6 |
|  | n66 | 0.3 |

Table 6.1.14.5-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_28_n66 | 28 | 0.2 |
|  | n66 | 0 |

6.1.14.6	Self-interference analysis

Based on the similarities with the existing DC combination DC_1A_n28A, the MSD due to 3rd harmonic interference can be specified as below.

Table 6.1.14.6-1: Reference sensitivity exceptions (MSD) due to UL harmonic for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| 28 | n668,9,10 | 10.2 | 7.6 | 6.2 | 5.3 | 4.0 | 3.2 | 2 |  |  |  |  |  |
| NOTE 8: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 9 The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LBsuch that in MHz and with the carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the low band. NOTE 10: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.1.14.6-2: Uplink configuration for reference sensitivity exceptions due to UL harmonic interference for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 30 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| 28 | n66 | 8 | 16 | 25 | 25 | 25 | 25 | 25 |  |  |  |  |  |

Since there is no reasonable MSD test frequency point, no need to specify the MSD due to IMD4 for DC_28_n66.

6.1.15	DC_4_n28

6.1.15.1	Configuration for DC

Table 6.1.15.1-1:  Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_4A_n28A | DC_4A_n28A | No |

6.1.15.2	Maximum output power for DC

Table 6.1.15.2-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_4A_n28A | 23 | +2/-3 |

6.1.15.3	Spurious emission band UE co-existence for DC

Table 6.1.15.3-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_4_n28 | E-UTRA Band 2, 5, 7, 25, 26, 27, 38, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 10, 42, 43, 50, 51, 66, 74, NR band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 5, 17 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 14 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 5 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180 kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 14: This requirement is applicable for 5 and 10 MHz E-UTRA channel bandwidth allocated within 718-728MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and RBstart < 48. NOTE 17: This requirement is applicable in the case of a 10 MHz E-UTRA carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. |  |  |  |  |  |  |  |

6.1.15.4	MSD analysis for DC

For 2UL/2DL UE coexistence study 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.1.15.4-1.

Table 6.1.15.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 | 1710 | 1755 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1406 | 1496 | 3420 | 3510 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2109 | 2244 | 5130 | 5265 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2812 | 2992 | 6840 | 7020 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3515 | 3740 | 8550 | 8775 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 962 | 1052 | 2413 | 2503 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 349 | 214 | 2672 | 2807 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3116 | 3251 | 4123 | 4258 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 354 | 534 | 4382 | 4562 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3819 | 3999 | 5833 | 6013 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2104 | 1924 | 4826 | 5006 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6317 | 6092 | 1282 | 1057 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 3859 | 3634 | 1176 | 1401 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7543 | 7768 | 4522 | 4747 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6536 | 6761 | 5529 | 5754 |

3rd harmonic interference may fall into Rx of band 4 for DC_4_n28.

6.1.15.5	∆TIB and ∆RIB values

For DC_4_n28, TIB,c and RIB,c values will be the same as for DC_1_n28 in 38.101-3. The TIB,c and RIB,c values are given in the tables below.

Table 6.1.15.5-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_4_n28 | 4 | 0.3 |
|  | n28 | 0.6 |

Table 6.1.15.5-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_4_n28 | 4 | 0 |
|  | n28 | 0.2 |

6.1.15.6	self-interference analysis

Based on the similarities with the existing DC combination DC_1A_n28A, the MSD due to 3rd harmonic interference can be specified as below.

Table 6.1.15.6-1: Reference sensitivity exceptions (MSD) due to UL harmonic for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n28 | 48,9,10 | 10.2 | 7.6 | 6.2 | 5.3 |  |  |  |  |  |  |  |  |
| NOTE 8: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 9 The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LBsuch that in MHz and with the carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the low band. NOTE 10: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.1.15.6-2: Uplink configuration for reference sensitivity exceptions due to UL harmonic interference for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 30 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| n28 | 4 | 8 | 16 | 25 | 25 |  |  |  |  |  |  |  |  |

6.1.16	DC_66_n28

6.1.16.1	Configuration for DC

Table 6.1.16.1-1:  Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_66A_n28A | DC_66A_n28A | No |

6.1.16.2	Maximum output power for DC

Table 6.1.16.2-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_66A_n28A | 23 | +2/-3 |

6.1.16.3	Spurious emission band UE co-existence for DC

Table 6.1.16.3-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_66_n28 | E-UTRA Band 2, 5, 7, 25, 26, 27, 38, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 10, 42, 43, 50, 51, 66, 74, NR band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 5, 17 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 14 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 5 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180 kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 14: This requirement is applicable for 5 and 10 MHz E-UTRA channel bandwidth allocated within 718-728MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and RBstart < 48. NOTE 17: This requirement is applicable in the case of a 10 MHz E-UTRA carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. |  |  |  |  |  |  |  |

6.1.16.4	MSD analysis for DC

For 2UL/2DL UE coexistence study 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.1.16.4-1.

Table 6.1.16.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 | 1710 | 1780 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1406 | 1496 | 3420 | 3560 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2109 | 2244 | 5130 | 5340 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2812 | 2992 | 6840 | 7120 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3515 | 3740 | 8550 | 8900 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 962 | 1077 | 2413 | 2528 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 374 | 214 | 2672 | 2857 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3116 | 3276 | 4123 | 4308 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 329 | 534 | 4382 | 4637 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3819 | 4024 | 5833 | 6088 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2154 | 1924 | 4826 | 5056 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6417 | 6092 | 1282 | 1032 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 3934 | 3634 | 1176 | 1451 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7543 | 7868 | 4522 | 4772 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6536 | 6836 | 5529 | 5804 |

3rd harmonic interference may fall into Rx of band 66 for DC_66_n28.

IMD4 may fall into Rx of band 66 for DC_66_n28.

6.1.16.5	∆TIB and ∆RIB values

For DC_66_n28, TIB,c and RIB,c values will be the same as for DC_1_n28 in 38.101-3. The TIB,c and RIB,c values are given in the tables below.

Table 6.1.16.5-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66_n28 | n28 | 0.6 |
|  | 66 | 0.3 |

Table 6.1.16.5-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_66_n28 | n28 | 0.2 |
|  | 66 | 0 |

6.1.16.6	Self-interference analysis

Based on the similarities with the existing DC combination DC_1A_n28A, the MSD due to 3rd harmonic interference can be specified as below.

Table 6.1.16.6-1: Reference sensitivity exceptions (MSD) due to UL harmonic for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n28 | 668,9,10 | 10.2 | 7.6 | 6.2 | 5.3 |  |  |  |  |  |  |  |  |
| NOTE 8: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 9 The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LBsuch that in MHz and with the carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the low band. NOTE 10: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.1.16.6-2: Uplink configuration for reference sensitivity exceptions due to UL harmonic interference for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 30 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| n28 | 66 | 8 | 16 | 25 | 25 |  |  |  |  |  |  |  |  |

Since there is no reasonable MSD test frequency point, no need to specify the MSD due to IMD4 for DC_66_n28.

6.1.17	DC_28_n1

6.1.17.1	Configuration for DC

Table 6.1.17.1-1:  Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_28A_n1A | DC_28A_n1A | No |

6.1.17.2	Maximum output power for DC

Table 6.1.17.2-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_28A_n1A | 23 | +2/-3 |

6.1.17.3	Spurious emission band UE co-existence for DC

Table 6.1.17.3-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_28_n1 | E-UTRA Band 5, 7, 8, 18, 19, 20, 26, 27, 31, 38, 40, 41, 72, 73 NR band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 22, 32, 42, 43, 50, 51, 52, 65, 74, 75, 76 NR band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |
|  | E-UTRA band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 5, 17 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 14 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 5 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 5,16 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 16 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 16 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180 kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 9: Applicable when the assigned E-UTRA carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 10: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 11: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 14: This requirement is applicable for 5 and 10 MHz E-UTRA channel bandwidth allocated within 718-728MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and RBstart < 48. NOTE 16: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 17: This requirement is applicable in the case of a 10 MHz E-UTRA carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. |  |  |  |  |  |  |  |

6.1.17.4	MSD analysis for DC

For 2UL/2DL UE coexistence study 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.1.17.4-1.

Table 6.1.17.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 | 1920 | 1980 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1406 | 1496 | 3840 | 3960 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2109 | 2244 | 5760 | 5940 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2812 | 2992 | 7680 | 7920 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3515 | 3740 | 9600 | 9900 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1172 | 1277 | 2623 | 2728 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 574 | 424 | 3092 | 3257 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3326 | 3476 | 4543 | 4708 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 129 | 324 | 5012 | 5237 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4029 | 4224 | 6463 | 6688 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2554 | 2344 | 5246 | 5456 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7217 | 6932 | 1072 | 832 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4534 | 4264 | 1596 | 1851 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8383 | 8668 | 4732 | 4972 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7166 | 7436 | 5949 | 6204 |

3rd harmonic interference may fall into Rx of band n1 for DC_28_n1.

6.1.17.5	∆TIB and ∆RIB values

For DC_28_n1, TIB,c and RIB,c values will be the same as for DC_1_n28 in 38.101-3. The TIB,c and RIB,c values are given in the tables below.

Table 6.1.17.5-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_28_n1 | n1 | 0.3 |
|  | 28 | 0.6 |

Table 6.1.17.5-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_28_n1 | n1 | 0 |
|  | 28 | 0.2 |

6.1.17.6	Self-interference analysis

Based on the similarities with the existing DC combination DC_1A_n28A, the MSD can be specified as below.

Table 6.1.17.6-1: Reference sensitivity exceptions (MSD) due to UL harmonic for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| 28 | n18,9,10 | 10.2 | 7.6 | 6.2 | 5.3 | 4.0 | 3.2 | 2 | 1.0 |  |  |  |  |
| NOTE 8: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 9 The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LBsuch that in MHz and with the carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the low band. NOTE 10: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.1.17.6-2: Uplink configuration for reference sensitivity exceptions due to UL harmonic interference for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 30 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| 28 | n1 | 8 | 16 | 25 | 25 | 25 | 25 | 25 | 25 |  |  |  |  |

6.1.18	DC_42_n3

6.1.18.1	Operating bands for DC

Table 6.1.18.1-1: DC band combination of 1 LTE band + 1 NR band

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_42_n3 | 42 | n3 | DC_42_n3 |

6.1.18.2	Configuration for DC

Table 6.1.18.2-1: Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_42A_n3A | DC_42A_n3A | 42A | n3A |
| DC_42C_n3A | DC_42A_n3A DC_42C_n3A | 42C | n3A |

6.1.18.3	Co-existence studies

Table 6.1.18.3-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_42A_n3A DC_42C_n3A | 23 | +2/-3 |

6.1.18.4	Spurious emission band UE co-existence for DC

Table 6.1.18.4-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_42_n3 | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 33, 34, 38, 40, 41, 44, 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

6.1.18.5	MSD analysis for DC

For 2UL/2DL UE coexistence study 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.1.18.5-1

Table 6.1.18.5-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 3400 | 3600 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 6800 | 7200 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 10200 | 10800 | 5130 | 5355 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 13600 | 14400 | 6840 | 7140 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 17000 | 18000 | 8550 | 8925 |
| 6th harmonics frequency limits | 6*fx_low | 6*fx_high | 6* fy_low | 6* fy_high |
| 6th harmonics frequency limits (MHz) | 20400 | 21600 | 10260 | 10710 |
| 7th harmonics frequency limits | 7*fx_low | 7*fx_high | 7* fy_low | 7* fy_high |
| 7th harmonics frequency limits (MHz) | 23800 | 25200 | 11970 | 12495 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1615 | 1890 | 5110 | 5385 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 5015 | 5490 | 180 | 170 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8510 | 8985 | 6820 | 7170 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 8415 | 9090 | 1530 | 1955 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 3230 | 3780 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 11910 | 12585 | 8530 | 8955 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 10220 | 10770 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 3740 | 3240 | 12690 | 11815 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1445 | 2070 | 7380 | 6630 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10240 | 10740 | 15310 | 16185 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11930 | 12555 | 13620 | 14370 |

Based on Table 6.1.18.5 -1, it can be seen that

-	2nd order harmonic may fall into Rx frequencies of band 22, 42, 48, 49, n77, n78.

-	3rd order harmonic may fall into Rx frequencies of band 46.

-	2nd order IMD products may fall into Rx frequencies of bands 3, 9, 35, 39, 46.

-	3rd order IMD products may fall into Rx frequencies of bands 46.

-	4th order IMD products may fall into Rx frequencies of bands 2, 3, 9, 22, 24, 25, 33, 35, 36, 37, 39, 42, 43, 48, 49, 52, n77, n78.

-	5th order IMD products may fall into Rx frequencies of bands 2, 3, 9, 11, 21, 22, 24, 25, 32, 33, 34, 35, 36, 37, 39, 42, 43, 45, 48, 49, 50, 52, 70, 74, 75, n77, n78, n92, n94.

Based on above,

-	the 2nd order harmonic of Band n3 will fall into Band 42.

-	the 2nd, 4th and 5th order IMD may fall into Rx frequencies of band n3.

-	the 4th and 5th or der IMD may fall into Rx frequencies of band n42.

Regarding the MSD due to harmonics of Band n3 fall into Band 42, the exceptions are needed. Based on CA_3-42, reference sensitivity exceptions for DC_42_n3 due to UL harmonic are proposed in Table 6-1.18.5-2 with uplink configuration specified in Table 6-1.18.5 -3.

Table 6-1.18.5-2: MSD due to UL harmonic for EN-DC in NR FR1

| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n3 | 422, 13 | 27.3 | 24.3 | 22.5 | 21.3 |  |  |  |  |  |  |  |  |
|  | 423 | 1.9 | 1.3 | 1.0 | 0.5 |  |  |  |  |  |  |  |  |
| NOTE 2: The requirements should be verified for UL EARFCN or NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 3: The requirements are only applicable to channel bandwidths no larger than 20 MHz and with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 13: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range ∆FHD above and below the edge of this downlink transmission bandwidth. The value ∆FHD depends on the EN-DC band combination: ∆FHD = 10 MHz for DC_1_n77, DC_2_n48, DC_2_n77, DC_48_n66, DC_66_n48, DC_66_n77, DC_3_n77, DC_3_n78, DC_11_n28, DC_28_n50 and DC_42_n3 |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6-1.18.5-3: Uplink configuration for reference sensitivity exceptions due to UL harmonic interference for EN-DC in NR FR1

|  | E-UTRA or NR Band / Channel bandwidth of the high band |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| n3 | 42 | 12 | 25 | 36 | 50 |  |  |  |  |  |  |  |  |

Regarding the MSD due to 2nd and 4th order IMD, the exceptions are needed. The reference sensitivity exceptions for DC_42_n3 due to IMD are proposed in Table 6-1.18.5-4.

Table 6-1.18.5-4: Reference sensitivity exceptions for PCell due to dual uplink operation for EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| DC_42_n3 | 42 | 3575 | 10 | 50 | 3575 | N/A | TDD | N/A |
|  | n3 | 1740 | 5 | 25 | 1835 | 26 | FDD | 2nd3 |
|  |  |  |  |  |  | 28.74 |  |  |
|  | 42 | 3435 | 10 | 50 | 3435 | N/A | TDD | N/A |
|  | n3 | 1765 | 5 | 25 | 1860 | 8.0 | FDD | 4th3 |
|  |  |  |  |  |  | 10.74 |  |  |
| NOTE 3: This band is subject to IMD5 also which MSD is not specified. NOTE 4: Applicable only if operation with 4 antenna ports is supported in the band with EN-DC configured. |  |  |  |  |  |  |  |  |

When 2UL inter-band EN-DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.1.18.5-5 lists up to 7th order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.1.18.5-5: Band 42 and Band n3 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | 4th IMD |
| Galileo | 1559 | - | 1591 | Yes |  | 4th IMD |
| GLONASS | 1591 | - | 1610 | Yes |  | 4th IMD |
| GPS | 1563 | - | 1587 | Yes |  | 4th IMD |
| ISM band (2.4GHz) | 2400 | - | 2483.5 |  | US/Europe |  |
|  | 2400 | - | 2494 |  | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 3rd Harmonic 2nd IMD |
|  | 5150 | - | 5350 | Yes | Europe | 3rd Harmonic 2nd IMD |
|  | 5470 | - | 5725 | Yes |  | 3rd IMD |
|  | 5150 | - | 5825 | Yes | Asia | 3rd Harmonic 2nd IMD |
| 45GHz Unlicensed Bands | 42300 | - | 47000 |  | China |  |
|  | 47200 | - | 48400 |  | China |  |
| 60GHz Unlicensed Bands | 57000 | - | 66000 |  | Europe |  |
|  | 57050 | - | 64000 |  | USA Canada |  |
|  | 57000 | - | 64000 |  | South Korea |  |
|  | 59000 | - | 66000 |  | Japan |  |
|  | 59000 | - | 64000 |  | China |  |
|  | 59400 | - | 62900 |  | Australia |  |

6.1.18.6	∆TIB and ∆RIB values

For DC_42_n3, the TIB,c and RIB,c values are given in the tables below.

Table 6.1.18.6-1: ΔTIB,c

| Inter-band EN-DC configuration | E-UTRA and NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_42_n3 | 42 | 0.8 |
|  | n3 | 0.6 |

Table 6.1.18.6-2: ΔRIB,c

| Inter-band EN-DC configuration | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_42_n3 | 42 | 0.5 |
|  | n3 | 0.2 |

6.1.18.7	Self-interference analysis

This section provides the formulation to determine whether the DL carriers would be impacted by UL harmonics, 2UL inter-modulation products (IMD), or DL harmonic mixing with UL interferer based on UL and DL channel combinations. The potential self-interference mechanism has been identified in section 6.1.18.5 except for harmonic mixing and UL harmonic order higher than 5 which are used to generate an interference mixing coefficient table for this DC combination, as shown in Table 6.1.18.7-1.

Table 6.1.18.7-1: Band 42 and Band n3 interference mixing coefficients

| EUTRA-NR DC Configuration | EUTRA/NR Band | UL Coefficient |  | DL Coefficient |  | Harmonic/IMD Order | Victim Band | Interference Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_42A_n3A | 42 | a | 0 | b | -1 | 2 | 42 | Harmonic |
|  | n3 | c | 2 | d | 0 |  |  |  |
| DC_42A_n3A | 42 | a | 1 | b | 0 | 2 | n3 | IMD |
|  | n3 | c | -1 | d | -1 |  |  |  |
| DC_42A_n3A | 42 | a | -1 | b | 0 | 4 | n3 | IMD |
|  | n3 | c | 3 | d | -1 |  |  |  |
| DC_42A_n3A | 42 | a | -2 | b | 0 | 5 | n3 | IMD |
|  | n3 | c | 3 | d | 1 |  |  |  |

Equations (1) and (2) below are used to calculate the interference center frequency (fINT) and its effective bandwidth (BWINT) where coefficients a, b, c, and d are defined in Table 6.1.18.6-1 and CBW stands for channel bandwidth.

(1)

(2)

Formula (3-1) and (3-2) below are then used to indicate when the interference is overlapping with RX1 and RX2, respectively.

(3-1)

(3-2)

6.1.19	DC_8_n2

6.1.19.1	Configuration for DC

Table 6.1.19.1-1:  Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration | Single UL allowed |
| --- | --- | --- |
| DC_8A_n2A | DC_8A_n2A | Yes |

6.1.19.2	Maximum output power for DC

Table 6.1.19.2-1: Maximum output power for inter-band EN-DC (two bands)

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_8A_n2A | 23 | +2/-3 |

6.1.19.3	Spurious emission band UE co-existence for DC

Table 6.1.19.3-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_8_n2 | E-UTRA Band 2, 8, 28, 50, 51, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41, 42, 43 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180 kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. |  |  |  |  |  |  |  |

6.1.19.4	MSD analysis for DC

For 2UL/2DL UE coexistence study 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.1.19.4-1

Table 6.1.19.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 880 | 915 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3700 | 3820 | 1760 | 1830 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5550 | 5730 | 2640 | 2745 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 7400 | 7640 | 3520 | 3660 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 9250 | 9550 | 4400 | 4575 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1030 | 935 | 2730 | 2825 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2785 | 2940 | 150 | 20 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4580 | 4735 | 3610 | 3740 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4635 | 4850 | 730 | 895 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6430 | 6645 | 4490 | 4655 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1870 | 2060 | 5460 | 5650 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1810 | 1610 | 6760 | 6485 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 955 | 1180 | 3970 | 3720 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5370 | 5570 | 8280 | 8555 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6340 | 6565 | 7310 | 7560 |
| UL frequency (MHz) | 1850 | 1910 | 880 | 915 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3700 | 3820 | 1760 | 1830 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |

Based on Table 6.1.19.4-1, it can be seen that:

-	2nd order IMD products may fall into own Rx frequencies of band 8

-	4th order IMD products may fall into own Rx frequencies of band n2

-	2nd order harmonic may fall into Rx frequencies of bands 3, 6, 43, n77 and n78.

-	3rd order harmonic may fall into Rx frequencies of bands 7, 41 and 46 .

-	3rd order IMD products may fall into Rx frequencies of bands 6, 43 n77, n78 and n79.

-	4th order IMD products may fall into Rx frequencies of bands 3, 5, 9, 12, 13, 14, 17, 18, 19, 20, 25, 26, 27, 28, 33, 34, 35, 36, 37, 39, 42, 44, 46, 67, 68, 70 and n79.

-	5th order IMD products may fall into Rx frequencies of band 3, 6, 43, 46, n77 and n78.

When 2UL inter-band EN-DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.1.19.4-2 lists up to 7th order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.1.19.4-2: Harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No |  |  |
|  | 2400 | - | 2494 | No |  |  |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 4th, 5th IMD 3rd harmonics |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | Yes |  | 3rd harmonics |
|  | 5150 | - | 5825 | Yes | Asia | 4th, 5th IMD 3rd harmonics |
| 45GHz Unlicensed Bands | 42300 | - | 47000 | No | China |  |
|  | 47200 | - | 48400 | No | China |  |
| 60GHz Unlicensed Bands | 57000 | - | 66000 | No | Europe |  |
|  | 57050 | - | 64000 | No | USA Canada |  |
|  | 57000 | - | 64000 | No | South Korea |  |
|  | 59000 | - | 66000 | No | Japan |  |
|  | 59000 | - | 64000 | No | China |  |
|  | 59400 | - | 62900 | No | Australia |  |

6.1.19.5	∆TIB and ∆RIB values

For DC_8_n2, the TIB,c and RIB,c values are reused from DC_2_n5 and are given in the tables below.

Table 6.1.19.5-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8_n2 | 8 | 0.3 |
|  | n2 | 0.3 |

Table 6.1.19.5-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_8_n2 | 8 | 0 |
|  | n2 | 0 |

6.1.19.6	MSD

Based on the co-existence studies MSD due to IMD2/4 is required. The MSD values can be reused from DC_3-n5.

Table 6.1.19.6-1: Reference sensitivity exceptions for PCell due to dual uplink operation for EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| DC_8_n2 | n2 | 1860 | 5 | 25 | 1940 | 4 | FDD | IMD4 |
|  | 8 | 890 | 5 | 25 | 935 | N/A |  | N/A |
|  | n2 | 1865 | 5 | 25 | 1945 | N/A | FDD | N/A |
|  | 8 | 910 | 5 | 25 | 955 | 24 |  | IMD2 |

6.1.20	DC_18_n28

6.1.20.1	Configuration for DC

Table 6.1.20.1-1:  Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration (NOTE1) | Uplink EN-DC configuration | Single UL allowed |
| --- | --- | --- |
| DC_18A_n28A | DC_18A_n28A | No |
| NOTE 1: The frequency range in band n28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL. |  |  |

6.1.20.2	Maximum output power for DC

Table 6.1.20.2-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_18A_n28A | 23 | +2/-3 |

6.1.20.3	Spurious emission band UE co-existence for DC

Table 6.1.20.3-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_18_n28 | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 4, 24 |
|  | E-UTRA Band 1, 65 | FDL_low | - | FDL_high | -50 | 1 | 2, 4, 23 |
|  | E-UTRA Band 42, 43 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 3, 34 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 25 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |
|  | Frequency range | 773 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 5 |
|  | Frequency range | 860 | - | 890 | -40 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 | 5 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180 kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval.. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz. NOTE 4: Applicable when the assigned E-UTRA carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the aggregated channel bandwidth. NOTE 23: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 24: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 25: This requirement is applicable for 5 and 10 MHz E-UTRA channel bandwidth allocated within 718-728MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and RBstart<48. |  |  |  |  |  |  |  |

6.1.20.4	MSD analysis for DC

For 2UL/2DL UE coexistence study 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.1.20.4-1.

Table 6.1.20.4-1: Harmonic and IMD analysis

| UE UL carriers | Fx_low | Fx_high | Fy_low | Fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 815 | 830 | 703 | 748 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1630 | 1660 | 1406 | 1496 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2445 | 2490 | 2109 | 2244 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 127 | 67 | 1518 | 1578 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 882 | 957 | 576 | 681 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2333 | 2408 | 2221 | 2326 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1697 | 1787 | 1279 | 1429 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 134 | 254 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3148 | 3238 | 2924 | 3074 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 3036 | 3156 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2177 | 1982 | 2617 | 2512 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 614 | 449 | 1084 | 949 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 3627 | 3822 | 3963 | 4068 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 3739 | 3904 | 3851 | 3986 |

No IMD and harmonic issue for this combination.

6.1.20.5	∆TIB and ∆RIB values

For DC_18_n28, TIB,c and RIB,c values are given in the tables below.

Table 6.1.20.5-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_18_n28 | 18 | 0.5 |
|  | n28 | 0.5 |

Table 6.1.20.5-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_18_n28 | 18 | 0 |
|  | n28 | 0 |

6.1.20.6	self-interference analysis

Table 6.1.20.6-1: Reference sensitivity exceptions (MSD) due to cross band isolation for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| 18 | n28 | 4.5 | 3 |  |  |  |  |  |  |  |  |  |  |

Table 6.1.20.6-2: Uplink configuration for reference sensitivity exceptions due to cross band isolation for EN-DC in NR FR1

| E-UTRA or NR Band / SCS / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 30 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| 18 | n28 | 15 | 18 | 18 |  |  |  |  |  |  |  |  |  |  |

6.1.21	DC_18_n41

6.1.21.1	Configuration for DC

Table 6.1.21.1-1: Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration | Single UL allowed |
| --- | --- | --- |
| DC_18A_n41A | DC_18A_n41A | No |
| The frequency range in band n41 is restricted for this band combination to 2595 – 2645 MHz. |  |  |

6.1.21.2	Maximum output power for DC

Table 6.1.21.2-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_18A_n41A | 23 | +2/-3 |

6.1.21.3	Spurious emission band UE co-existence for DC

Table 6.1.21.3-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_18_n41 | E-UTRA Band 1, 3, 11, 19, 21, 28, 34, 42, 65 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180 kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz |  |  |  |  |  |  |  |

6.1.21.4	MSD analysis for DC

For 2UL/2DL UE coexistence study 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.1.21.4-1.

Table 6.1.21.4-1: Harmonic and IMD analysis

| UE UL carriers | Fx_low | Fx_high | Fy_low | Fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 815 | 830 | 2496 | 2690 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1630 | 1660 | 4992 | 5380 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2445 | 2490 | 7488 | 8070 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1666 | 1875 | 3311 | 3520 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1060 | 836 | 4162 | 4565 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4126 | 4350 | 5807 | 6210 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | -245 | -6 | 6658 | 7255 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 3750 | 3332 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4941 | 5180 | 8303 | 8900 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 6622 | 7040 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9945 | 9154 | 824 | 570 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6440 | 5828 | 2502 | 2935 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10799 | 11590 | 5756 | 6010 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9118 | 9730 | 7437 | 7870 |

Based on the co-existence studies, impact on own Rx is as follows.

- 3nd IMD may fall into own Rx of band 18. Since band 18 is KDDI only band, No IMD issue with considering KDDI spectrum.

- 5th order IMD may also fall into own Rx of band n41. There is no need to estimate MSD value for own Rx of TDD band.

No IMD and harmonic issue for this combination..

6.1.21.5	∆TIB and ∆RIB values

For DC_18_n41, TIB,c and RIB,c values are given in the tables below.

Table 6.1.21.5-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_18_n41 | 18 | 0.3 |
|  | n41 | 0.31 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. |  |  |

Table 6.1.21.5-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_18_n41 | 18 | 0 |
|  | n41 | 01 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. |  |  |

6.1.21.6	self-interference analysis

Table 6.1.21.6-1: Reference sensitivity exceptions (MSD) due to receiver harmonic mixing for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n41 | 1811 | N/A | N/A | N/A |  |  |  |  |  |  |  |  |
| NOTE 11: No requirements apply for the case that there is at least one individual RE within the uplink transmission bandwidth of the relative higher band and when the frequency range of relative higher band’s uplink channel bandwidth or uplink 1st adjacent channel bandwidth is fully or partially overlapped with the 3 times of the frequency range of the relative lower band’s downlink channel bandwidth. The reference sensitivity is only verified when this is not the case (the requirements specified in clause 7.3B.1 apply). |  |  |  |  |  |  |  |  |  |  |  |  |

6.1.22	DC_7_n2

6.1.22.1	Configuration for DC

Table 6.1.22.1-1:  Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration | Single UL allowed |
| --- | --- | --- |
| DC_7A_n2A DC_7C_n2A | DC_7A_n2A | No |

6.1.22.2	Maximum output power for DC

The maximum output power for the uplink EN-DC configurations is given as.

Table 6.1.22.2-1: Maximum output power for inter-band EN-DC (two bands)

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_7A_n2A | 23 | +2/-3 |

6.1.22.3	Spurious emission band UE co-existence for DC

The requirements to be specified in TS 38.101-3 Table 6.5B.3.3.2-1 is given below.

Table 6.1.22.3-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_7_n2 | E-UTRA Band 4, 5, 7, 10, 12, 13, 14, 17, 26, 27, 28, 29, 30, 42, 50, 51, 66, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 43 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA band 2 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | Frequency range | 2570 | - | 2575 | 1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180 kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 6: This requirement is applicable for any channel bandwidths within the range 2500 - 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 15: Applicable when NS_05 in clause 6.6.3.3.1 is signalled by the network. |  |  |  |  |  |  |  |

6.1.22.4	MSD analysis for DC

For 2UL/2DL UE coexistence study 2nd, 3rd, 4th, 5th, 6th and 7th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented below.

Table 6.1.22.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 2500 | 2570 | 1850 | 1910 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 5000 | 5140 | 3700 | 3820 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7500 | 7710 | 5550 | 5730 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 10000 | 10280 | 7400 | 7640 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 12500 | 12850 | 9250 | 9550 |
| 6th harmonics frequency limits | 6*fx_low | 6*fx_high | 6* fy_low | 6* fy_high |
| 6th harmonics frequency limits (MHz) | 15000 | 15420 | 11100 | 11460 |
| 7th harmonics frequency limits | 7*fx_low | 7*fx_high | 7* fy_low | 7* fy_high |
| 7th harmonics frequency limits (MHz) | 17500 | 17990 | 12950 | 13370 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 720 | 590 | 4350 | 4480 |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3090 | 3290 | 1130 | 1320 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6850 | 7050 | 6200 | 6390 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 5590 | 5860 | 2980 | 3230 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1180 | 1440 | 8700 | 8960 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 9350 | 9620 | 8050 | 8300 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5140 | 4830 | 8430 | 8090 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 730 | 410 | 4010 | 3680 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9900 | 10210 | 11850 | 12190 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10550 | 10870 | 11200 | 11530 |

Based on Table 6.1.22.4-1, it can be seen that no harmonics or IMD products fall into own Rx frequencies.

When 2UL inter-band EN-DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. A summary of this is given below.

Table 6.1.22.4-2: Harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | YES | US | 2nd HAM, 4th IMD |
|  | 5150 | - | 5350 | YES | Europe | 2nd HAM, 4th IMD |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | YES | Asia | 2nd HAM, 4th IMD |
| 45GHz Unlicensed Bands | 42300 | - | 47000 | No | China |  |
|  | 47200 | - | 48400 | No | China |  |
| 60GHz Unlicensed Bands | 57000 | - | 66000 | No | Europe |  |
|  | 57050 | - | 64000 | No | USA Canada |  |
|  | 57000 | - | 64000 | No | South Korea |  |
|  | 59000 | - | 66000 | No | Japan |  |
|  | 59000 | - | 64000 | No | China |  |
|  | 59400 | - | 62900 | No | Australia |  |

6.1.22.5	∆TIB and ∆RIB values

The TIB,c and RIB,c values are reused from CA_2-7 as specified in TS 36.101.

Table 6.1.22.5-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7_n2 | 7 | 0.5 |
|  | n2 | 0.5 |

Table 6.1.22.5-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_7_n2 | 7 | 0 |
|  | n2 | 0 |

6.1.22.6	Self-interference analysis

Based on the co-existence studies no MSD is needed.

6.1.23	DC_25_n77

6.1.23.1	Configuration for DC

Table 6.1.23.1-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration | Single UL allowed |
| --- | --- | --- |
| DC_25A_n77A | DC_25A_n77A | YES |
| DC_25A-25A_n77A | DC_25A_n77A | YES |

6.1.23.2	Maximum output power for DC

Table 6.1.23.2-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| EN-DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_25A_n77A | 23 | +2/-3 |

6.1.23.3	Spurious emission band UE co-existence for DC

Table 6.1.23.3-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| EN-DC CA combination | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected Band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_25_n77 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 25, 26, 29, 30, 41, 65, 66, 70, 71 | FDL_low | - | FDL_high | -50 | 1 |  |

6.1.23.4	MSD analysis for DC

For study of 2UL/2DL, Table 6.1.23.4-1 lists up to 5th order harmonics and intermodulation products for DC_25A_n77A.

Table 6.1.23.4-1: Harmonic and IMD analysis

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 1850 | 1915 | 3300 | 4200 |
| 2nd harmonic | 2* f1_low | 2*f1_high | 2*f2_low | 2*f2_high |
| harmonic frequency limit (MHz) | 3700 | 3830 | 6600 | 8400 |
| 3rd harmonic | 3* f1_low | 3*f1_high | 3*f2_low | 3*f2_high |
| harmonic frequency limit (MHz) | 5550 | 5745 | 9900 | 12600 |
| 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency limit (MHz) | 1385 | 2350 | 5150 | 6115 |
| 3rd order IMD products | 2*f1_low – f2_high | 2*f1_high – f2_low | 2*f2_low – f1_high | 2*f2_high – f1_low |
| IMD frequency limit (MHz) | 500 | 530 | 4685 | 6550 |
| 3rd order IMD products | 2*f1_low + f2_low | 2*f1_high + f2_high | 2*f2_low + f1_low | 2*f2_high + f1_high |
| IMD frequency limit (MHz) | 7000 | 8030 | 8450 | 10315 |
| 4th order IMD products | 3*f1_low – f2_high | 3*f1_high – f2_low | 3*f2_low – f1_high | 3*f2_high – f1_low |
| IMD frequency limit (MHz) | 1350 | 2445 | 7985 | 10750 |
| 4th order IMD products | 3*f1_low + f2_low | 3*f1_high + f2_high | 3*f2_low + f1_low | 3*f2_high + f1_high |
| IMD frequency limit (MHz) | 8850 | 9945 | 11750 | 14515 |
| 4th order IMD products | 2*f1_low – 2*f2_high | 2*f1_high – 2*f2_low | 2*f1_low + 2*f2_low | 2*f1_high + 2*f2_high |
| IMD frequency limit (MHz) | 4700 | 2770 | 10300 | 12230 |
| 5th order IMD products | f1_low – 4*f2_high | f1_high – 4*f2_low | f2_low – 4*f1_high | f2_high – 4*f1_low |
| IMD frequency limit (MHz) | 14950 | 11285 | 4360 | 3200 |
| 5th order IMD products | f1_low + 4*f2_low | f1_high + 4*f2_high | f2_low + 4*f1_low | f2_high + 4*f1_high |
| IMD frequency limit (MHz) | 15050 | 18715 | 10700 | 11860 |
| 5th order IMD products | 2*f1_low – 3*f2_high | 2*f1_high - 3*f2_low | 2*f2_low – 3*f1_high | 2*f2_high – 3*f1_low |
| IMD frequency limit (MHz) | 8900 | 6070 | 855 | 2850 |
| 5th order IMD products | 2*f1_low + 3*f2_low | 2*f1_high + 3*f2_high | 2*f2_low + 3*f1_low | 2*f2_high + 3*f1_high |
| IMD frequency limit (MHz) | 13600 | 16430 | 12150 | 14145 |

Table 6.1.23.4-2: Band 25 and Band n77 UL harmonic mixing products

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| 25 | 1850 | 1915 | 1930 | 1995 | 3860 | 3990 | 5790 | 5985 | 7720 | 7980 | 9650 | 9975 |
| n77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 | 16500 | 21000 |

Based on co-existence study as presented in the table 6.1.23.4-1 and 6.1.23.4-2, own Rx impact is shown in the following.

-	The 2nd harmonic of Band 25 uplink may fall into own Rx of band n77

-	The 2nd harmonic mixing of Band 25 may fall into UL of band n77

-	The 2nd, 4th and 5th order IMD generated by dual uplink of Band 25 + Band n77 may fall into own Rx of band 25

-	The 4th and 5th order IMD generated by dual uplink of Band 25 + Band n77 may fall into own Rx of Band n77

The same MSD as CA_n25-n77 in TS 38.101-1 can be applied to DC_25_n77 as specified in Table 6.1.23.4-3 and 6.1.23.4-4 for the harmonic issue.

Table 6.1.23.4-3: MSD due to harmonic issue for DC_25A_n77A

| MSD due to harmonic exception for the DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) |  | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| 25 | n772, 4 |  | 23.9 | 22.1 | 20.9 | 19.8 | 19.0 | 17.9 | 16.8 | 16.0 | 15.5 | 14.8 |  | 14.3 | 13.8 |
|  | n773 |  | 1.1 | 0.8 | 0.3 | 0.1 | 0 | 0 | 0 | 0 | 0 | 0 |  | 0 | 0 |
| NOTE 2: The requirements should be verified for UL EARFCN or NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 3: The requirements are only applicable to channel bandwidths no larger than 20 MHz and with a carrier frequency at MHz offset from in the victim (higher band) with , where andare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 4: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.1.23.4-4 Uplink configuration due to UL harmonic interference

|  |  | NR Band / Channel bandwidth of the affected DL band / UL RB allocation of the aggressor band |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 30 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 70 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| 25 | n77 |  | 25 | 36 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 |

The same MSD as CA_n25-n77 in TS 38.101-1 can be applied to DC_25_n77 as specified in Table 6.1.23.4-5 and 6.1.23.4-6 for the harmonic mixing issue.

Table 6.1.23.4-5: MSD due to receiver harmonic mixing for EN-DC in NR FR1

| NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n77 | 25 | [6.1] | [5.0] | [4.0] | [3.7] |  |  |  |  |  |  |  |  |

Table 6.1.23.4-6: Uplink configuration due to receiver harmonic mixing for EN-DC in NR FR1

| NR Band / SCS / Channel bandwidth of the affected DL band / UL RB allocation of the aggressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 70 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| n77 | 25 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |  |

The same MSD as CA_n25-n77 in TS 38.101-1 can be applied to DC_25_n77 as specified in Table 6.1.23.4-7 for the IMD issues.

Table 6.1.23.4-7: MSD due to IMD issue

|  | Operating band/ Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC CA Configuration | Operating band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| DC_25A_n77A DC_25A-25A_n77A | 25 | 1855 | 5 | 25 | 1935 | 26 | FDD | IMD2 |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
|  | 25 | 1885 | 5 | 25 | 1965 | 8 | FDD | IMD4 |
|  | n77 | 3690 | 10 | 50 | 3690 | N/A | TDD | N/A |
|  | 25 | 1885 | 5 | 25 | 1965 | 5 | FDD | IMD5 |
|  | n77 | 3810 | 10 | 50 | 3810 | N/A | TDD | N/A |

6.1.23.5	∆TIB and ∆RIB values

For DC_25_n77, the ΔTIB,c and ΔRIB,c values are given the same as CA_n25-n77 in the tables below.

Table 6.1.23.5-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_25_n77 DC_25-25_n77 | 25 | 0.6 |
|  | n77 | 0.8 |

Table 6.1.23.5-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_25_n77 DC_25-25_n77 | 25 | 0.2 |
|  | n77 | 0.5 |

6.1.24	DC_25_n78

6.1.24.1	Configuration for DC

Table 6.1.24.1-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration | Single UL allowed |
| --- | --- | --- |
| DC_25A_n78A | DC_25A_n78A | YES |
| DC_25A-25A_n78A | DC_25A_n78A | YES |

6.1.24.2	Maximum output power for DC

Table 6.1.24.2-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| EN-DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_25A_n78A | 23 | +2/-3 |

6.1.24.3	Spurious emission band UE co-existence for DC

Table 6.1.24.3-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| EN-DC CA combination | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected Band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_25_n78 | E-UTRA Band 2, 5, 7, 12, 13, 25, 26, 28, 41, 66 | FDL_low | - | FDL_high | -50 | 1 |  |

6.1.24.4	MSD analysis for DC

For study of 2UL/2DL, Table 6.1.24.4-1 lists up to 5th order harmonics and intermodulation products for DC_25A_n78A.

Table 6.1.24.4-1: Harmonic and IMD analysis

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 1850 | 1915 | 3300 | 3800 |
| 2nd harmonic | 2* f1_low | 2*f1_high | 2*f2_low | 2*f2_high |
| harmonic frequency limit (MHz) | 3700 | 3830 | 6600 | 7600 |
| 3rd harmonic | 3* f1_low | 3*f1_high | 3*f2_low | 3*f2_high |
| harmonic frequency limit (MHz) | 5550 | 5745 | 9900 | 11400 |
| 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency limit (MHz) | 1385 | 1950 | 5150 | 5715 |
| 3rd order IMD products | 2*f1_low – f2_high | 2*f1_high – f2_low | 2*f2_low – f1_high | 2*f2_high – f1_low |
| IMD frequency limit (MHz) | 100 | 530 | 4685 | 5750 |
| 3rd order IMD products | 2*f1_low + f2_low | 2*f1_high + f2_high | 2*f2_low + f1_low | 2*f2_high + f1_high |
| IMD frequency limit (MHz) | 7000 | 7630 | 8450 | 9515 |
| 4th order IMD products | 3*f1_low – f2_high | 3*f1_high – f2_low | 3*f2_low – f1_high | 3*f2_high – f1_low |
| IMD frequency limit (MHz) | 1750 | 2445 | 7985 | 9550 |
| 4th order IMD products | 3*f1_low + f2_low | 3*f1_high + f2_high | 3*f2_low + f1_low | 3*f2_high + f1_high |
| IMD frequency limit (MHz) | 8850 | 9545 | 11750 | 13315 |
| 4th order IMD products | 2*f1_low – 2*f2_high | 2*f1_high – 2*f2_low | 2*f1_low + 2*f2_low | 2*f1_high + 2*f2_high |
| IMD frequency limit (MHz) | 3900 | 2770 | 10300 | 11430 |
| 5th order IMD products | f1_low – 4*f2_high | f1_high – 4*f2_low | f2_low – 4*f1_high | f2_high – 4*f1_low |
| IMD frequency limit (MHz) | 13350 | 11285 | 4360 | 3600 |
| 5th order IMD products | f1_low + 4*f2_low | f1_high + 4*f2_high | f2_low + 4*f1_low | f2_high + 4*f1_high |
| IMD frequency limit (MHz) | 15050 | 17115 | 10700 | 11460 |
| 5th order IMD products | 2*f1_low – 3*f2_high | 2*f1_high - 3*f2_low | 2*f2_low – 3*f1_high | 2*f2_high – 3*f1_low |
| IMD frequency limit (MHz) | 7700 | 6070 | 855 | 2050 |
| 5th order IMD products | 2*f1_low + 3*f2_low | 2*f1_high + 3*f2_high | 2*f2_low + 3*f1_low | 2*f2_high + 3*f1_high |
| IMD frequency limit (MHz) | 13600 | 15230 | 12150 | 13345 |

Table 6.1.24.4-2: Band 25 and Band n78 UL harmonic mixing products

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| 25 | 1850 | 1915 | 1930 | 1995 | 3860 | 3990 | 5790 | 5985 | 7720 | 7980 | 9650 | 9975 |
| n78 | 3300 | 3800 | 3300 | 3800 | 6600 | 7600 | 9900 | 11400 | 13200 | 15200 | 16500 | 19000 |

Based on co-existence study as presented in the table 6.1.24.4-1 and 6.1.24.4-2, own Rx impact is shown in the following.

-	The 2nd harmonic of Band 25 uplink may fall into own Rx of band n78

-	The 2nd, 4th and 5th order IMD generated by dual uplink of Band 25 + Band n78 may fall into own Rx of band 25

-	The 4th and 5th order IMD generated by dual uplink of Band 25 + Band n78 may fall into own Rx of Band n78

The same MSD as CA_n25-n78 in TS 38.101-1 can be applied to DC_25_n78 as specified in Table 6.1.24.4-3 and 6.1.24.4-4 for the harmonic issue.

Table 6.1.24.4-3: MSD due to harmonic issue for DC_25A_n78A

| MSD due to harmonic exception for the DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) |  | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| 25 | n782, 4 |  | 23.9 | 22.1 | 20.9 | 19.8 | 19.0 | 17.9 | 16.8 | 16.0 | 15.5 | 14.8 |  | 14.3 | 13.8 |
|  | n783 |  | 1.1 | 0.8 | 0.3 | 0.1 | 0 | 0 | 0 | 0 | 0 | 0 |  | 0 | 0 |
| NOTE 2: The requirements should be verified for UL EARFCN or NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 3: The requirements are only applicable to channel bandwidths no larger than 20 MHz and with a carrier frequency at MHz offset from in the victim (higher band) with , where andare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 4: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.1.24.4-4 Uplink configuration due to UL harmonic interference

|  |  | NR Band / Channel bandwidth of the affected DL band / UL RB allocation of the aggressor band |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 30 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 70 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| 25 | n78 |  | 25 | 36 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 |

The same MSD as CA_n25-n77 in TS 38.101-1 can be applied to DC_25_n78 as specified in Table 6.1.24.4-5.

Table 6.1.24.4-5: MSD due to IMD issue

|  | Operating band/ Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC CA Configuration | Operating band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| DC_25A_n78A DC_25A-25A_n78A | 25 | 1855 | 5 | 25 | 1935 | 26 | FDD | IMD2 |
|  | n78 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
|  | 25 | 1885 | 5 | 25 | 1965 | 8 | FDD | IMD4 |
|  | n78 | 3690 | 10 | 50 | 3690 | N/A | TDD | N/A |
|  | 25 | 1875 | 5 | 25 | 1955 | 5 | FDD | IMD5 |
|  | n78 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |

6.1.24.5	∆TIB and ∆RIB values

For DC_25_n78, the ΔTIB,c and ΔRIB,c values are given the same as CA_n25-n78 in the tables below.

Table 6.1.24.5-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_25_n78 DC_25-25_n78 | 25 | 0.6 |
|  | n78 | 0.8 |

Table 6.1.24.5-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_25_n78 DC_25-25_n78 | 25 | 0.2 |
|  | n78 | 0.5 |

6.1.25	DC_21_n28

6.1.25.1	Configuration for DC

Table 6.1.25.1-1: Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration | Single UL allowed |
| --- | --- | --- |
| DC_21A_n28AX1 | DC_21A_n28A | DC_21_n28 |
| NOTE X1: The frequency range in band n28 is restricted for this band combination to 728 - 738 MHz for the UL and 783 - 793 MHz for the DL. This restriction applies also for these band combinations when applicable EN-DC configuration is part of a higher order EN-DC configuration. |  |  |

6.1.25.2	Maximum output power for DC

Table 6.1.25.2-1: Maximum output power for inter-band EN-DC (two bands)

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_21A_n28A | 23 | +2/-3 |

6.1.25.3	Spurious emission band UE co-existence for DC

Table 6.1.25.3-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_21_n28 | E-UTRA Band 1, 42, 65, NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |
|  | E-UTRA Band 3, 18, 19, 34, NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 5, 17 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 14 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 5 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180 kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 9: Applicable when the assigned E-UTRA carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 10: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 11: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 14: This requirement is applicable for 5 and 10 MHz E-UTRA channel bandwidth allocated within 718-728MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and RBstart < 48. NOTE 17: This requirement is applicable in the case of a 10 MHz E-UTRA carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. |  |  |  |  |  |  |  |

6.1.25.4	MSD analysis for DC

For 2UL/2DL UE coexistence study 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.1.25.4-1.

Table 6.1.25.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1447.9 | 1462.9 | 703 | 748 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 2895.8 | 2925.8 | 1406 | 1496 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 4343.7 | 4388.7 | 2109 | 2244 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 5791.6 | 5851.6 | 2812 | 2992 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 7239.5 | 7314.5 | 3515 | 3740 |
| 6th harmonics frequency limits | 6*fx_low | 6*fx_high | 6* fy_low | 6* fy_high |
| 6th harmonics frequency limits (MHz) | 8687.4 | 8777.4 | 4218 | 4488 |
| 7th harmonics frequency limits | 7*fx_low | 7*fx_high | 7* fy_low | 7* fy_high |
| 7th harmonics frequency limits (MHz) | 10135.3 | 10240.3 | 4921 | 5236 |
| Two tone 2nd order IMD products | ¦fx_low – fy_high¦ | ¦fx_high – fy_low¦ | ¦fx_low + fy_low¦ | ¦fx_high + fy_high¦ |
| IMD frequency limits (MHz) | 699.9 | 759.9 | 2150.9 | 2210.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2147.8 | 2222.8 | 56.9 | 48.1 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3598.8 | 3673.8 | 2853.9 | 2958.9 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3595.7 | 3685.7 | 646.1 | 796.1 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 1399.8 | 1519.8 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5046.7 | 5136.7 | 3556.9 | 3706.9 |
| Two-tone 4th order IMD products | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |  |  |
| IMD frequency limits (MHz) | 4301.8 | 4421.8 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1544.1 | 1349.1 | 5148.6 | 5043.6 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 651.8 | 816.8 | 2982.7 | 2847.7 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4259.9 | 4454.9 | 6494.6 | 6599.6 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5004.8 | 5169.8 | 5749.7 | 5884.7 |

Based on Table 6.1.25.4-1, it can be seen that

-	the 2nd order harmonic may fall into Rx frequencies of band 21.

-	the 2nd order IMD may fall into Rx frequencies of band n28.

-	the 4th order IMD may fall into Rx frequencies of bands 21 and n28.

-	the 5th order IMD may fall into Rx frequencies of bands 21 and n28.

6.1.25.5	∆TIB and ∆RIB values

For DC_21_n28, the TIB,c and RIB,c values are reused from the LTE combination CA_21-28, and are given in the tables below.

Table 6.1.25.5-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_21_n28 | 21 | 0.4 |
|  | n28 | 0.3 |

Table 6.1.25.5-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_21_n28 | 21 | 0 |
|  | n28 | 0 |

6.1.25.6		Self-interference analysis

The 2nd order harmonic and the 4th order IMD may fall into Rx frequencies of band 21.

-	Harmonic and IMD was calculated based on the frequency range in band n28 that the operator actually owned, which resulted in that 2nd order harmonic and IMD4 doesn’t fall into own Rx of band 21. Therefore, we didn’t specify MSD.

The 2nd, 4th and 5th order IMD may fall into Rx frequencies of band n28.

-	IMD was calculated based on the frequency range in band n28 that the operator actually owned, which resulted in that IMD2, IMD4 and IMD5 doesn’t fall into own Rx of band n28. Therefore, we didn’t specify MSD.

The 5th order IMD may fall into Rx frequencies of bands 21.

-	For DC_21_n28, REFSENS exceptions are reused from LTE CA_21-28, already defined in TS 36.101. The MSD values are shown in the following table.

Table 6.1.25.6-1: Reference sensitivity exceptions (MSD) due to UL harmonic for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n28 | 21X1 | N/A | N/A | N/A |  |  |  |  |  |  |  |  |  |  |
| NOTE X1: The frequency range in band n28 is restricted for this band combination to 728 - 738 MHz for the UL. This band is subject to 2nd harmonic fall in B21 also which MSD is not specified. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.1.25.6-2: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_21A_n28AX1 | 21 | 1450.4 | 5 | 25 | 1498.4 | 2.5 | IMD5 |
|  | n28 | 735.5 | 5 | 25 | 790.5 | N/A | N/A |
| NOTE X1: The frequency range in band n28 is restricted for this band combination to 728 - 738 MHz for the UL and 783 - 793 MHz for the DL. This band is subject to IMD2, IMD4 and IMD5 fall in n28 also which MSD is not specified. In addition, this band is subject to IMD4 fall in B21 also which MSD is not specified. |  |  |  |  |  |  |  |

6.1.26	DC_12_n71

6.1.26.1	Configuration for DC

Table 6.1.26.1-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_12A_n71A | DC_12A_n71AX,Y | Yes |
| NOTE X: Only single switched UL is supported. NOTE Y: The implementation with 4 antennas is targeted for FWA form factor for this band combination. |  |  |

6.1.26.2	Maximum output power for DC

Table 6.1.26.2-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_12A_n71AX | 23 | +2/-3 |
| NOTE X: Only single switched UL is supported. |  |  |

6.1.26.3	Spurious emission band UE co-existence for DC

Note that only Single Tx Switched UL mode is supported for this combination, spurious emission band UE co-existence requirement for band 12 and n71 respectively is applied.

Table 6.1.26.3-1: Requirements for band 12 spurious emissions for UE co-existence

| E-UTRA Band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| 12 | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 27, 30, 41, 48, 53, 71, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 10, 50, 51, 66, 70, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 12, 85 | FDL_low | - | FDL_high | -50 | 1 | 15 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 15: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

Table 6.1.26.3-2: Requirements for band n71 spurious emissions for UE co-existence

| NR Band | Spurious emission for UE co-existence |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| n71 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 30, 48, 53, 66, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25, 41, 70, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 15 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -50 | 1 | 15 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x RBsize kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 15: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

6.1.26.4	MSD analysis for DC

Note that only Single Tx Switched UL mode is supported for this combination, so the MSD requirements due to cross band isolation need to be checked. Based on the analysis in the discussion part, the MSD requirements due to cross band isolation can be specified as below.

Table 6.1.26.4-1: Reference sensitivity exceptions (MSD) due to cross band isolation for PC3 EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n71 | 12 | 2.3 | 2.3 |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied. NOTE 2: The B41 requirements are modified by -0.5dB when carrier frequency of the assigned E-UTRA channel bandwidth is within 2515 – 2690 MHz. NOTE 3: These requirements apply when the uplink is active in Band n1, n84 and the separation between the lower edge of the uplink channel in Band n1, n84 and the upper edge of the downlink channel in Band 3 is < 60 MHz. For each channel bandwidth in Band 3, the requirement applies regardless of channel bandwidth in Band n1, n84. NOTE 4: The DL victim band should be configured using the lowest SCS that is compatible with the highest CBW for which an MSD is specified. NOTE 5: MSD test point can be chosen according to supported BW and lowest SCS supported by the UE. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.1.26.4-2: Uplink configuration for reference sensitivity exceptions due to cross band isolation for EN-DC in NR FR1

| E-UTRA or NR Band / SCS / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 30 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 70 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| n71 | 12 | 15 | 20 | 20 |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: The UL configuration applies regardless of the channel bandwidth of the UL band. UL resource blocks allocation in the table shall be further limited to that specified in Table 7.3.1-2 in TS 36.101 [4] or Table 7.3.2-3 in TS 38.101-1 [2]. NOTE 2: The UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth. NOTE 3: When the maximum UL RB allocation “LCRB” value is less than the maximum transmission bandwidth configuration “NRB” defined in Table 5.3.2-1 in 38.101-1 [2] for the specified UL band SCS, the UL band should be configured using the lowest CBW that is compatible with the maximum specified LCRB value. NOTE 4: If the aggressor band is NR band, the test SCS and UL RB can be adjusted according to supported BW and lowest SCS supported by the UE. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.1.26.5	∆TIB and ∆RIB values

For DC_12_n71, the ΔTIB,c and ΔRIB,c values are given in the tables below.

Table 6.1.26.5-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_12_n71 | 12 | 1 |
|  | n71 | 1 |

Table 6.1.26.5-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_12_n71 | 12 | 0.8 |
|  | n71 | 0.8 |

6.1.27	DC_71_n41

6.1.27.1	Operating bands for DC

Table 6.1.27.1-1: DC band combination of 1 LTE band + 1 NR band

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_71_n41 | 71 | n41 | No |

6.1.27.2	Configuration for DC

Table 6.1.27.2-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_71A_n41A | DC_71A_n41A | 71A | n41A |

6.1.27.3	Maximum output power for DC

Table 6.1.27.3-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_71A_n41A | 23 | +2/-3 |

6.1.27.4	Spurious emission band UE co-existence for DC

Table 6.1.27.4-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_71A_n41A | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 30, 48, 66, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 2, 25, 70 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA band 71 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 5 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180 kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

6.1.27.5	MSD analysis for DC

For 2UL/2DL UE coexistence study 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.1.27.5-1

Table 6.1.27.5-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 663 | 698 | 2496 | 2690 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high | 2*fx_low | 2*fx_high |
| 2nd harmonics frequency limits (MHz) | 1326 | 1396 | 4992 | 5380 |
| 3rd harmonics frequency limits | 3* fy_low | 3* fy_high | 3*fx_low | 3*fx_high |
| 3rd harmonics frequency limits (MHz) | 1989 | 2094 | 7488 | 8070 |
| 4th harmonics frequency limits | 4* fy_low | 4* fy_high | 4*fx_low | 4*fx_high |
| 4th harmonics frequency limits (MHz) | 2652 | 2792 | 9984 | 10760 |
| 5th harmonics frequency limits | 5* fy_low | 5* fy_high | 5*fx_low | 5*fx_high |
| 5th harmonics frequency limits (MHz) | 3315 | 3490 | 12480 | 13450 |
| 6th harmonics frequency limits | 6* fy_low | 6* fy_high | 6*fx_low | 6*fx_high |
| 6th harmonics frequency limits (MHz) | 3978 | 4188 | 14976 | 16140 |
| 7th harmonics frequency limits | 7* fy_low | 7* fy_high | 7*fx_low | 7*fx_high |
| 7th harmonics frequency limits (MHz) | 4641 | 4886 | 17472 | 18830 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2027 | 1798 | 3159 | 3388 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1364 | 1100 | 4294 | 4717 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3822 | 4086 | 5655 | 6078 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 4054 | 3596 | 6318 | 6776 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 701 | 402 | 6790 | 7407 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4485 | 4784 | 8151 | 8768 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 10097 | 9286 | 296 | 38 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10647 | 11458 | 5148 | 5482 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 6744 | 6092 | 2898 | 3391 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8814 | 9466 | 6981 | 7474 |

Based on Table 6.1.27.5-1, it can be seen that

-	4th order IMD products may fall into own Rx frequencies of band 71.

-	4th harmonics may fall into Rx frequencies of band n41

6.1.27.6	∆TIB and ∆RIB values

For DC_71_n41, the TIB,c and RIB,c values are reused from CA_n41-n71 and DC_7_n71 and are given in the tables below.

Table 6.1.27.6-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_71_n41 | 71 | 0.6 |
|  | n41 | 0.3 |

Table 6.1.27.6-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_71_n41 | 71 | 0.2 |
|  | n41 | 0 |

6.1.27.7	MSD

Co-existence studies shows that MSD need to be defined for IMD4 falling into own Rx frequencies of band 71. MSD value same as in CA_n41-n71.

Table 6.1.27.7-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_71A_n41A | 71 | 666 | 5 | 25 | 620 | 11 | IMD4 |
|  | n41 | 2618 | 5 | 25 | 2618 | N/A | N/A |

Reference exception due to 4th harmonics into DL n41 need to be defined as in tables below.

Table 6.1.27.7-2: Reference sensitivity exceptions (MSD) due to UL harmonic for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| 71 | n416,7 |  | 10.8 | 9.1 | 8.0 |  | 6.1 | 5.1 | 4.2 | 3.5 | 2.7 | 2.3 | 2.1 | 1.4 |
| NOTE 6: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 4th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 7: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.1.27.7-3: Uplink configuration for reference sensitivity exceptions due to UL harmonic interference for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 30 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 70 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| 71 | n41 | 15 |  | 16 | 25 | 25 |  | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 |

6.1.28	DC_71_n2

6.1.28.1	Operating bands for DC

Table 6.1.28.1-1: DC band combination of 1 LTE band + 1 NR band

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_71_n2 | 71 | n2 | No |

6.1.28.2	Configuration for DC

Table 6.1.28.2-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_71A_n2A | DC_71A_n2A | 71A | n2A |

6.1.28.3	Maximum output power for DC

Table 6.1.28.3-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_71A_n2A | 23 | +2/-3 |

6.1.28.4	Spurious emission band UE co-existence for DC

Table 6.1.28.4-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_71A_n2A | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 30, 48, 66 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 25, 41, 70, NR Band n2, n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA band 71 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 5 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180 kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

6.1.28.5	MSD analysis for DC

For 2UL/2DL UE coexistence study 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.1.28.5-1

Table 6.1.28.5-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 663 | 698 | 1850 | 1910 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high | 2*fx_low | 2*fx_high |
| 2nd harmonics frequency limits (MHz) | 1326 | 1396 | 3700 | 3820 |
| 3rd harmonics frequency limits | 3* fy_low | 3* fy_high | 3*fx_low | 3*fx_high |
| 3rd harmonics frequency limits (MHz) | 1989 | 2094 | 5550 | 5730 |
| 4th harmonics frequency limits | 4* fy_low | 4* fy_high | 4*fx_low | 4*fx_high |
| 4th harmonics frequency limits (MHz) | 2652 | 2792 | 7400 | 7640 |
| 5th harmonics frequency limits | 5* fy_low | 5* fy_high | 5*fx_low | 5*fx_high |
| 5th harmonics frequency limits (MHz) | 3315 | 3490 | 9250 | 9550 |
| 6th harmonics frequency limits | 6* fy_low | 6* fy_high | 6*fx_low | 6*fx_high |
| 6th harmonics frequency limits (MHz) | 3978 | 4188 | 11100 | 11460 |
| 7th harmonics frequency limits | 7* fy_low | 7* fy_high | 7*fx_low | 7*fx_high |
| 7th harmonics frequency limits (MHz) | 4641 | 4886 | 12950 | 13370 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1247 | 1152 | 2513 | 2608 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 584 | 454 | 3002 | 3157 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3176 | 3306 | 4363 | 4518 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2494 | 2304 | 5026 | 5216 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 79 | 244 | 4852 | 5067 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3839 | 4004 | 6213 | 6428 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6977 | 6702 | 942 | 742 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8063 | 8338 | 4502 | 4702 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4404 | 4154 | 1606 | 1831 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6876 | 7126 | 5689 | 5914 |

It can be found that

-	There is no IMD

-	There is 3rd harmonic from 71 UL into band n2 DL.

-	There is 3rd harmonic mixing from 71 DL into band n2 UL

6.1.28.6	∆TIB and ∆RIB values

For DC_71_n2, the TIB,c and RIB,c values are reused from DC_2_n71 and are given in the tables below.

Table 6.1.28.6-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_71_n2 | 71 | 0.3 |
|  | n2 | 0.3 |

Table 6.1.28.6-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_71_n2 | 71 | 0 |
|  | n2 | 0 |

6.1.28.7	MSD

Reference exception due to 3rd harmonics and harmonic mixing into DL n2 need to be defined as in tables below.

Table 6.1.28.7-1: Reference sensitivity exceptions (MSD) due to UL harmonic for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| 71 | n211 | 4.6 | 1.0 | 0.7 | 0.6 |  |  |  |  |  |  |  |  |  |
|  | n212 | 1.7 | 1.0 | 0.7 | 0.6 |  |  |  |  |  |  |  |  |  |
| NOTE 11: These requirements apply when the lower edge frequency of the 5 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. NOTE 12: These requirements apply when the lower edge frequency of the 10 MHz, 15 MHz, or 20 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.1.28.7-2: Uplink configuration for reference sensitivity exceptions due to UL harmonic interference for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 30 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 70 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| 71 | n2 | 15 | 254 85 | 254 85 | 204 85 | 204 85 |  |  |  |  |  |  |  |  |  |
| NOTE 4: These requirements apply when the lower edge frequency of the 5 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. NOTE 5: These requirements apply when the lower edge frequency of the 10 MHz, 15 MHz, or 20 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.1.28.7-3: Reference sensitivity exceptions (MSD) due to receiver harmonic mixing for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n2 | 714 | 26.8 | 23.6 | 21.2 | 15.6 |  |  |  |  |  |  |  |
| NOTE 4: The requirements should be verified for DL EARFCN or NR ARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.1.28.7-4: Uplink configuration for reference sensitivity exceptions due to receiver harmonic mixing for EN-DC in NR FR1

| E-UTRA or NR Band / SCS / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| n2 | 71 | 15 | 25 | 50 | 50 | 50 |  |  |  |  |  |  |  |

6.1.29	DC_2_n30

6.1.29.1	Operating bands for DC

Table 6.1.29.1-1: DC band combination of 1 LTE band + 1 NR band

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_2_n30 | 2 | n30 | No |

6.1.29.2	Configuration for DC

Table 6.1.29.2-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_2A_n30A | DC_2A_n30A | 2A | n30A |

6.1.29.3	Maximum output power for DC

Table 6.1.29.3-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_2A_n30A | 23 | +2/-3 |

6.1.29.4	Spurious emission band UE co-existence for DC

Table 6.1.29.4-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_2A_n30A | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 27, 29, 30, 41, 48, 53, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180 kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

6.1.29.5	MSD analysis for DC

For 2UL/2DL UE coexistence study 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.1.29.5-1.

Table 6.1.29.5-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 2305 | 2315 |
| DL frequency (MHz) | 1930 | 1990 | 2350 | 2360 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3700 | 3820 | 4610 | 4630 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5550 | 5730 | 6915 | 6945 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 7400 | 7640 | 11525 | 9260 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 9250 | 9550 | 11525 | 11575 |
| Two tone 2nd order IMD products | ¦fy_low - fx_high¦ | ¦fy_high - fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 395 | 465 | 4155 | 4225 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1385 | 1515 | 2700 | 2780 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6005 | 6135 | 6460 | 6540 |
| Two-tone 4th order IMD products | ¦3*fx_low - fy_high¦ | ¦3*fx_high - fy_low¦ | ¦3*fy_low - fx_high¦ | ¦3*fy_high - fx_low¦ |
| IMD frequency limits (MHz) | 3235 | 3425 | 5005 | 5095 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7855 | 8045 | 8765 | 8855 |
| Two-tone 4th order IMD products | ¦2*fx_low - 2*fy_high¦ | ¦2*fx_high - 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 930 | 790 | 8310 | 8450 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7410 | 7310 | 5335 | 5085 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11070 | 11170 | 9705 | 9955 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3245 | 3095 | 1120 | 920 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10615 | 10765 | 10160 | 10360 |

Based on above table,

-	There is no harmonic interference to own higher DL band

-	There is no intermodulation interference to own DL bands

6.1.29.6	∆TIB and ∆RIB values

For DC_2_30, the TIB,c and RIB,c values are given in the tables below and are re-used from E-UTRA CA_2-30.

Table 6.1.29.6-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2_n30 | 2 | 0.5 |
|  | n30 | 0.3 |

Table 6.1.29.6-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2_n30 | 2 | 0.4 |
|  | n30 | 0.5 |

6.1.30	DC_5_n30

6.1.30.1	Operating bands for DC

Table 6.1.30.1-1: DC band combination of 1 LTE band + 1 NR band

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_5_n30 | 5 | n30 | No |

6.1.30.2	Configuration for DC

Table 6.1.30.2-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_5A_n30A | DC_5A_n30A | 5A | n30A |

6.1.30.3	Maximum output power for DC

Table 6.1.30.3-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_5A_n30A | 23 | +2/-3 |

6.1.30.4	Spurious emission band UE co-existence for DC

Table 6.1.30.4-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_5A_n30A | E-UTRA Band 2, 4, 5, 7,12, 13, 14, 17, 24, 25, 26, 29, 30, 38, 48, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41, 53 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180 kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz |  |  |  |  |  |  |  |

6.1.30.5	MSD analysis for DC

For 2UL/2DL UE coexistence study 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.1.30.5-1.

Table 6.1.30.5-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 2305 | 2315 |
| DL frequency (MHz) | 869 | 894 | 2350 | 2360 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 | 1698 | 4610 | 4630 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 | 2547 | 6915 | 6945 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3296 | 3396 | 11525 | 9260 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 4120 | 4245 | 11525 | 11575 |
| Two tone 2nd order IMD products | ¦fy_low - fx_high¦ | ¦fy_high - fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1456 | 1491 | 3129 | 3164 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 667 | 607 | 3761 | 3806 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3953 | 4013 | 5434 | 5479 |
| Two-tone 4th order IMD products | ¦3*fx_low - fy_high¦ | ¦3*fx_high - fy_low¦ | ¦3*fy_low - fx_high¦ | ¦3*fy_high - fx_low¦ |
| IMD frequency limits (MHz) | 157 | 242 | 6066 | 6121 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4777 | 4862 | 7739 | 7794 |
| Two-tone 4th order IMD products | ¦2*fx_low - 2*fy_high¦ | ¦2*fx_high - 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2982 | 2912 | 6258 | 6328 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8436 | 8371 | 1091 | 981 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10044 | 10109 | 5601 | 5711 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 5297 | 5217 | 2063 | 2158 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8563 | 8643 | 7082 | 7177 |

Based on above table,

-	There is no harmonic interference to own higher DL band

-	There is no intermodulation interference to own DL bands

6.1.30.6	∆TIB and ∆RIB values

For DC_5_n30, the TIB,c and RIB,c values are given in the tables below and are re-used from E-UTRA CA_5-30.

Table 6.1.30.6-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5_n30 | 5 | 0.3 |
|  | n30 | 0.3 |

Table 6.1.30.6-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5_n30 | 5 | 0 |
|  | n30 | 0 |

6.1.31	DC_12_n30

6.1.31.1	Operating bands for DC

Table 6.1.31.1-1: DC band combination of 1 LTE band + 1 NR band

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_12_n30 | 12 | n30 | No |

6.1.31.2	Configuration for DC

Table 6.1.31.2-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_12A_n30A | DC_12A_n30A | 12A | n30A |

6.1.31.3	Maximum output power for DC

Table 6.1.31.3-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_12A_n30A | 23 | +2/-3 |

6.1.31.4	Spurious emission band UE co-existence for DC

Table 6.1.31.4-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_12A_n30A | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 27, 30, 41, 48, 53, 71 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 66, 70, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 12, 85 | FDL_low | - | FDL_high | -50 | 1 | 5 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180 kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz |  |  |  |  |  |  |  |

6.1.31.5	MSD analysis for DC

For 2UL/2DL UE coexistence study 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.1.31.5-1.

Table 6.1.31.5-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 699 | 716 | 2305 | 2315 |
| DL frequency (MHz) | 729 | 746 | 2350 | 2360 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1398 | 1432 | 4610 | 4630 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2097 | 2148 | 6915 | 6945 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2796 | 2864 | 11525 | 9260 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3495 | 3580 | 11525 | 11575 |
| Two tone 2nd order IMD products | ¦fy_low - fx_high¦ | ¦fy_high - fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1589 | 1616 | 3004 | 3031 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 917 | 873 | 3894 | 3931 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3703 | 3747 | 5309 | 5346 |
| Two-tone 4th order IMD products | ¦3*fx_low - fy_high¦ | ¦3*fx_high - fy_low¦ | ¦3*fy_low - fx_high¦ | ¦3*fy_high - fx_low¦ |
| IMD frequency limits (MHz) | 218 | 157 | 6199 | 6246 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4402 | 4463 | 7614 | 7661 |
| Two-tone 4th order IMD products | ¦2*fx_low - 2*fy_high¦ | ¦2*fx_high - 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3232 | 3178 | 6008 | 6062 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8561 | 8504 | 559 | 481 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9919 | 9976 | 5101 | 5179 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 5547 | 5483 | 2462 | 2533 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8313 | 8377 | 6707 | 6778 |

Based on above table,

-	There is no harmonic interference to own higher DL band

-	There is no intermodulation interference to own DL bands

6.1.31.6	∆TIB and ∆RIB values

For DC_12_n30, the TIB,c and RIB,c values are given in the tables below and are re-used from E-UTRA CA_12-30.

Table 6.1.31.6-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_12_n30 | 12 | 0.3 |
|  | n30 | 0.3 |

Table 6.1.31.6-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_12_n30 | 12 | 0 |
|  | n30 | 0 |

6.1.32	DC_66_n30

6.1.32.1	Operating bands for DC

Table 6.1.32.1-1: DC band combination of 1 LTE band + 1 NR band

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_66_n30 | 66 | n30 | No |

6.1.32.2	Configuration for DC

Table 6.1.32.2-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_66A_n30A | DC_66A_n30A | 66A | n30A |

6.1.32.3	Maximum output power for DC

Table 6.1.32.3-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_66A_n30A | 23 | +2/-3 |

6.1.32.4	Spurious emission band UE co-existence for DC

Table 6.1.32.4-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_66A_n30A | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 24, 25, 26, 27, 29, 30, 38, 41, 53, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 48, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180 kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. |  |  |  |  |  |  |  |

6.1.32.5	MSD analysis for DC

For 2UL/2DL UE coexistence study 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.1.32.5-1.

Table 6.1.32.5-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1780 | 2305 | 2315 |
| DL frequency (MHz) | 2110 | 2200 | 2350 | 2360 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3560 | 4610 | 4630 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5340 | 6915 | 6945 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 6840 | 7120 | 11525 | 9260 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 8550 | 8900 | 11525 | 11575 |
| Two tone 2nd order IMD products | ¦fy_low - fx_high¦ | ¦fy_high - fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 525 | 605 | 4015 | 4095 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1105 | 1255 | 2830 | 2920 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5725 | 5875 | 6320 | 6410 |
| Two-tone 4th order IMD products | ¦3*fx_low - fy_high¦ | ¦3*fx_high - fy_low¦ | ¦3*fy_low - fx_high¦ | ¦3*fy_high - fx_low¦ |
| IMD frequency limits (MHz) | 2815 | 3035 | 5135 | 5235 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7435 | 7655 | 8625 | 8725 |
| Two-tone 4th order IMD products | ¦2*fx_low - 2*fy_high¦ | ¦2*fx_high - 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1210 | 1050 | 8030 | 8190 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7550 | 7440 | 4815 | 4525 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10930 | 11040 | 9145 | 9435 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3525 | 3355 | 730 | 500 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10335 | 10505 | 9740 | 9970 |

Based on above table,

-	There is no harmonic interference to own higher DL band

-	There is no intermodulation interference to own DL bands

6.1.32.6	∆TIB and ∆RIB values

For DC_66_n30, the TIB,c and RIB,c values are given in the tables below and are re-used from E-UTRA CA_30-66.

Table 6.1.32.6-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66_n30 | 66 | 0.5 |
|  | n30 | 0.8 |

Table 6.1.32.6-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_66_n30 | 66 | 0.5 |
|  | n30 | 0.4 |

6.1.33	DC_11_n41

6.1.33.1	Configuration for DC

Table 6.1.33.1-1:  Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_11A_n41A | DC_11A_n41A | No |

6.1.33.2	Maximum output power for DC

Table 6.1.33.2-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_11A_n41A | 23 | +2/-3 |

6.1.33.3	Spurious emission band UE co-existence for DC

Table 6.1.33.3-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_11_n41 | E-UTRA Band 1, 3, 11, 18, 19, 21, 28, 34, 42, 65 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180 kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz |  |  |  |  |  |  |  |

6.1.33.4	MSD analysis for DC

For 2UL/2DL UE coexistence study 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.1.33.4-1

Table 6.1.33.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1427.9 | 1447.9 | 2496 | 2690 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high | 2*fx_low | 2*fx_high |
| 2nd harmonics frequency limits (MHz) | 2855.8 | 2895.8 | 4992 | 5380 |
| 3rd harmonics frequency limits | 3* fy_low | 3* fy_high | 3*fx_low | 3*fx_high |
| 3rd harmonics frequency limits (MHz) | 4283.7 | 4343.7 | 7488 | 8070 |
| 4th harmonics frequency limits | 4* fy_low | 4* fy_high | 4*fx_low | 4*fx_high |
| 4th harmonics frequency limits (MHz) | 5711.6 | 5791.6 | 9984 | 10760 |
| 5th harmonics frequency limits | 5* fy_low | 5* fy_high | 5*fx_low | 5*fx_high |
| 5th harmonics frequency limits (MHz) | 7139.5 | 7239.5 | 12480 | 13450 |
| 6th harmonics frequency limits | 6* fy_low | 6* fy_high | 6*fx_low | 6*fx_high |
| 6th harmonics frequency limits (MHz) | 8567.4 | 8687.4 | 14976 | 16140 |
| 7th harmonics frequency limits | 7* fy_low | 7* fy_high | 7*fx_low | 7*fx_high |
| 7th harmonics frequency limits (MHz) | 9995.3 | 10135.3 | 17472 | 18830 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1262.1 | 1048.1 | 3923.9 | 4137.9 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 399.8 | 165.8 | 3544.1 | 3952.1 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5351.8 | 5585.8 | 6419.9 | 6827.9 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2096.2 | 2524.2 | 7847.8 | 8275.8 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1593.7 | 1847.7 | 6040.1 | 6642.1 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6779.7 | 7033.7 | 8915.9 | 9517.9 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8536.1 | 9332.1 | 3021.6 | 3295.6 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11411.9 | 12207.9 | 8207.6 | 8481.6 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4592.2 | 5214.2 | 648.3 | 1096.3 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10343.8 | 10965.8 | 9275.7 | 9723.7 |

Based on above, there is no harmonic/harmonic mixing and IMD issues.

6.1.33.5	∆TIB and ∆RIB values

For DC_11_n41, the TIB,c and RIB,c values are reused from CA_11-41 and are given in the tables below.

Table 6.1.33.5-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_11_n41 | 11 | 0.3 |
|  | n41 | 0.3 |

Table 6.1.33.5-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_11_n41 | 11 | 0 |
|  | n41 | 0 |

6.1.33.6	MSD

There is no additional MSD exception requirement.

6.1.34	DC_38_n28

6.1.34.1	Configuration for DC

Table 6.1.34.1-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_38A_n28A | DC_38A_n28A | No |

6.1.34.2	Maximum output power for DC

Table 6.1.34.2-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_38A_n28A | 23 | +2/-3 |

6.1.34.3	Spurious emission band UE co-existence for DC

Table 6.1.34.3-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_38_n28 | E-UTRA Band 2, 3, 5, 8, 20, 27, 31, 34, 40, 72, NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 4, 22, 32, 42, 43, 50, 51, 52, 65, 66, 74, 75, 76, NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 5, 17 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 14 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 5 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180 kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 14: This requirement is applicable for 5 and 10 MHz E-UTRA channel bandwidth allocated within 718-728MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and RBstart < 48. NOTE 17: This requirement is applicable in the case of a 10 MHz E-UTRA carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. |  |  |  |  |  |  |  |

6.1.34.4	MSD analysis for DC

For 2UL/2DL UE coexistence study 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.1.34.4-1.

Table 6.1.34.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 | 2570 | 2620 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1406 | 1496 | 5140 | 5240 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2109 | 2244 | 7710 | 7860 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2812 | 2992 | 10280 | 10480 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3515 | 3740 | 12850 | 13100 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1822 | 1917 | 3273 | 3368 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1214 | 1074 | 4392 | 4537 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3976 | 4116 | 5843 | 5988 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 511 | 326 | 6962 | 7157 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4679 | 4864 | 8413 | 8608 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3834 | 3644 | 6546 | 6736 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9777 | 9532 | 422 | 192 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6454 | 6214 | 2896 | 3131 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10983 | 11228 | 5382 | 5612 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9116 | 9356 | 7249 | 7484 |

There is no harmonic and IMD issue for this band combination.

6.1.34.5	∆TIB and ∆RIB values

For DC_38_n28, the ΔTIB,c and ΔRIB,c values are given in the tables below.

Table 6.1.34.5-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_38_n28 | 38 | 0.3 |
|  | n28 | 0.3 |

Table 6.1.34.5-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_38_n28 | 38 | 0 |
|  | n28 | 0 |

6.1.35	DC_12_n77

6.1.35.1	Operating bands for DC

Table 6.1.35.1-1: DC band combination of 1 LTE band + 1 NR band

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_12_n77 | 12 | n77 | DC_12_n77 |

6.1.35.2	Configuration for DC

Table 6.1.35.2-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_12A_n77A | DC_12A_n77A | 12A | n77A |

6.1.35.3	Maximum output power for DC

Table 6.1.35.3-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_12A_n77A | 23 | +2/-3 |

6.1.35.4	Spurious emission band UE co-existence for DC

Table 6.1.35.4-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_12_n77 | E-UTRA Band 2, 5, 7. 13, 17, 24, 25, 26, 27, 30, 41, 53, 70, 71 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 66 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA band 12, 85 | FDL_low | - | FDL_high | -50 | 1 | 5 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180 kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

6.1.35.5	MSD analysis for DC

For 2UL/2DL UE coexistence study 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.1.35.5-1

Table 6.1.35.5-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 699 | 716 | 3300 | 4200 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high | 2*fx_low | 2*fx_high |
| 2nd harmonics frequency limits (MHz) | 1398 | 1432 | 6600 | 8400 |
| 3rd harmonics frequency limits | 3* fy_low | 3* fy_high | 3*fx_low | 3*fx_high |
| 3rd harmonics frequency limits (MHz) | 2097 | 2148 | 9900 | 12600 |
| 4th harmonics frequency limits | 4* fy_low | 4* fy_high | 4*fx_low | 4*fx_high |
| 4th harmonics frequency limits (MHz) | 2796 | 2864 | 13200 | 16800 |
| 5th harmonics frequency limits | 5* fy_low | 5* fy_high | 5*fx_low | 5*fx_high |
| 5th harmonics frequency limits (MHz) | 3495 | 3580 | 16500 | 21000 |
| 6th harmonics frequency limits | 6* fy_low | 6* fy_high | 6*fx_low | 6*fx_high |
| 6th harmonics frequency limits (MHz) | 4194 | 4296 | 19800 | 25200 |
| 7th harmonics frequency limits | 7* fy_low | 7* fy_high | 7*fx_low | 7*fx_high |
| 7th harmonics frequency limits (MHz) | 4893 | 5012 | 23100 | 29400 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3501 | 2584 | 3999 | 4916 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2802 | 1868 | 5884 | 7701 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4698 | 5632 | 7299 | 9116 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 7002 | 5168 | 7998 | 9832 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2103 | 1152 | 9184 | 11901 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5397 | 6348 | 10599 | 13316 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 16101 | 12484 | 436 | 1404 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 13899 | 17516 | 6096 | 7064 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 11202 | 8468 | 4452 | 6303 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11298 | 14032 | 8697 | 10548 |

Based on Table 6.1.35.5-1, it can be seen that

-	5th order harmonics may fall into own Rx frequencies of band n77.

-	n77 UL might affect 12 DL via 5th order harmonic mixing

-	5th order IMD products may fall into own Rx frequencies of band 12.

6.1.35.6	∆TIB and ∆RIB values

For DC_12_n77, the TIB,c and RIB,c values are reused from DC_12_n78 and are given in the tables below.

Table 6.1.35.6-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_12_n77 | 12 | 0.5 |
|  | n77 | 0.8 |

Table 6.1.35.6-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_12_n77 | 12 | 0.2 |
|  | n77 | 0.5 |

6.1.35.7	MSD

Co-existence studies shows that MSD need to be defined for IMD5 falling into own Rx frequencies of band 12. MSD value same as in DC_12_n78.

Table 6.1.35.7-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_12A_n77A | 12 | 710 | 5 | 20 | 740 | 5.5 | IMD5 |
|  | n77 | 3580 | 10 | 50 | 3580 | N/A | N/A |

Co-existence studies shows that 5th order harmonic and harmonic mixing need to be defined. MSD values as in DC_13_n77.

Table 6.1.35.7-2: Reference sensitivity exceptions (MSD) due to UL harmonic for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| 12 | n774,5 |  | 10.4 | 8.9 | 7.8 | 6.7 | 5.7 | 4.7 | 3.7 | 3 | 2.3 | 1.7 | 1.2 | 0.7 |
| NOTE 4: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 5th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 5: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.1.35.7-3: Uplink configuration for reference sensitivity exceptions due to UL harmonic interference for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 30 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 70 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| 12 | n77 | 15 |  | 10 | 15 | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 |

Table 6.1.35.7-4: Reference sensitivity exceptions (MSD) due to receiver harmonic mixing for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n77 | 122 | 31 | 28 |  |  |  |  |  |  |  |  |  |
| NOTE 2: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.1.35.7-5: Uplink configuration for reference sensitivity exceptions due to receiver harmonic mixing for EN-DC in NR FR1

| E-UTRA or NR Band / SCS / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| n77 | 12 | 15 | 25 | 50 |  |  |  |  |  |  |  |  |  |

6.1.36	DC_14_n77

6.1.36.1	Operating bands for DC

Table 6.1.36.1-1: DC band combination of 1 LTE band + 1 NR band

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_14_n77 | 14 | n77 | No |

6.1.36.2	Configuration for DC

Table 6.1.36.2-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_14A_n77A | DC_14A_n77A | 14A | n77A |

6.1.36.3	Maximum output power for DC

Table 6.1.36.3-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_14A_n77A | 23 | +2/-3 |

6.1.36.4	Spurious emission band UE co-existence for DC

Table 6.1.36.4-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_14_n77 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 27, 29, 30, 41, 53, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 | 5 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 5 |
| NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

6.1.36.5	MSD analysis for DC

For 2UL/2DL UE coexistence study 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.1.36.5-1

Table 6.1.36.5-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 788 | 798 | 3300 | 4200 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high | 2*fx_low | 2*fx_high |
| 2nd harmonics frequency limits (MHz) | 1576 | 1596 | 6600 | 8400 |
| 3rd harmonics frequency limits | 3* fy_low | 3* fy_high | 3*fx_low | 3*fx_high |
| 3rd harmonics frequency limits (MHz) | 2364 | 2394 | 9900 | 12600 |
| 4th harmonics frequency limits | 4* fy_low | 4* fy_high | 4*fx_low | 4*fx_high |
| 4th harmonics frequency limits (MHz) | 3152 | 3192 | 13200 | 16800 |
| 5th harmonics frequency limits | 5* fy_low | 5* fy_high | 5*fx_low | 5*fx_high |
| 5th harmonics frequency limits (MHz) | 3940 | 3990 | 16500 | 21000 |
| 6th harmonics frequency limits | 6* fy_low | 6* fy_high | 6*fx_low | 6*fx_high |
| 6th harmonics frequency limits (MHz) | 4728 | 4788 | 19800 | 25200 |
| 7th harmonics frequency limits | 7* fy_low | 7* fy_high | 7*fx_low | 7*fx_high |
| 7th harmonics frequency limits (MHz) | 5516 | 5586 | 23100 | 29400 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3412 | 2502 | 4088 | 4998 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2624 | 1704 | 5802 | 7612 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4876 | 5796 | 7388 | 9198 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 6824 | 5004 | 8176 | 9996 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1836 | 906 | 9102 | 11812 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5664 | 6594 | 10688 | 13398 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 16012 | 12402 | 108 | 1048 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 13988 | 17598 | 6452 | 7392 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 11024 | 8304 | 4206 | 6036 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11476 | 14196 | 8964 | 10794 |

Based on Table 6.1.36.5-1, it can be seen that

-	5th order harmonics may fall into own Rx frequencies of band n77.

-	n77 UL might affect 14 DL via 5th order harmonic mixing

-	5th order IMD products may fall into own Rx frequencies of band 14.

6.1.36.6	∆TIB and ∆RIB values

For DC_14_n77, the TIB,c and RIB,c values are reused from DC_13_n77 and are given in the tables below.

Table 6.1.36.6-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_14_n77 | 14 | 0.5 |
|  | n77 | 0.8 |

Table 6.1.36.6-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_14_n77 | 14 | 0.2 |
|  | n77 | 0.5 |

6.1.36.7	MSD

Co-existence studies shows that MSD need to be defined for IMD5 falling into own Rx frequencies of band 14. MSD value same as in DC_13_n77.

Table 6.1.36.7-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_14A_n77A | 14 | 795.5 | 5 | 15 | 765.5 | 5.5 | IMD5 |
|  | n77 | 3947.5 | 10 | 50 | 3947.5 | N/A | N/A |

Co-existence studies shows that 5th or harmonic and harmonic mixing need to be defined. Harmonic MSD values same as in DC_13_n77.

Table 6.1.36.7-2: Reference sensitivity exceptions (MSD) due to UL harmonic for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| 14 | n774, 5 |  | 10.4 | 8.9 | 7.8 | 6.7 | 5.7 | 4.7 | 3.7 | 3 | 2.3 | 1.7 | 1.2 | 0.7 |
| NOTE 4: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 5th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 5: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.1.36.7-3: Uplink configuration for reference sensitivity exceptions due to UL harmonic interference for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 30 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 70 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| 14 | n77 | 15 |  | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 |

Table 6.1.36.7-4: Reference sensitivity exceptions (MSD) due to receiver harmonic mixing for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n77 | 142 | 31 | 28 |  |  |  |  |  |  |  |  |  |
| NOTE 2: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.1.36.7-5: Uplink configuration for reference sensitivity exceptions due to receiver harmonic mixing for EN-DC in NR FR1

| E-UTRA or NR Band / SCS / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| n77 | 14 | 15 | 25 | 50 |  |  |  |  |  |  |  |  |  |

6.1.37	DC_30_n77

6.1.37.1	Operating bands for DC

Table 6.1.37.1-1: DC band combination of 1 LTE band + 1 NR band

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_30_n77 | 30 | n77 | No |

6.1.37.2	Configuration for DC

Table 6.1.37.2-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_30A_n77A | DC_30A_n77A | 30A | n77A |

6.1.37.3	Maximum output power for DC

Table 6.1.37.3-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_30A_n77A | 23 | +2/-3 |

6.1.37.4	Spurious emission band UE co-existence for DC

Table 6.1.37.4-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_30_n77 | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 24, 25, 26, 27, 29, 30, 41, 53, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |

6.1.37.5	MSD analysis for DC

For 2UL/2DL UE coexistence study 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.1.37.5-1

Table 6.1.37.5-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2305 | 2315 | 3300 | 4200 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high | 2*fx_low | 2*fx_high |
| 2nd harmonics frequency limits (MHz) | 4610 | 4630 | 6600 | 8400 |
| 3rd harmonics frequency limits | 3* fy_low | 3* fy_high | 3*fx_low | 3*fx_high |
| 3rd harmonics frequency limits (MHz) | 6915 | 6945 | 9900 | 12600 |
| 4th harmonics frequency limits | 4* fy_low | 4* fy_high | 4*fx_low | 4*fx_high |
| 4th harmonics frequency limits (MHz) | 9220 | 9260 | 13200 | 16800 |
| 5th harmonics frequency limits | 5* fy_low | 5* fy_high | 5*fx_low | 5*fx_high |
| 5th harmonics frequency limits (MHz) | 11525 | 11575 | 16500 | 21000 |
| 6th harmonics frequency limits | 6* fy_low | 6* fy_high | 6*fx_low | 6*fx_high |
| 6th harmonics frequency limits (MHz) | 13830 | 13890 | 19800 | 25200 |
| 7th harmonics frequency limits | 7* fy_low | 7* fy_high | 7*fx_low | 7*fx_high |
| 7th harmonics frequency limits (MHz) | 16135 | 16205 | 23100 | 29400 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1895 | 985 | 5605 | 6515 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 410 | 1330 | 4285 | 6095 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7910 | 8830 | 8905 | 10715 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3790 | 1970 | 11210 | 13030 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2715 | 3645 | 7585 | 10295 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 10215 | 11145 | 12205 | 14915 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14495 | 10885 | 5960 | 5020 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15505 | 19115 | 12520 | 13460 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 7990 | 5270 | 345 | 1485 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14510 | 17230 | 13515 | 15345 |

Based on Table 6.1.37.5-1, it can be seen that

-	4th order IMD products may fall into own Rx frequencies of band 30.

#### 6.1.37.6	∆TIB and ∆RIB values

For DC_30_n77, the TIB,c and RIB,c values are reused from CA_n7-n78 and are given in the tables below.

Table 6.1.37.6-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_30_n77 | 30 | 0.5 |
|  | n77 | 0.8 |

Table 6.1.37.6-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_30_n77 | 30 | 0 |
|  | n77 | 0.5 |

6.1.37.7	MSD

Co-existence studies shows that MSD need to be defined for IMD4 falling into own Rx frequencies of band 30. MSD value same as in DC_1A_n77A.

Table 6.1.37.7-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_30A_n77A | 30 | 2310 | 5 | 25 | 2355 | 8.0 | IMD4 |
|  | n77 | 3487.5 | 10 | 50 | 3487.5 | N/A | N/A |

6.1.38	DC_14_n30

6.1.38.1	Operating bands for DC

Table 6.1.38.1-1: DC band combination of 1 LTE band + 1 NR band

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_14_n30 | 14 | n30 | No |

6.1.38.2	Configuration for DC

Table 6.1.38.2-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_14A_n30A | DC_14A_n30A | 14A | n30A |

6.1.38.3	Maximum output power for DC

Table 6.1.38.3-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_14A_n30A | 23 | +2/-3 |

6.1.38.4	Spurious emission band UE co-existence for DC

Table 6.1.38.4-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_14_n30 | E-UTRA Band 2, 4, 5,12, 13, 14, 17, 24, 25, 26, 27, 29, 30, 41, 48, 53, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 | 5 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 5 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180 kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

6.1.38.5	MSD analysis for DC

For 2UL/2DL UE coexistence study 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.1.38.5-1.

Table 6.1.38.5-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 788 | 798 | 2305 | 2315 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high | 2*fx_low | 2*fx_high |
| 2nd harmonics frequency limits (MHz) | 1576 | 1596 | 4610 | 4630 |
| 3rd harmonics frequency limits | 3* fy_low | 3* fy_high | 3*fx_low | 3*fx_high |
| 3rd harmonics frequency limits (MHz) | 2364 | 2394 | 6915 | 6945 |
| 4th harmonics frequency limits | 4* fy_low | 4* fy_high | 4*fx_low | 4*fx_high |
| 4th harmonics frequency limits (MHz) | 3152 | 3192 | 9220 | 9260 |
| 5th harmonics frequency limits | 5* fy_low | 5* fy_high | 5*fx_low | 5*fx_high |
| 5th harmonics frequency limits (MHz) | 3940 | 3990 | 11525 | 11575 |
| 6th harmonics frequency limits | 6* fy_low | 6* fy_high | 6*fx_low | 6*fx_high |
| 6th harmonics frequency limits (MHz) | 4728 | 4788 | 13830 | 13890 |
| 7th harmonics frequency limits | 7* fy_low | 7* fy_high | 7*fx_low | 7*fx_high |
| 7th harmonics frequency limits (MHz) | 5516 | 5586 | 16135 | 16205 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1527 | 1507 | 3093 | 3113 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 739 | 709 | 3812 | 3842 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3881 | 3911 | 5398 | 5428 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3054 | 3014 | 6186 | 6226 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 49 | 89 | 6117 | 6157 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4669 | 4709 | 7703 | 7743 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8472 | 8422 | 887 | 837 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10008 | 10058 | 5457 | 5507 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 5369 | 5319 | 2216 | 2266 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8491 | 8541 | 6974 | 7024 |

Based on Table 6.1.38.5-1, it can be seen that there are no IMD issues.

6.1.38.6	∆TIB and ∆RIB values

For DC_14_n30, the TIB,c and RIB,c values are reused from CA_14-30 and are given in the tables below.

Table 6.1.38.6-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_14_n30 | 14 | 0.3 |
|  | n30 | 0.3 |

Table 6.1.38.6-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_14_n30 | 14 | 0 |
|  | n30 | 0 |

6.1.38.7	MSD

Based on the co-existence analysis, there is no need to define MSD for DC_14_n30.

6.1.39	DC_7_n25

6.1.39.1	Operating bands for DC

Table 6.1.39.1-1: DC band combination of 1 LTE band + 1 NR band

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_7_n25 | 7 | n25 | No |

6.1.39.2	Configuration for DC

Table 6.1.39.2-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_7A_n25A | DC_7A_n25A | 7A | n25A |
| DC_7A-7A_n25A | DC_7A_n25A | CA_7A-7A | n25A |
| DC_7C_n25A | DC_7A_n25A | CA_7C | n25A |

6.1.39.3	Maximum output power for DC

Table 6.1.39.3-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_7A_n25A | 23 | +2/-3 |

6.1.39.4	Spurious emission band UE co-existence for DC

Table 6.1.39.4-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_7_n25 | E-UTRA Band 4, 5, 12, 13, 14, 17, 26, 27, 28, 29, 30, 42, 66, 71, 85 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 43 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 2 NR Band n25 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | Frequency range | 2570 | - | 2575 | 1.6 | 5 | 5, 7, 19 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 7, 19 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 19 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180 kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 19: This requirement is applicable for any channel bandwidths within the range 2500 – 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 – 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. |  |  |  |  |  |  |  |

6.1.39.5	MSD analysis for DC

For 2UL/2DL UE coexistence study 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.1.39.5-1

Table 6.1.39.5-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1915 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high | 2*fx_low | 2*fx_high |
| 2nd harmonics frequency limits (MHz) | 3700 | 3830 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3* fy_low | 3* fy_high | 3*fx_low | 3*fx_high |
| 3rd harmonics frequency limits (MHz) | 5550 | 5745 | 7500 | 7710 |
| 4th harmonics frequency limits | 4* fy_low | 4* fy_high | 4*fx_low | 4*fx_high |
| 4th harmonics frequency limits (MHz) | 7400 | 7660 | 10000 | 10280 |
| 5th harmonics frequency limits | 5* fy_low | 5* fy_high | 5*fx_low | 5*fx_high |
| 5th harmonics frequency limits (MHz) | 9250 | 9575 | 12500 | 12850 |
| 6th harmonics frequency limits | 6* fy_low | 6* fy_high | 6*fx_low | 6*fx_high |
| 6th harmonics frequency limits (MHz) | 11100 | 11490 | 15000 | 15420 |
| 7th harmonics frequency limits | 7* fy_low | 7* fy_high | 7*fx_low | 7*fx_high |
| 7th harmonics frequency limits (MHz) | 12950 | 13405 | 17500 | 17990 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 720 | 585 | 4350 | 4485 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1130 | 1330 | 3085 | 3290 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6200 | 6400 | 6850 | 7055 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1440 | 1170 | 8700 | 8970 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2980 | 3245 | 5585 | 5860 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8050 | 8315 | 9350 | 9625 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8430 | 8085 | 5160 | 4830 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11850 | 12195 | 9900 | 10230 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4010 | 3670 | 745 | 410 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11200 | 11540 | 10550 | 10885 |

Based on Table 6.1.39.5-1, it can be seen that there are no IMD issues.

6.1.39.6	∆TIB and ∆RIB values

For DC_7_n25, the TIB,c and RIB,c values are reused from CA_n7-n25 and are given in the tables below.

Table 6.1.39.6-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7_n25 | 7 | 0.5 |
|  | n25 | 0.5 |

Table 6.1.39.6-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_7_n25 | 7 | 0 |
|  | n25 | 0 |

6.1.39.7	MSD

Co-existence studies shows that there is no need to defined MSD for DC_7_n25.

6.1.40	DC_13_n25

6.1.40.1	Operating bands for DC

Table 6.1.40.1-1: DC band combination of 1 LTE band + 1 NR band

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_13_n25 | 13 | n25 | No |

6.1.40.2	Configuration for DC

Table 6.1.40.2-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_13A_n25A | DC_13A_n25A | 13A | n25A |

6.1.40.3	Maximum output power for DC

Table 6.1.40.3-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_13A_n25A | 23 | +2/-3 |

6.1.40.4	Spurious emission band UE co-existence for DC

Table 6.1.40.4-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_13A_n25A | E-UTRA Band 4, 5,12,13,17, 26, 29, 41, 48, 66, 70, 71 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2,14 NR Band n25 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 30 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 | 5 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 5 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180 kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

6.1.40.5	MSD analysis for DC

For 2UL/2DL UE coexistence study 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.1.40.5-1

Table 6.1.40.5-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 777 | 787 | 1850 | 1915 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high | 2*fx_low | 2*fx_high |
| 2nd harmonics frequency limits (MHz) | 1554 | 1574 | 3700 | 3830 |
| 3rd harmonics frequency limits | 3* fy_low | 3* fy_high | 3*fx_low | 3*fx_high |
| 3rd harmonics frequency limits (MHz) | 2331 | 2361 | 5550 | 5745 |
| 4th harmonics frequency limits | 4* fy_low | 4* fy_high | 4*fx_low | 4*fx_high |
| 4th harmonics frequency limits (MHz) | 3108 | 3148 | 7400 | 7660 |
| 5th harmonics frequency limits | 5* fy_low | 5* fy_high | 5*fx_low | 5*fx_high |
| 5th harmonics frequency limits (MHz) | 3885 | 3935 | 9250 | 9575 |
| 6th harmonics frequency limits | 6* fy_low | 6* fy_high | 6*fx_low | 6*fx_high |
| 6th harmonics frequency limits (MHz) | 4662 | 4722 | 11100 | 11490 |
| 7th harmonics frequency limits | 7* fy_low | 7* fy_high | 7*fx_low | 7*fx_high |
| 7th harmonics frequency limits (MHz) | 5439 | 5509 | 12950 | 13405 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1138 | 1063 | 2627 | 2702 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 361 | 276 | 2913 | 3053 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3404 | 3489 | 4477 | 4617 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2276 | 2126 | 5254 | 5404 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 416 | 511 | 4763 | 4968 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4181 | 4276 | 6327 | 6532 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6883 | 6613 | 1298 | 1193 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8177 | 8447 | 4958 | 5063 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4191 | 3976 | 1339 | 1499 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7104 | 7319 | 6031 | 6191 |

Based on Table 6.1.40.5-1, it can be seen that there are no IMD issues.

6.1.40.6	∆TIB and ∆RIB values

For DC_13_n25, the TIB,c and RIB,c values are reused from DC_13_n2 and are given in the tables below.

Table 6.1.40.6-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_13_n25 | 13 | 0.3 |
|  | n25 | 0.3 |

Table 6.1.40.6-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_13_n25 | 13 | 0 |
|  | n25 | 0 |

6.1.40.7	MSD

Co-existence studies shows that there is no need to defined MSD for DC_13_n25.

6.1.41	DC_3-3_n8

6.1.41.1	Configuration for DC

Table 6.1.41.1-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration | Single UL allowed |
| --- | --- | --- |
| DC_3A-3A_n8A | DC_3A_n8A | No |

6.1.41.2	Maximum output power for DC

The Maximum output power requirement was already coverd in the fallback DC_3A_n8A, which was specified in Rel.16.

6.1.41.3	Spurious emission band UE co-existence for DC

The Spurious emission requirement was already coverd in the fallback DC_3A_n8A, which was specified in Rel.16.

6.1.41.4	MSD analysis for DC

The Spurious emission requirement was already coverd in the fallback DC_3A_n8A, which was studied in Rel.16 TR 36.716-11-11 section 6.1.91.4. The needed MSD requirements were already specified in Rel.16 specs.

6.1.41.5	∆TIB and ∆RIB values

For DC_3-3_n8, the ΔTIB,c and ΔRIB,c values are given in the tables below.

Table 6.1.41.5-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-3_n8 | 3 | 0.3 |
|  | n8 | 0.3 |

Table 6.1.41.5-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3-3_n8 | 3 | 0 |
|  | n8 | 0 |

6.1.42	DC_7-7_n8

6.1.42.1	Configuration for DC

Table 6.1.42.1-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration | Single UL allowed |
| --- | --- | --- |
| DC_7A-7A_n8A | DC_7A_n8A | No |

6.1.42.2	Maximum output power for DC

The Maximum output power requirement was already coverd in the fallback DC_7A_n8A, which was specified in Rel.16.

6.1.42.3	Spurious emission band UE co-existence for DC

The Spurious emission requirement was already coverd in the fallback DC_7A_n8A, which was specified in Rel.16.

6.1.42.4	MSD analysis for DC

The Spurious emission requirement was already coverd in the fallback DC_7A_n8A, which was studied in Rel.16 TR 36.716-11-11 section 6.1.92.5. The needed MSD requirements were already specified in Rel.16 specs.

6.1.42.5	∆TIB and ∆RIB values

For DC_7-7_n8, the ΔTIB,c and ΔRIB,c values are given in the tables below.

Table 6.1.42.5-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-7_n8 | 7 | 0.3 |
|  | n8 | 0.6 |

Table 6.1.42.5-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_7-7_n8 | 7 | 0 |
|  | n8 | 0.2 |

6.1.43	DC_48_n77

6.1.43.1	Configuration for DC

Table 6.1.43.1-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration | Single UL allowed |
| --- | --- | --- |
| DC_48A_n77A3. 4. 9, 11 | N/A | N/A |
| NOTE 3: The minimum requirements apply only when there is non-simultaneous Tx/Rx operation between E-UTRA and NR carriers. This restriction applies also for these carriers when applicable EN-DC configuration is part of a higher order EN-DC configuration. NOTE 4: The minimum requirements for intra-band non-contiguous EN-DC apply. When UE capability interBandContiguousMRDC is indicated, the minimum requirements for intra-band-contiguous EN-DC also should be met in addtion to intra-band non-contiguous EN-DC. The intra-band requirements also apply for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. NOTE 9: The combination is not used alone as fall back mode of other band combinations in which UL in Band 42 or Band 48 is not used. NOTE 11: The minimum requirements for inter-band EN-DC apply when the maximum power spectral density imbalance between downlink carriers is within 6 dB. The power spectral density imbalance condition also applies for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. |  |  |

6.1.43.2	Maximum output power for DC

This combination is not used alone, and no uplink inter-band EN-DC is specified for the configuration.

6.1.43.3	Spurious emission band UE co-existence for DC

This combination is not used alone, and no additional spurious emission requirement.

Table 6.1.43.3-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| EN-DC Configuration | Spurious emission |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_48_n77 | N/A |  |  |  |  |

6.1.43.4	MSD analysis for DC

This combination is not used alone and is as fall back mode of other band combinations. Also, there is non-simultaneous Tx/Rx operation between E-UTRA and NR carriers even in other or higher order configuration. Thus, there is no coexistence issue or MSD requirement.

6.1.43.5	∆TIB and ∆RIB values

The ΔTIB,c and ΔRIB,c values are given in tables below,

Table 6.1.43.5-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_48_n77 | 48 | 0.0 |
|  | n77 | 0.0 |

Table 6.1.43.5-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_48_n77 | 48 | 0.0 |
|  | n77 | 0.0 |

6.1.43.6		MSD

There is no additional MSD requirement.

6.1.44	DC_n28_3

      - 6.1.44.1	Configuration for DC

Table 6.1.44.1-1: Inter-band NE-DC configurations within FR1 (two bands)

| NE-DC configuration | Uplink NE-DC configuration | Single UL allowed |
| --- | --- | --- |
| DC_n28A_3A DC_n28A_3C | DC_n28A_3A | No |

      - 6.1.44.2	Maximum output power for DC

Table 6.1.44.2-1: Maximum output power for inter-band NE-DC (two bands)

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_n28A_3A | 23 | +2/-3 |

      - 6.1.44.3	Spurious emission band UE co-existence for DC

The protected bands for UE co-existence for DC_n28A_3A are the same with DC_3A_n28A, which have been already defined in the TS38.101-3 spec.

      - 6.1.44.4	MSD analysis for DC

The MSD analysis for DC_n28A_3A are the same with DC_3A_n28A, where there are no harmonic and IMD falling into own receive bands.

      - 6.1.44.5	∆TIB and ∆RIB values

The ΔTIB,c and ΔRIB,c for DC_n28_3 are the same with DC_3_n28, which have already been defined in the TS38.101-3 spec.

      - 6.1.44.6	Self-interference analysis

Based on the co-existence study and self-interference analysis, it can be seen there are no harmonic and intermodulation issue for DC_n28A_3A and DC_n28A_3C configuration. Thus, no MSD is required for this combination.

      - 6.1.44.7	OOB blocking exception requirements

No OOB blocking exception for DC_n28_3.

6.1.45	DC_n28_8

      - 6.1.45.1	Configuration for DC

Table 6.1.45.1-1: Inter-band NE-DC configurations within FR1 (two bands)

| NE-DC configuration | Uplink NE-DC configuration | Single UL allowed |
| --- | --- | --- |
| DC_n28A_8A DC_n28A_8B | DC_n28A_8A | No |

      - 6.1.45.2	Maximum output power for DC

Table 6.1.45.2-1: Maximum output power for inter-band NE-DC (two bands)

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_n28A_8A | 23 | +2/-3 |

      - 6.1.45.3	Spurious emission band UE co-existence for DC

The protected bands for UE co-existence for DC_n28_8 are the same with DC_8A_n28A, which have been already defined in the TS38.101-3 spec.

      - 6.1.45.4	MSD analysis for DC

The MSD analysis for DC_n28_8 are the same with DC_8A_n28A, where there are no harmonic and IMD falling into own receive bands.

      - 6.1.45.5	∆TIB and ∆RIB values

The ΔTIB,c and ΔRIB,c for DC_n28_8 are the same with DC_8A_n28A, which have already been defined in the TS38.101-3 spec.

      - 6.1.45.6		Self-interference analysis

Based on the co-existence study and self-interference analysis, it can be seen there are no harmonic and intermodulation issue for DC_n28_8 configuration. Thus, no MSD is required for this combination.

      - 6.1.45.7	OOB blocking exception requirements

No OOB blocking exception for DC_n28_8.

6.1.46	DC_n28_40

      - 6.1.46.1	Configuration for DC

Table 6.1.46.1-1: Inter-band NE-DC configurations within FR1 (two bands)

| NE-DC configuration | Uplink NE-DC configuration | Single UL allowed |
| --- | --- | --- |
| DC_n28A_40A DC_n28A_40C | DC_n28A_40A | No |

      - 6.1.46.2	Maximum output power for DC

Table 6.1.46.2-1: Maximum output power for inter-band NE-DC (two bands)

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_n28A_40A | 23 | +2/-3 |

      - 6.1.46.3	Spurious emission band UE co-existence for DC

The protected bands for UE co-existence for DC_n28A_40A are the same with DC_28A_n40A, which have been already defined in the TS38.101-3 spec.

      - 6.1.46.4	MSD analysis for DC

The MSD analysis for DC_n28A_40A are the same with DC_28A_n40A, where there are no harmonic and IMD falling into own receive bands.

Moreover, there exist 3rd harmonic mixing issue for this combination.

      - 6.1.46.5	∆TIB and ∆RIB values

The ΔTIB,c and ΔRIB,c for DC_n28_40 are the same with DC_28_n40, which have already been defined in the TS38.101-3 spec.

      - 6.1.46.6	Self-interference analysis

Based on the co-existence study and self-interference analysis, it can be seen there are no harmonic and intermodulation issue for DC_28A_n40A and DC_n28A_40C configuration.

The MSD for the 3rd harmonic mixing issue are defined in table 6.1.46.6-1, where the MSD values for 5/10/15/20MHz are reused from DC_28A_n40A.

Table 6.1.46.6-1: Reference sensitivity exceptions (MSD) due to receiver harmonic mixing for NE-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| 40 | n284 | 37.8 | 34.8 | 33 | 30.3 |  | 18 |  |  |  |  |  |  |
| NOTE 4: The requirements should be verified for DL EARFCN or NR ARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.1.46.6-2: Uplink configuration for reference sensitivity exceptions due to receiver harmonic mixing for EN-DC in NR FR1

| E-UTRA or NR Band / SCS / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 30 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| 40 | n28 | 15 | 25 | 50 | 75 | 100 |  | 100 |  |  |  |  |  |  |

      - 6.1.46.7	OOB blocking exception requirements

No OOB blocking exception for DC_n28_40.

6.1.47	DC_n41_3

      - 6.1.47.1	Configuration for DC

Table 6.1.47.1-1: Inter-band NE-DC configurations within FR1 (two bands)

| NE-DC configuration | Uplink NE-DC configuration | Single UL allowed |
| --- | --- | --- |
| DC_n41A_3A DC_n41A_3C DC_n41C_3A | DC_n41A_3A | No |

      - 6.1.47.2	Maximum output power for DC

Table 6.1.47.2-1: Maximum output power for inter-band NE-DC (two bands)

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_n41A_3A | 23 | +2/-3 |

      - 6.1.47.3	Spurious emission band UE co-existence for DC

The protected bands for UE co-existence for DC_n41_3 are the same with DC_3A_n41A, which have been already defined in the TS38.101-3 spec.

      - 6.1.47.4	MSD analysis for DC

The MSD analysis for DC_n41_3 are the same with DC_3A_n41A, where:

-	IMD4 may falls into band 3 own Rx band.

      - 6.1.47.5	∆TIB and ∆RIB values

The ΔTIB,c and ΔRIB,c for  DC_n41_3 are the same with DC_3A_n41A, which have already been defined in the TS38.101-3 spec.

      - 6.1.47.6	Self-interference analysis

The MSD caused by cross-band isolation and IMD4 for DC_n41_3 can reuse the same values of DC_3A_n41A, which have been already defined in the TS38.101-3 spec.

      - 6.1.47.7	OOB blocking exception requirements

No OOB blocking exception for DC_n41_3.

6.1.48	DC_n41_8

      - 6.1.48.1	Configuration for DC

Table 6.1.48.1-1: Inter-band NE-DC configurations within FR1 (two bands)

| NE-DC configuration | Uplink NE-DC configuration | Single UL allowed |
| --- | --- | --- |
| DC_n41A_8A DC_n41A_8B DC_n41C_8A | DC_n41A_8A | No |

      - 6.1.48.2	Maximum output power for DC

Table 6.1.48.2-1: Maximum output power for inter-band NE-DC (two bands)

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_n41A_8A | 23 | +2/-3 |

      - 6.1.48.3	Spurious emission band UE co-existence for DC

The protected bands for UE co-existence for DC_n41_8 are the same with DC_8A_n41A, which have been already defined in the TS38.101-3 spec.

      - 6.1.48.4	MSD analysis for DC

The MSD analysis for DC_n41_8 are the same with DC_8A_n41A, where:

-	3rd harmonic of band 8 may falls into band n41 own Rx band.

-	3rd  and 5th IMD caused by 2UL may falls into band 8 own Rx band.

      - 6.1.48.5	∆TIB and ∆RIB values

The ΔTIB,c and ΔRIB,c for  DC_n41_8 are the same with DC_8A_n41A, which have already been defined in the TS38.101-3 spec.

      - 6.1.48.6	Self-interference analysis

The MSD caused by harmonic issue and IMD issue for DC_n41_8 can reuse the same values of DC_8A_n41A, which have been already defined in the TS38.101-3 spec.

6.1.48.7	OOB blocking exception requirements

No OOB blocking exception for DC_n41_8.

6.1.49	DC_n41_34

      - 6.1.49.1	Configuration for DC

Table 6.1.49.1-1: Inter-band NE-DC configurations within FR1 (two bands)

| NE-DC configuration | Uplink NE-DC configuration | Single UL allowed |
| --- | --- | --- |
| DC_n41A_34A3 | DC_n41A_34A | No |
| NOTE 3: The minimum requirements apply only when there is non-simultaneous Tx/Rx operation between E-UTRA and NR carriers. This restriction applies also for these carriers when applicable EN-DC configuration is part of a higher order EN-DC configuration. |  |  |

      - 6.1.49.2	Maximum output power for DC

Table 6.1.49.2-1: Maximum output power for inter-band NE-DC (two bands)

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_n41A_34A | 23 | +2/-3 |

      - 6.1.49.3	Spurious emission band UE co-existence for DC

Table 6.1.49.3-1: Spurious emissions for inter-band DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_n41_34 | E-UTRA Band 1, 3, 8, 26, 28, 39, 42, 44, 45, 50, 51, 52, 65, 73, 74 NR band n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA 40 | FDL_low | - | FDL_high | -40 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| NOTE 1: Void NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180 kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz |  |  |  |  |  |  |  |

      - 6.1.49.4	MSD analysis for DC

Since both NR band n41 and LTR band 34 are TDD band, also as mentioned above, non-simultaneous Tx/Rx operation between NR band n41 and LTE band 34 is supported, therefore there are no harmonic and intermodulation for this DC configuration.

      - 6.1.49.5	∆TIB and ∆RIB values

For DC_n41_34, same TIB,c and RIB,c values are given in the tables below.

Table 6.1.49.5-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_n41_34 | n41 | 0.3 |
|  | 34 | 0.3 |

Table 6.1.49.5-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_n41_34 | n41 | 0 |
|  | 34 | 0 |

6.1.49.6	Self-interference analysis

Based on the co-existence study and self-interference analysis, it can be seen there are no harmonic and intermodulation issue for DC_n41A_34A configuration, i.e. no MSD needs to be defined.

6.1.49.7	OOB blocking exception requirements

No OOB blocking exception for DC_n41A_34A.

6.1.50	DC_n41_39

      - 6.1.50.1	Configuration for DC

Table 6.1.50.1-1: Inter-band NE-DC configurations within FR1 (two bands)

| NE-DC configuration | Uplink NE-DC configuration | Single UL allowed |
| --- | --- | --- |
| DC_n41A_39A3 DC_n41A_39C3 DC_n41C_39A3 | DC_n41A_39A | No |
| NOTE 3: The minimum requirements apply only when there is non-simultaneous Tx/Rx operation between E-UTRA and NR carriers. This restriction applies also for these carriers when applicable EN-DC configuration is part of a higher order EN-DC configuration. |  |  |

      - 6.1.50.2	Maximum output power for DC

Table 6.1.50.2-1: Maximum output power for inter-band NE-DC (two bands)

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_n41A_39A | 23 | +2/-3 |

      - 6.1.50.3	Spurious emission band UE co-existence for DC

The protected bands for UE co-existence for DC_n41_39 are the same with DC_39A_n41A, which have been already defined in the TS38.101-3 spec.

      - 6.1.50.4	MSD analysis for DC

Since both NR band n41 and LTR band 39 are TDD band, also as mentioned above, non-simultaneous Tx/Rx operation between NR band n41 and LTE band 39 is supported, therefore there are no harmonic and intermodulation for these DC configuration.

      - 6.1.50.5	∆TIB and ∆RIB values

The ΔTIB,c and ΔRIB,c for  DC_n41_39 are the same with DC_39A_n41A, which have already been defined in the TS38.101-3 spec.

6.1.50.6	Self-interference analysis

Based on the co-existence study and self-interference analysis, it can be seen there are no harmonic and intermodulation issue for DC_n41_39 configuration, i.e. no MSD needs to be defined.

      - 6.1.50.7	OOB blocking exception requirements

No OOB blocking exception for DC_n41_39.

6.1.51	DC_n41_40

      - 6.1.51.1	Configuration for DC

Table 6.1.51.1-1: Inter-band NE-DC configurations within FR1 (two bands)

| NE-DC configuration | Uplink EN-DC configuration | Single UL allowed |
| --- | --- | --- |
| DC_n41A_40A3 DC_n41A_40C3 DC_n41C_40A3 | DC_n41A_40A | No |
| NOTE 3: The minimum requirements apply only when there is non-simultaneous Tx/Rx operation between E-UTRA and NR carriers. This restriction applies also for these carriers when applicable EN-DC configuration is part of a higher order EN-DC configuration. |  |  |

      - 6.1.51.2	Maximum output power for DC

Table 6.1.51.2-1: Maximum output power for inter-band NE-DC (two bands)

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_n41A_40A | 23 | +2/-3 |

      - 6.1.51.3	Spurious emission band UE co-existence for DC

The protected bands for UE co-existence for DC_n41_40 are the same with DC_40A_n41A, which have been already defined in the TS38.101-3 spec.

      - 6.1.51.4	MSD analysis for DC

Since both NR band n41 and LTR band 40 are TDD band, also as mentioned above, non-simultaneous Tx/Rx operation between NR band n41 and LTE band 40 is supported, therefore there are no harmonic and intermodulation for this DC configuration.

      - 6.1.51.5	∆TIB and ∆RIB values

The ΔTIB,c and ΔRIB,c for  DC_n41_40 are the same with DC_40A_n41A, which have already been defined in the TS38.101-3 spec.

      - 6.1.51.6	Self-interference analysis

Based on the co-existence study and self-interference analysis, it can be seen there are no harmonic and intermodulation issue for DC_n41_40 configuration, i.e. no MSD needs to be defined.

      - 6.1.51.7	OOB blocking exception requirements

No OOB blocking exception for DC_n41_40.

6.1.52	DC_38_n3

      - 6.1.52.1	Configuration for DC

Table 6.1.52.1-1: Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration | Single UL allowed |
| --- | --- | --- |
| DC_38A_n3A | DC_38A_n3A | No |

      - 6.1.52.2	Maximum output power for DC

Table 6.1.52.2-1: Maximum output power for inter-band EN-DC (two bands)

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_38A_n3A | 23 | +2/-31 |
| NOTE 1: For the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

      - 6.1.52.3	Spurious emission band UE co-existence for DC

Table 6.1.52.3-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_38_n3 | E-UTRA Band 1, 5, 8, 20, 27, 28, 31, 32, 33, 34, 40, 43, 50, 51, 65, 67, 68, 72, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 22, 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180 kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. |  |  |  |  |  |  |  |

      - 6.1.52.4	MSD analysis for DC

Same co-existence analysis including harmonic and IMD as DC_3_n38 defined in TR 37.716-11-11 are applied, where:

-	IMD4 caused by 2UL may fall into band n3 DL range.

      - 6.1.52.5	∆TIB and ∆RIB values

For DC_38_n3, same TIB,c and RIB,c values as DC_3_n38 are reused, which are given in the tables below.

Table 6.1.52.5-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_38_n3 | 38 | 0.5 |
|  | n3 | 0.5 |

Table 6.1.52.5-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_38_n3 | 38 | 0 |
|  | n3 | 0 |

      - 6.1.52.6	Self-interference analysis

Based on the co-existence study and self-interference analysis, it can be seen that IMD4 could potentially desensitize band n3 DL carrier. Therefore, MSD value and test configuration need to be defined. It is proposed to reused the same MSD values of 8.2dB as DC_3A_n38A for DC_38A_n3A

Table 6.1.52.6-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| DC_38A_n3A | n3 | 1713 | 5 | 25 | 1808 | 8.2 | FDD | IMD4 |
|  | 38 | 2617 | 5 | 25 | 2617 | N/A | TDD | N/A |

      - 6.1.52.7	OOB blocking exception requirements

No OOB blocking exception for DC_38A_n3A.

6.1.53	DC_38_n1

6.1.53.1	Configurations for DC

Table 6.1.53.1-1: Inter-band DC configurations (two bands)

| DC configuration | Uplink configuration | Single UL allowed |
| --- | --- | --- |
| DC_38A_n1A | DC_38A_n1A | No |

6.1.53.2	Maximum Ouput Power for DC

Table 6.1.53.2-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_38A_n1A | 23 | +2/-3 |

6.1.53.3	Co-existence studies

Table 6.1.53.3-1 lists the Band 38A + Band n1A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.1.53.3-1: Band 38 and Band n1 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fn_low | fn_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2570 | 2620 | 1920 | 1980 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fn_low | 2* fn_high |
| 2nd harmonics frequency limits (MHz) | 5140 – 5240 |  | 3840 – 3960 |  |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fn_low | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 7710 – 7860 |  | 5760 – 5940 |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |
| IMD frequency limits (MHz) | 590 – 700 |  | 4490 – 4600 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |
| IMD frequency limits (MHz) | 3160 – 3320 |  | 1220 – 1390 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |
| IMD frequency limits (MHz) | 7060 – 7220 |  | 6410 – 6580 |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) | (fx_high + max BW fn) | (fn_low – max BW fx) | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 2520 – 2670 |  | 1900 – 2000 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 5730 – 5940 |  | 3140 – 3370 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ | ¦2*fx_high –2* fn_low¦ | ¦2*fx_low +2* fn_low¦ | ¦2*fx_high +2* fn_high¦ |
| IMD frequency limits (MHz) | 1180 – 1400 |  | 8980 – 9200 |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 9630 – 9840 |  | 8330 – 8560 |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5060 – 5350 |  | 8300 – 8560 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 520 – 800 |  | 3750 – 4020 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10250 – 10540 |  | 12200 – 12460 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10900 – 11180 |  | 11550 – 11820 |  |

Based on Table 6.1.53.3-1,

-	2nd order harmonics may fall into Rx frequencies of band 77

-	3rd order harmonics may fall into Rx frequencies of bands 46 and 47

-	2nd order IMD may fall into Rx frequencies of bands 71 and 79

-	3rd order IMD may fall into Rx frequencies of bands 52, 77 and	 78

-	4th order IMD may fall into Rx frequencies of bands 46, 47, 52, 77 and 78

-	5th order IMD may fall into Rx frequencies of bands 12, 13, 14, 17, 20, 28, 29, 43, 44, 46, 67, 68, 71, 77, 78 and 85

When a 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS, the harmonics and intermodulation products can have an impact on these systems. Table 6.1.53.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.1.53.3-2: 2UL Band 38 + Band n1 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 3rd Harmonic, IMD4, IMD5 |
|  | 5150 | - | 5350 | Yes | Europe | 3rd Harmonic, IMD5 |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | Yes | Asia | 3rd Harmonic, IMD4, IMD5 |

The following requirements for spurious emission band UE coexistence are proposed for DC_38_n1 mirroring those already used for DC_1_n38 in 38101-3.

Table 6.1.53.2-3: Requirements

| EN-DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_38_n1 | E-UTRA Band 1, 3, 5, 8, 20, 22, 27, 28, 31, 32, 34, 40, 42, 43, 50, 51, 65, 67, 68, 72, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |

6.1.53.4	∆TIB and ∆RIB values

Table 6.1.53.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_38_n1 | 38 | 0.5 |
|  | n1 | 0.5 |

Table 6.1.53.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_38_n1 | 38 | 0 |
|  | n1 | 0 |

6.1.53.5	Reference sensitivity exceptions

Table 6.1.53.5-1: Reference sensitivity exceptions (MSD) due to cross band isolation for PC3 EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| 38 | n1 | 1.9 | 1.9 | 1.9 | 1.9 | 1.9 | 1.9 | 1.9 | 1.9 |  |  |  |  |  |
| n1 | 38 | 2.9 | 2.9 | 2.9 | 2.9 |  |  |  |  |  |  |  |  |  |

Table 6.1.53.5-2: Uplink configuration for reference sensitivity exceptions due to cross band isolation for EN-DC in NR FR1

| E-UTRA or NR Band / SCS / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 30 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 70 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| 38 | n1 | 15 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |  |  |  |  |  |
| n1 | 38 | 15 | 100 | 100 | 100 | 100 |  |  |  |  |  |  |  |  |  |

6.1.54	DC_38_n8

6.1.54.1	Configurations for DC

Table 6.1.54.1-1: Inter-band DC configurations (two bands)

| DC configuration | Uplink configuration | Single UL allowed |
| --- | --- | --- |
| DC_38A_n8A | DC_38A_n8A | No |

6.1.54.2	Maximum Ouput Power for DC

Table 6.1.54.2-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_38A_n8A | 23 | +2/-3 |

6.1.54.3	Co-existence studies

Table 6.1.54.3-1 lists the Band 38A + Band n8A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.1.54.3-1: Band 38 and Band n8 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fn_low | fn_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2570 | 2620 | 880 | 915 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fn_low | 2* fn_high |
| 2nd harmonics frequency limits (MHz) | 5140 – 5240 |  | 1760 – 1830 |  |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fn_low | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 7710 – 7860 |  | 2640 – 2745 |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |
| IMD frequency limits (MHz) | 1655 – 1740 |  | 3450 – 3535 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |
| IMD frequency limits (MHz) | 4225 – 4360 |  | 740 – 860 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |
| IMD frequency limits (MHz) | 6020 – 6155 |  | 4330 – 4450 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 6795 – 6980 |  | 20 – 175 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ | ¦2*fx_high –2* fn_low¦ | ¦2*fx_low +2* fn_low¦ | ¦2*fx_high +2* fn_high¦ |
| IMD frequency limits (MHz) | 3310 – 3480 |  | 6900 – 7070 |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8590 – 8775 |  | 5210 – 5365 |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 900 – 1090 |  | 9365 – 9600 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 2395 – 2600 |  | 5880 – 6100 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 6090 – 6280 |  | 11160 – 11395 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7780 – 7985 |  | 9470 – 9690 |  |

Based on Table 6.1.54.3-1,

-	2nd order harmonics may fall into Rx frequencies of band 3

-	3rd order harmonics may fall into Rx frequencies of bands  7, 41, 46 and 90

-	2nd order IMD may fall into Rx frequencies of bands 22, 42, 77 and 78

-	3rd order IMD may fall into Rx frequencies of bands 12, 13, 14, 17, 20, 26, 27, 28, 44, 67, 68, 79 and 85

-	4th order IMD may fall into Rx frequencies of bands 42, 46, 52, 77 and 78

-	5th order IMD may fall into Rx frequencies of bands 8, 38, 40, 41, 46, 47, 53, 69 and 90

When a 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS, the harmonics and intermodulation products can have an impact on these systems. Table 6.1.54.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.1.54.3-2: 2UL Band 38 + Band n8 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD5 |
|  | 2400 | - | 2494 | Yes | Asia | IMD5 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 3rd Harmonic, IMD4, IMD5 |
|  | 5150 | - | 5350 | Yes | Europe | 3rd Harmonic, IMD4 |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | Yes | Asia | 3rd Harmonic, IMD4 |

The following requirements for spurious emission band UE coexistence are proposed for DC_38_n8 reflecting a combination of those used for bands 8 and 38 in 36101.

Table 6.1.54.3-3: Requirements

| EN-DC Configuration | Spurious emission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band |  | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_38_n8 | E-UTRA Band 1, 2, 4, 12, 13, 14, 17 20, 27, 28, 29, 30, 31, 32, 33, 34, 39, 40, 45, 50, 51, 65, 66, 67, 68, 72, 73, 74, 75, 76, 85, 87, 88 | FDL_low |  | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3, 22, 42, 43, 52 NR Band n77, n78, n79 | FDL_low |  | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 8 | FDL_low |  | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 11, 21 | FDL_low |  | - | FDL_high | -50 | 1 | 12 |
|  | Frequency range | 860 |  | - | 890 | -40 | 1 | 5, 12 |
|  | Frequency range | 1884.5 |  |  | 1915.7 | -41 | 0.3 | 3, 12 |
|  | Frequency range | 2620 |  | - | 2645 | -15.5 | 5 | 5, 7, 22 |
|  | Frequency range | 2645 |  | - | 2690 | -40 | 1 | 5, 22 |
| NOTE 1 Void NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 in TS 36.101 [4] and Table 6.5.3.1-2 in TS 38.101-1 [2] are permitted for each assigned carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180 kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz NOTE 4 Void NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1, Table 6.6.3.1A-1 in TS 36.101 [4] or in Table 6.5.3.1-1 in TS 38.101-1 [2] from the edge of the channel bandwidth. NOTE 6 Void NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 8 Void NOTE 9 Void NOTE 10 Void NOTE 11 Void NOTE 12: This requirement is applicable only for the following cases: A: for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 902.5 MHz ≤ Fc < 907.5 MHz with an uplink transmission bandwidth less than or equal to 20 RB; B: for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 907.5 MHz ≤ Fc ≤ 912.5 MHz without any restriction on uplink transmission bandwidth; C: for carriers of 10 MHz channel bandwidth when carrier centre frequency (Fc) is Fc = 910 MHz with an uplink transmission bandwidth less than or equal to 32 RB with RBstart > 3. NOTE 13 Void NOTE 14 Void NOTE 15 Void NOTE 16 Void NOTE 17 Void NOTE 18 Void NOTE 19 Void NOTE 20 Void NOTE 21 Void NOTE 22: This requirement is applicable for power class 3 UE for any channel bandwidths within the range 2570 - 2615 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2605.5 - 2607.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2597 - 2605 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. |  |  |  |  |  |  |  |  |

6.1.54.4	∆TIB and ∆RIB values

Table 6.1.54.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_38_n8 | 38 | 0.6 |
|  | n8 | 0.3 |

Table 6.1.54.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_38_n8 | 38 | 0 |
|  | n8 | 0 |

6.1.54.5	Reference sensitivity exceptions

Table 6.1.54.5-1: MSD test points for PCell due to dual uplink operation for EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_38A_n8A | 38 | 2617.5 | 5 | 25 | 2617.5 | N/A | N/A |
|  | n8 | 887.5 | 5 | 25 | 932.5 | 8.1 | IMD5 |

6.1.55	DC_n3_1

      - 6.1.55.1	Configuration for DC

Table 6.1.55.1-1: Inter-band NE-DC configurations within FR1 (two bands)

| NE-DC configuration | Uplink NE-DC configuration | Single UL allowed |
| --- | --- | --- |
| DC_n3A_1A | DC_n3A_1A | DC_n3_1 |

      - 6.1.55.2	Maximum output power for DC

Table 6.1.55.2-1: Maximum output power for inter-band NE-DC (two bands)

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_n3A_1A | 23 | +2/-3 |

6.1.55.3	Spurious emission band UE co-existence for DC

The protected bands for UE co-existence for DC_n3A_1A are the same as DC_1A_n3A, which have been already defined in the TS38.101-3 spec.

      - 6.1.55.4	MSD analysis for DC

The MSD analysis for DC_n3A_1A are the same as DC_1A_n3A, as specified in 6.1.37.5 of3GPP TR 37.716-11-11 V2.0.0.

      - 6.1.55.5	∆TIB and ∆RIB values

The ΔTIB,c and ΔRIB,c for DC_n3A_1A are the same as DC_1A_n3A, which have already been defined in the TS38.101-3 spec.

      - 6.1.55.6		Self-interference analysis

Based on the co-existence study and self-interference analysis, The MSD test points for DC_n3A_1A are the same as DC_1A _n3A, as specified in Table 7.3B.2.3.5.1-1 of TS38.101-3 spec.

      - 6.1.55.7	OOB blocking exception requirements

No OOB blocking exception for DC_n3A_1A.

6.1.56	DC_n8_1

6.1.56.1	Operating bands for DC

Table 6.1.56.1-1: DC band combination of 1 LTE band + 1 NR band

| NE-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_n8_1 | 1 | n8 | No |

6.1.56.2	Configuration for DC

Table 6.1.56.2-1:  Inter-band NE-DC configurations of 1 LTE band + 1 NR band

| NE-DC configuration | Uplink NE-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_n8A_1A | DC_n8A_1A | 1A | n8A |

6.1.56.3	Maximum output power for DC

Table 6.1.56.3-1: Maximum output power for inter-band NE-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance(dB) |
| --- | --- | --- |
| DC_n8A_1A | 23 | +2/-3 |

      - 6.1.56.3	Spurious emission band UE co-existence for DC

The protected bands for UE co-existence for DC_n8A_1A are the same as DC_1A_n8A, which have been already defined in the TS38.101-3 spec.

      - 6.1.56.4	MSD analysis for DC

The MSD analysis for DC_n8A_1A is the same as DC_1A_n8A which is available in 3GPP TR 37.716-11-11 V2.0.0:

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 880 | 915 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 1760 | 1830 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 2640 | 2745 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 7680 | 7920 | 3520 | 3660 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 9600 | 9900 | 4400 | 4575 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1100 | 1005 | 2800 | 2895 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2925 | 3080 | 220 | 90 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4720 | 4875 | 3680 | 3810 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4845 | 5060 | 660 | 825 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6640 | 6855 | 4560 | 4725 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2010 | 2200 | 5600 | 5790 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1740 | 1540 | 7040 | 6765 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1095 | 1320 | 4180 | 3930 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5440 | 5640 | 8560 | 8835 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6480 | 6705 | 7520 | 7770 |

The 4th  order IMD may fall into own Rx frequency of Band 1.

      - 6.1.56.5	MSD

MSD values for DC_n8A_1A are the same as DC_1A_n8A, which have been already defined in the TS 38.101-3 spec.

      - 6.1.56.6		∆TIB and ∆RIB values

The ΔTIB,c and ΔRIB,c for DC_n8A_1A are the same as DC_1A_n8A, which have already been defined in the TS 38.101-3 spec.

      - 6.1.56.7		Self-interference analysis

The analysis of DC_n8A_1A is the same as DC_1A_n8A which is available in 3GPP TR 37.716-11-11 V2.0.0.

      - 6.1.56.8		OOB blocking exception requirements

No OOB blocking exception for DC_n8A_1A.

6.1.57	DC_n77_1, DC_n77(*)_1

6.1.57.1	Operating bands for DC

Table 6.1.57.1-1: DC band combination of 1 LTE band + 1 NR band

| NE-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_n77_1 | 1 | n77 | DC_n77_1 |

6.1.57.2	Configuration for DC

Table 6.1.57.2-1:  Inter-band NE-DC configurations of 1 LTE band + 1 NR band

| NE-DC configuration | Uplink NE-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_n77A_1A DC_n77(2A)_1A | DC_n77A_1A | 1A | n77A |

6.1.57.3	Maximum output power for DC

Table 6.1.57.3-1: Maximum output power for inter-band NE-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance(dB) |
| --- | --- | --- |
| DC_n77A_1A | 23 | +2/-3 |

      - 6.1.57.3	Spurious emission band UE co-existence for DC

The protected bands for UE co-existence for DC_n77A_1A are the same with DC_1A_n77A, which have been already defined in the TS38.101-3 spec.

      - 6.1.57.4	MSD analysis for DC

The MSD analysis for DC_n77A_1A are the same with DC_1A_n77A, as given in TR 37.863-01-01_V2.0.0:

-	2nd harmonics of band 1 fall on n77 DL range.

-	IMD2 and IMD4 fall on band 1 DL range.

      - 6.1.57.5	∆TIB and ∆RIB values

The ΔTIB,c and ΔRIB,c for DC_n77A_1A are the same with DC_1A_n77A, which have already been defined in the TS38.101-3 spec.

      - 6.1.57.6	Self-interference analysis

The analysis for DC_n77A_1A (and DC_n77(2A)_1A) are the same with DC_1A_n77A, as given in TR 37.863-01-01_V2.0.0.

      - 6.1.57.7	OOB blocking exception requirements

No OOB blocking exception for DC_n77A_1A.

6.1.58	DC_n77_3, DC_n77(*)_3

6.1.58.1	Operating bands for DC

Table 6.1.58.1-1: DC band combination of 1 LTE band + 1 NR band

| NE-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_n77A_3A | 3 | n77 | DC_n77_3 |

6.1.58.2	Configuration for DC

Table 6.1.58.2-1:  Inter-band NE-DC configurations of 1 LTE band + 1 NR band

| NE-DC configuration | Uplink NE-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_n77A_3A DC_n77(2A)_3A | DC_n77A_3A | 3A | n77A |
| Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability |  |  |  |

6.1.58.3	Maximum output power for DC

Table 6.1.58.3-1: Maximum output power for inter-band NE-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance(dB) |
| --- | --- | --- |
| DC_n77A_3A | 23 | +2/-3 |

6.1.58.4	Spurious emission band UE co-existence for DC

The protected bands for UE co-existence for DC_n77A_3A are the same as DC_3A_n77A, which have been already defined in the TS38.101-3 spec Table 6.5B.3.3.2-1.

6.1.58.5	MSD analysis for DC

The MSD analysis for DC_n77A_3A are the same as DC_3A_n77A which is available in 3GPP TR 37.716-11-11 V2.0.0:

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 3300 | 4200 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 6600 | 8400 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 9900 | 12600 | 5130 | 5355 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 13200 | 16800 | 6840 | 7140 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 16500 | 21000 | 8550 | 8925 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2490 | 1515 | 5010 | 5985 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 4815 | 6690 | 780 | 270 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8310 | 10185 | 6720 | 7770 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 8115 | 10890 | 930 | 2055 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 11610 | 14385 | 8430 | 9555 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3030 | 4980 | 10020 | 11970 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 3840 | 2640 | 15090 | 11415 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1245 | 3270 | 9180 | 6330 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10140 | 11340 | 14910 | 18585 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11730 | 13755 | 13320 | 16170 |

2nd order harmonics of band 3 could fall on the n77 downlink frequency range.

6.1.58.6	MSD

MSD values for DC_n77A_3A are the same as DC_3A_n77A, which have been already defined in the TS38.101-3 spec.

6.1.58.7	∆TIB and ∆RIB values

The ΔTIB,c and ΔRIB,c for DC_n77A_3A are the same as DC_3A_n77A, which have already been defined in the TS38.101-3 spec.

6.1.58.8	Self-interference analysis

The analysis of DC_n77A_3A (and DC_n77(2A)_3A) are the same as DC_3A_n77A which is available in 3GPP TR 37.716-11-11 V2.0.0.

      - 6.1.58.9		OOB blocking exception requirements

No OOB blocking exception for DC_n77A_3A.

6.1.59	DC_n3_8

6.1.59.1	Operating bands for DC

Table 6.1.59.1-1: DC band combination of 1 LTE band + 1 NR band

| NE -DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_n3_8 | 8 | n3 | No |

6.1.59.2	Configuration for DC

Table 6.1.59.2-1:  Inter-band NE-DC configurations of 1 LTE band + 1 NR band

| NE-DC configuration | Uplink NE-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_n3A_8A | DC_n3A_8A | 8A | n3A |

6.1.59.3	Maximum output power for DC

Table 6.1.59.3-1: Maximum output power for inter-band NE-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance(dB) |
| --- | --- | --- |
| DC_n3A_8A | 23 | +2/-3 |

      - 6.1.59.4	Spurious emission band UE co-existence for DC

The protected bands for UE co-existence for DC_n3A_8A are the same as DC_8_n3 which have been already defined in the TS38.101-3 spec Table 6.5B.3.3.2-1.

      - 6.1.59.5	MSD analysis for DC

The MSD analysis for DC_n3A_8A are the same as DC_8_n3 which is available in 3GPP TR 37.716-11-11 V2.0.0.

6.1.59.6	MSD

MSD values for DC_n3A_8A are the same as DC_8_n3, which have been already defined in TR 37.716-11-11.

      - 6.1.59.7	∆TIB and ∆RIB values

The ΔTIB,c and ΔRIB,c for DC_n3A_8A are the same as DC_8_n3, which have already been defined in TR 37.716-11-11.

      - 6.1.59.8	Self-interference analysis

The analysis of DC_n3A_8A are the same as DC_8_n3 which is available in 3GPP TR 37.716-11-11 V2.0.0.

      - 6.1.59.9	OOB blocking exception requirements

No OOB blocking exception for DC_n3A_8A.

6.1.60	DC_n8_3

6.1.60.1	Operating bands for DC

Table 6.1.60.1-1: DC band combination of 1 LTE band + 1 NR band

| NE-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_n8_3 | 3 | n8 | No |

6.1.60.2	Configuration for DC

Table 6.1.60.2-1:  Inter-band NE-DC configurations of 1 LTE band + 1 NR band

| NE-DC configuration | Uplink NE-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_n8A_3A | DC_n8A_3A | 3A | n8A |

6.1.60.3	Maximum output power for DC

Table 6.1.60.3-1: Maximum output power for inter-band NE-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance(dB) |
| --- | --- | --- |
| DC_n8A_3A | 23 | +2/-3 |

      - 6.1.60.4	Spurious emission band UE co-existence for DC

The protected bands for UE co-existence for DC_n8A_3A are the same as DC_3A_n8A, which have been already defined in the TS38.101-3 spec Table 6.5B.3.3.2-1.

      - 6.1.60.5	MSD analysis for DC

The MSD analysis for DC_n8A_3A are the same as DC_3A_n8A which is available in 3GPP TR 37.716-11-11 V2.0.0:

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1760 | 1830 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2640 | 2745 | 5130 | 5355 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3520 | 3660 | 6840 | 7140 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 4400 | 4575 | 8550 | 8925 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 795 | 905 | 2590 | 2700 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 25 | 120 | 2505 | 2690 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3470 | 3615 | 4300 | 4485 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 855 | 1035 | 4215 | 4475 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4350 | 4530 | 6010 | 6270 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1810 | 1590 | 5180 | 5400 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6260 | 5925 | 1950 | 1735 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 3595 | 3300 | 675 | 930 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7720 | 8055 | 5230 | 5445 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6890 | 7185 | 6060 | 6315 |

2nd order harmonic of n8 could fall on the band 3 downlink frequency range.

-	IMD4 could fall on the n8 downlink frequency range.

-	IMD4 could fall on the downlink frequency range of band 3 and n8.

6.1.60.6	MSD

MSD values for DC_n8A_3A are the same as DC_3A_n8A, which have been already defined in the TS38.101-3 spec.

      - 6.1.60.7	∆TIB and ∆RIB values

The ΔTIB,c and ΔRIB,c for DC_n8A_3A are the same as DC_3A_n8A, which have already been defined in the TS38.101-3 spec.

      - 6.1.60.8	Self-interference analysis

The analysis of DC_n8A_3A are the same as DC_3A_n8A which is available in 3GPP TR 37.716-11-11 V2.0.0.

      - 6.1.60.9	OOB blocking exception requirements

No OOB blocking exception for DC_n8A_3A.

6.1.61	DC_n77_8, DC_n77(*)_8

6.1.61.1	Operating bands for DC

Table 6.1.61.1-1: DC band combination of 1 LTE band + 1 NR band

| NE-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_n77A_8A DC_n77(2A)_8A | 8 | n77 | No |

6.1.61.2	Configuration for DC

Table 6.1.61.2-1: Inter-band NE-DC configurations of 1 LTE band + 1 NR band

| NE-DC configuration | Uplink NE-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_n77A_8A DC_n77(2A)_8A | DC_n77A_8A | 8A | n77A |

6.1.61.3	Maximum output power for DC

Table 6.1.61.3-1: Maximum output power for inter-band NE-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance(dB) |
| --- | --- | --- |
| DC_n77A_8A | 23 | +2/-3 |

      - 6.1.61.4	Spurious emission band UE co-existence for DC

The protected bands for UE co-existence for DC_n77A_8A (and DC_n77(2A)_8A) are the same as DC_8_n77 which have been already defined in the TS38.101-3 spec Table 6.5B.3.3.2-1.

      - 6.1.61.5	MSD analysis for DC

The MSD analysis for DC_n77A_8A (and DC_n77(2A)_8A)  are the same as DC_8_n77 which is available in 3GPP TR 37.716-11-11 V2.0.0.

6.1.61.6	MSD

MSD values for DC_n77A_8A (and DC_n77(2A)_8A)  are the same as DC_8_n77, which have been already defined in TR 37.716-11-11.

      - 6.1.61.7	∆TIB and ∆RIB values

The ΔTIB,c and ΔRIB,c for DC_n77A_8A (and DC_n77(2A)_8A)  are the same as DC_8_n77, which have already been defined in TR 37.716-11-11.

      - 6.1.61.8	Self-interference analysis

The analysis of DC_n77A_8A (and DC_n77(2A)_8A)  are the same as DC_8A_n77A  which is available in 3GPP TR 37.863-01-01_V2.0.0

      - 6.1.61.9	OOB blocking exception requirements

No OOB blocking exception for DC_n77A_8A and DC_n77(2A)_8A.

6.1.62	DC_n28_34

      - 6.1.62.1	Configuration for DC

Table 6.1.62.1-1: Inter-band NE-DC configurations within FR1 (two bands)

| NE-DC configuration | Uplink NE-DC configuration | Single UL allowed |
| --- | --- | --- |
| DC_n28A_34A | DC_n28A_34A | No |

      - 6.1.62.2	Maximum output power for DC

Table 6.1.62.2-1: Maximum output power for inter-band NE-DC (two bands)

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_n28A_34A | 23 | +2/-3 |

      - 6.1.62.3	Spurious emission band UE co-existence for DC

Table 6.1.62.3-1: Spurious emissions for inter-band DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_n28_34 | E-UTRA Band 3, 7, 8, 18, 19 ,20, 26, 31, 38, 40, 41, 72 NR band n77, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 22, 32, 42, 43, 50, 51, 52, 65, 73, 74, 75 NR band n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 5, 17 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 14 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 5 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180 kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1, Table 6.6.3.1A-1 in TS 36.101 [4] or in Table 6.5.3.1-1 in TS 38.101-1 [2] from the edge of the channel bandwidth. NOTE 9: Applicable when the assigned E-UTRA carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 10: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 11: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 14: This requirement is applicable for 5 and 10 MHz E-UTRA channel bandwidth allocated within 718-728MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and RBstart < 48. NOTE 17: This requirement is applicable in the case of a 10 MHz E-UTRA carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. |  |  |  |  |  |  |  |

      - 6.1.62.4	MSD analysis for DC

For study of 2UL/2DL, Table 6.1.62.4-1 lists up to 5th order harmonics and intermodulation products for DC_n28A_34A.

Table 6.1.62.4-1: Harmonic and IMD analysis

| UL frequency | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
|  | 703 | 748 | 2010 | 2025 |
| 2nd order harmonics frequency range | 1406 | 1496 | 4020 | 4050 |
| 3rd order harmonics frequency range | 2109 | 2244 | 6030 | 6075 |
| 4th order harmonics frequency range | 2812 | 2992 | 8040 | 8100 |
| 5th order harmonics frequency range | 3515 | 3740 | 10050 | 10125 |
| 2nd order IMD products | ¦fy_low-fx_high¦ | ¦fy_high-fx_low¦ | ¦fy_low+fx_low¦ | ¦fy_high+fx_high¦ |
| IMD frequency limits (MHz) | 1262 | 1322 | 2713 | 2773 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 619 | 514 | 3272 | 3347 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3416 | 3521 | 4723 | 4798 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2644 | 2524 | 5426 | 5546 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 84 | 234 | 5282 | 5372 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4119 | 4269 | 6733 | 6778 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7397 | 7292 | 982 | 787 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8743 | 8848 | 4822 | 5017 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4669 | 4534 | 1776 | 1941 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7436 | 7571 | 6129 | 6294 |

As we can see from Table 6.1.62.4-1 :

- No harmonic issue for DC_n28A_34A

- 5th intermodulation issue may falls into band n28 own Rx.

      - 6.1.62.5	∆TIB and ∆RIB values

For DC_n28_34, same TIB,c and RIB,c values are given in the tables below.

Table 6.1.62.5-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_n28_34 | n28 | 0.3 |
|  | 34 | 0.3 |

Table 6.1.62.5-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_n28_34 | n28 | 0 |
|  | 34 | 0 |

      - 6.1.62.6	Self-interference analysis

Based on the co-existence study and self-interference analysis, it can be seen that IMD5 could potentially desensitize band n28 DL carrier. However, after checking the test point, we see there are no suitable test points considering both symmetric UL/DL frequency point and the 5th intermodulation product of |4*fn28_ul-f34|. For example, DL Fc= 760.5MHz when UL Fc = 705.5MHz, however,  |4*fn28_ul-f34|  IMD5 product will not fall into  DL Fc range, which means no MSD are needed to be defined.

However, according to the analysis in R4-2117609, the cross band isolation MSD should be defined

Table 6.1.62.6-1: Reference sensitivity exceptions (MSD) due to cross band isolation for PC3 NE-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| 34 | n28 | 3.4 | 3.4 | 3.4 | 2.7 |  | 0.3 |  |  |  |  |  |  |  |

Table 6.1.62.6-2: Uplink configuration for reference sensitivity exceptions due to cross band isolation for EN-DC in NR FR1

| E-UTRA or NR Band / SCS / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 30 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 70 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| 34 | n28 | 15 | 75 | 75 | 75 | 75 |  | 75 |  |  |  |  |  |  |  |

      - 6.1.62.7	OOB blocking exception requirements

No OOB blocking exception for DC_n28_34.

6.1.63	DC_n28_39

6.1.63.1	Configuration for DC

Table 6.1.63.1-1: Inter-band NE-DC configurations within FR1 (two bands)

| NE-DC configuration | Uplink NE-DC configuration | Single UL allowed |
| --- | --- | --- |
| DC_n28A_39A DC_n28A_39C | DC_n28A_39A | No |

6.1.63.2	Maximum output power for DC

Table 6.1.63.2-1: Maximum output power for inter-band NE-DC (two bands)

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_n28A_39A | 23 | +2/-3 |

      - 6.1.63.3	Spurious emission band UE co-existence for DC

Table 6.1.63.3-1: Spurious emissions for inter-band DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_n28_39 | E-UTRA Band 8, 26, 34, 40, 41 NR band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 22, 42, 50, 51, 52, 73, 74 NR band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 5, 17 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 14 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 5 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180 kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1, Table 6.6.3.1A-1 in TS 36.101 [4] or in Table 6.5.3.1-1 in TS 38.101-1 [2] from the edge of the channel bandwidth. NOTE 9: Applicable when the assigned E-UTRA carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 11: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 14: This requirement is applicable for 5 and 10 MHz E-UTRA channel bandwidth allocated within 718-728MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and RBstart < 48. NOTE 17: This requirement is applicable in the case of a 10 MHz E-UTRA carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. |  |  |  |  |  |  |  |

6.1.63.4	MSD analysis for DC

For study of 2UL/2DL, Table 6.1.63.4-1 lists up to 5th order harmonics and intermodulation products for DC_n28A_39A.

Table 6.1.63.4-1: Harmonic and IMD analysis

| UL frequency | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
|  | 703 | 748 | 1880 | 1920 |
| 2nd order harmonics frequency range | 1406 | 1496 | 3760 | 3840 |
| 3rd order harmonics frequency range | 2109 | 2244 | 5640 | 5760 |
| 4th order harmonics frequency range | 2812 | 2992 | 7520 | 7680 |
| 5th order harmonics frequency range | 3515 | 3740 | 9400 | 9600 |
| 2nd order IMD products | ¦fy_low-fx_high¦ | ¦fy_high-fx_low¦ | ¦fy_low+fx_low¦ | ¦fy_high+fx_high¦ |
| IMD frequency limits (MHz) | 1132 | 1217 | 2583 | 2668 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 514 | 384 | 3012 | 3137 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3286 | 3416 | 4463 | 4588 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2434 | 2264 | 5166 | 5336 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 189 | 364 | 4892 | 5057 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3989 | 4164 | 6343 | 6463 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6977 | 6772 | 1112 | 892 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8223 | 8428 | 4692 | 4912 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4354 | 4144 | 1516 | 1731 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7046 | 7256 | 5869 | 6084 |

As we can see from Table 6.1.63.4-1 :

- No harmonic issue

- No intermodulation issue may falls into band n28 own Rx.

      - 6.1.63.5	∆TIB and ∆RIB values

For DC_n28_39, same TIB,c and RIB,c values are given in the tables below.

Table 6.1.63.5-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_n28_39 | n28 | 0.3 |
|  | 39 | 0.3 |

Table 6.1.63.5-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_n28_39 | n28 | 0 |
|  | 39 | 0 |

      - 6.1.63.6		Self-interference analysis

Based on the co-existence study and self-interference analysis, it can be seen there are no harmonic and intermodulation issue for DC_n28A_39A configuration, i.e. no MSD needs to be defined.

However, according to the analysis in R4-2117609, the cross band isolation MSD should be defined

Table 6.1.63.6-1: Reference sensitivity exceptions (MSD) due to cross band isolation for PC3 NE-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| 39 | n28 | 3.4 | 3.4 | 3.4 | 2.7 |  | 0.3 |  |  |  |  |  |  |  |

Table 6.1.63.6-2: Uplink configuration for reference sensitivity exceptions due to cross band isolation for NE-DC in NR FR1

| E-UTRA or NR Band / SCS / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 30 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 70 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| 39 | n28 | 15 | 100 | 100 | 100 | 100 |  | 100 |  |  |  |  |  |  |  |

      - 6.1.63.7	OOB blocking exception requirements

No OOB blocking exception for DC_n28_39.

6.1.64	DC_14_n5

6.1.64.1	Configuration for DC

Table 6.1.64.1-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration | Single UL allowed |
| --- | --- | --- |
| DC_14A_n5A | DC_14A_n5A | DC_14A_n5A |

6.1.64.2	Maximum output power for DC

Table 6.1.64.2-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_14A_n5A | 23 | +2/-3 |

6.1.64.3	Spurious emission band UE co-existence for DC

Table 6.1.64.3-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_14_n5 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 24, 25, 26, 29, 30, 48, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 41, 53 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 in TS 36.101 [4] and Table 6.5.3.1-2 in TS 38.101-1 [2] are permitted for each assigned carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180 kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. |  |  |  |  |  |  |  |

6.1.64.4	MSD analysis for DC

For 2UL/2DL UE coexistence study 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.1.64.4-1

Table 6.1.64.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 788 | 798 | 824 | 849 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1576 | 1596 | 1648 | 1698 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2364 | 2394 | 2472 | 2547 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3152 | 3192 | 3296 | 3396 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3940 | 3990 | 4120 | 4245 |
| Two-tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 26 | 61 | 1612 | 1647 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 727 | 772 | 850 | 910 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2400 | 2445 | 2436 | 2496 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1515 | 1570 | 1674 | 1759 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 122 | 52 | 3224 | 3294 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3188 | 3243 | 3260 | 3345 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2608 | 2498 | 2368 | 2303 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 971 | 876 | 746 | 666 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4084 | 4194 | 3976 | 4041 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 4048 | 4143 | 4012 | 4092 |

Based on Table 6.1.64.4-1, it can be seen that:

- 3rd and 5th order IMD products might fall into Rx frequencies of band n5. However, it is not possible to configure the UL test points such that the IMD5 product will overlap the corresponding DL frequency in band n5.

- 3rd order IMD products might fall into Rx frequencies of band 14.

6.1.64.5	∆TIB and ∆RIB values

For DC_14_n5, the TIB,c and RIB,c values are reused from CA_n5-n14 and are given in the tables below.

Table 6.1.64.5-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_14_n5 | 14 | 0.5 |
|  | n5 | 0.5 |

Table 6.1.64.5-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_14_n5 | 14 | 0 |
|  | n5 | 0 |

6.1.64.6	MSD

Based on the co-existence studies for DC_14_n5, MSD is needed. MSD values from CA_n5A-n14A are re-used as shown in the table below.

Table 6.1.64.6-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_14A_n5A | 14 | 791 | 5 | 25 | 761 | N/A | N/A |
|  | n5 | 836 | 5 | 25 | 881 | 25 | IMD3 |
|  | 14 | 795.5 | 5 | 25 | 765.5 | 25 | IMD3 |
|  | n5 | 826.5 | 5 | 25 | 871.5 | N/A | N/A |

6.1.65	DC_11_n1

6.1.65.1	Configuration for DC

Table 6.1.65.1-1: Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration | Single UL allowed |
| --- | --- | --- |
| DC_11A_n1A | DC_11A_n1A | No |

6.1.65.2	Maximum output power for DC

Table 6.1.65.2-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_11A_n1A | 23 | +2/-3 |

6.1.65.3	Spurious emission band UE co-existence for DC

For DC_11_n1, the TIB,c and RIB,c values for UEs not supporting simultaneous Rx/Tx are given in the tables below.

Table 6.1.65.3-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_11_n1 | E-UTRA Band 1, 11, 18, 19, 21, 27, 28, 40, 42, 65 NR Band n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 5, 16 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 16 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 16 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 in TS 36.101 [4] and Table 6.5.3.1-2 in TS 38.101-1 [2] are permitted for each assigned carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180 kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1, Table 6.6.3.1A-1 in TS 36.101 [4] or in Table 6.5.3.1-1 in TS 38.101-1 [2] from the edge of the channel bandwidth. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 16: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. |  |  |  |  |  |  |  |

6.1.65.4	MSD analysis for DC

For 2UL/2DL UE coexistence study 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.1.65.4-1

Table 6.1.65.4 -1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1427.9 | 1447.9 | 1920 | 1980 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 2855.8 | 2895.8 | 3840 | 3960 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 4283.7 | 4343.7 | 5760 | 5940 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 5711.6 | 5791.6 | 7680 | 7920 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 7139.5 | 7239.5 | 9600 | 9900 |
| 6th harmonics frequency limits | 6*fx_low | 6*fx_high | 6* fy_low | 6* fy_high |
| 6th harmonics frequency limits (MHz) | 8567.4 | 8687.4 | 11520 | 11880 |
| 7th harmonics frequency limits | 7*fx_low | 7*fx_high | 7* fy_low | 7* fy_high |
| 7th harmonics frequency limits (MHz) | 9995.3 | 10135.3 | 13440 | 13860 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 552.1 | 472.1 | 3347.9 | 3427.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 875.8 | 975.8 | 2392.1 | 2532.1 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4775.8 | 4875.8 | 5267.9 | 5407.9 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2303.7 | 2423.7 | 4312.1 | 4512.1 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 1104.2 | 944.2 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6203.7 | 6323.7 | 7187.9 | 7387.9 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 6695.8 | 6855.8 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6492.1 | 6232.1 | 3871.6 | 3731.6 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 3084.2 | 2864.2 | 503.7 | 323.7 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9107.9 | 9367.9 | 7631.6 | 7771.6 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8615.8 | 8835.8 | 8123.7 | 8303.7 |

The frequencies of harmonic mixing products are captured in Table 6.1.65.4-2.

Table 6.1.65.4-2: Harmonic mixing products

|  |  |  |  |  | 2nd Harmonic mixing |  | 3rd Harmonic mixing |  | 4th Harmonic mixing |  | 5th Harmonic mixing |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| 11 | 1427.9 | 1447.9 | 1475.9 | 1495.9 | 2951.8 | 2991.8 | 4427.7 | 4487.7 | 5903.6 | 5983.6 | 7379.5 | 7479.5 |
| n1 | 1920 | 1980 | 2110 | 2170 | 4220 | 4340 | 6330 | 6510 | 8440 | 8680 | 10550 | 10850 |

Based on Table 6.1.65.4 -1 and 6.1.65.4-2, it can be seen that

-	2nd order harmonic may fall into Rx frequencies of band n77.

-	3rd order harmonic may fall into Rx frequencies of band 46, 47, n96.

-	4th order harmonic may fall into Rx frequencies of band 46.

-	2nd order IMD products may fall into Rx frequencies of bands 42, 52, n77, n78.

-	3rd order IMD products may fall into Rx frequencies of bands 5, 6, 8, 19, 26, 40, 41, 46, 53, n47, n79, n90.

-	4th order IMD products may fall into Rx frequencies of bands 8, 30, 40, n79, n96.

-	5th order IMD products may fall into Rx frequencies of bands 31, 43, 72, 73, 87, 88, n77, n78, n96.

-	Tx of Band 11 may affect Rx of Band n1 due to the harmonic mixing products due to 3rd order Tx in Band 11 and 2nd order Rx harmonics in Band n1.

Considering that MSD due to 2nd order harmonic mixing with fundamental UL aggressor is low for FDD-TDD band combinations (for example only 6.1dB B2 MSD in DC_2_n77), and, considering that for this FDD-FDD band combination, the 3rd harmonic B11 Tx aggressor power level is much lower than the fundamental Tx power level, it is expected that the DL interference level caused by the second order receiver harmonic mixing will be lower than the thermal noise floor. Consequently, band n1 MSD can be neglected and does not need to be specified.

When 2UL inter-band EN-DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.1.65.4-5 lists up to 7th order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.1.65.4-5: Band 11 and Band n1 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 |  |  |  |
| Galileo | 1559 | - | 1591 |  |  |  |
| GLONASS | 1591 | - | 1610 |  |  |  |
| GPS | 1563 | - | 1587 |  |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 |  | US/Europe | 3rd IMD |
|  | 2400 | - | 2494 |  | Asia | 3rd IMD |
| ISM band (5GHz) | 5150 | - | 5925 |  | US | 3rd Harmonics 3rd IMD |
|  | 5150 | - | 5350 |  | Europe | 3rd IMD |
|  | 5470 | - | 5725 |  |  | 4th Harmonics |
|  | 5150 | - | 5825 |  | Asia | 3rd Harmonics 3rd IMD |
| 45GHz Unlicensed Bands | 42300 | - | 47000 |  | China |  |
|  | 47200 | - | 48400 |  | China |  |
| 60GHz Unlicensed Bands | 57000 | - | 66000 |  | Europe |  |
|  | 57050 | - | 64000 |  | USA Canada |  |
|  | 57000 | - | 64000 |  | South Korea |  |
|  | 59000 | - | 66000 |  | Japan |  |
|  | 59000 | - | 64000 |  | China |  |
|  | 59400 | - | 62900 |  | Australia |  |

6.1.65.5		∆TIB and ∆RIB values

For DC_11_n1, the TIB,c and RIB,c values are given in the tables below.

Table 6.1.65.5-1: ΔTIB,c

| Inter-band EN-DC configuration | E-UTRA and NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_11_n1 | 11 | 0.3 |
|  | n1 | 0.3 |

Table 6.1.65.5-2: ΔRIB,c

| Inter-band EN-DC configuration | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_11_n1 | 11 | 0 |
|  | n1 | 0 |

6.1.66	DC_41_n1

6.1.66.1	Configuration for DC

Table 6.1.66.1-1: Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration | Single UL allowed |
| --- | --- | --- |
| DC_41A_n1A DC_41C_n1A | DC_41A_n1A DC_41C_n1A | No |

6.1.66.2	Maximum output power for DC

Table 6.1.66.2-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_41A_n1A DC_41C_n1A | 23 | +2/-3 |

6.1.66.3	Spurious emission band UE co-existence for DC

For DC_41_n1, the TIB,c and RIB,c values for UEs not supporting simultaneous Rx/Tx are given in the tables below.

Table 6.1.66.3-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_41_n1 | E-UTRA Band 1, 5, 8, 11, 18, 19, 21, 26, 27, 28, 40, 42, 44, 45, 50, 51, 52, 65, 73, 74 NR Band n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | NR Band n77, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 5, 16 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 16 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 16 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 in TS 36.101 [4] and Table 6.5.3.1-2 in TS 38.101-1 [2] are permitted for each assigned carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180 kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1, Table 6.6.3.1A-1 in TS 36.101 [4] or in Table 6.5.3.1-1 in TS 38.101-1 [2] from the edge of the channel bandwidth. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 16: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. |  |  |  |  |  |  |  |

6.1.66.4	MSD analysis for DC

For 2UL/2DL UE coexistence study 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.1.66.4-1

Table 6.1.66.4 -1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2496 | 2690 | 1920 | 1980 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 4992 | 5380 | 3840 | 3960 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7488 | 8070 | 5760 | 5940 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 9984 | 10760 | 7680 | 7920 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 12480 | 13450 | 9600 | 9900 |
| 6th harmonics frequency limits | 6*fx_low | 6*fx_high | 6* fy_low | 6* fy_high |
| 6th harmonics frequency limits (MHz) | 14976 | 16140 | 11520 | 11880 |
| 7th harmonics frequency limits | 7*fx_low | 7*fx_high | 7* fy_low | 7* fy_high |
| 7th harmonics frequency limits (MHz) | 17472 | 18830 | 13440 | 13860 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 516 | 770 | 4416 | 4670 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3012 | 3460 | 1150 | 1464 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6912 | 7360 | 6336 | 6650 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 5508 | 6150 | 3070 | 3444 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 1032 | 1540 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 9408 | 10050 | 8256 | 8630 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 8832 | 9340 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5424 | 4990 | 8840 | 8004 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 948 | 380 | 4230 | 3528 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10176 | 10610 | 11904 | 12740 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10752 | 11320 | 11328 | 12030 |

Based on Table 6.1.66.4 -1, it can be seen that

-	2nd order harmonic may fall into Rx frequencies of band 46, n77, n79.

-	3rd order harmonic may fall into Rx frequencies of band 46, 47, n96.

-	2nd order IMD products may fall into Rx frequencies of bands 12, 13,14, 17, 28, 29, 44, 67, 68, 71, 85, n79.

-	3rd order IMD products may fall into Rx frequencies of bands 32, 42, 45, 50, 51, 52, 75, 76, n77, n78, n91, n92, n93, n94, n96.

-	4th order IMD products may fall into Rx frequencies of bands 11, 21, 24, 32, 42, 45, 46, 47, 50, 51, 52, 74, 75, 76, n77, n78, n91, n92, n93, n94.

-	5th order IMD products may fall into Rx frequencies of bands 5, 6, 8, 12, 13,14, 17, 18, 19, 20, 22, 26, 27, 28, 29, 31, 42, 43, 44, 46, 48, 49, 67, 68, 71, 72, 73, 85, 87, 88, n77, n78, n79.

Because of no harmonic and IMD products falling into the own Rx Band, there is no MSD issue for this DC configuration.

When 2UL inter-band EN-DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.1.66.4-2 lists up to 7th order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.1.66.4-2: Band 41 and Band n1 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 |  |  |  |
| Galileo | 1559 | - | 1591 |  |  |  |
| GLONASS | 1591 | - | 1610 |  |  |  |
| GPS | 1563 | - | 1587 |  |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 |  | US/Europe |  |
|  | 2400 | - | 2494 |  | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 |  | US | 2nd Harmonics 4th IMD |
|  | 5150 | - | 5350 |  | Europe | 2nd Harmonics 4th IMD |
|  | 5470 | - | 5725 |  |  | 4th IMD |
|  | 5150 | - | 5825 |  | Asia | 2nd Harmonics 4th IMD |
| 45GHz Unlicensed Bands | 42300 | - | 47000 |  | China |  |
|  | 47200 | - | 48400 |  | China |  |
| 60GHz Unlicensed Bands | 57000 | - | 66000 |  | Europe |  |
|  | 57050 | - | 64000 |  | USA Canada |  |
|  | 57000 | - | 64000 |  | South Korea |  |
|  | 59000 | - | 66000 |  | Japan |  |
|  | 59000 | - | 64000 |  | China |  |
|  | 59400 | - | 62900 |  | Australia |  |

The REFSENS exceptions for cross band isolation for DC_41_n1 are proposed in Table 6.1.66.4-3 and Table 6.1.66.4-4.

Table 6.1.66.4-3: Reference sensitivity exceptions (MSD) due to cross band isolation for PC3 EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n1 | 41 | 6.1 | 6.1 | 6.1 | 6.1 |  |  |  |  |  |  |  |  |  |
| 41 | n1 | 9.1 | 9.1 | 9.1 | 9.1 | 9.1 | 9.1 | 9.1 | 9.1 |  |  |  |  |  |

Table 6.1.66.4-4: Uplink configuration for reference sensitivity exceptions due to cross band isolation for EN-DC in NR FR1

| E-UTRA or NR Band / SCS / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 30 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 70 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| n1 | 41 | 15 | 100 | 100 | 100 | 100 |  |  |  |  |  |  |  |  |  |
| 41 | n1 | 15 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |  |  |  |  |  |

6.1.66.5	∆TIB and ∆RIB values

For DC_41_n1, the TIB,c and RIB,c values are given in the tables below.

Table 6.1.66.5-1: ΔTIB,c

| Inter-band EN-DC configuration | E-UTRA and NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_41_n1 | 41 | 0.5 |
|  | n1 | 0.5 |

Table 6.1.66.5-2: ΔRIB,c

| Inter-band EN-DC configuration | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_41_n1 | 41 | 0 |
|  | n1 | 0 |

6.1.67	DC_7_n79

6.1.67.1	Configuration for DC

Table 6.1.67.1-1:  Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_7A_n79A | DC_7A_n79A | No |
| DC_7A_n79C | DC_7A_n79A | No |

6.1.67.2	Maximum output power for DC

Table 6.1.67.2-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_7A_n79A | 23 | +2/-3 |

6.1.67.3	Spurious emission band UE co-existence for DC

Table 6.1.67.3-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_7_n79 | E-UTRA Band 1, 3, 5, 8, 28, 34, 40, 42, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1, Table 6.6.3.1A-1 in TS 36.101 [4] or in Table 6.5.3.1-1 in TS 38.101-1 [2] from the edge of the channel bandwidth. NOTE 6: This requirement is applicable for any channel bandwidths within the range 2500 - 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. |  |  |  |  |  |  |  |

6.1.67.4	MSD analysis for DC

For 2UL/2DL UE coexistence study 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.1.67.4-1.

Table 6.1.67.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2500 | 2570 | 4400 | 5000 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 5000 | 5140 | 8800 | 10000 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7500 | 7710 | 13200 | 15000 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 10000 | 10280 | 17600 | 20000 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 12500 | 12850 | 22000 | 25000 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1830 | 2500 | 6900 | 7570 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 0 | 740 | 6230 | 7500 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 9400 | 10140 | 11300 | 12570 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2500 | 3310 | 10630 | 12500 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 11900 | 12710 | 15700 | 17570 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 5000 | 3660 | 13800 | 15140 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 17500 | 15030 | 5880 | 5000 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 10000 | 8060 | 1090 | 2500 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 20100 | 22570 | 14400 | 15280 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 18200 | 20140 | 16300 | 17710 |

IMD4 interference for DC_7_n79 may fall into DL frequency of band 7.

There is no harmonic issue for DC_7_n79.

6.1.67.5	∆TIB and ∆RIB values

For DC_7_n79, TIB,c and RIB,c values will be the same as for DC_7_n78 in 38.101-3. The TIB,c and RIB,c values are given in the tables below.

Table 6.1.67.5-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7_n79 | 7 | 0.5 |
|  | n79 | 0.8 |

Table 6.1.67.5-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_7_n79 | 7 | 0 |
|  | n79 | 0.5 |

6.1.67.6	self-interference analysis

Referring to DC_30A_n77A, the MSD due to IMD4 is specified for DC_7_n79 as below.

Table 6.1.67.6-1: MSD test points for PCell due to dual uplink operation for EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_7_n79 | 7 | 2510 | 5 | 25 | 2630 | [8] | IMD4 |
|  | n79 | 4900 | 40 | 216 | 4900 | N/A | N/A |

The frequency range for n79 is near condition of Band 7 2nd harmonic interference, e.g band 7 is located in 2500~2520MHz (2nd harmonic interference is 5000~5040MHz). Referring to DC_1_n77, the MSD of band n79 can be specified after 10, 15 and 20MHz are introduced for band n79.

Table 6.1.67.6-2: Reference sensitivity exceptions (MSD) due to UL harmonic for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| 7 | n793 |  | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |  |
| NOTE 3: The requirements are only applicable to channel bandwidths no larger than 20 MHz and with a carrier frequency at MHz offset from in the victim (higher band) with , where andare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.1.67.6-3: Uplink configuration for reference sensitivity exceptions due to UL harmonic interference for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 30 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 70 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| 7 | n79 | 15 |  | 25 | 36 | 50 |  |  |  |  |  |  |  |  |  |

6.1.68	DC_38_n79

6.1.68.1	Configuration for DC

Table 6.1.68.1-1:  Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_38A_n79A | DC_38A_n79A | No |
| DC_38A_n79C | DC_38A_n79A | No |

6.1.68.2	Maximum output power for DC

Table 6.1.68.2-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_38A_n79A | 23 | +2/-3 |

6.1.68.3	Spurious emission band UE co-existence for DC

Table 6.1.68.3-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_38_n79 | E-UTRA Band 1, 3, 5, 8, 28, 34, 40, 42, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2620 | - | 2645 | -15.5 | 5 | 5, 7, 22 |
|  | Frequency range | 2645 | - | 2690 | -40 | 1 | 5, 22 |
| NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1, Table 6.6.3.1A-1 in TS 36.101 [4] or in Table 6.5.3.1-1 in TS 38.101-1 [2] from the edge of the channel bandwidth. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 22: This requirement is applicable for power class 3 UE for any channel bandwidths within the range 2570 - 2615 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2605.5 - 2607.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2597 - 2605 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. |  |  |  |  |  |  |  |

6.1.68.4	MSD analysis for DC

For 2UL/2DL UE coexistence study 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.1.68.4-1.

Table 6.1.68.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2570 | 2620 | 4400 | 5000 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 5140 | 5240 | 8800 | 10000 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7710 | 7860 | 13200 | 15000 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 10280 | 10480 | 17600 | 20000 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 12850 | 13100 | 22000 | 25000 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1780 | 2430 | 6970 | 7620 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 140 | 840 | 6180 | 7430 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 9540 | 10240 | 11370 | 12620 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2710 | 3460 | 10580 | 12430 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 12110 | 12860 | 15770 | 17620 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 4860 | 3560 | 13940 | 15240 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 17430 | 14980 | 6080 | 5280 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 9860 | 7960 | 940 | 2290 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 20170 | 22620 | 14680 | 15480 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 18340 | 20240 | 16510 | 17860 |

There is no MSD issue for DC_38_n79 since band 38 and n79 are TDD bands.

There is no harmonic issue for DC_38_n79.

6.1.68.5	∆TIB and ∆RIB values

For DC_38_n79, TIB,c and RIB,c values will be the same as for DC_41_n79 in 38.101-3. The TIB,c and RIB,c values are given in the tables below.

Table 6.1.68.5-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_38_n79 | 38 | 0.3 |
|  | n79 | 0.8 |

Table 6.1.68.5-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_38_n79 | 38 | 0 |
|  | n79 | 0.5 |

6.1.68.6	self-interference analysis

There is no REFSENS exception for DC_38_n79.

6.1.69	DC_5_n1

6.1.69.1	Operating bands for DC

Table 6.1.69.1-1: DC band combination of 1 LTE band + 1 NR band

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_5_n1 | 5 | n1 | No |

6.1.69.2	Configuration for DC

Table 6.1.69.2-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_5A_n1A | DC_5A_n1A | 5A | n1A |

6.1.69.3	Maximum output power for DC

Table 6.1.69.3-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_5A_n1A | 23 | +2/-3 |

6.1.69.4	Spurious emission band UE co-existence for DC

Table 6.1.69.4-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_5_n1 | E-UTRA Band 5, 7, 8, 11, 18, 19, 21, 22, 26, 28, 31, 38, 40, 42, 43, 50, 51, 65, 73, 74 NR band n1 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA band 41, 52 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180 kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

6.1.69.5	MSD analysis for DC

For 2UL/2DL UE coexistence study 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.1.69.5-1

Table 6.1.69.5-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 1920 | 1980 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high | 2*fx_low | 2*fx_high |
| 2nd harmonics frequency limits (MHz) | 1648 | 1698 | 3840 | 3960 |
| 3rd harmonics frequency limits | 3* fy_low | 3* fy_high | 3*fx_low | 3*fx_high |
| 3rd harmonics frequency limits (MHz) | 2472 | 2547 | 5760 | 5940 |
| 4th harmonics frequency limits | 4* fy_low | 4* fy_high | 4*fx_low | 4*fx_high |
| 4th harmonics frequency limits (MHz) | 3296 | 3396 | 7680 | 7920 |
| 5th harmonics frequency limits | 5* fy_low | 5* fy_high | 5*fx_low | 5*fx_high |
| 5th harmonics frequency limits (MHz) | 4120 | 4245 | 9600 | 9900 |
| 6th harmonics frequency limits | 6* fy_low | 6* fy_high | 6*fx_low | 6*fx_high |
| 6th harmonics frequency limits (MHz) | 4944 | 5094 | 11520 | 11880 |
| 7th harmonics frequency limits | 7* fy_low | 7* fy_high | 7*fx_low | 7*fx_high |
| 7th harmonics frequency limits (MHz) | 5768 | 5943 | 13440 | 13860 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1156 | 1071 | 2744 | 2829 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 332 | 222 | 2991 | 3136 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3568 | 3678 | 4664 | 4809 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2312 | 2142 | 5488 | 5658 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 492 | 627 | 4911 | 5116 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4392 | 4527 | 6584 | 6789 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7096 | 6831 | 1476 | 1316 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8504 | 8769 | 5216 | 5376 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4292 | 4062 | 1293 | 1488 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7408 | 7638 | 6312 | 6507 |

Based on Table 6.1.69.5-1, it can be seen that

- 4th order IMD products may fall into own Rx frequencies of band n1.

6.1.69.6	∆TIB and ∆RIB values

For DC_5_n1, the TIB,c and RIB,c values are reused from DC_1_n5 and are given in the tables below.

Table 6.1.69.6-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5_n1 | 5 | 0.3 |
|  | n1 | 0.3 |

Table 6.1.69.6-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_5_n1 | 5 | 0 |
|  | n1 | 0 |

6.1.69.7	MSD

Co-existence studies shows that IMD4 theoretically may be falling into own Rx frequencies of band n1. However, it is not possible to find test-points making this happen with a direct hit to band n1. Therefor it is seen that no MSD need to be defined.

6.1.70	DC_5_n3

6.1.70.1	Operating bands for DC

Table 6.1.70.1-1: DC band combination of 1 LTE band + 1 NR band

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_5_n3 | 5 | n3 | DC_5_n3 |

6.1.70.2	Configuration for DC

Table 6.1.70.2-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_5A_n3A | DC_5A_n3A | 5A | n3A |

6.1.70.3	Maximum output power for DC

Table 6.1.70.3-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_5A_n3A | 23 | +2/-3 |

6.1.70.4	Spurious emission band UE co-existence for DC

Table 6.1.70.4-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_5_n3 | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 21, 26, 28, 31, 38, 40, 43, 50, 51, 65, 73, 74 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 34 NR band n3 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 22, 42, 52 Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180 kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

6.1.70.5	MSD analysis for DC

For 2UL/2DL UE coexistence study 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.1.70.5-1

Table 6.1.70.5-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high | 2*fx_low | 2*fx_high |
| 2nd harmonics frequency limits (MHz) | 1648 | 1698 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3* fy_low | 3* fy_high | 3*fx_low | 3*fx_high |
| 3rd harmonics frequency limits (MHz) | 2472 | 2547 | 5130 | 5355 |
| 4th harmonics frequency limits | 4* fy_low | 4* fy_high | 4*fx_low | 4*fx_high |
| 4th harmonics frequency limits (MHz) | 3296 | 3396 | 6840 | 7140 |
| 5th harmonics frequency limits | 5* fy_low | 5* fy_high | 5*fx_low | 5*fx_high |
| 5th harmonics frequency limits (MHz) | 4120 | 4245 | 8550 | 8925 |
| 6th harmonics frequency limits | 6* fy_low | 6* fy_high | 6*fx_low | 6*fx_high |
| 6th harmonics frequency limits (MHz) | 4944 | 5094 | 10260 | 10710 |
| 7th harmonics frequency limits | 7* fy_low | 7* fy_high | 7*fx_low | 7*fx_high |
| 7th harmonics frequency limits (MHz) | 5768 | 5943 | 11970 | 12495 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 961 | 861 | 2534 | 2634 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 137 | 12 | 2571 | 2746 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3358 | 3483 | 4244 | 4419 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1922 | 1722 | 5068 | 5268 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 687 | 837 | 4281 | 4531 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4182 | 4332 | 5954 | 6204 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6316 | 5991 | 1686 | 1511 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7664 | 7989 | 5006 | 5181 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3707 | 3432 | 873 | 1098 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6778 | 7053 | 5892 | 6117 |

Based on Table 6.1.70.5-1, it can be seen that

- 4th order IMD products may fall into own Rx frequencies of band n3.

- 2nd and 5th order IMD products may fall into own Rx frequencies of band 5

6.1.70.6	∆TIB and ∆RIB values

For DC_5_n3, the TIB,c and RIB,c values are reused from DC_3_n5 and are given in the tables below.

Table 6.1.70.6-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5_n3 | 5 | 0.3 |
|  | n3 | 0.3 |

Table 6.1.70.6-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_5_n3 | 5 | 0.2 |
|  | n3 | 0.2 |

6.1.70.7	MSD

Co-existence studies shows that MSD need to be defined for IMD4 falling into own Rx frequencies of band 3 and IMD2 falling into band n5. MSD value same as in DC_3_n5.

Table 6.1.70.7-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_5A_n3A | 5 | 838 | 5 | 25 | 883 | N/A | N/A |
|  | n3 | 1771 | 10 | 50 | 1866 | 4 | IMD4 |
|  | 5 | 838 | 5 | 25 | 883 | 24 | IMD23 |
|  | n3 | 1721 | 10 | 50 | 1816 | N/A | N/A |
| NOTE 3: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |

6.1.71	DC_2_n25

6.1.71.1	Operating bands for EN-DC

Table 6.1.71.1-1: DC band combination of 1 LTE band + 1 NR band

| EN-DC band | E-UTRA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 2_n25 | 2 | n25 | Yes |

6.1.71.2	Configuration for DC

Table 6.1.71.2-1: Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A_n25A10, 11, 13 | n25A | 2A | n25A |
| NOTE 10: Void. The combination is not used alone as fallback mode of other band combinations in which UL in Band 2 is not used. NOTE 11: The minimum requirements for inter-band EN-DC apply when the maximum power spectral density imbalance between downlink carriers is within 6 dB. The power spectral density imbalance condition also applies for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. NOTE 13: The minimum requirements apply for synchronized DL carriers with a maximum receive time difference ≤ 3 usec. The requirements also apply for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. |  |  |  |

6.1.71.3	Maximum output power for DC

Table 6.1.71.3-1: Maximum output power for Inter-band EN-DC

| DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_2A_n25A2 |  |  | 23 | +2/-3 |
| NOTE 2: Only single switched UL is supported |  |  |  |  |

6.1.71.4	Spurious emission band UE co-existence for DC

Only Single Tx Switched UL mode is supported for this combination, therefore no spurious emission band UE co-existence requirement is needed

6.1.71.5	MSD analysis for DC

Cross band isolation need to be defined.

Table 6.1.71.5-1: Reference sensitivity exceptions (MSD) due to cross band isolation for PC3 EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n25 | 2 | 33 | 33 | 33 | 33 |  |  |  |  |  |  |  |  |  |

Table 6.1.71.5-2: Uplink configuration for reference sensitivity exceptions due to cross band isolation for EN-DC in NR FR1

| E-UTRA or NR Band / SCS / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 30 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 70 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| n25 | 2 | 15 | 40 | 40 | 40 | 40 |  |  |  |  |  |  |  |  |  |
| NOTE 1: The UL configuration applies regardless of the channel bandwidth of the UL band. UL resource blocks allocation in the table shall be further limited to that specified in Table 7.3.1-2 in TS 36.101 [4] or Table 7.3.2-3 in TS 38.101-1 [2]. NOTE 2: The UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.1.71.6	MPR, AMPR requirements

Since only Single Tx Switched UL mode is supported for this combination, no MPR or A-MPR requirement is needed

6.2	Inter-band DC including FR2

6.2.1	DC_66_n261

6.2.1.1	Configuration for DC

Table 6.2.1.1-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_66A_n261(A-L) | DC_66A_n261A DC_66A_n261G DC_66A_n261H DC_66A_n261I |
| DC_66A_66A_n261(A-L) | DC_66A_n261A DC_66A_n261G DC_66A_n261H DC_66A_n261I |

6.2.1.2	Spurious emission band UE co-existence for DC

Co-existence was studied for lower order combinations DC_66_n261 in Rel-15, and the results are captured in TR 37.863-01-01. No further studies are needed for this combination.

6.2.1.3	MSD analysis for DC

MSD values was studied for lower order combinations DC_66_n261 in Rel-15, and the results are captured in TR 37.863-01-01. No further studies are needed for this combination

6.2.1.4	∆TIB and ∆RIB values

TIB,c and RIB,c was studied for lower order combinations DC_66_n261 in Rel-15, and the results are captured in TR 37.863-01-01. No further studies are needed for this combination.

6.2.2	DC_48_n261

6.2.2.1	Configuration for DC

Table 6.2.2.1-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_48A_n261(A-L) | DC_48A_n261A DC_48A_n261G DC_48A_n261H DC_48A_n261I |

6.2.2.2	Spurious emission band UE co-existence for DC

Co-existence was studied for lower order combinations DC_48_n261 in Rel-16, and the results are captured in TR 37.716-11-11. No further studies are needed for this combination.

6.2.2.3	MSD analysis for DC

MSD values was studied for lower order combinations DC_48_n261 in Rel-16, and the results are captured in TR 37.716-11-11.

6.2.2.4	∆TIB and ∆RIB values

TIB,c and RIB,c was studied for lower order combinations DC_48_n261 in Rel-16, and the results are captured in TR 37.716-11-11. No further studies are needed for this combination.

6.2.3	DC_13_n261

6.2.3.1	Configuration for DC

Table 6.2.3.1-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_13A_n261(A-L) | DC_13A_n261A DC_13A_n261G DC_13A_n261H DC_13A_n261I |

6.2.3.2	Spurious emission band UE co-existence for DC

Co-existence was studied for lower order combinations DC_13_n261 in Rel-16, and the results are captured in TR 37.716-11-11. No further studies are needed for this combination.

6.2.3.3	MSD analysis for DC

MSD values was studied for lower order combinations DC_13_n261 in Rel-16, and the results are captured in TR 37.716-11-11. No further studies are needed for this combination

6.2.3.4	∆TIB and ∆RIB values

TIB,c and RIB,c was studied for lower order combinations DC_13_n261 in Rel-16, and the results are captured in TR 37.716-11-11.

6.2.4	DC_5_n261

6.2.4.1	Configuration for DC

Table 6.2.4.1-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_5A_n261(A-J) | DC_5A_n261A DC_5A_n261G DC_5A_n261H DC_5A_n261I |
| DC_5A_n261(A-L) | DC_5A_n261A DC_5A_n261G DC_5A_n261H DC_5A_n261I |

6.2.4.2	Spurious emission band UE co-existence for DC

Co-existence was studied for lower order combinations DC_5_n261 in Rel-15, and the results are captured in 37.863-01-01. No further studies are needed for this combination.

6.2.4.3	MSD analysis for DC

MSD value was studied for lower order combinations DC_5_n261 in Rel-15, and the results are captured in 37.863-01-01. No further studies are needed for this combination.

6.2.4.4	∆TIB and ∆RIB values

TIB,c and RIB,c was studied for lower order combinations DC_5_n261 in Rel-15, and the results are captured in 37.863-01-01. No further studies are needed for this combination.

6.2.5	DC_2_n261

6.2.5.1	Configuration for DC

Table 6.2.5.1-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_2A_n261(A-L) | DC_2A_n261A DC_2A_n261G DC_2A_n261H DC_2A_n261I |
| DC_2A-2A_n261A | DC_2A_n261A |
| DC_2A-2A_n261I | DC_2A_n261A DC_2A_n261G DC_2A_n261H DC_2A_n261I |
| DC_2A-2A_n261M | DC_2A_n261A DC_2A_n261G DC_2A_n261H DC_2A_n261I |

6.2.5.2	Spurious emission band UE co-existence for DC

Co-existence was studied for lower order combinations DC_2_n261in Rel-16, and the results are captured in 37.716-11-11. No further studies are needed for this combination.

6.2.5.3	MSD analysis for DC

MSD value was studied for lower order combinations DC_2_n261in Rel-16, and the results are captured in 37.716-11-11. No further studies are needed for this combination.

6.2.5.4	∆TIB and ∆RIB values

TIB,c and RIB,c were studied for lower order combinations DC_2_n261 in Rel-16, and the results are captured in 37.716-11-11. No further studies are needed for this combination.

6.2.6	DC_20_n257

6.2.6.1	Configuration for DC

Table 6.2.6.1-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_20A_n257A DC_20A_n257B DC_20A_n257C DC_20A_n257D DC_20A_n257E DC_20A_n257F DC_20A_n257G DC_20A_n257H DC_20A_n257I DC_20A_n257J DC_20A_n257K DC_20A_n257L DC_20A_n257M | DC_20A_n257A |

6.2.6.2	Maximum output power for DC

UE maximum output power reduction requirement for E-UTRA single carrier and CA operation specified in clauses 6.2.3 and 6.2.3A of TS 36.101 [4] and for NR single carrier and CA operation specified in clause 6.2.2, 6.2A.2 , and 6.2D.2 of TS 38.101-2 [3] apply.

6.2.6.3	Spurious emission band UE co-existence for DC

General requirements specified in clause 6.5B.3.4 from TS 38.101-3 apply.

6.2.6.4	MSD analysis for DC

There is no MSD requirement for these band combinations.

6.2.6.5	∆TIB and ∆RIB values

For DC_20_n257, the ΔTIB,c and ΔRIB,c values are given in the tables below.

Table 6.2.6.5-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_20_n257 | 20 | 0 |
|  | n257 | 0 |

Table 6.2.6.5-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_20_n257 | 20 | 0 |
|  | n257 | 0 |

6.2.7	DC_3-3_n257

6.2.7.1	Configuration for DC

Table 6.2.7.1-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_3A-3A_n257G DC_3A-3A_n257H DC_3A-3A_n257I DC_3A-3A_n257J DC_3A-3A_n257K DC_3A-3A_n257L DC_3A-3A_n257M | DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K |

6.2.7.2	Spurious emission band UE co-existence for DC

General requirements specified in clause 6.5B.3.4 from TS 38.101-3 apply.

6.2.7.3	MSD analysis for DC

No IMD issues are expected for DC configuration between the LTE bands below 6GHz and FR2 NR band.

6.2.7.4	∆TIB and ∆RIB values

MSD values was studied for lower order combinations DC_3-3_n257 in Rel-16, and the results are captured in TR 37.716-11-11. No further studies are needed for this combination.

6.2.8	DC_7_n257, DC_7-7_n257

6.2.8.1	Configuration for DC

Table 6.2.8.1-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_7A_n257J DC_7A_n257K DC_7A_n257L DC_7A_n257M | DC_7A_n257J DC_7A_n257K |
| DC_7A-7A_n257J DC_7A-7A_n257K DC_7A-7A_n257L DC_7A-7A_n257M | DC_7A_n257J DC_7A_n257K |

6.2.8.2	Spurious emission band UE co-existence for DC

General requirements specified in clause 6.5B.3.4 from TS 38.101-3 apply.

6.2.8.3	MSD analysis for DC

No IMD issues are expected for DC configuration between the LTE bands below 6GHz and FR2 NR band.

6.2.8.4	∆TIB and ∆RIB values

MSD values was studied for lower order combinations DC_7_n257, DC_7-7_n257 in Rel-16, and the results are captured in TR 37.716-11-11. No further studies are needed for this combination.

6.3	Intra-band contiguous DC

6.3.1	DC_(n)66

6.3.1.1	Channel bandwidths per operating band for DC

Table 6.3.1.1-1: Supported channel bandwidths per DC configuration

|  |  | E-UTRA – NR configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink EN-DC configuration | Uplink EN-DC configurations | Component carriers in order of increasing carrier frequency |  |  | Maximum aggregated bandwidth (MHz) | BCS |
|  |  | Channel bandwidths for E-UTRA carrier (MHz) | Channel bandwidths NR for carrier (MHz) | Channel bandwidths for E-UTRA carrier (MHz) |  |  |
| DC_(n)66AA | DC_(n)66AA1 | 5, 10, 15, 20 | 5,10,15,20, 25, 30, 40 |  | 60 | 0 |
|  |  |  | 5,10,15,20, 25, 30, 40 | 5, 10, 15, 20 |  |  |
| NOTE 1: Only single switched UL is supported |  |  |  |  |  |  |

6.3.1.2	Configuration for DC

Table 6.3.1.1-2:  Intra-band contiguous EN-DC configurations

| EN-DC configuration | Uplink EN-DC configuration | Single UL allowed |
| --- | --- | --- |
| DC_(n)66AA | DC_(n)66AA1 | Yes |
| NOTE 1: Only single switched UL is supported |  |  |

6.3.1.3	Maximum output power for DC

Table 6.3.1.3-1: Maximum output power for EN-DC (continuous sub-blocks)

| EN-DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_(n)66AA1 | 23 | +2/-3 |
| NOTE 1: Only single switched UL is supported |  |  |

6.3.1.4	Spurious emission band UE co-existence for DC

Note that only single switched UL mode is supported for this combination hence no additional spurious emission band UE co-existence requirement is needed.

6.3.1.5	MSD analysis for DC

Since only single swithched UL mode is supported and UL-DL separation is large for this combination, no MSD requirement is needed.

6.3.1.6	MPR, AMPR requirements

Note that only single switched UL mode is supported for this combination hence no additional MPR or A-MPR requirement is needed.

6.3.2	DC_(n)3

6.3.2.1	Channel bandwidths per operating band for DC

Table 6.3.2.1-1: Supported channel bandwidths per DC configuration

|  |  | E-UTRA – NR configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink EN-DC configuration | Uplink EN-DC configurations | Component carriers in order of increasing carrier frequency |  |  | Maximum aggregated bandwidth (MHz) | BCS |
|  |  | Channel bandwidths for E-UTRA carrier (MHz) | Channel bandwidths NR for carrier (MHz) | Channel bandwidths for E-UTRA carrier (MHz) |  |  |
| DC_(n)3AA | DC_(n)3AA1 | 5, 10, 15, 20 | 5,10,15,20, 25, 30 |  | 50 | 0 |
|  |  |  | 5,10,15,20, 25, 30 | 5, 10, 15, 20 |  |  |
| NOTE 1: Only single switched UL is supported |  |  |  |  |  |  |

6.3.2.2	Configuration for DC

Table 6.3.2.2-1:  Intra-band contiguous EN-DC configurations

| EN-DC configuration | Uplink EN-DC configuration | Single UL allowed |
| --- | --- | --- |
| DC_(n)3AA | DC_(n)3AA1 | DC_(n)3AA |
| NOTE 1: Only single switched UL is supported |  |  |

6.3.2.3	Maximum output power for DC

Table 6.3.2.3-1: Maximum output power for EN-DC (continuous sub-blocks)

| EN-DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_(n)3AA1 | 23 | +2/-3 |
| NOTE 1: Only single switched UL is supported |  |  |

6.3.2.4	Spurious emission band UE co-existence for DC

Note that only single switched UL mode is supported for this combination hence no additional spurious emission band UE co-existence requirement is needed.

6.3.2.5	MSD analysis for DC

Only single switched UL mode is supported and UL-DL separation is not large (20 MHz), the MSD needs to be studied.

6.3.2.6	MPR, AMPR requirements

Note that only single switched UL mode is supported for this combination hence no additional MPR or A-MPR requirement is needed.

6.4	Intra-band non-contiguous DC

6.4.1	DC_71_n71

6.4.1.1	Channel bandwidths per operating band for DC

Table 6.4.1.1-1: Supported channel bandwidths per DC configuration

|  |  | E-UTRA – NR configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink EN-DC configuration | Uplink EN-DC configurations | Component carriers in order of increasing carrier frequency |  |  | Maximum aggregated bandwidth (MHz) | BCS |
|  |  | Channel bandwidths for E-UTRA carrier (MHz) | Channel bandwidths NR for carrier (MHz) | Channel bandwidths for E-UTRA carrier (MHz) |  |  |
| DC_71A_n71A | DC_71A_n71A2 | 15 | 5 |  | 20 | 0 |
|  |  | 10 | 5, 10 |  |  |  |
|  |  | 5 | 5, 10, 15 |  |  |  |
|  |  |  | 5 | 15 |  |  |
|  |  |  | 5, 10 | 10 |  |  |
|  |  |  | 5, 10, 15 | 5 |  |  |
|  |  | 5, 10, 15, 20 | 5, 10, 15, 20 |  | 30 | 1 |
|  |  |  | 5, 10, 15, 20 | 5, 10, 15, 20 |  |  |
| NOTE 2: Only single switched UL is supported. |  |  |  |  |  |  |

6.4.1.2	Configuration for DC

The configuration to be specified in Table 5.5B.3-1 of 38.101-3 is as follows:

Table 6.4.1.2-1: Intra-band non-contiguous EN-DC configurations

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_71A_n71A3 | DC_71A_n71A5 | Yes5 |
| NOTE 3: The minimum requirements only apply for non-simultaneous Tx/Rx between all carriers. NOTE 5: Only single switched UL is supported. |  |  |

6.4.1.3	Maximum output power for DC

The maximum output power for the uplink EN-DC configuration in Table 6.2B.1.2-1 of 38.101-3 is given as.

Table 6.4.1.3-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| EN-DC configuration | Power class 1.5 (dBm) | Tolerance (dB) | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| DC_71A_n71A4 |  |  |  |  | 23 | +2/-3 |
| NOTE 4: Only single switched UL is supported |  |  |  |  |  |  |

6.4.1.4	Spurious emission band UE co-existence for DC

Since single switched UL only – no need for defining spurious emissions

6.4.1.5	MSD analysis for DC

For 2UL/2DL UE coexistence study 2nd, 3rd, 4th, 5th, 6th and 7th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented below.

Table 6.4.1.5-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 663 | 698 | 663 | 698 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1326 | 1396 | 1326 | 1396 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 1989 | 2094 | 1989 | 2094 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2652 | 2792 | 2652 | 2792 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3315 | 3490 | 3315 | 3490 |
| 6th harmonics frequency limits | 6*fx_low | 6*fx_high | 6* fy_low | 6* fy_high |
| 6th harmonics frequency limits (MHz) | 3978 | 4188 | 3978 | 4188 |
| 7th harmonics frequency limits | 7*fx_low | 7*fx_high | 7* fy_low | 7* fy_high |
| 7th harmonics frequency limits (MHz) | 4641 | 4886 | 4641 | 4886 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 35 | 35 | 1326 | 1396 |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 628 | 733 | 628 | 733 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1989 | 2094 | 1989 | 2094 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1291 | 1431 | 1291 | 1431 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 70 | 70 | 2652 | 2792 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 2652 | 2792 | 2652 | 2792 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2129 | 1954 | 2129 | 1954 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 768 | 593 | 768 | 593 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 3315 | 3490 | 3315 | 3490 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 3315 | 3490 | 3315 | 3490 |

Based on Table 6.4.1.5-1, it can be seen that 3rd and 5th order IMD can fall in the Rx frequency of band 71. However, this is not relevant for single switched UL only operation.

When 2UL inter-band EN-DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. A summary of this is given below.

Table 6.4.1.5-2: Harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | No | US |  |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | No | Asia |  |
| 45GHz Unlicensed Bands | 42300 | - | 47000 | No | China |  |
|  | 47200 | - | 48400 | No | China |  |
| 60GHz Unlicensed Bands | 57000 | - | 66000 | No | Europe |  |
|  | 57050 | - | 64000 | No | USA Canada |  |
|  | 57000 | - | 64000 | No | South Korea |  |
|  | 59000 | - | 66000 | No | Japan |  |
|  | 59000 | - | 64000 | No | China |  |
|  | 59400 | - | 62900 | No | Australia |  |

6.4.1.6	∆RIBNC values

Based on the co-existence studies for DC_71_n71 it is found that a Wgap exception is needed defined due to the configuration having 2DL carriers (1NR and 1LTE) in the DL of band 71 with an UL NR carrier. This is to be defined in Table 7.3B.3.2-2 of 38.101-3.

Table 6.4.1.6-1: Intra-band non-contiguous EN-DC with one uplink configuration on NR for reference sensitivity (NR carrier is higher than the E-UTRA carrier)

| DC configuration | Aggregated bandwidth |  | Wgap / (MHz) | UL NR allocation | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- |
|  | NR | E-UTRA |  |  |  |  |
| DC_71A_n71A | 5 MHz | 5 MHz | 5 < Wgap ≤ 25 | 5 | 4.0 | FDD |
|  |  |  | 0 < Wgap ≤ 5 | 20 | 0 |  |
|  | 10 MHz | 5 MHz | 5 < Wgap ≤ 20 | 51 | 4.6 |  |
|  |  |  | 0 < Wgap ≤ 5 | 201 | 2.3 |  |
|  | 10 MHz | 10 MHz | 5 < Wgap ≤ 15 | 51 | 4.3 |  |
|  |  |  | 0 < Wgap ≤ 5 | 201 | 3.2 |  |
|  | 15 MHz | 10 MHz | 5 < Wgap ≤ 10 | 52 | 22.2 |  |
|  |  |  | 0 < Wgap ≤ 5 | 203 | 5.2 |  |
|  | 20 MHz | 10 MHz | Wgap = 5 | 5 | [22.2] |  |
| NOTE 1: Uplink resource block starts at RB position 9 for SCS=15KHz. NOTE 2: Uplink resource block starts at RB position 2 for SCS=15KHz. NOTE 3: Uplink resource block starts at RB position 19 for SCS=15KHz. |  |  |  |  |  |  |

6.4.1.7	MPR, AMPR requirements

Not applicable for DC_71_n71, since it is defined as single switched UL only.

6.4.2	DC_66_n66

6.4.2.1	Channel bandwidths per operating band for DC

Table 6.4.2.1-1: Supported channel bandwidths per DC configuration

| E-UTRA – NR configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink EN-DC configuration | Uplink EN-DC configurations | Component carriers in order of increasing carrier frequency |  |  | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
|  |  | Channel bandwidths for E-UTRA carrier (MHz) | Channel bandwidths for NR carrier (MHz) | Channel bandwidths for E-UTRA carrier (MHz) |  |  |
| DC_66A_n66A | DC_66A_n66A2 | 5, 10, 15, 20 | 5, 10, 15, 20, 25, 30, 40 |  | 60 | 1 |
|  |  |  | 5, 10, 15, 20, 25, 30, 40 | 5, 10, 15, 20 |  |  |
| NOTE 2: Only single switched UL is supported. |  |  |  |  |  |  |

6.4.2.2	Configuration for DC

Table 6.4.2.2-1: Intra-band non-contiguous EN-DC configurations

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_66A_n66A | DC_66A_n66A | Yes5 |
| NOTE 3: The minimum requirements only apply for non-simultaneous Tx/Rx between all carriers. NOTE 5: Only single switched UL is supported. |  |  |

6.4.2.3	Maximum output power for DC

Table 6.4.2.3-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| EN-DC configuration | Power class 1.5 (dBm) | Tolerance (dB) | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| DC_66A_n66A4 |  |  |  |  | 23 | +2/-3 |
| NOTE 4: Only single switched UL is supported |  |  |  |  |  |  |

6.4.2.4	Spurious emission band UE co-existence for DC

Since single switched UL only – no need for defining spurious emissions

6.4.2.5	MSD analysis for DC

Note that only Single Tx Switched UL mode is supported for this combination, no MSD requirement is needed.

6.4.2.6	∆RIB values

The delta R IBNC values for intra-band non-contiguous EN-DC DC_66_n66 are already specified in TS38.101-3 and shown below, there is no additional requirement.

Table 6.4.2.6-1: Intra-band non-contiguous EN-DC with one uplink configuration for reference sensitivity

| DC configuration | Aggregated channel bandwidth (LTE+NR) | Wgap / (MHz) | UL LTE allocation | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- |
| DC_66A_n66A | NOTE 4 | NOTE 8 | NOTE 9 | 0 | FDD |
| NOTE 1: 1 refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission. NOTE 2: Wgap is the sub-block gap between the two sub-blocks. NOTE 3: The table only applies when the center frequency of LTE carrier is higher than the NR carrier, and the ΔRIBNC applies to the NR DL carrier only. NOTE 4: All combinations of channel bandwidths defined in Table 5.3B.1.3-1. NOTE 5: 5 refers to the UL resource blocks shall be located at RBstart=25. NOTE 6: 6 refers to the UL resource blocks shall be located at RBstart=35. NOTE 7: 7 refers to the UL resource blocks shall be located at RBstart=50. NOTE 8: All applicable sub-block gap sizes. NOTE 9: The UL LTE allocation is same as Transmission bandwidth configuration NRB as defined in Table 5.6-1 in [4]. |  |  |  |  |  |

6.4.2.7	MPR, AMPR requirements

Note that only Single Tx Switched UL mode is supported for this combination, no MPR or A-MPR requirement is needed.

Annex A (informative): Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2020-08 | RAN4-96e | R4-2010367 |  |  |  | Initial TR skeleton | 0.0.0 |
| 2020-08 | RAN4-96e | R4-2010369 |  |  |  | included TPs/pCRs: R4-2010595 DC_8A_n7A R4-2011599 DC_66_n77 R4-2011600 DC_13_n77 R4-2011601 DC_5_n77 R4-2011602 DC_2_n77 R4-2010384 TP for DC_19_n1 R4-2010394 TP for DC_21_n1 R4-2010396 TP for DC_42_n1 R4-2011604 DC_48_n25A R4-2011611 DC_28A_n2A and DC_2A_n28A R4-2011615 DC_4A_n2A R4-2011617 DC_4A_n5A R4-2010555 DC_4A_n7A R4-2010877 DC_28A_n66A R4-2011622 DC_4A_n28A R4-2011623 DC_66A_n28A R4-2011882 DC_28A_n1A R4-2011592 DC_42_n3 R4-2010317 DC_66_n261 R4-2010319 DC_48_n261 R4-2010325 DC_13_n261 R4-2010327 DC_5_n261 R4-2010328 DC_2_n261 | 0.1.0 |
| 2020-11 | RAN4-97e | R4-2014786 |  |  |  | included TPs/pCRs: R4-2016655 DC_8_n2 R4-2016658 DC_18A_n28A R4-2016663 DC_18A_n41A R4-2016667 DC_7_n2A R4-2016670 DC_71A_n71A R4-2014877 DC_2_n261 | 0.2.0 |
| 2021-02 | RAN4-98e | R4-2101097 |  |  |  | included TPs/pCRs: R4-2103012 DC_25_n77 R4-2103013 DC_25_n78 R4-2103021 DC_21_n28 R4-2103026 DC_12_n71 R4-2103027 DC_71A_n41A R4-2103028 DC_71A_n2A | 0.3.0 |
| 2021-04 | RAN4-98-bis-e | R4-2106888 |  |  |  | included TPs/pCRs: R4-2105285 DC_2A_n30A R4-2105286 DC_5A_n30A R4-2105287 DC_12A_n30A R4-2105288 DC_66A_n30A R4-2105290 DC_71A_n71A R4-2105296 DC_11_n41 R4-2105298 DC_66_n66 R4-2105307 DC_38A_n28A R4-2105308 DC_12_n77 R4-2105309 DC_14_n77 R4-2105310 DC_30_n77 R4-2105311 DC_14_n30 R4-2105318 DC_7_n25 R4-2106735 DC_13_n25 R4-2106919 DC_3-3_n8, DC_7-7_n8 R4-2106639 DC_20_n257 | 0.4.0 |
| 2021-08 | RAN4-100e | R4-2112520 |  |  |  | included TPs/pCRs: R4-2114810 DC_48_n77 R4-2112848 DC_n28A_3A R4-2112849 DC_n28A_8A R4-2114815 DC_n28A_40A R4-2112853 DC_n41A_3A R4-2114816 DC_n41A_8A R4-2114817 DC_n41A_34A R4-2112856 DC_n41A_39A R4-2112857 DC_n41A_40A R4-2112928 DC_38A_n3A R4-2113063 DC_(n)66AA R4-2114818 DC_38_n1 R4-2114819 DC_38_n8 R4-2112650 addition of uplink configurations for DC_3-3_n257, DC_7_n257, DC_7-7_n257 | 0.5.0 |
| 2021-11 | RAN4-101e | R4-2117866 |  |  |  | included TPs/pCRs: R4-2119802 DC_n3A_1A R4-2119803 DC_(n)3AA R4-2119809 DC_n8A_1A R4-2119818 DC_n8A_3A R4-2119819 DC_n77(2A)_1A R4-2119822 DC_n77(2A)_3A R4-2119825 DC_n3A_8A R4-2119827 DC_n77A_8A, DC_n77(2A)_8A R4-2119795 DC_n28A_34A R4-2119796 DC_n28A_39A R4-2119797 DC_n28A_3C R4-2119798 DC_n28A_40C R4-2119800 Addition of DC_14_n5 R4-2119814 DC_11_n1 R4-2119815 DC_41_n1 R4-2118563 DC_38A_n79A and DC_38A_n79C R4-2119823 DC_7A_n79A and DC_7A_n79C R4-2118052 correction for DC_7_n257 EN-DC configurations | 0.6.0 |
| 2021-12 | RAN-94e | RP-213072 |  |  |  | TR as provided for information to TSG RAN | 1.0.0 |
| 2022-03 | RAN4-102e | R4-2204045 |  |  |  | included TPs/pCRs: R4-2200237 TP update for DC_11_n1 R4-2202153 DC_5_n3 R4-2202157 DC_n28A_8C R4-2202158 DC_n41_39 R4-2202159 DC_n41_3 R4-2202160 DC_n41_8 R4-2202161 DC_n41_40 R4-2202163 DC_5_n1 R4-2204047 general part update R4-2206384 Including band combinations DC_2_n25 | 1.1.0 |