3GPP TSG-RAN WG4 Meeting # 102-e	R4-2204145

# Electronic Meeting, February 21-March 3, 2022

Source:	LG Electronics

Title:	RRM requirements for SL-DRX

Agenda item:	10.15.5.2

Document for:	Discussion

Introduction

In this paper, we provide our views on the remaining issues of RRM requirements for intra-band concurrent SL operation.

Discussion

According to WF[1] which was agreed in the last RAN4#101bis-e meeting, the remaining issues are as follows. We would like to continue the discussion on the remained issues which are highlighted in yellow color.

- Among agreements of requirements related to intra-band con-current SL operation - 1.2 Scheduling restriction calculation when switching TDM based intra-band con-current SL operation - Consider only RF switching time for scheduling availability - When switch from uplink transmision to NR V2X sidelink transmision occurs in sidelink slot ‘n’, - UE is not expected to transmit or receive on NR V2X sidelink on the sidelink slot ‘n’. - When switch from NR V2X sidelink transmision to uplink transmision occurs in sidelink slot ‘n-1’, - UE is not expected to transmit or receive on NR V2X sidelink on the sidelink slot ‘n-1’. - When switch from NR V2X sidelink transmision to uplink transmision occurs in Uu slot ‘n’, - UE is not expected to transmit uplink or receive downlink on the Uu slot ‘n’. - When switch from uplink transmision to NR V2X sidelink transmision occurs in Uu slot ‘n-1’, - UE is not expected to transmit uplink or receive downlink on the Uu slot ‘n-1’. - FFS: whether the requirement is different when NTA is smaller than the switching time - 1.2.1 Timeline applicability when switching TDM based intra-band con-current SL operation - FFS Tx preparation time by - SL to Uu Tx switch: K2 >= 2 slots for the first UL grant when switching from SL to Uu - Uu to SL Tx switch: N2 >= one slot + PSSCH preparation time for the first SL grant when switching from Uu to SL - Among agreements of requirements related to SL-DRX - 2.2.3 UE Rx(Data) drop rate requirements for Asynchronized SLSS measurement & search - Further discuss in next meeting - Option 1: maximum drop rate(X) during Tdetect,SyncRef UE_V2X - Option 1a : X = 0.3%(Rel-16) (CATT, vivo, Xiaomi) - Option 1b : X = 0.3%(Rel-16) with additional clarification that “The dropping rate is calculated ignoring SL-DRX configuration” (Oppo) - Option 1c : X = max{2 slots, 0.3% of its V2X data reception} (LGE) - Option 2 : maximum aggregated drop window(Y) during Tdetect,SyncRef UE_V2X (Oppo) - Y = 480ms - Moderator’s suggestion based on 2nd round - Up to 24 slots(or 5%) of V2X data reception during maximum aggregated drop window of 480ms is allowed to be dropped for PSBCH monitoring - 2.2.4 Conditional SyncRef UE detection requirements for Asynchronized SLSS measurement & search - Further discuss in next meeting - Option 1 : Define conditional SyncRef UE detection requirements (QC, LGE, vivo) - Option 1a: (Qualcomm) - UE can skip asynchronized SyncRef UE search to save power when the following conditions are all satisfied over an evaluation period: - SLSS RSRP is larger than a threshold - SLSS RSRP variation is lower than a threshold. The SLSS RSRP variation is the average value of (instantaneous RSRP - current filtered RSRP)^2 during the evaluation period - Data connection is maintained with the current SyncRef UE source - The evaluation period is the same as SLSS Tx initiation/cease evaluation period when SLSS is the synchronization source - Option 1b: (LGE) - UE can skip asynchronized SyncRef UE search to save power when the following conditions are all satisfied over an evaluation period, Tevaluate,SLSS: - All SLSS RSRPs are larger than a threshold, syncTxThreshOoC. - Data connection is maintained with the current SyncRef UE source - Option 1c : FFS (vivo) - Option 2 : Not define conditional SyncRef UE detection requirements for asynchronized SLSS measurement & search in R17 (Huawei, Xiaomi) - 2.2.5 UE Tx(Data & SLSS) drop rate requirements for Asynchronized - Specify based on the previous agreements - Allow Tx dropping at most in an aggregated window of 480ms during Tdetect,SyncRef UE_V2X async search - If needed, revisit in next meeting - 2.3.1 Whether to define interruption to WAN due to SL-DRX - Agreement (GTW) - Define interruption requirements on NR transmission if configured due to NR SL transitions between active and non-active in SL DRX when NR SL is in SL-DRX but NR is in non-DRX - used as baseline - FFS whether interruptions are applicable for the following WAN conditions and impact on SL transitions between active and non-active SL DRX if interruptions are not applicable: - reception of paging - reception of system information - while onDurationTimer is running - while RLF timer is running - while UE is performing CBD - FFS on interruptions for the case when NR is in DRX and SL is in SL-DRX - 2.3.4 Interruption to WAN due to SyncRef UE detection and/or Sensing during SL DRX off duration - Define interruption - Further discuss how to specify it in next meeting

Related to Intra-band con-current SL operation

- 1.2 Scheduling restriction calculation when switching TDM based intra-band con-current SL operation

The requirement for switching from uplink transmission to NR V2X sidelink transmission is not different depending on whether NTA is smaller than the switching time. Figure 2.1 shows it.

Figure 2.1: Switching from UL to SL transmission

Proposal 1: The scheduling restriction requirement is same no matter what NTA is smaller than the switching time.

- 1.2.1 Timeline applicability when switching TDM based intra-band con-current SL operation

For Tx preparation time, our understanding is that it is already specified in RAN1 specification. Therefore, RAN4 does not need to consider it if the additional difference is not identified.

Proposal 2: Not consider Tx preparation time requirement in RAN4 if the additional difference is not identified.

Related to SL-DRX

- 2.2.3 UE Rx(Data) drop rate requirements for Asynchronized SLSS measurement & search

SL-DRX cycle and on-duration timer were agreed in RAN2 as follows.

  - SL-DRX cycle :

    - ms10, ms20, ms32, ms40, ms60, ms64, ms70, ms80, ms128, ms160, ms256, ms320, ms512, ms640, ms1024, ms1280, ms2048, ms2560, ms5120, ms10240

  - SL-DRX on-duration :

    - ms1, ms2, ms3, ms4, ms5,ms6, ms8, ms10, ms20, ms30, ms40, ms50, ms60,                                                     ms80, ms100, ms200, ms300, ms400, ms500, ms600, ms800, ms1000, ms1200,                                                         ms1600

Table 2.1 summarizes the aggregated on-duration window during Tdetect,SyncRef UE_V2X (8s) for all possible combinations of SL-DRX cycle and SL-DRX on-duration.

Table 2.1: Aggregated on-duration window

| SL-DRX on-duration(ms) | SL-DRX cycle (ms) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 20 | 32 | 40 | 60 | 64 | 70 | 80 | 128 | 160 | 256 | 320 | 512 | 640 | 1024 | 1280 | 2048 | 2560 | 5120 | 10240 |
| 1 | 800 | 400 | 250 | 200 | 133 | 125 | 114 | 100 | 62 | 50 | 31 | 25 | 15 | 12 | 7 | 6 | 3 | 3 | 1 | 0 |
| 2 | 1600 | 800 | 500 | 400 | 266 | 250 | 228 | 200 | 125 | 100 | 62 | 50 | 31 | 25 | 15 | 12 | 7 | 6 | 3 | 1 |
| 3 | 2400 | 1200 | 750 | 600 | 400 | 375 | 342 | 300 | 187 | 150 | 93 | 75 | 46 | 37 | 23 | 18 | 11 | 9 | 4 | 2 |
| 4 | 3200 | 1600 | 1000 | 800 | 533 | 500 | 457 | 400 | 250 | 200 | 125 | 100 | 62 | 50 | 31 | 25 | 15 | 12 | 6 | 3 |
| 5 | 4000 | 2000 | 1250 | 1000 | 666 | 625 | 571 | 500 | 312 | 250 | 156 | 125 | 78 | 62 | 39 | 31 | 19 | 15 | 7 | 3 |
| 6 | 4800 | 2400 | 1500 | 1200 | 800 | 750 | 685 | 600 | 375 | 300 | 187 | 150 | 93 | 75 | 46 | 37 | 23 | 18 | 9 | 4 |
| 7 | 5600 | 2800 | 1750 | 1400 | 933 | 875 | 800 | 700 | 437 | 350 | 218 | 175 | 109 | 87 | 54 | 43 | 27 | 21 | 10 | 5 |
| 10 |  | 4000 | 2500 | 2000 | 1333 | 1250 | 1142 | 1000 | 625 | 500 | 312 | 250 | 156 | 125 | 78 | 62 | 39 | 31 | 15 | 7 |
| 20 |  |  | 5000 | 4000 | 2666 | 2500 | 2285 | 2000 | 1250 | 1000 | 625 | 500 | 312 | 250 | 156 | 125 | 78 | 62 | 31 | 15 |
| 30 |  |  | 7500 | 6000 | 4000 | 3750 | 3428 | 3000 | 1875 | 1500 | 937 | 750 | 468 | 375 | 234 | 187 | 117 | 93 | 46 | 23 |
| 40 |  |  |  |  | 5333 | 5000 | 4571 | 4000 | 2500 | 2000 | 1250 | 1000 | 625 | 500 | 312 | 250 | 156 | 125 | 62 | 31 |
| 50 |  |  |  |  | 6666 | 6250 | 5714 | 5000 | 3125 | 2500 | 1562 | 1250 | 781 | 625 | 390 | 312 | 195 | 156 | 78 | 39 |
| 60 |  |  |  |  |  |  |  | 6000 | 3750 | 3000 | 1875 | 1500 | 937 | 750 | 468 | 375 | 234 | 187 | 93 | 46 |
| 80 |  |  |  |  |  |  |  |  | 5000 | 4000 | 2500 | 2000 | 1250 | 1000 | 625 | 500 | 312 | 250 | 125 | 62 |
| 100 |  |  |  |  |  |  |  |  | 6250 | 5000 | 3125 | 2500 | 1562 | 1250 | 781 | 625 | 390 | 312 | 156 | 78 |
| 200 |  |  |  |  |  |  |  |  |  |  | 6250 | 5000 | 3125 | 2500 | 1562 | 1250 | 781 | 625 | 312 | 156 |
| 300 |  |  |  |  |  |  |  |  |  |  |  | 7500 | 4687 | 3750 | 2343 | 1875 | 1171 | 937 | 468 | 234 |
| 400 |  |  |  |  |  |  |  |  |  |  |  |  | 6250 | 5000 | 3125 | 2500 | 1562 | 1250 | 625 | 312 |
| 500 |  |  |  |  |  |  |  |  |  |  |  |  | 7812 | 6250 | 3906 | 3125 | 1953 | 1562 | 781 | 390 |
| 600 |  |  |  |  |  |  |  |  |  |  |  |  |  | 7500 | 4687 | 3750 | 2343 | 1875 | 937 | 468 |
| 800 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 6250 | 5000 | 3125 | 2500 | 1250 | 625 |
| 1000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 7812 | 6250 | 3906 | 3125 | 1562 | 781 |
| 1200 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 7500 | 4687 | 3750 | 1875 | 937 |
| 1600 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 6250 | 5000 | 2500 | 1250 |

Green blocks are that ratio of  SL-DRX on-durationSL-DRX cycle×100 is less than 10%.

Blue and Brown blocks correspond to the aggregated minimum window of on-duration which is higher than an aggregated window(Y) of 480ms, and 160ms respectively during Tdetect,SyncRef UE_V2X (8s). Overall drop rate needs to be defined considering the aggregated minimum window of on-duration. Here, the Blue and Brown blocks are included in the green blocks.

  - 24ms,  Y≥480

  - 8ms,    160 ≤ Y < 480

  - 4ms,    Y < 160

Proposal 3: Overall drop shall not exceed X ms of its V2X data reception in Table 2.2 during Tdetect,SyncRef UE_V2X for the purpose of selection / reselection to the SyncRef UE when SL-DRX is used.

Table 2.2: Overall UE Rx data drop for Asynchronized SLSS measurement & search when SL-DRX is used

| Aggregated window of on-duration (Y) during Tdetect,SyncRef UE_V2X [ms] | UE Rx data drop(X) [ms] |
| --- | --- |
| Y ≥ 480ms | 24 |
| 160 ms ≤ Y < 480ms | 8 |
| Y < 160ms | 4 |

- 2.2.4 Conditional SyncRef UE detection requirements for Asynchronized SLSS measurement & search

We proposed to consider the conditional SyncRef UE detection requirements in aspect of power saving as follows.

  - For Asynchronized SLSS measurement & search, UE can skip asynchronized SyncRef UE search to save power when the following conditions are all satisfied over an evaluation period, Tevaluate,SLSS :

    - All SLSS RSRPs are larger than a threshold, syncTxThreshOoC.

    - Data connection is maintained with the current SyncRef UE source

However, some companies raised the concern of impact on performance. Regarding that this meeting is the last meeting to complete Rel-17, we suggest not to consider in Rel-17 and to postpone to Rel-18.

Proposal 4: Not consider issue 2.2.4 in Rel-17 and postpone to Rel-18.

- 2.3.1 Whether to define interruption to WAN due to SL-DRX

The case when NR is in DRX and SL is in SL-DRX can reuse the following EN-DC requirement when both E-UTRA PCell and PSCell are in DRX.

  - When both E-UTRA PCell and PSCell are in DRX, no interruption is allowed.

Proposal 5: When NR is in DRX and SL is in SL-DRX, no interruption is allowed.

For whether interruptions are applicable for the following WAN conditions and impact on SL transitions between active and non-active SL DRX if interruptions are not applicable:

  - reception of paging

  - reception of system information

  - while onDurationTimer is running

  - while RLF timer is running

  - while UE is performing CBD

Above 1st, 2nd, and 3rd bullets are from LTE Prose interruption requirements for inter-band LTE Prose and LTE operating band. The bands are licensed band. It means network can control LTE Prose configuration. The case is similar to SL mode 1.

However, 4th and 5th bullets are new. There were no cases of 4th and 5th in EN-DC interruption requirements. The reason is that network can control the DRX configuration of PCell and PScell. Therefore, SL mode 1 does not apply to 4th and 5th bullet.

Proposal 6: Interruptions to WAN are not applicable for the following WAN conditions when SL is mode1.

  - reception of paging

  - reception of system information

  - while onDurationTimer is running

- 2.4.3 Interruption to WAN due to SyncRef UE detection and/or Sensing during SL DRX off duration

RAN4 agreed to define interruption. The remaining issue is how to specify it.

When SL DRX is configured, a UE has to measure RSRP on sensing resources within sensing window, and the sensing resources could be positioned during DRX off. In the worst case, interruption due to sensing operation might be considered during all sensing resources. However, it would cause serious data transmission and reception problem over the WAN. So, some additional percentage of missed ACK/NACK should be allowed in case of sensing operation taking into account the limited interruption to the WAN (e.g., less than 1.5% probability of missed ACK/NACK).

Proposal 7: When UE performs SyncRef UE detection and/or sensing operation during SL DRX off duration, interruption is allowed

  - up to 0.5% probability of missed ACK/NACK when the configured SL-DRX cycle is less than 640,

  - up to 0.375% probability of missed ACK/NACK when the configured SL-DRX cycle is less than 640.

Conclusion

In this contribution, we provided our views on the remained RRM requirements related to SL DRX [1]. Based on the views, we propose as follows.

Related to Intra-band con-current SL operation

- 1.2 Scheduling restriction calculation when switching TDM based intra-band con-current SL operation

Proposal 1: The scheduling restriction requirement is same no matter what NTA is smaller than the switching time.

- 1.2.1 Timeline applicability when switching TDM based intra-band con-current SL operation

Proposal 2: Not consider Tx preparation time requirement in RAN4 if the additional difference is not identified.

Related to SL-DRX

- 2.2.3 UE Rx(Data) drop rate requirements for Asynchronized SLSS measurement & search

Proposal 3: Overall drop shall not exceed X ms of its V2X data reception in Table 2.2 during Tdetect,SyncRef UE_V2X for the purpose of selection / reselection to the SyncRef UE when SL-DRX is used.

Table 2.2: Overall UE Rx data drop for Asynchronized SLSS measurement & search when SL-DRX is used

| Aggregated window of on-duration (Y) during Tdetect,SyncRef UE_V2X [ms] | UE Rx data drop(X) [ms] |
| --- | --- |
| Y ≥ 480ms | 24 |
| 160 ms ≤ Y < 480ms | 8 |
| Y < 160ms | 4 |

- 2.2.4 Conditional SyncRef UE detection requirements for Asynchronized SLSS measurement & search

Proposal 4: Not consider issue 2.2.4 in Rel-17 and postpone to Rel-18.

- 2.3.1 Whether to define interruption to WAN due to SL-DRX

Proposal 5: When NR is in DRX and SL is in SL-DRX, no interruption is allowed.

Proposal 6: Interruptions to WAN are not applicable for the following WAN conditions when SL is mode1.

  - reception of paging

  - reception of system information

  - while onDurationTimer is running

- 2.4.3 Interruption to WAN due to SyncRef UE detection and/or Sensing during SL DRX off duration

Proposal 7: When UE performs SyncRef UE detection and/or sensing operation during SL DRX off duration, interruption is allowed

  - up to 0.5% probability of missed ACK/NACK when the configured SL-DRX cycle is less than 640,

  - up to 0.375% probability of missed ACK/NACK when the configured SL-DRX cycle is less than 640.

- Reference

[1] R4-2202650, “WF on NR SL enhancements RRM requirements”, LG Electronics

[2] R4-2202732, “Email discussion summary for [101bis-e][ 215] NR_SL_enh_RRM”, Moderator (LG Electronics)