3GPP TSG-RAN WG4 Meeting # 102-e	R4-2204170

# Electronic Meeting, February 21 – March 3, 2022

Source: 	MediaTek Inc.

Title: 	Draft text proposal to update TS 38.101-5 on clause 7.6 Blocking characteristics

Agenda Item:	10.13.4.2

Document for:	Approval

Introduction

Regarding NR NTN WF [1][2] and Tdoc [3] from RAN4 meetings, there were agreements about the UE blocking characteristics. In the contribution, the draft CR text is proposed for NR NTN UE blocking characteristics.

Discussion

From [1], the agreement of out-of-band blocking for NR NTN UE is shown below.

Out-of band blocking Current requirement can be reused.

Observation 1: From [1], it was agreed that current OOB requirement based on TS 38.101-1 can be reused for NTN UE.

Proposal 1: To reuse NR UE OBB definition and requirements from TS 38.101-1 for NR NTN UE.

From [2] and [3], the agreement of NBB and IBB for NR NTN UE is shown below.

NBB and IBB for band n255/n256 Option 1: For frequency lower than 2.7GHz, there are potential TN interferers. To consider reusing TN UE NBB and IBB requirements for NTN UE as assumption.

Observation 2: From [2] and [3], the agreement is to consider reusing TN UE NBB and IBB requirements for NTN UE as assumption.

Proposal 2: To reuse NR UE IBB and NBB definition and requirements from TS 38.101-1 for NR NTN UE.

Observation 3: Two options are provided here for text proposal. Option 2 is preferred as it is straightforward.

Conclusion

In the contribution, regarding NR NTN UE, the CR for blocking characteristics is proposed.

Observation 1: From [1], it was agreed that current OOB requirement based on TS 38.101-1 can be reused for NTN UE.

Proposal 1: To reuse NR UE OBB definition and requirements from TS 38.101-1 for NR NTN UE.

Observation 2: From [2] and [3], the agreement is to consider reusing TN UE NBB and IBB requirements for NTN UE as assumption.

Proposal 2: To reuse NR UE IBB and NBB definition and requirements from TS 38.101-1 for NR NTN UE.

Observation 3: Two options are provided here for text proposal. Option 2 is preferred as it is straightforward.

- 4 References

[1] R4-2115642, “WF on NTN UE RF requirement”, Huawei, HiSilicon

[2] R4-2203036, “WF on UE RF requirement for NTN UE”, Huawei

[3] R4-2203113, “Email discussion summary for [101-bis-e][308] NTN_Solutions_Part3”, CATT

- Option 1 : Draft proposed CR text compared with TS 38.101-1

<Start of change>

- 7.6	Blocking characteristics

- 7.6.1	General

The blocking characteristic is a measure of the receiver's ability to receive a wanted signal at its assigned channel frequency in the presence of an unwanted interferer on frequencies other than those of the spurious response or the adjacent channels, without this unwanted input signal causing a degradation of the performance of the receiver beyond a specified limit. The blocking performance shall apply at all frequencies except those at which a spurious response occurs.

- 7.6.2	In-band blocking

For NR NTN bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz in-band blocking (IBB) is defined for an unwanted interfering signal falling into the UE receive band or into the first 15 MHz below or above the UE receive band.

The throughput of the wanted signal shall be ≥ 95 % of the maximum throughput of the reference measurement channels with parameters specified in Table 7.6.2-1 and Table 7.6.2-2. The relative throughput requirement shall be met for any SCS specified for the channel bandwidth of the wanted signal.

Table 7.6.2-1: In-band blocking parameters for NR NTN bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| RX parameter | Units | Channel bandwidth (MHz) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 5, 10 | 15 | 20 |
| Power in transmission bandwidth configuration3 | dBm | REFSENS + 6 dB | REFSENS + 7 dB | REFSENS + (9 + 10log10(BWChannel /20)) dB |
| BWinterferer | MHz | 5 |  |  |
| FIoffset, case 1 | MHz | 7.5 |  |  |
| FIoffset, case 2 | MHz | 12.5 |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in clause 7.3.2 with PCMAX_L,f,c defined in clause 6.2.4. |  |  |  |  |

<If we have the same Annexes for NTN, we may even not need the table. >

Table 7.6.2-2: In-band blocking for NR NTN bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| NR band | Parameter | Unit | Case 1 | Case 2 | Case 3 | Case 4 |
| --- | --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -56 | -44 | -15 | -38 |
|  | Finterferer (offset) | MHz | -BWChannel/2 – FIoffset, case 1 and BWChannel/2 + FIoffset, case 1 | ≤ -BWChannel/2 – FIoffset, case 2 and ≥ BWChannel/2 + FIoffset, case 2 |  | -BWChannel/2-11 |
| n255, n256 | Finterferer | MHz | NOTE 2 | FDL_low – 15 to FDL_high + 15 |  |  |
| NOTE 1: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 2: For each carrier frequency, the requirement applies for two interferer carrier frequencies: a: -BWChannel/2 – FIoffset, case 1; b: BWChannel/2 + FIoffset, case 1 |  |  |  |  |  |  |

< For bands n255 and n256, we can also just refer to requirement for bands in TS 38.101-1 if we have the same IBB for NTN, we may even not need the table. >

- 7.6.3	Out-of-band blocking

For NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz out-of-band band blocking is defined for an unwanted CW interfering signal falling outside a frequency range 15 MHz below or above the UE receive band.

The throughput of the wanted signal shall be ≥ 95% of the maximum throughput of the reference measurement channels with parameters specified in Table 7.6.3-1 and Table 7.6.3-2. The relative throughput requirement shall be met for any SCS specified for the channel bandwidth of the wanted signal.

Table 7.6.3-1: Out-of-band blocking parameters for NR NTN bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| RX parameter | Units | Channel bandwidth (MHz) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 5, 10 | 15 | 20 |
| Power in transmission bandwidth configuration2 | dBm | REFSENS + 6 dB | REFSENS + 7 dB | REFSENS + (9 + 10log10(BWChannel /20)) dB |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in clause 7.3.2 with PCMAX_L,f,c defined in clause 6.2.4. |  |  |  |  |

Table 7.6.3-2: Out of-band blocking for NR NTN bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| NR band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| n255, n256 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 6000 MHz. NOTE 2: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 2580 MHz and FInterferer < 2775 MHz. |  |  |  |  |  |

For interferer frequencies across ranges 1, 2 and 3 in Table 7.6.3-2, a maximum of

exceptions are allowed for spurious response frequencies in each assigned frequency channel when measured using a step size of  MHz withthe number of resource blocks in the downlink transmission bandwidth configuration, BWChannel the bandwidth of the frequency channel in MHz and n = 1, 2, 3 for SCS = 15, 30, 60 kHz, respectively. For these exceptions, the requirements in clause 7.7 apply.

<If we have the same clause 7.7 requirements for NTN, we may even not need the text. >

- 7.6.4	Narrow band blocking

This requirement is measure of a receiver's ability to receive a NR signal at its assigned channel frequency in the presence of an unwanted narrow band CW interferer at a frequency, which is less than the nominal channel spacing. The relative throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels with parameters specified in Table 7.6.4-1.

Table 7.6.4-1: Narrow Band Blocking

| NR band | Parameter | Unit | Channel Bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 |
| n255, 256 | Pw | dBm | PREFSENS + channel-bandwidth specific value below |  |  |  |
|  |  |  | 16 | 13 | 14 | 16 |
|  | Puw (CW) | dBm | -55 |  |  |  |
|  | Fuw (offset SCS= 15 kHz) 4 | MHz | BWChannel2+0.2SCS+0.5+0.5SCS |  |  |  |
|  | Fuw (offset SCS= 30 kHz)4 | MHz | NA |  |  |  |
| NOTE 1: The transmitter shall be set a 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in clause 7.3.2 with PCMAX_L,f,c defined in clause 6.2.4 NOTE 2: The PREFSENS power level is specified in clause 7.3.2. NOTE 3: Fuw shall be rounded to half of SCS. |  |  |  |  |  |  |

<End of change>

- Option 2 : Draft proposed CR text

<Start of change>

- 7.6	Blocking characteristics

- 7.6.1	General

The definition for general blocking characteristics in sub-clause 7.6.1 from TS 38.101-1 is applicable for NR NTN UE.

- 7.6.2	In-band blocking

<Same approach as for Option 1>

- 7.6.3	Out-of-band blocking

The definition and requirements for out-of-band blocking in sub-clause 7.6.3 from TS 38.101-1 are applicable for NR NTN UE for NTN bands of the corresponding FDL_high and FUL_high range.

For NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz, interferer frequencies across ranges 1, 2 and 3 in Table 7.6.3-2 of [38.101-1], a maximum of

exceptions are allowed for spurious response frequencies in each assigned frequency channel when measured using a step size of  MHz withthe number of resource blocks in the downlink transmission bandwidth configuration, BWChannel the bandwidth of the frequency channel in MHz and n = 1, 2, 3 for SCS = 15, 30, 60 kHz, respectively. For these exceptions, the requirements in clause 7.7 apply.

<If we have the same clause 7.7 requirements for NTN, we may even not need the 2nd paragraph text. >

- 7.6.4	Narrow band blocking

The definition and requirements for narrow blocking in clause 7.6.4 from TS 38.101-1 are applicable for NR NTN UE.

<End of change>