3GPP TSG-RAN WG4 Meeting # 102-e	R4-2204188

# Electronic Meeting, February 21 – March 3, 2022

Agenda Item:	10.16.8.1

Source:	MediaTek Inc.

Title:	Discussion on RRM requirements in FR2-2

Document for:		Discussion

Introduction

In this paper, our views on RRM measurement requirements in FR2-2 are provided, including scheduling restriction and deriveSSB-IndexFromCell.

deriveSSB-IndexFromCell

Assumptions on deriveSSB-IndexFromCell

- For 480 kHz SSB SCS

  - deriveSSB-IndexFromCell is always enabled by the network

- For 960 kHz SSB SCS

  - deriveSSB-IndexFromCell is up to network configuration (i.e. can be enabled or disabled)

  - FFS: NOTE: RAN4 to also specify the frame boundary alignment tolerance even for the case when deriveSSB-IndexFromCell is disabled

  - FFS: If deriveSSB-IndexFromCell is not enabled, the requirement of SSB index detection for intra-frequency measurement for FR2-2 need to be specified for 960kHz SCS

- FFS whether the deriveSSB_IndexFromCell is always enabled or not in unlicensed band in FR2-2.

  - Note: It’s not always enabled in FR1 NR-U.

Tolerance for frame boundary alignment

- Keep the existing requirements, i.e., 2 SSB symbols, for SCS up to 240kHz

- FFS: For 480kHz SCS:

  - Option 1a: 3 SSB symbols

  - Option 1b: 2 SSB symbols

- FFS: For 960kHz SCS: when deriveSSB-IndexFromCell is enabled

  - Option 2a: 3 SSB symbols

  - Option 2b: 2 SSB symbols

- FFS: For 960kHz SCS: when deriveSSB-IndexFromCell is disabled

  - Option 3a: 6 SSB symbols

  - Option 3b: 4 SSB symbols

- FFS: Revise PDSCH symbols to 2 for 480 kHz and to 3 for 960 kHz SCS

In the last meeting, the deriveSSB-IndexFromCell for 480kHz and 960kHz SCS are discussed in [1].

Given the cell phase synchronization accuracy of 3 us is unchanged, the timing misalignment among cells could be up to 3 us, which is almost 3 symbols for SCS of 960 kHz. Thus, the deriveSSB-IndexFromCell tolerance, specified in clause 7.7 can be relaxed to be 3 SSB symbols for SCS of 960 kHz.

Proposal 1: For 960kHz SCS, when deriveSSB-IndexFromCell is enabled, relax deriveSSB-IndexFromCell tolerance to be 3 SSB symbols for SSB SCS (Option 2a).

For the SCS of 480 kHz, with the unchanged tolerance of 2 symbols (Option 1b), it already supports the propagation delay difference of 350 m in the worst case, while with 3 symbols (Option 1a) it supports the propagation delay difference of around 1 km. Not very sure in FR2-2 the propagation delay difference of around 1 km needs to be considered and update the requirement. Thus we would like to keep the current tolerance value.

Proposal 2: For 480kHz SCS, when deriveSSB-IndexFromCell is enabled, reuse the existing deriveSSB-IndexFromCell tolerance as 2 SSB symbols (Option 1b).

- 9.2A.5.3.1	Scheduling availability of UE performing measurements in TDD bands

When UE performs intra-frequency measurements in a TDD band, the following restrictions apply due to SS-RSRP or SS-SINR measurement

-	The UE is not expected to transmit PUCCH/PUSCH/SRS on SSB symbols configured to be measured, and on 1 data symbol before each consecutive SSB symbols configured to be measured and 1 data symbol after each consecutive SSB symbols configured to be measured within SMTC window duration if deriveSSB_IndexFromCell is enabled. If the high layer in TS 38.331[2]  signaling of smtc2 is configured, the SMTC periodicity follows smtc2; Otherwise SMTC periodicity follows smtc1.

-	The UE is not expected to transmit PUCCH/PUSCH/SRS on all symbols within SMTC window duration if deriveSSB_IndexFromCell is not enabled. If the high layer in TS 38.331 [2] signaling of smtc2 is configured, the SMTC periodicity follows smtc2; Otherwise SMTC periodicity follows smtc1.

In the last meeting, we agreed for 480 kHz SSB SCS deriveSSB-IndexFromCell is always enabled by the network and unlicensed band is FFS. It can be observed in the current FR1 with CCA, e.g. as specified in 9.2A5.3.1 for NR-U, the deriveSSB_IndexFromCell is not always enabled in TDD band. The corresponding clause is provided below for reference.

However, as the FR2-2 deployement could be well-planned and we could reuse the agreeement in the last meeting for 480 kHz in the licensed band.

Proposal 3: On the unlicensed band, for 480 kHz SSB SCS, deriveSSB-IndexFromCell is always enabled by the network.

Scheduling restriction

Scheduling restrictions

- Introduce scheduling restrictions for one symbol before and one symbol after the measurement resources (SSB, CSI-RS etc.) during L1 measurements for 480/960kHz SCS

- FFS: Introduce a total of K (due to synchronization error) + L (due to beam switching time) + M (due to propagation delay difference, TA adjustment etc) symbols scheduling restriction before and after SSB transmission –

  - For 480 kHz SCS,

    - Option 1: K=2

    - Option 2: K=3

  - For 960 kHz SCS,

    - K=3

  - L = 1 for 480/960kHz SCS

  - FFS: M for 480/960kHz SCS

    - Consider propagation delay difference of 3.33us considering FR2-2 coverage of 1km

In the last meeting, the scheduling restriction for 480kHz and 960kHz SCS are discussed in [1].

The open issue is the scheduling restrictions during L3 measurements for 480/960kHz SCS. As the previous topic, for 480 kHz SCS, K=2 is sufficient to cover synchronization error. And L=1 for due to beam switching time is reasonable. Regarding the “ M (due to propagation delay difference, TA adjustment etc)”, although the FR2-2 coverage of 1km is considered, the difference of propagation delay may still be within 1 us and can be covered by K=2. Besides, it is beneficial to minimize the scheduling restrictions. Thus, we are not sure about the M is necessary for 480 kHz.

Proposal 4: Regarding the scheduling restrictions during L3 measurements, for 480kHz SCS, support K=2 (Option 1), L=1 and M=0. It gives 3 data symbols restrictions before and after the SSBs.

For 960 kHz SCS, K=3 and L=1 is also needed, while it still support more than 200 ms propagation delay difference with the beam switching time of 200ns. Thus M=0 would be sufficient, if it is not targeting to support 1 km propagation delay difference.

Proposal 5: Regarding the scheduling restrictions during L3 measurements, for 480kHz SCS, support K=3, L=1 and M=0. It gives 4 data symbols restrictions before and after the SSBs.

PBCH detection for SSB index acquisition

To our understanding, enabling the deriveSSB-IndexFromCell can help UE to derive the SBI of the neighbouring cells from the serving cells and thus UE can skip SSB index detection on PBCH. If the deriveSSB-IndexFromCell is not enabled, the delay budget for PBCH detection for SSB index acquisition should be provided.

Proposal 6: If deriveSSB-IndexFromCell is not enabled, additional time for PBCH detection for SSB index acquisition is allowed.

Conclusion

In this contribution, we discuss the RRM measurement requirement in FR2-2. We have the following observations and proposals.

Proposal 1: For 960kHz SCS, when deriveSSB-IndexFromCell is enabled, relax deriveSSB-IndexFromCell tolerance to be 3 SSB symbols for SSB SCS (Option 2a).

Proposal 2: For 480kHz SCS, when deriveSSB-IndexFromCell is enabled, reuse the existing deriveSSB-IndexFromCell tolerance as 2 SSB symbols (Option 1b).

Proposal 3: On the unlicensed band, for 480 kHz SSB SCS, deriveSSB-IndexFromCell is always enabled by the network.

Proposal 4: Regarding the scheduling restrictions during L3 measurements, for 480kHz SCS, support K=2 (Option 1), L=1 and M=0. It gives 3 data symbols restrictions before and after the SSBs.

Proposal 5: Regarding the scheduling restrictions during L3 measurements, for 480kHz SCS, support K=3, L=1 and M=0. It gives 4 data symbols restrictions before and after the SSBs.

Proposal 6: If deriveSSB-IndexFromCell is not enabled, additional time for PBCH detection for SSB index acquisition is allowed.

- References

- R4-2202657, WF on R17 NR_ext_to_71GHz_RRM_1, Qualcomm.

- R4-2202659, WF on NR extension to 71 GHz – RRM - Part 2, Intel Corporation.