3GPP TSG-RAN WG4 Meeting # 102-e															R4-2204277

Electronic Meeting, February 21 – March 3, 2022

Agenda Item:	10.11.2.1

Source:	OPPO

Title:	On Pre-configured MG for NR_MG_enh

Document for:	Approval

- Introduction

A WF on pre-configured MG pattern was approved in last RAN4#101-bis e-meeting [1]. An LS reply was also sent from RAN2 on Pre-configured MG [2]. In this contribution, we provide our views on the remaining issues for pre-configured MG pattern.

- Discussion

The exact configuration of Pre-MG used for PRS measurement

- UE shall inform network about the PRS measurement by LocationMeasurementIndication IE

- FFS on when this indication shall be informed to NW

  - Option 1. when UE is going to perform PRS measurement upon the measurement gap is needed [TS38.331]

  - Option 2. when UE is going to perform PRS with the configured Pre-MG only if UE has not informed NW before pre-MG configuration.

For legacy gap, there is no need for UE to send any indication to network for gap-needed PRS measurement. In our view, it is a specific case of Pre-MG that UE shall inform network about the PRS measurement by LocationMeasurementIndication IE. So, we think option 2 is more acceptable. This indication shall be informed to NW dedicated for Pre-MG if UE has not informed NW before pre-MG configuration.

Proposal 1: This indication shall be informed to NW when UE is going to perform PRS with the configured Pre-MG only if UE has not informed NW before pre-MG configuration.

Trigger events for pre-MG activation/deactivation - For UE to autonomous pre-MG activation/deactivation the following trigger events may change the pre-MG activation status - BWP switching by DCI/Timer based - activation/de-activation of SCell(s) - FFS how to handle other cases in terms of UE and NW behavior - addition/removal of any measurement object(s) - addition/release/change of a SCell under CA - BWP switching by RRC - LPP positioning request

Though we agree that all conditions involving RRC (re)configuration could have impact on the activation status of Pre-MG. As enhancement for pre-configured MG, pre-configured MG triggered by other than BWP switch can be postponed to the later release.

Proposal 2: Pre-configured MG triggered by other than BWP switch and activation/de-activation of SCell(s) can be postponed to the later release.

How to determine pre-MG (de)activation status when multiple trigger events happened FFS on Option 1: pre-MG can be (de)activated by BWP switching and SCell activation independently

RAN4 has also agreed activation/de-activation of SCell(s) as trigger events to change the pre-MG activation status. Pre-MG can be (de)activated by BWP switching and SCell activation independently. If DCI or timer-based BWP switch could collide with SCell activation, UE shall follow the rules by combining the pre-MG status indication for all CCs for all configured measurement objects.

Proposal 3: Pre-MG can be (de)activated by BWP switching and SCell activation independently.

The principle for the signaling-based Pre-MG (de)activation under CA can be based on:

-	NW will forward the per-BWP pre-MG status indications to UE by RRC message, which is defined for each of induvial CCs (e.g. Table S1, S2 for the CC1 and CC2 respectively)

-	UE will combined the pre-MG status indication from NW for each induvial CC for all configured measurement objects.

- FFS on the rules on how to combine the individual per-BWP pre-MG status . - Option 1a - When configured with per-UE gap, - assume the per-UE gap is ON as long as the pre-MG status of active DL BWP in one of the CCs is ON, - and assume the per-UE gap is OFF only if the pre-MG status of active DL BWP in all CCs are OFF. - When configured with per-FR gap, - assume the gap is ON as long as the pre-MG status of active DL BWP in one of the CCs in the same FR is ON, - and assume the gap is OFF only if the pre-MG status of active DL active DL BWP in all CCs in the same FR are OFF - Option 1b - For per-UE pre-MG, - UE assumes the pre-MG is ON as long as the pre-MG status for active DL BWP in one of the activated CCs or for one of the deactivated SCCs is ON, - and assume the pre-MG is OFF only if the pre-MG status for active DL BWP in all activated CCs and for all deactivated SCCs are OFF. - For per-FR pre-MG, - UE assumes the pre-MG is ON as long as the pre-MG status for active DL BWP in one of the activated CCs or for one of the deactivated SCCs in the same FR is ON, - and assume the pre-MG is OFF only if the pre-MG status for active DL BWP in all activated CCs and for all deactivated SCCs in the same FR are OFF

For the single carrier case the activation/deactivation status of Pre-MG should follow the active DL BWP in this CC. For CA case, the pre-MG could be related to one or more CCs, but pre-MG’s status could be different per BWP. The gap status should consider all the Pre-MG status of active DL BWPs in all CCs.

Proposal 4: The rules on how to combine the individual per-BWP pre-MG status can be

  - When configured with per-UE gap, UE should assume the gap is ON as long as the pre-MG status of active DL BWP in one of the CCs is ON, and assume the gap is OFF only if the pre-MG status of active DL BWP in all CCs are OFF.

  - When configured with per-FR gap, UE should assume the gap is ON as long as the pre-MG status of active DL BWP in one of the CCs in the same FR is ON, and assume the gap is OFF only if the pre-MG status of active DL active DL BWP in all CCs in the same FR are OFF.

Activation/deactivation delay under CA: FFS on: - The Pre-MG activation/deactivation delay for CA with single CC based BWP switching shall be the same as for non-CA with single CC.

We can confirm that the Pre-MG activation/deactivation delay for CA with single CC based BWP switching shall be the same as for non-CA with single CC.

Proposal 5: The Pre-MG activation/deactivation delay for CA with single CC based BWP switching shall be the same as for non-CA with single CC.

As agreed, pre-configured MG(s) and legacy MG(s) can be configured simultaneously needs to be considered for concurrent gaps operation at 2nd stage. Whether multiple Pre-MGs can be configured also needs to be discussed then.

We suggest to only focus on single pre-MG and BWP switching on single CC in Rel17. If per FR1 gap and per FR2 gap are configured simultaneously, Pre-MG per FR could also work separately.

Proposal 6: RAN4 only consider single pre-MG and BWP switching on single CC in Rel17, leaving joint operation with concurrent gaps to R18.

- Summary

In this contribution, we provided our views on Pre-MG.

Proposal 1: This indication shall be informed to NW when UE is going to perform PRS with the configured Pre-MG only if UE has not informed NW before pre-MG configuration.

Proposal 2: Pre-configured MG triggered by other than BWP switch and activation/de-activation of SCell(s) can be postponed to the later release.

Proposal 3: Pre-MG can be (de)activated by BWP switching and SCell activation independently.

Proposal 4: The rules on how to combine the individual per-BWP pre-MG status can be

  - When configured with per-UE gap, UE should assume the gap is ON as long as the pre-MG status of active DL BWP in one of the CCs is ON, and assume the gap is OFF only if the pre-MG status of active DL BWP in all CCs are OFF.

  - When configured with per-FR gap, UE should assume the gap is ON as long as the pre-MG status of active DL BWP in one of the CCs in the same FR is ON, and assume the gap is OFF only if the pre-MG status of active DL active DL BWP in all CCs in the same FR are OFF.

Proposal 5: The Pre-MG activation/deactivation delay for CA with single CC based BWP switching shall be the same as for non-CA with single CC.

Proposal 6: RAN4 only consider single pre-MG and BWP switching on single CC in Rel17, leaving joint operation with concurrent gaps to R18.

References

[1] R4-2202614, WF on R17 NR MG enhancements – Pre-configured MG, Intel Corporation

[2] R4-2202615,	 LS on R17 NR MG enhancements – Pre-configured MG, Intel

[3]	R4-2118364, On pre-configured MG pattern(s) for NR_MG_enh, OPPO