3GPP TSG-RAN WG4 Meeting # 102-e	R4-2204287

# Electronic Meeting, February 21 – March 3, 2022

Agenda item:	10.22.2.1.

Source: 	OPPO

Title: 	Discussion on efficient activation/de-activation mechanism for SCells

Document for:	Discussion and decision

Introduction

In last meeting, A WF [1] was approved for Rel-17 further Multi-RAT Dual-Connectivity enhancements.

In this document, we provide our views on efficient activation/de-activation mechanism for SCells.

Discussion

Issue 1-2-2: Tuncertainty_MAC for scenario #3 Scenario #3: SCell to be activated belongs to FR2, if there is no active serving cell on that FR2 band, and target SCell is known to UE. - Option 1(MTK, QC, Apple, OPPO): Assuming PDCCH TCI and PDSCH TCI (when applicable) shall be associated with the triggered temporary RS burst: - if semi-persistent CSI-RS is used for CSI reporting, Tactivation_time is 3ms + max (Ttemp_RS+ 2ms, Tuncertainty_SP) - if periodic CSI-RS is used for CSI reporting, Tactivation_time is max (Ttemp_RS + 5ms, Tuncertainty_RRC + TRRC_delay-THARQ) under the condition that - One of the candidate TCI states configured in TCI-StatesPDCCH-ToAddList has the same QCL source of the triggered A-TRS, - The QCL source of CSI-RS for CQI reporting is the same as the triggered A-TRS, - The TCI state for PDCCH/PDSCH that is the same as A-TRS is assumed during SCell activation until changed by network after SCell activation. - Option 2 (Huawei, Ericsson, Nokia): Not assuming PDCCH TCI and PDSCH TCI (when applicable) is associated with the triggered temporary RS burst. - Option 2a (Huawei): - if semi-persistent CSI-RS is used for CSI reporting, Tactivation_time is 3ms + max(Ttemp_RS+ 2ms, Tuncertainty_MAC, Tuncertainty_SP) - if periodic CSI-RS is used for CSI reporting, Tactivation_time is max(Ttemp_RS + 5ms, Tuncertainty_MAC+3ms, Tuncertainty_RRC + TRRC_delay-THARQ) - Option 2b (Ericsson): - if semi-persistent CSI-RS is used for CSI reporting, Tactivation_time is 3ms + max(TATRS + 2ms, Tuncertainty_MAC + 2ms, Tuncertainty_SP) - if periodic CSI-RS is used for CSI reporting, Tactivation_time is max(TATRS + 5ms, Tuncertainty_MAC + 5ms, Tuncertainty_RRC + TRRC_delay-THARQ) (Moderator note: The difference between option 2a and 2b are in yellow highlight)

RAN4 has agreed on Scenario#1: SCell to be activated is known and belongs to FR1, and Scenario#2: SCell to be activated is unknown and belongs to FR1, and SCell is contiguous to an active serving cell in the same band.

As agreed in RAN4, UE performs the time-frequency tracking based on only TRS or only SSBs, depending on the network configuration. RAN4 will specify requirements for the temporary RS based SCell activation requirements assuming UE uses TRS triggered in the same MAC CE as SCell activation command.

As the majority views in RAN1, TCI state for TRS is contained in the same MAC CE for Scell activation with TRS. If it was agreed, TCI states for TRS shall be configured with associated RS (e.g., SSB) for known scell which has been used much earlier than Scell activation command. The TRS with active TCI state can be used immediately after SCell activation command for known case. UE can perform time-frequency tracking once the first TRS is available (e.g., T_TRS), but does not need to wait for first complete available SSB corresponding to the TCI state. Therefore, for Scenario#3: SCell to be activated belongs to FR2,  if there is no active serving cell on that FR2 band, and target SCell is known to UE, we prefer option 1 to replace (T_uncertainty_MAC + T_FineTiming) with T_ATRS, assuming A-TRS burst triggered by the MAC-CE shall be associated with the SSB that might have been used and PDCCH TCI and PDSCH TCI (when applicable) shall be associated with the triggered A-TRS burst.

Observation 1: (T_uncertainty_MAC + T_FineTiming) can be replaced by with T_ATRS.

Proposal 1: Assuming PDCCH TCI and PDSCH TCI (when applicable) shall be associated with the triggered temporary RS burst:

          - if semi-persistent CSI-RS is used for CSI reporting, Tactivation_time is 3ms + max (Ttemp_RS+ 2ms, Tuncertainty_SP)

          - if periodic CSI-RS is used for CSI reporting, Tactivation_time is max (Ttemp_RS + 5ms, Tuncertainty_RRC + TRRC_delay-THARQ)

under the condition that

          - One of the candidate TCI states configured in TCI-StatesPDCCH-ToAddList has the same QCL source of the triggered A-TRS,

          - The QCL source of CSI-RS for CQI reporting is the same as the triggered A-TRS,

          - The TCI state for PDCCH/PDSCH that is the same as A-TRS is assumed during SCell activation until changed by network after SCell activation.

Conclusion

In this document, we provide our views on efficient activation/de-activation mechanism for SCells.

Observation 1: (T_uncertainty_MAC + T_FineTiming) can be replaced by with T_ATRS.

Proposal 1: Assuming PDCCH TCI and PDSCH TCI (when applicable) shall be associated with the triggered temporary RS burst:

          - if semi-persistent CSI-RS is used for CSI reporting, Tactivation_time is 3ms + max (Ttemp_RS+ 2ms, Tuncertainty_SP)

          - if periodic CSI-RS is used for CSI reporting, Tactivation_time is max (Ttemp_RS + 5ms, Tuncertainty_RRC + TRRC_delay-THARQ)

under the condition that

          - One of the candidate TCI states configured in TCI-StatesPDCCH-ToAddList has the same QCL source of the triggered A-TRS,

          - The QCL source of CSI-RS for CQI reporting is the same as the triggered A-TRS,

          - The TCI state for PDCCH/PDSCH that is the same as A-TRS is assumed during SCell activation until changed by network after SCell activation.

- References

[1] R4-2202688, WF on R17 further Multi-RAT Dual-Connectivity enhancements, Huawei

[2] R4-2201149, Discussion on efficient activation/de-activation mechanism for SCells, OPPO