3GPP TSG-RAN WG4 Meeting # 102-e                                      R4-2204407

Electronic Meeting, February 21 – March 3, 2022

Agenda Item:	5.1.3.2.

Source:	Intel Corporation

Title:	Discussion on Rel16 NR positioning measurement accuracy requirements

Document for:		Discussion

# Introduction

In the previous RAN4 meeting some open issues for the performance requirements were left in [1]. In this contribution we will focus on the discussion of remaining issue below.

- RF margin

- Applicability of UE Rx-Tx time difference accuracy requirements in case of NTA_offset change

# RSTD accuracy requirements

- RF margin

Group delay calibration margin for RSTD - in FR1 when the reference and target PRS resources are in the same PFL (single PFL case) PRS BW (MHz); Margin (Tc). ≥ 5; [TBD]. ≥ 10; [TBD]. ≥ 20; [TBD]. ≥ 50; [TBD]. ≥ 100; [TBD]. - in FR2 when the reference and target PRS resources are in the same PFL (single PFL case). FFS if additional margin should be added for OTA calibration. PRS BW (MHz); Margin (Tc). ≥ 20; [TBD]. ≥ 50; [TBD]. ≥ 100; [TBD]. ≥ 200; [TBD].

- Frequency drift margin

Frequency drift margin for RSTD

- RAN4 will define a single value for the frequency drift margin based on a single value of the maximum time offset

- FFS on the exact value

In principle, as in for RF working assumptions, the allowed frequency drift is +/-0.1 ppm/s[3,TS38.101-1]. On the other hand, typically the time offset between of the measured TRPs( e.g. reference cell and neighbor cell for RSTD measurements) can be within [- expectedRSTD-Uncertainty3Ts, expectedRSTD-Uncertainty3Ts]. Thus, we can see

Proposal 1 : The maximum frequency drift between the measured TRPs can be dependent the maximum time offsets among the measured TRPs/cells (e.g. 2* expectedRSTD)

# UE Rx-Tx time difference accuracy requirements

- Applicability of accuracy requirements in case TA adjustment

In previous RAN4 meeting, the impacts of TA adjustment on the UE Rx-Tx time difference requirements was addressed.

Applicability of UE Rx-Tx time difference accuracy requirements under TA adjustment FFS on Applicability of UE Rx-Tx time difference accuracy requirements under TA adjustment - Option 1. - UE Rx-Tx measurement accuracy requirements shall apply if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment - Option 2. - UE Rx-Tx measurement accuracy requirements shall apply for a cell, which is also the downlink reference cell (defined in section 7.1.1) for SRS transmission even if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment. - the UE Rx-Tx measurement accuracy requirements shall not apply for a cell, which is not the downlink reference cell (defined in section 7.1.1) for SRS transmission, if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment.

In the last meeting, the option 4 was proposed as the compromised approach for further checking. In our views, in case of autonomous TA adjustment for applying UE Rx-Tx accuracy for a cell based on whether SRS is transmitted on that cell is sufficient and unambiguous.

Proposal 2:  Applicability of accuracy requirements under TA adjustment if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment  can be:

  - UE Rx-Tx measurement accuracy requirements shall apply for a serving cell even if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment.

  - the UE Rx-Tx measurement accuracy requirements shall not apply for a neighbor cell if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment

# Conclusion

In this contribution, some considerations on NR positioning measurement accuracy requirements are provided and the following observations and proposals can be drawn:

Proposal 1 : The maximum frequency drift between the measured TRPs can be dependent the maximum time offsets among the measured TRPs/cells (e.g. 2* expectedRSTD)

Proposal 2:  Applicability of accuracy requirements under TA adjustment if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment  can be:

  - UE Rx-Tx measurement accuracy requirements shall apply for a serving cell even if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment.

  - the UE Rx-Tx measurement accuracy requirements shall not apply for a neighbor cell if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment

# References

- R4-2115307, “WF on requirements on NR Pos performance requirements”

- 3GPP TS 38.133 v16.8.0: "NR: Requirements for support of radio resource management "

- 3GPP TS 38.101-4 v16.8.0