3GPP TSG-RAN WG4 Meeting # 102-e      	R4-2204528

Electronic Meeting, February 21 – March 3, 2022

Agenda Item:	10.12.2.3.3

Source: 	CMCC

Title: 	Discussion on the test setup for CRS-IM 15kHz

Document for:	Discussion

- Introduction

In RAN4#101bis-e meeting, the test setup was discussed and a way forward was agreed in [1]. In this contribution, we further provide our views about remaining issues for test setup.

- Discussion

  - General issues

PDSCH loading level

- Option 1: Add one of the combinations below that achieves about 1dB performance gain over the reference scheme

  - Loading 30 %, INR 1 = 9.69 dB, INR 2 = 3.7 dB

  - Loading 40 %, INR 1 = 8.79 dB, INR 2 = 2.7 dB

  - Loading 50 %, INR 1 = 8.36 dB, INR 2 = 1.7 dB

- Option 2: Only consider 20% PDSCH loading level

- Option 3: Consider 30% interference PDSCH loading for Scenario 1 and 20% interference PDSCH loading for Scenario 2

- Option 4: Also include 80% loading

- Option 5: Further check 30% loading level

- Option 6: Either 30% or 20%

We don’t support high PDSCH loading level considering of reasonable CRS-IM performance gain. In our view, 20% should be starting point. We are open to check 30% loading level for Scenario 1.

Proposal 1: Take 20% PDSCH loading level as baseline. Further check 30% loading level for Scenario 1.

Tx antenna and LTE CRS port number

- Option 1: Only cover 4 CRS ports

- Option 2: Cover 2 and 4 for CRS ports

  - Option 2A: Consider 2 CRS ports for Scenario 1 and 4 CRS ports for Scenario 2

  - Option 2B: Use different CRS port number in the tests with different INR levels

- Option 3: Only cover 2 CRS ports (Nokia)

In last meeting, we use CRS port aligned with serving cell for Scenario 1, and 4 CRS ports as default assumption for Scenario 2. Therefore, we support to cover 2 and 4 for CRS ports for Scenario 1, and use 4 CRS ports as test setup for Scenario 2.

Proposal 2:

- For LTE CRS port in Scenario 1, over 2 and 4 for CRS ports

- For LTE CRS port in Scenario 2, only cover 4 CRS ports.

- Conclusion

In this contribution, we discuss some test setup issues for LTE CRS-IM in scenarios with overlapping spectrum for LTE and NR. The proposals are:

Proposal 1: Take 20% PDSCH loading level as baseline. Further check 30% loading level for Scenario 1.

Proposal 2:

- For LTE CRS port in Scenario 1, over 2 and 4 for CRS ports

- For LTE CRS port in Scenario 2, only cover 4 CRS ports.

- Reference

[1] R4-2203131, WF on general part and 15 kHz NR SCS scenario for CRS-IM receiver, China Telecom.

[2] R4-2120705, WF on CRS-IM receiver in scenarios with overlapping spectrum for LTE and NR, China Telecom.

[3] RP-213656, Revised WID: Further enhancement on NR demodulation performance, China Telecom.