3GPP TSG-RAN WG4 Meeting # 102-e										R4-2204556

Electronic Meeting, February 21 – March 3, 2022

Agenda item:	10.5.1.3

Source: 	CMCC

Title: 	Discussion on switching related requirements

Document for:	Discussion

1. Introduction

In RAN4 #101 bis e-meeting, most TDD switching related requirements have been finished and the only remaining issue is whether the group delay needs to be declared or not.

In this contribution, we show our views on group delay requirements.

2. Discussion

If repeater is deployed where the target receiver could receive both the signal directly from target transmitter or through repeater, group delay should be less than CP to avoid ISI interference. However, current state of technology could not support group delay less than CP and above scenario should be avoided by planning deployment.

Group delay could be regarded as one part of propagation delay. If the group delay of repeater is too long, the total propagation time could be enlarged and maybe larger than current specified GP in current spec, leading to additional DL-UL interference. Following analysis shows if group delay is relatively large, cell coverage should be shrunk if we still want to avoid UL-DL interference.

The Total coverage range from donor gNB to UE through repeater=(TGUARD- TAoffset – group delay) * c (light speed), among which TGUARD is the total allocated GP and is regarded as 1 symbol in following analysis and TAoffset= NTAoffset * Tc.

Table 1: total coverage range from donor gNB to UE to avoid U-D interference

|  | Group delay |  |  |  |
| --- | --- | --- | --- | --- |
| Frequency range | 0us | 5us | 10us | 20us |
| FR1 assuming 30kHz SCS | 6810m | 5310m | 3810m | 810m |
| FR2 assuming 120kHz SCS | 3375m | 1875m | 375m | No coverage |

Based on above analysis, we should regulate group delay requirements to avoid U->D interference.

Observation 1: we should regulate group delay requirements to avoid U->D interference.

According to our survey, current art of technology could achieve 5-10us group delay. And according to above analysis in table 1, such requirement could avoid extra UL-DL interference. So 5~10us group delay limit is reasonable for normal repeater. But as approved in last meeting, the spec also considers the repeater that has long group delay. Therefore, our preference for group delay is based on declaration. Repeater vendors could declare it based on practical deployment scenario.

Proposal 1: repeater vendors should declare its group delay based on practical deployment.

3. Conclusions

In this contribution, we discuss TDD switching related RF requirements with following observations and proposals:

Observation 1: we should regulate group delay requirements to avoid U->D interference.

Proposal 1: repeater vendors should declare its group delay based on practical deployment.