3GPP TSG-RAN WG4 Meeting # 102-e										R4-2204566

Electronic Meeting, February 21 – March 3, 2022

Agenda item:	9.3.3

Source: 	CMCC

Title: 	Discussion on UE requirements for 6GHz licensed spectrum

Document for:	Discussion

1. Introduction

In last RAN4 #101 bis e-meeting, the remaining issues on UE requirements include MPR, ACLR, SEM, REFSENSE and ACS.

In this contribution, we focus on the remaining issues for UE.

2. Discussion

For Tx requirements, the remaining issues are both related to co-existence scenario, i.e. ACLR. The main controversy exists in whether to reuse the same requirements in TS 38.101-1 or the requirements in TR 38.921.

For the simulations performed for ACLR in TS 38.101-1, it assumes 2GHz reference frequency. Although the frequency of new 5G band is becoming higher, but the same ACLR is reused considering the frequency difference is not that much. For 6GHz licensed spectrum, new simulation is performed and final simulation shows RF requirements could be relax compared with legacy FR1 requirements. from our understanding, we may need to respect the co-existence work and reuse the same value as defined in TR 38.921 which is already approved for 6GHz license spectrum.

Proposal 1: it’s better to reuse the same value as in TR 38.921 for ACLR and SEM requirements.

For Rx requirements, the main issue is ACS and REFSENSE.

For ACS, as stated above, we suggest to respect the results approved in TR 38.921.

Proposal 2: it’s better to reuse the same value as in TR 38.921 for ACS requirements.

For REFSENSE, it is related to NF which is stated as 9dB for baseline assumption in TR 38.921. Final REFSENSE is calculated as:

-174 + 10*log(106*15K*12)+9(NF)-1(SNR)=-93.2dBm/20MHz, which is equal to n78 REFSENSE if we assume 0.5dB IM. Therefore, we suggest to reuse the same REFSENSE as n78 for 6GHz license spectrum.

Proposal 3: it’s better to reuse the same REFSENSE as n78 and n79 for 6GHz license spectrum.

3. Conclusions

In this contribution, our understanding of remaining UE RF requirement of 6GHz is listed with following observations and proposals:

Proposal 1: it’s better to reuse the same value as in TR 38.921 for ACLR and SEM requirements.

Proposal 2: it’s better to reuse the same value as in TR 38.921 for ACS requirements.

Proposal 3: it’s better to reuse the same REFSENSE as n78 and n79 for 6GHz license spectrum.

4. Reference

[1] R4-2202267, WF on UE RF requirements for 6GHz licensed band, Ericsson.