3GPP TSG-RAN WG4 Meeting # 102-e										R4-2204567

Electronic Meeting, February 21 – March 3, 2022

Agenda item:	9.3.4

Source: 	CMCC

Title: 	Discussion on BS requirements for 6GHz licensed spectrum

Document for:	Discussion

1. Introduction

In last RAN4 #101 bis e-meeting, the remaining issues on BS requirements include fOBUE and fOOB.

In this contribution, we focus on the remaining issues for BS.

2. Discussion

The legacy fOBUE requirements for FR1 gNB are listed as below for information.

Table 6.6.1-1: Maximum offset of OBUE outside the downlink operating band

| BS type | Operating band characteristics | ΔfOBUE (MHz) |
| --- | --- | --- |
| BS type 1-H | FDL,high – FDL,low < 100 MHz | 10 |
|  | 100 MHz FDL,high – FDL,low 900 MHz | 40 |
| BS type 1-C | FDL,high – FDL,low 200 MHz | 10 |
|  | 200 MHz < FDL,high – FDL,low 900 MHz | 40 |

Table 9.7.1-1: Maximum offset ΔfOBUE outside the downlink operating band

| BS type | Operating band characteristics | ΔfOBUE (MHz) |
| --- | --- | --- |
| BS type 1-O | FDL,high – FDL,low < 100 MHz | 10 |
|  | 100 MHz ≤ FDL,high – FDL,low ≤ 900 MHz | 40 |

The legacy fOOB requirements for FR1 gNB are listed as below for information.

Table 7.4.2.2-0: ΔfOOB offset for NR operating bands

| BS type | Operating band characteristics | ΔfOOB (MHz) |
| --- | --- | --- |
|  | FUL,high – FUL,low ≤ 200 MHz | 20 |
| BS type 1-C | 200 MHz < FUL,high – FUL,low ≤ 900 MHz | 60 |
|  | FUL,high – FUL,low < 100 MHz | 20 |
| BS type 1-H | 100 MHz ≤ FUL,high – FUL,low ≤ 900 MHz | 60 |

Table 10.5.2.2-0: ΔfOOB offset for NR operating bands in FR1

| BS type | Operating band characteristics | ΔfOOB (MHz) |
| --- | --- | --- |
| BS type 1-O | FUL,high – FUL,low < 100 MHz | 20 |
|  | 100 MHz ≤ FUL,high – FUL,low ≤ 900 MHz | 60 |

Observation 1: the fOBUE and fOOB requirements are the same for 1-H and 1-O but relax compared with 1-C.

Above same principle is suggested to be applicable for 6GHz, so it’s suggested to define the same fOBUE/fOOB requirements for 1-H and 1-O 6GHz.

As studied in TR 38.921, more relax requirements are suggested considering the higher frequency and larger operation band frequency range. So it’s suggested to respect the requirement in TR 38.921 and reuse 100MHz for 1-H and 1-O as the breakpoint. But for 1-C, the same as current FR1 requirements.

Proposal 1: fOBUE is 100MHz for 1-H and 1-O 6GHz license spectrum while 40MHz for 1-C type.

Proposal 2: fOOB is 100MHz for 1-H and 1-O 6GHz license spectrum while 60MHz for 1-C type.

3. Conclusions

In this contribution, our understanding of remaining BS RF requirement of 6GHz is listed with following observations and proposals:

Observation 1: the fOBUE and fOOB requirements are the same for 1-H and 1-O while relax compared with 1-C.

Proposal 1: fOBUE is 100MHz for 1-H and 1-O 6GHz license spectrum while 40MHz for 1-C type.

Proposal 2: fOOB is 100MHz for 1-H and 1-O 6GHz license spectrum while 60MHz for 1-C type.

4. Reference

[1] R4-2202268, WF on BS RF requirements for 6GHz licensed band.