3GPP TSG-RAN WG4 Meeting # 102-e	R4-2204733

# Electronic Meeting, February 21-March 3, 2022

Source:	LG Electronics

Title:	A-MPR analysis results for NR-U(VLP) considering regulatory parameters in Korea.

Agenda item:	9.2.3

Document for:	Approval

Introduction

In RAN#92-e meeting, New WI [1] on “Introduction of operation in full unlicensed band 5925-7125MHz” was approved.

In this contribution, based on the WF[4] of RAN4#101-e, we provide the simulation results of A-MPR  for NR-U(VLP) considering regulatory parameters in Korea.

Discussion

We consider the following simulation assumptions for A-MPR and test scenarios in Table 1.

- PA calibration : 1dB MPR for PC5 and DFT-s-OFDM QPSK 100RB3 20MHz waveform - Determine back-off required to meet Korea regulatory parameters - Out-of-band emissions [3] : -34 dBm/MHz (f ≤ 5925MHz, f ≥ 6445MHz) - Maximum PSD for in-band emissions [3] : +1 dBm/MHz(5925 – 6425MHz) - Maximum mean EIRP for in-band emissions[3] : +14dBm (5925 – 6425MHz) - Waveform : CP-OFDM , DFT-s-OFDM - A-MPR approach for VLP mode - AMPR of VLP mode is calculated relative to +20 dBm(PC5). - Modulation order - QPSK/16QAM/64QAM/256QAM - Channel Bandwidth - 20/40/60/80MHz - Uplink configuration - Full allocation, Interlaced Allocation - Test Channel : edge channel & non-edge channel (total 8EA channel(#1~#8) as seen in Figure 1)

Table 1.  Simulation set-up for A-MPR of VLP mode in South Korea

| Test scenario | DFT/CP | Modulation | Allocation |
| --- | --- | --- | --- |
| 1 | DFT-S-OFDM | QPSK | Full |
| 2 | CP-OFDM | QPSK | Full |
| 3 | DFT-S-OFDM | QPSK | Interlace_0 |
| 4 | CP-OFDM | QPSK | Interlace_0 |
| 5 | DFT-S-OFDM | 16QAM | Full |
| 6 | CP-OFDM | 16QAM | Full |
| 7 | DFT-S-OFDM | 16QAM | Interlace_0 |
| 8 | CP-OFDM | 16QAM | Interlace_0 |
| 9 | DFT-S-OFDM | 64QAM | Full |
| 10 | CP-OFDM | 64QAM | Full |
| 11 | DFT-S-OFDM | 64QAM | Interlace_0 |
| 12 | CP-OFDM | 64QAM | Interlace_0 |
| 13 | DFT-S-OFDM | 256QAM | Full |
| 14 | CP-OFDM | 256QAM | Full |
| 15 | DFT-S-OFDM | 256QAM | Interlace_0 |
| 16 | CP-OFDM | 256QAM | Interlace_0 |

Figure 1. Possible Channel locations for operation in 5925MHz~6425MHz

Figure 2~Figure 6 show the simulation results of Tx Power backoff for PC5 UE to meet +1dBm/MHz PSD limit & 
-34dBm/MHz OOBE limit in Korea considering edge channel(#1) and non-edge channels(#2~#8);

- Figure 2 : all CBW of 20MHz, 40MHz, 60MHz and 80MHz

- Figure 3 : CBW of 20MHz

- Figure 4 : CBW of 40MHz

- Figure 5 : CBW of 60MHz

- Figure 6 : CBW of 80MHz

Figure 2. All CBW of 20MHz, 40MHz, 60MHz and 80MHz

Figure 3.  CBW of 20MHz (edge CH & non-edge CH)

Figure 4.  CBW of 40MHz (edge CH & non-edge CH)

Figure 5.  CBW of 60MHz (edge CH & non-edge CH)

Figure 6.  CBW of 80MHz (edge CH & non-edge CH)

From Figure 1, we can know there is a guardband(GB) of 20 MHz between both edge channels and out-of-band emissions frequency range. Due to the GB, a big difference between edge channel and non-edge channel is not observed from Figure 2 to Figure 6.

For simulation results, 6dB Tx power backoff for PC5 UE should be considered to meet VLP UE’s maximum output power of 14dBm in South Korea.

- A-MPR for PC5 VLP in South Korea = max(6dB, simulation results of A-MPR based on PC5)

And, the A-MPR should be equal to or larger than MPR in Table 6.2F.2-1.

Observation 1: For all edge channel of 20MHz, 40MHz, 60MHz and 80MHz, there is GuardBand of 20MHz between both edge channels and Out-of-band emissions frequency range.

Observation 2: Considering VLP UE’s maximum output power of 14dBm in South Korea,

- A-MPR for PC5 VLP in South Korea = Max(6dB, A-MPR based on PC5 UE to meet  +1dBm/MHz PSD limit & -34dBm/MHz OOBE limit).

Observation 3: Regarding Table 6.2F.2-1, 7 dB can apply to A-MPR for 256QAM in CP-OFDM when CBW is 40MHz, 60MHz and 80MHz.

Observation 4: There is no big difference in A-MPR between edge CH and non-edge CH because of GB.

Based on the simulation results and the observations, we propose A-MPR for PC5 VLP in South Korea as follows.

Proposal 1: Specify A-MPR without considering edge CH and non-edge CH.

Proposal 2: Define PC5 A-MPR in Table 2 for VLP in South Korea.

Table 2. Proposed A-MPR for PC5 VLP in South Korea

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz |  | 40 MHz |  | 60 MHz |  | 80 MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| DFT-s-ODFM | QPSK | ≤ 7.5 | ≤ 10.5 | ≤ 6.0 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 16 QAM | ≤ 7.5 | ≤ 10.5 | ≤ 6.0 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 64 QAM | ≤ 7.5 | ≤ 10.5 | ≤ 6.0 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 256 QAM | ≤ 7.5 | ≤ 10.5 | ≤ 6.0 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
| CP-OFDM | QPSK | ≤ 7.5 | ≤ 10.5 | ≤ 6.0 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 16 QAM | ≤ 7.5 | ≤ 10.5 | ≤ 6.0 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 64 QAM | ≤ 7.5 | ≤ 10.5 | ≤ 6.0 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 256 QAM | ≤ 7.5 | ≤ 10.5 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies. |  |  |  |  |  |  |  |  |  |

Conclusion

In this contribution, we provide A-MPR analysis results for NR-U(VLP) considering regulatory parameters in Korea.

Observation 1: For all edge channel of 20MHz, 40MHz, 60MHz and 80MHz, there is GuardBand of 20MHz between both edge channels and Out-of-band emissions frequency range.

Observation 2: Considering VLP UE’s maximum output power of 14dBm in South Korea,

- A-MPR for PC5 VLP in South Korea = Max(6dB, A-MPR based on PC5 UE to meet  +1dBm/MHz PSD limit & -34dBm/MHz OOBE limit)

Observation 3: Regarding Table 6.2F.2-1, 7 dB can apply to A-MPR for 256QAM in CP-OFDM when CBW is 40MHz, 60MHz and 80MHz.

Observation 4: There is no big difference in A-MPR between edge CH and non-edge CH because of GB.

Based on the A-MPR simulation and observations, we propose as follow

Proposal 1: Specify  A-MPR without considering  edge CH and non-edge CH.

Proposal 2: Define PC5 A-MPR in Table 2 for VLP in South Korea.

Table 2. Proposed A-MPR for PC5 VLP in South Korea

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz |  | 40 MHz |  | 60 MHz |  | 80 MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| DFT-s-ODFM | QPSK | ≤ 7.5 | ≤ 10.5 | ≤ 6.0 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 16 QAM | ≤ 7.5 | ≤ 10.5 | ≤ 6.0 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 64 QAM | ≤ 7.5 | ≤ 10.5 | ≤ 6.0 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 256 QAM | ≤ 7.5 | ≤ 10.5 | ≤ 6.0 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
| CP-OFDM | QPSK | ≤ 7.5 | ≤ 10.5 | ≤ 6.0 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 16 QAM | ≤ 7.5 | ≤ 10.5 | ≤ 6.0 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 64 QAM | ≤ 7.5 | ≤ 10.5 | ≤ 6.0 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 256 QAM | ≤ 7.5 | ≤ 10.5 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies. |  |  |  |  |  |  |  |  |  |

TP for PC5 VLP A-MPR table in South Korea to TR 38.849 updating clause 6.1.1.2.2

******************************* Start of TP ***************************************

- 6	RF requirements

- 6.1	UE specific

- 6.1.1	Transmitter characteristics

This section details specific transmitter characteristics for a UE operating in the lower 6 GHz NR.

Table 6.1.1-1: Summary of NS values.

| Country | Mode |  |  |
| --- | --- | --- | --- |
|  | SP | LPI | VLP |
| Region 1 |  |  |  |
| EU/CEPT | N/A | [NS_xx] (new) | TBD |
| Region 2 |  |  |  |
| US | NS_54 | NS_53 | N/A |
| Canada | NS_54 | [NS_xx] (new) | TBD |
| Brazil | N/A | NS_53 | TBD |
| Peru | N/A | NS_53 | N/A |
| Chile | N/A | NS_53 | N/A |
| Costa Rica | N/A | NS_01 | TBD |
| Colombia | N/A | NS_53 | N/A |
| Region 3 |  |  |  |
| South Korea | N/A | [NS_xy] (new) | [NS_xz] (new) |

- < Unchanged sections are omitted >

#### 6.1.1.2	 A-MPR for a NS(s) for the full 6GHz NR unlicensed operation

##### 6.1.1.2.1	Canada

Table 6.1.1.2.1-1: A-MPR for PC5 LPI in Canada

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |
| --- | --- | --- | --- |
|  |  | 20 MHz |  |
|  |  | Full (dB) | Partial (dB) |
| DFT-s-ODFM | PI/2 BPSK | ≤ 3.0 | ≤ 5.5 |
|  | QPSK | ≤ 3.0 | ≤ 5.5 |
|  | 16 QAM | ≤ 3.0 | ≤ 5.5 |
|  | 64 QAM | ≤ 3.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 5.0 | ≤ 5.5 |
| CP-OFDM | QPSK | ≤ 3.5 | ≤ 5.5 |
|  | 16 QAM | ≤ 4.0 | ≤ 5.5 |
|  | 64 QAM | ≤ 5.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 7.0 | ≤ 7.0 |

##### 6.1.1.2.2	South Korea

Table 6.1.1.2.2-1: A-MPR for PC5 LPI in South Korea

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz |  | 40 MHz |  | 60 MHz |  | 80 MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| DFT-s-ODFM | QPSK | ≤ 6.0 | ≤ 8.5 | ≤ 4.0 | ≤ 5.5 | ≤ 3.5 | ≤ 4.5 | ≤ 3.0 | ≤ 4.5 |
|  | 16 QAM | ≤ 6.0 | ≤ 8.5 | ≤ 4.0 | ≤ 5.5 | ≤ 4.0 | ≤ 5.0 | ≤ 3.5 | ≤ 5.0 |
|  | 64 QAM | ≤ 6.0 | ≤ 8.5 | ≤ 4.0 | ≤ 5.5 | ≤ 4.0 | ≤ 5.0 | ≤ 3.5 | ≤ 5.0 |
|  | 256 QAM | ≤ 6.0 | ≤ 8.5 | ≤ 5.0 | ≤ 5.5 | ≤ 5.0 | ≤ 5.5 | ≤ 5.0 | ≤ 5.5 |
| CP-OFDM | QPSK | ≤ 6.0 | ≤ 8.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.0 | ≤ 5.5 | ≤ 4.5 | ≤ 5.5 |
|  | 16 QAM | ≤ 6.0 | ≤ 8.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.0 | ≤ 5.5 | ≤ 4.5 | ≤ 5.5 |
|  | 64 QAM | ≤ 6.0 | ≤ 8.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 6.0 | ≤ 8.5 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 |

Table 6.1.1.2.2-2: A-MPR for PC5 VLP in South Korea

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz |  | 40 MHz |  | 60 MHz |  | 80 MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| DFT-s-ODFM | QPSK | ≤ 7.5 | ≤ 10.5 | ≤ 6.0 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 16 QAM | ≤ 7.5 | ≤ 10.5 | ≤ 6.0 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 64 QAM | ≤ 7.5 | ≤ 10.5 | ≤ 6.0 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 256 QAM | ≤ 7.5 | ≤ 10.5 | ≤ 6.0 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
| CP-OFDM | QPSK | ≤ 7.5 | ≤ 10.5 | ≤ 6.0 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 16 QAM | ≤ 7.5 | ≤ 10.5 | ≤ 6.0 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 64 QAM | ≤ 7.5 | ≤ 10.5 | ≤ 6.0 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 256 QAM | ≤ 7.5 | ≤ 10.5 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies. |  |  |  |  |  |  |  |  |  |

- 6.1.1.2.3	Peru and Chile

Table 6.1.1.2.3-1: A-MPR for PC5 LPI in Peru and Chile

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz |  | 40 MHz |  | 60 MHz |  | 80 MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| DFT-s-ODFM | QPSK | ≤ 9.0 | ≤ 12.0 | ≤ 6.0 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 3.0 | ≤ 5.5 |
|  | 16 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.0 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 3.0 | ≤ 5.5 |
|  | 64 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.0 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 3.0 | ≤ 5.5 |
|  | 256 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.0 | ≤ 8.5 | ≤ 5.0 | ≤ 6.5 | ≤ 5.0 | ≤ 5.5 |
| CP-OFDM | QPSK | ≤ 9.0 | ≤ 12.0 | ≤ 6.0 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 4.0 | ≤ 5.5 |
|  | 16 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.0 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 4.0 | ≤ 5.5 |
|  | 64 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.0 | ≤ 8.5 | ≤ 5.5 | ≤ 6.5 | ≤ 5.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 7.0 | ≤ 8.5 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 |

The A-MPR results for PC5 LPI in Peru and Chile are mostly identical to the A-MPR results defined by NS_53 (LPI in US). In some cases, A-MPR results are 0.5-1dB relaxed comparing to NS_53. Thus, accounting for the marginal difference between these A-MPR values and NS_53, the latter can be re-used to support Peru and Chile.

******************************* End of TP ***************************************

- Reference

[1] RP-211602, “New WID for Introduction of operation in full unlicensed band 5925-7125MHz, Apple Inc”

[2] TR 37.890, “3GPP;Technical Specification Group Radio Access Network;Feasibility Study on 6 GHz for LTE and NR

in Licensed and Unlicensed Operations (Release 17)”

[3] Korea’s Ministry of Science and ICT, "Technical standards for radio equipment for radio stations"

- URL: https://www.law.go.kr/admRulLsInfoP.do?admRulSeq=2100000196974

- URL: https://www.law.go.kr/admRulLsInfoP.do?chrClsCd=010202&admRulSeq=2100000196973

[4] R4-2119863,”WF on introduction of the full unlicensed band, Apple Inc”

[5] R4-2201125, “Discussion on the impact of the New power class definitions for NR-U UE in other countries, LGE”