3GPP TSG-RAN WG4 Meeting # 102-e                                                        R4-2204765			R4-22xxxxx

# Electronic Meeting, February 21 – March 3, 2022

Source:	ZTE Corporation

Title:	On RedCap FR1 Operating band

Agenda Item:	10.20.2.1.1

Document for:	Approval

- 1	Introduction

In last meeting, some general issues for FR1 RedCap UE were discussed, and also some tentative agreements were achieved and included in the moderator’s summary paper [1] [2] during the meeting. However, the WF [2] was not approved in the end of the meeting, even no notes in the chairman note. Although the WF was noted in the end, some of the tentative agreements needs to be approved in official approved contributions.

Meanwhile, some other issues such as band n79 was also discussed.

- In this contribution, we first summarize some tentative agreements, then provide some discussions on band n79 from RAN4 aspect.

- 2	Discussion

2.1  Tentative agreements in last meeting

Some tentative agreements in [2] related to Power class and UL architecture in RedCap in FR1 are summarized below:1. 1 TX architecture of 23 dBm PA

2. 1 TX of 26 dBm PA in Rel-17 and 2 TX architecture is excluded in Rel-17

3. PC2 support based on operator request

In terms of the discussion in [1], no objections on the above three bullets, therefore they could be approved officially. Actually there were no 2nd email discussion for bullet 1 and bullet 3 since agreements were achieved in the 1st round discussion.

Proposal 1. Approve the follow tentative agreements:

1. 1 TX architecture of 23 dBm PA

2. 1 TX of 26 dBm PA in Rel-17 and 2 TX architecture is excluded in Rel-17

3. PC2 support based on operator request

Band n79

There were no agreements on whether or not introduce the n79 to RedCap UE. Actually, the new channel bandwidths of {10, 20, 30, 70, 90}MHz for band n79 have been introduced in TS38.101-1 via the CR [3], associated all the RF requirements, which means all the issues in RAN4 have been resloved.

Observation: From RAN4 aspect, all the issues have been resloved for band n79 introducing new channel bandwidths

Now the only issue for band n79 is the RAN1 NBC issue, i.e. how to determine the CORESET#0 configuration table for legacy UE and new UE. Actually this issue is general issue for band n79 which should be resloved for normal band n79 UE, rather than RedCap UE. Therefore, we think n79 can be supported for RedCap UE in Rel-17 from RAN4 perspective.

Proposal 2. From RAN4 aspect, band n79 can be supported for RedCap UE in Rel-17.

- 3 Conclusion

- The conclusions and the proposals are summarized below:

Proposal 1. Approve the follow tentative agreements:

1. 1 TX architecture of 23 dBm PA

2. 1 TX of 26 dBm PA in Rel-17 and 2 TX architecture is excluded in Rel-17

3. PC2 support based on operator request

Obversation: From RAN4 aspect, all the issues have been resloved for band n79 introducing new channel bandwidths

Proposal 2. From RAN4 aspect, band n79 can be supported for RedCap UE in Rel-17.

- 4	Reference

[1] R4-2202332 Email discussion summary for [101-bis-e][132] NR_RedCap, Moderator (Ericsson)

[2] R4-2202371 WF on the RedCap RF, Ericsson

[3] R4-2117846, Draft CR to TS 38.101-1: Introduction of additional CBWs for band n79, Samsung