3GPP TSG-RAN4 Meeting #102-e	R4-2204830

# Online,  , 21st Feb 2022 - 3rd Mar 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | draftCR | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR for 38.101-4 Interference model for enhanced performance requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_demod_enh2-Perf |  |  |  |  |  | Date: |  |  | 2022-02-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | New interference model appendix for MMSE-IRC feature |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added interference model appendix for MMSE-IRC feature |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No testable requirements for MMSE-IRC feature. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | B.X (new appendix) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of Changes >>

## B.X	Interference model for enhanced performance requirements, scenario 1

### B.X.1	Interference to Noise ratio (INR)

Each interfering cell involved in enhanced performance requirements characterized by its associated dominant interferer to noise ratio (INR) value:

INRi=Ior(i+1)Noc'

where is  is the average received power spectral density from the i-th strongest interfering cell involved in the requirement scenario (is assumed to be the power spectral density associated with the serving cell) and Noc'=j=M+2NIor(j)+Noc

where

Noc is the average power spectral density of a white noise source consistent with definition provided in section [4.4.3/4.5.3].

N is the total number of cells involved in a given requirement scenario.

M≤N-1 is the number of dominant interferer cells involved in a given requirement scenario.

In the case of M=N-1, the EsNoc and INR values are the same for dominant interferers.

### B.X.2	Interference model for enhanced performance requirements, synchronous deployment

This subclause provides synchronous network deployment interference model for each explicitly modelled interfering cell in the requirement scenario. In each subframe, each interfering cell shall transmit 16 QAM based randomly modulated data over the entire PDSCH region and over the full transmission bandwidth of the specified reference measurement channel. Transmitted physical channels shall include PSS, SSS, PBCH and TRS/CSI-RS.

Transmission rank of the interfering cell shall be randomly determined with probabilities as defined in Table B.X.2-1. The rank configuration will be independent for each interfering cell. Each interfering cell has 80% DTX probability

For each TDRA a single precoding matrix for the number of layers  associated to the selected rank shall be selected randomly from Table 5.2.2.2.1-5 to Table 5.2.2.2.1-6 of TS 38.214 with PRB bundling granularity with PRB bundling size of 2.

Table B.X.2-1 Rank configuration for the random interference model

|  | Rank 1 | Rank 2 |
| --- | --- | --- |
| Rank probability | 70% | 30% |
| Note 1: The rank should follow the probability indicated in the table randomly per TDRA. Note 2: The probabilities for rank are used for both 1st and 2nd interfering cells. The MCS and rank configurations are independent for each interfering cell. |  |  |

The generic beamforming model in subclause B.4.1 shall be applied assuming number of antenna ports as specified in the requirement scenario.

Random precoding with selected rank and precoding matrices for each TDRA shall be applied to 16 QAM randomly modulated layer symbols including the demodulation reference symbols over antenna port 1000 when the rank is one and antenna ports 1000, 1001 when the rank is two. DMRS type 1 with front loaded single symbol and one additional DMRS position with FDM applied between DMRS and data (number of CDM groups without data is 1)

For each TDRA, the scrambling ID value nSCID is randomly assigned from the set of {0,1}.

[For unallocated REs in the control region, random precoding for the number of antenna ports in the requirement scenario shall be applied to QPSK randomly modulated symbols. ]

<< End of Changes >>