3GPP TSG-RAN4 Meeting #102-e	R4-2204850

# Electronic Meeting, 21st Feb– 3rd Mar 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DRAFT | rev | - | Current version: | 16.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of NR Sidelink reference configurations_R16 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Perf |  |  |  |  |  | Date: |  |  | 2022-02-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Reference configuration for NR sidelink communication given in A.3.21 needs to be corrected in following aspects: Table A.3.21.2-1: SL-BWP configuration - The wording " maxNrofRXPool-r16 = 1", " maxNrofTXPool-r16 = 1" are confusing since maxNrofRXPool-r16 = 16 according to 38.331. The intention here obviously should be "number of entries is 1". - Value of sl-TxPoolExceptional-r16 is wrongly written in comment column. Table A.3.21.2-2: Resource pool configuration - Configurations of sl-PSCCH-Config-r16, sl-PSSCH-Config-r16 and sl-PSFCH-Config-r16 are missing. sl-PSCCH-Config-r16, sl-PSSCH-Config-r16 should be set according to Table A.3.21.3-1 and Table A.3.21.3-2 respectively. And sl-PSFCH-Config-r16 should be set to not present according to Table A.3.21.3-2. - sl-SyncAllowed-r16 is a sequence of 3 flags according to 38.331 rather than enumeration of "gnss", "gnbEnb" and "ue". For sake of simplify we suggest set all three flags to true. Then this reference configuration of resource pool can be used under any kind of sync source. - Value of sl-SubchannelSize-r16, sl-MaxNumPerReserve should be "n10" and "n2" - Redudant curly braces should be removed from value of sl-RS-ForSensing and sl-SelectionWindow - sl-MCS-Table should be configured in sl-MinMaxMCS-List-r16 rather than SL-ResourcePool-r16. - sl-RxParametersNcell-r16 field is optional itself according to 38.331. It can be directly set to "Not present". - Value of sl-SensingWindow should be ms100; - sl-SelectionWindow-r16 should be configured for all 8 priority levels. - Resource reservation period field is not included in PSCCH RMC according to Table A.3.21.3-1. Which means that sl-MultiReserveResource should be explictly set to "Not present" according to 38.212 cl.8.3.1.1. Furthermore, including sl-ResourceReservePeriodList is meaningless since multi-reservation is not allowed. - Value of sl-ZoneConfigMCR-List-r16 is wrongly written in comment column. Table A.3.21.2-3: UE autonomous resource selection configuration - Value of sl-ProbResourceKeep-r16 should be " v0dot8"; - Value of sl-ReselectAfter-r16 should be "n1"; - Reference configuration of sl-PreemptionEnable-r16 should be given in Table A.3.21.2-2 rather than Table A.3.21.2-3 - Value of sl-MaxTxTransNumPSSCH-r16 in sl-ParametersAboveThres-r16 should be "1" rather than "both". - sl-TypeTxSync-r16 can be set to "not present" for simplfy. Table A.3.21.3-1: PSCCH RMC - 2nd stage SCI format should be set to "00" rather than "0" to indicate SCI format 2-A |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Reference configuration for NR sidelink is updated. - Table A.3.21.2-1/2/3 are reformatted to improve readablity. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Reference configuration is incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.21 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

A.3.21	V2X sidelink communication

A.3.21.1	Introduction

This clause also defines the principle and the reference configurations that are applicable to test cases verifying RRM core requirements for V2X sidelink communication.

A.3.21.2	Reference resource pool configurations for V2X Sidelink Communication

Table A.3.21.2-1: V2X sidelink SL-BWP configuration for NR

| Field | Value | Comment |
| --- | --- | --- |
| SL-BWP-ConfigCommon-r16 |  |  |
| sl-BWP-Generic-r16 |  |  |
| sl-LengthSymbols-r16 | sym14 |  |
| sl-StartSymbol-r16 | sym0 |  |
| sl-BWP-PoolConfigCommon-r16 |  |  |
| sl-RxPool-r16 |  | Indicates the resource pool for reception on the configured BWP. 1 entry |
| SL-ResourcePool-r16[1] | Set according to Table A.3.21.2-2 | Entry 1 |
| sl-TxPoolSelectedNormal-r16 |  | Indicates the resources pool for mode 2 sidelink communication on the configured BWP. 1 entry |
| SL-ResourcePoolConfig-r16[1] |  | Entry 1 |
| sl-ResourcePool-r16 | Set according to Table A.3.21.2-2 |  |
| sl-TxPoolExceptional-r16 | Not present |  |

Table A.3.21.2-2: V2X sidelink resource pool configuration for NR

| Field | Value | Comment |
| --- | --- | --- |
| SL-ResourcePool-r16 |  |  |
| sl-PSCCH-Config-r16 | Set according to Table A.3.21.3-1 |  |
| sl-PSSCH-Config-r16 | Set according to Table A.3.21.3-2 |  |
| sl-PSFCH-Config-r16 | Not present |  |
| sl-SyncAllowed-r16 |  |  |
| gnss-Sync-r16 | true |  |
| gnbEnb-Sync-r16 | true |  |
| ue-Sync-r16 | true |  |
| sl-SubchannelSize-r16 | n10 | Minimum bandwidth of subchannel for adjacent transmission |
| sl-StartRB-Subchannel-r16 | 0 | Indicates the lowest RB index of the subchannel with the lowest index. |
| sl-NumSubchannel-r16 | 1 | Number of subchannels for SL transmission |
| sl-UE-SelectedConfigRP-r16 |  |  |
| sl-Thres-RSRP-List-r16 | Set according to the specific test configuration | Indicates a list of 64 thresholds, and the threshold should be selected based on the priority in the decoded SCI and the priority in the SCI to be transmitted. A resource is excluded if it is indicated or reserved by a decoded SCI and PSSCH RSRP in the associated data resource is above a threshold. |
| sl-MultiReserveResource-r16 | Not present |  |
| sl-MaxNumPerReserve-r16 | n2 | Indicates the maximum number of reserved PSCCH/PSSCH resources that can be indicated by an SCI. |
| sl-SensingWindow-r16 | ms100 | Parameter that indicates the start of the sensing window. |
| sl-SelectionWindowList-r16 |  | Parameter that determines the end of the selection window for each priority level 8 entries |
| SL-SelectionWindowConfig-r16[k,k=1..8] |  | entry k |
| sl-Priority-r16 | k | priority k |
| sl-SelectionWindow-r16 | n20 |  |
| sl-ResourceReservePeriodList-r16 | Not present |  |
| sl-RS-ForSensing-r16 | pssch | PSSCH-RSRP measurement is used in the sensing operation. |
| sl-RxParametersNcell | Not present |  |
| sl-ZoneConfigMCR-List-r16 | Not present |  |
| sl-PreemptionEnable-r16 | enabled |  |
| sl-MinMaxMCS-List-r16 |  | 1 entry |
| SL-MinMaxMCS-Config-r16[1] |  | Entry 1 |
| sl-MCS-Table-r16 | qam64 | Indicates the MCS table used in the resource pool. |
| sl-TimeResource-r16 | 11111111111111111111 | Indicates the time resource of resource pool |

Table A.3.21.2-3: V2X sidelink UE autonomous resource selection configuration for NR

| Field | Value | Comment |
| --- | --- | --- |
| SL-UE-SelectedConfig-r16 |  |  |
| sl-PSSCH-TxConfigList-r16 |  | 1 entry |
| SL-PSSCH-TxConfig-r16[1] |  | Entry 1 |
| sl-TypeTxSync-r16 | Not present | When this filed is absent, the configuration is applicable for all synchronization reference types. |
| sl-ThresUE-Speed-r16 | kmph200 | This filed indicates a UE absolute speed threshold. |
| sl-ParametersAboveThres-r16 |  |  |
| sl-MinMCS-PSSCH-r16 | 0 | These fields indicate the minimum anMCS value used for transmissions on PSSCH. |
| sl-MaxMCS-PSSCH-r16 | 15 | This field indicates the maximum MCS value used for transmissions on PSSCH. |
| sl-MinSubChannelNumPSSCH-r16 | 1 | This field indicates the minimum number of sub-channels which may be used for transmissions on PSSCH. |
| sl-MaxSubchannelNumPSSCH-r16 | 1 | This field indicates the maximum number of sub-channels which may be used for transmissions on PSSCH. |
| sl-MaxTxTransNumPSSCH-r16 | 1 | Indicates the maximum transmission number (including new transmission and retransmission) for PSSCH. |
| sl-MaxTxPower-r16 | Not present | This filed indicates the maximum transmission power for transmission on PSSCH and PSCCH. |
| sl-ParametersBelowThres-r16 |  |  |
| sl-MinMCS-PSSCH-r16 | 4 |  |
| sl-MaxMCS-PSSCH-r16 | 25 |  |
| sl-MinSubChannelNumPSSCH-r16 | 1 |  |
| sl-MaxSubchannelNumPSSCH-r16 | 1 |  |
| sl-MaxTxTransNumPSSCH-r16 | 1 |  |
| sl-MaxTxPower-r16 | Not present |  |
| sl-ProbResourceKeep-r16 | v0dot8 | Indicates the probability with which the UE keeps the current resource when the resource reselection counter reaches zero for sensing based UE autonomous resource selection (see TS 38.321 [7]). |
| sl-ReselectAfter-r16 | n1 | Indicates the number of consecutive skipped transmissions before triggering resource reselection for sidelink communication (see TS 38.321 [7]). |

A.3.21.3	Reference measurement channels for V2X Sidelink Communication

Table A.3.21.3-1: PSCCH Reference Measurement Channels

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Reference channel |  |  | CC.1A HD |
| Channel bandwidth |  | MHz | Note2 |
| Number of PSCCH symbols per slot |  |  | 2 |
| Number of PSCCH RB |  |  | 10 |
| Modulation |  |  | QPSK |
| Information Bit Payload (without CRC) |  | Bits | 26 |
| Information Bit | Number of DMRS ports |  | 0 (1 port) |
|  | Priority |  | As set by higher layers |
|  | Resource reservation period |  | N/A |
|  | Modulation and coding scheme |  | Set as the PSSCH MCS specified in the test |
|  | DMRS pattern |  | 0 (2 DMRS) |
|  | 2nd stage SCI format |  | 00 (SCI format 2-A) |
|  | Beta offset indicator |  | Set as specified in the test |
|  | Frequency resource assignment |  | Set as per PSSCH RB allocation specific in the test |
|  | Time resource assignment |  | Set as per PSSCH slot allocation specific in the test |
|  | Reserved bits |  | Set all these bits to 0 |
| Transport block CRC |  | Bits | 24 |
| Binary Channel Bits (see Note 1) |  | Bits | 360 |
| Note 1: Binary channel bits calculated under assumption of 2 CP-OFDM symbols per subframe. Note 2: Channel bandwidth depends on test configuration. |  |  |  |

Table A.3.21.3-2: PSSCH Reference Measurement Channels

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | CD.1A HD |
| Sidelink transmission mode |  | 2 |
| Channel bandwidth | MHz | Note1 |
| Allocated PSSCH resource blocks |  | 10 |
| Number of PSSCH symbols per slot |  | 10 |
| Modulation |  | QPSK |
| Target Code Rate |  | 1/3 |
| Information Bit Payload (Transport block size) | Bits | 672 |
| Transport block CRC | Bits | 24 |
| Number of PSSCH HARQ retransmissions |  | 0 |
| Binary Channel Bits | Bits | 2160 |
| Note 1: Channel bandwidth depends on test configuration. Note 2: 2nd state SCI and PSFCH are not allocated per slot. |  |  |

<End of modified section 1>