3GPP TSG-RAN WG4#101bis-e	R4-2205027

Electronic meeting, 17th – 25th January 2022

Source: 	Ericsson

Title:  	Draft TP to TS 38.106: Frequency stability and out of band gain requirements

Agenda Item:	10.5.2.3

Document for:	Approval

# Introduction

The work split for repeaters spec drafting was agreed in [1]. This contribution presents proposed text for frequency stability and out of band gain.

# Discussion

In [2], it was agreed that frequency stability would be 0.01 PPM for both FR1 and FR2.

The out of band gain requirement has not yet been agreed. This contribution presents an example based on the 36.106 text. The OOB gain indicated is exemplary and dependent on agreements.

Due to the need to develop a new OOB gain for FR2, OTA OOB gain is for the time being left open.

# Text Proposal

## 6.3	Frequency stability

Frequency stability is the ability to maintain the same frequency on the output signal with respect to the input signal.

## 6.3.1	Minimum requirement

The frequency deviation of the output signal with respect to the input signal shall be no more than ±0,01 PPM.

## 6.4	Out of band gain

Out of band gain refers to the gain of the repeater outside the pass band.

## Minimum requirement

The intended use of a repeater in a system is to amplify the in-band signals and not to amplify signals outside of the passband.

In the intended application of the repeater, the out of band gain is less than lowest expected the coupling loss to the nearest source of emissions.

The repeater minimum coupling loss to the nearest source of emissions shall be declared by the manufacturer. This is the minimum required attenuation between any emissions source outside of the passband and the repeater for proper repeater operation.

The gain outside the pass band shall not exceed the maximum level specified in table 6.4.1-1 or table 6.4.1-2, where:

-	f_offset_CW is the offset between the outer channel edge frequency of the outer channel in the pass band and a CW signal.

# Table 6.4.1-1: Out of band gain limits 1 for bands below 2[.5] GHz

| Frequency offset, f_offset_CW | Maximum gain |
| --- | --- |
| 0,2 f_offset_CW < 1,0 MHz | 60 dB |
| 1,0 f_offset_CW < 5,0 MHz | 45 dB |
| 5,0 f_offset_CW < 10,0 MHz | 45 dB |
| 10,0 MHz f_offset_CW | 35 dB |

Table 6.4.1-2: Out of band gain limits 1 for bands above 2[.5] GHz [and passband bandwidths >20MHz]

| Frequency offset, f_offset_CW | Maximum gain |
| --- | --- |
| [0,2]< f_offset_CW < [4,0] MHz | 60 dB |
| [4,0]< f_offset_CW < [15,0] MHz | 45 dB |
| [15,0] MHz <f_offset_CW | 35 dB |

For 10,0 MHz  f_offset_CW the out of band gain shall not exceed the maximum gain of table 6.4.1.1-3 or the maximum gain stated in table 6.4.1-1 or table 6.4.1-2, whichever is lower.

Table 6.4.1-2: Out of band gain limits 3

| Frequency offset, f_offset_CW | Maximum gain |
| --- | --- |
| 10 MHz f_offset_CW | Out of band gain minimum coupling loss to nearest interference source |

## 7.3	OTA frequency stability

### Frequency stability is the ability to maintain the same frequency on the output signal with respect to the input signal.

### 7.3.1	Minimum requirement

The frequency deviation of the output signal with respect to the input signal shall be no more than ±0,01 PPM.

## 7.4	OTA out of band gain

<Text will be added.>

# References

CMCC, “WF on specification drafting””, R4-2120656, RAN4#101-e, November 2021