3GPP TSG-RAN WG4 Meeting #102-e	R4-2205097

Electronic Meeting, 21 February – 3 March 2022

Agenda Item:	10.20.4.3.1

Source:	Ericsson

Title:	CQI reporting requirements for RedCap

Document for:	Discussion

# 1	Introduction

RAN4#101-bis-e agreed on RedCap demodulation and CQI reporting requirements [1]. This contribution discusses the open issues on CQI reporting requirements for RedCap.

# 2	Discussion

## 2.1	 Test setup

- For 2Rx tests for FR1 FDD, - Define new 2Rx tests for FR1 FDD with CQI table 1 - FFS whether to define test with CQI table 2 for Redcap UE supporting 256 QAM. If agreed, existing test requirements can be reused

RAN4 agreed to define CQI reporting tests for both static and fading condition for RedCap 1Rx and 2Rx UEs. RAN4 has also agreed to define wideband CQI reporting test only for the fading condition. Since supporting 256QAM is optional for RedCap UE, RAN4 uses CQI table 1 for CQI reporting tests.

We are fine to define CQI reporting test with CQI table 2 for RedCap 2Rx UE with applicability as far as RAN4 can reuse the existing requirements.

Proposal 1: Define CQI reporting test with CQI table 1 (Test 1) for RedCap 2Rx UE in FR1 FDD. Define also CQI reporting test with CQI table 2 (Test 2) for RedCap 2Rx UE in FR1 FDD by reusing the existing requirements. If UE is capable of 256QAM, UE need to pass Test 2. Otherwise UE need to pass Test 1.

As we discussed in our companion paper [2], we also propose to remove 1Rx case from FR2.

Proposal 2: Define the following CQI reporting tests for RedCap UE.

FR1 FDD SCS=15kHz

| 1Rx UE or 2Rx UE | Reference | CQI test | CBW / SCS | CQI table | Rank | Antenna configuration | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | Table 6.2.2.1.1.1-1 Test 1/2 | CQI report definition test | 10 / 15 | Table 1 | 1 | 2 x 1 | New |
| 1Rx UE | Table 6.2.2.1.2.1-1 Test 1/2 | Wideband CQI reporting test | 10 / 15 | Table 1 | 1 | 2 x 1 | New |
| 2Rx UE | Table 6.2.2.1.1.1-1 Test 1/2 | CQI report definition test | 10 / 15 | Table 1 | 2 | 2 x 2 Low | New for UE not supporting 256QAM |
| 2Rx UE | Table 6.2.2.1.2.1-1 Test 1/2 | Wideband CQI reporting test | 10 / 15 | Table 1 | 1 | 2 x 2 Low | New for UE not supporting 256QAM |
| 2Rx UE | Table 6.2.2.1.1.1-1 Test 1/2 | CQI report definition test | 10 / 15 | Table 2 | 2 | 2 x 2 Low | Reuse for UE supporting 256QAM |
| 2Rx UE | Table 6.2.2.1.2.1-1 Test 1/2 | Wideband CQI reporting test | 10 / 15 | Table 2 | 1 | 2 x 2 Low | Reuse for UE supporting 256QAM |

FR1 TDD SCS=30kHz

| 1Rx UE or 2Rx UE | Reference | CQI test | CBW / SCS | CQI table | Rank | Antenna configuration | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | Table 6.2.2.1.1.1-1 Test 1/2 | CQI report definition test | 20 / 30 | Table 1 | 1 | 2 x 1 | New |
| 1Rx UE | Table 6.2.2.1.2.1-1 Test 1/2 | Wideband CQI reporting test | 20 / 30 | Table 1 | 1 | 2 x 1 | New |
| 2Rx UE | Table 6.2.2.1.1.1-1 Test 1/2 | CQI report definition test | 20 / 30 | Table 1 | 2 | 2 x 2 Low | New |
| 2Rx UE | Table 6.2.2.1.2.1-1 Test 1/2 | Wideband CQI reporting test | 20 / 30 | Table 1 | 1 | 2 x 2 Low | New |

FR2 TDD SCS=120kHz

| 1Rx UE or 2Rx UE | Reference | CQI test | CBW / SCS | CQI table | Rank | Antenna configuration | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | Table 8.2.2.2.1.1-1 Test 1/2 | CQI report definition test | 100 / 120 | Table 1 | 1 | 2 x 1 | New |
| 1Rx UE | Table 8.2.2.2.2.1-1 Tests 1/2 | Wideband CQI reporting test | 100 / 120 | Table 1 | 1 | 2 x 1 | New |
| 2Rx UE | Table 8.2.2.2.1.1-1 Test 1/2 | CQI report definition test | 100 / 120 | Table 1 | 2 | 2 x 2 | Reuse |
| 2Rx UE | Table 8.2.2.2.2.1-1 Tests 1/2 | Wideband CQI reporting test | 100 / 120 | Table 1 | 1 | 2 x 2 | Reuse |

## 2.2	Test points for CQI reporting tests

Figure 1 shows our initial simulation results of median CQI for RedCap UE in static condition in FR1 FDD, based on the CQI-TBS mapping table shown in Table 1. In Rel-15, RAN4 set the SNR test points to 8/9dB and 14/15dB for FR1/FR2. For RedCap, since we agreed to configure CQI table 1, if we reuse the same test points as the existing test cases with CQI table 2 for RedCap 2Rx , SNR=8/9dB and SNR=14/15dB corresponds to CQI index with 16QAM and 64QAM for CQI table 1 with rank 2. Then we prefer to set SNR test points corresponding to 16QAM and 64QAM for 1Rx case.

Proposal 3: For the static condition CQI reporting test with CQI table 1, set two SNR test points corresponding to CQI indexes of 16QAM and 64QAM for RedCap 1Rx and 2Rx UEs.

Proposal 4: For the static condition CQI reporting test with CQI table 1 for 2Rx UEs, set SNR=8/9dB and SNR=14/15dB as the starting point.

We also apply the same assumption to fading condition.

Proposal 5: For the wideband CQI reporting test with CQI table 1 in fading condition, set two SNR test points corresponding to CQI indexes of 16QAM and 64QAM for RedCap 1Rx and 2Rx UEs.

Figure 1	Median CQI for RedCap UE in FR1 FDD.

Table 1	Mapping of CQI Index to Information Bit payload (CQI table 1)

| CBW/SCS |  |  |  | 10MHz / 15kHz | 10MHz / 15kHz | 20MHz / 30kHz | 20MHz / 30kHz |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MCS table |  |  |  | 64QAM | 64QAM | 64QAM | 64QAM |
| Number of allocated PDSCH resource blocks |  |  |  | 52 | 52 | 51 | 51 |
| Number of consecutive PDSCH symbols |  |  |  | 12 | 12 | 12 | 12 |
| Number of PDSCH MIMO layers |  |  |  | 1 | 2 | 1 | 2 |
| Number of DMRS REs |  |  |  | 24 | 24 | 24 | 24 |
| Overhead for TBS determination |  |  |  | 0 | 0 | 0 | 0 |
| Available REs |  |  |  | 6240 | 6240 | 6120 | 6120 |
| CQI index | Spectrum efficiency | MCS index | Modulation |  |  |  |  |
| 0 | OOR | OOR | OOR | N/A | N/A | N/A | N/A |
| 1 | 0.2344 | 0 | 2 | 1480 | 2976 | 1480 | 2856 |
| 2 | 0.2344 | 0 | 2 | 1480 | 2976 | 1480 | 2856 |
| 3 | 0.3770 | 2 | 2 | 2408 | 4744 | 2408 | 4616 |
| 4 | 0.6016 | 4 | 2 | 3752 | 7424 | 3752 | 7296 |
| 5 | 0.8770 | 6 | 2 | 5504 | 11016 | 5376 | 10760 |
| 6 | 1.1758 | 8 | 2 | 7296 | 14600 | 7168 | 14344 |
| 7 | 1.4766 | 11 | 4 | 9224 | 18432 | 8968 | 17928 |
| 8 | 1.9141 | 13 | 4 | 12040 | 24072 | 11784 | 23568 |
| 9 | 2.4062 | 15 | 4 | 15112 | 30216 | 14600 | 29192 |
| 10 | 2.7305 | 18 | 6 | 16896 | 33816 | 16896 | 33816 |
| 11 | 3.3223 | 20 | 6 | 20496 | 40976 | 20496 | 40976 |
| 12 | 3.9023 | 22 | 6 | 24576 | 49176 | 24072 | 48168 |
| 13 | 4.5234 | 24 | 6 | 28168 | 56368 | 27656 | 55304 |
| 14 | 5.1152 | 26 | 6 | 31752 | 63528 | 31240 | 62504 |
| 15 | 5.5547 | 28 | 6 | 31752 | 69672 | 33816 | 67584 |

# 3	Summary

Proposal 1: Define CQI reporting test with CQI table 1 (Test 1) for RedCap 2Rx UE in FR1 FDD. Define also CQI reporting test with CQI table 2 (Test 2) for RedCap 2Rx UE in FR1 FDD by reusing the existing requirements. If UE is capable of 256QAM, UE need to pass Test 2. Otherwise UE need to pass Test 1.

Proposal 2: Define the following CQI reporting tests for RedCap UE.

FR1 FDD SCS=15kHz

| 1Rx UE or 2Rx UE | Reference | CQI test | CBW / SCS | CQI table | Rank | Antenna configuration | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | Table 6.2.2.1.1.1-1 Test 1/2 | CQI report definition test | 10 / 15 | Table 1 | 1 | 2 x 1 | New |
| 1Rx UE | Table 6.2.2.1.2.1-1 Test 1/2 | Wideband CQI reporting test | 10 / 15 | Table 1 | 1 | 2 x 1 | New |
| 2Rx UE | Table 6.2.2.1.1.1-1 Test 1/2 | CQI report definition test | 10 / 15 | Table 1 | 2 | 2 x 2 Low | New for UE not supporting 256QAM |
| 2Rx UE | Table 6.2.2.1.2.1-1 Test 1/2 | Wideband CQI reporting test | 10 / 15 | Table 1 | 1 | 2 x 2 Low | New for UE not supporting 256QAM |
| 2Rx UE | Table 6.2.2.1.1.1-1 Test 1/2 | CQI report definition test | 10 / 15 | Table 2 | 2 | 2 x 2 Low | Reuse for UE supporting 256QAM |
| 2Rx UE | Table 6.2.2.1.2.1-1 Test 1/2 | Wideband CQI reporting test | 10 / 15 | Table 2 | 1 | 2 x 2 Low | Reuse for UE supporting 256QAM |

FR1 TDD SCS=30kHz

| 1Rx UE or 2Rx UE | Reference | CQI test | CBW / SCS | CQI table | Rank | Antenna configuration | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | Table 6.2.2.1.1.1-1 Test 1/2 | CQI report definition test | 20 / 30 | Table 1 | 1 | 2 x 1 | New |
| 1Rx UE | Table 6.2.2.1.2.1-1 Test 1/2 | Wideband CQI reporting test | 20 / 30 | Table 1 | 1 | 2 x 1 | New |
| 2Rx UE | Table 6.2.2.1.1.1-1 Test 1/2 | CQI report definition test | 20 / 30 | Table 1 | 2 | 2 x 2 Low | New |
| 2Rx UE | Table 6.2.2.1.2.1-1 Test 1/2 | Wideband CQI reporting test | 20 / 30 | Table 1 | 1 | 2 x 2 Low | New |

FR2 TDD SCS=120kHz

| 1Rx UE or 2Rx UE | Reference | CQI test | CBW / SCS | CQI table | Rank | Antenna configuration | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | Table 8.2.2.2.1.1-1 Test 1/2 | CQI report definition test | 100 / 120 | Table 1 | 1 | 2 x 1 | New |
| 1Rx UE | Table 8.2.2.2.2.1-1 Tests 1/2 | Wideband CQI reporting test | 100 / 120 | Table 1 | 1 | 2 x 1 | New |
| 2Rx UE | Table 8.2.2.2.1.1-1 Test 1/2 | CQI report definition test | 100 / 120 | Table 1 | 2 | 2 x 2 | Reuse |
| 2Rx UE | Table 8.2.2.2.2.1-1 Tests 1/2 | Wideband CQI reporting test | 100 / 120 | Table 1 | 1 | 2 x 2 | Reuse |

Proposal 3: For the static condition CQI reporting test with CQI table 1, set two SNR test points corresponding to CQI indexes of 16QAM and 64QAM for RedCap 1Rx and 2Rx UEs.

Proposal 4: For the static condition CQI reporting test with CQI table 1 for 2Rx UEs, set SNR=8/9dB and SNR=14/15dB as the starting point.

Proposal 5: For the wideband CQI reporting test with CQI table 1 in fading condition, set two SNR test points corresponding to CQI indexes of 16QAM and 64QAM for RedCap 1Rx and 2Rx UEs.

# References

- R4-2203032 “WF on RedCap demodulation and CQI reporting requirements”, Ericsson.

- R4-2205094, “General issues on UE demodulation and CSI requirements for RedCap”, Ericsson.