3GPP TSG-RAN WG4 Meeting # 102-e 	R4-2205233

# Electronic Meeting, February 21 – March 03, 2022

Source:	LG Electronics

Title:	60GHz channel and synchronization raster

Agenda item:	10.16.2	Operation bands and system parameters (channelization, raster, CBW, etc)

Document for:	Discussion and Approval

# Introduction

In RAN4#101bis-e meeting following agreement [1] was made on channel and sync raster arrangement for FR2-2.

Agreement: Consider the different channelization for licensed band(s) and unlicensed band(s)

- Fixed sync raster for unlicensed bands

  - Fixed scheme should not be constrained by IEEE channel raster

  - Send LS to RAN1 to make sure that RAN1 accommodates the solution already now for both fixed and floating sync raster.

  - For the contiguous carrier aggregation, the channel spacing of adjacent channels should be multiple of the larger SCS, i.e., 960KHz, used by two channels/CCs

- Floating sync raster for licensed bands

- Stick to the agreement last meeting for 3 x 17.28MHz as the minimum granularity.

  - Refer to gap between adjacent GSCN values is not smaller than 3 x 17.28MHz

- FFS: Unlicensed bands tries to use the sub-set of sync raster for licensed bands

In addition to agreement above following clarification was added to LS [2] that was sent out to RAN1:

In order to facilitate discussions in RAN1, RAN4 would like to also note for licensed operations the minimum granularity, RAN4 is considering 3 x 17.28 MHz for 120 kHz, 12 x 17.28 MHz for 480 kHz and 6 x 17.28 MHz for 960 kHz. RAN1 may use this information for further discussions.

In this document we provide our views and proposal for channel and synchronization raster for 52.6-71GHz frequency range intended for licensed and un-licensed operation.

# Channel raster for 52.6-71GHz frequency range

Proposal #1:

In order to have common and simplified ARFCN scheme that could be applied to all FR2-2 bands a proposal to use channel raster step size of 960kHz is made, which is the same as the largest supported SCS.

More detailed analysis and the start/end points for frequency ranges of 52.6-57GHz, 57-71GHz (n263) and 66-71GHz is shown below. Nref= 2 490 011, which corresponds to CF= 52 650.72MHz, has been selected as lowest channel location and all other channel locations are having offset that is multiple of <16> from this one (16x60kHz= 960kHz).

Last channel location has Nref= 2 794 987, which corresponds to CF= 70 949,28MHz and the number of channel locations within FR2-2 band is (2 794 987 - 2 490 011) / 16 + 1 = 19 062.

For 480k SCS and 960kHz SCS the lowest and highest channel center frequency is 200MHz away from the band edge, which is half of the 400MHz minimum CBW.

All the proposed channel raster entries have frequency offset to GSCN grid, which is a multiple of the 960kHz, and hence the NRU type of wide-band operation is possible.

| 52600-57000 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | x16 | Nref Delta | Nref | Band LL/UL | CF |
| 120k | 0 | 0 | 2 490 011 | 52 600 | 52 650,72 |
| 120k | … | … | … |  | … |
| 120k | 4478 | 71648 | 2 561 659 | 57 000 | 56 949,60 |
|  | x16 | Nref Delta | Nref | Band LL/UL | CF |
| 480k | 156 | 2496 | 2 492 507 | 52 600 | 52 800,48 |
| 480k | … | … | … |  | … |
| 480k | 4322 | 69152 | 2 559 163 | 57 000 | 56 799,84 |
|  | x16 | Nref Delta | Nref | Band LL/UL | CF |
| 960k | 156 | 2496 | 2 492 507 | 52 600 | 52 800,48 |
| 960k | … | … | … |  | … |
| 960k | 4322 | 69152 | 2 559 163 | 57 000 | 56 799,84 |
| 57000-71000 |  |  |  |  |  |
|  | x16 | Nref Delta | Nref | Band LL/UL | CF |
| 120k | 4583 | 73328 | 2 563 339 | 57 000 | 57 050,40 |
| 120k | … | … | … |  | … |
| 120k | 19061 | 304976 | 2 794 987 | 71 000 | 70 949,28 |
|  | x16 | Nref Delta | Nref | Band LL/UL | CF |
| 480k | 4739 | 75824 | 2 565 835 | 57 000 | 57 200,16 |
| 480k | … | … | … |  | … |
| 480k | 18905 | 302480 | 2 792 491 | 71 000 | 70 799,52 |
|  | x16 | Nref Delta | Nref | Band LL/UL | CF |
| 960k | 4739 | 75824 | 2 565 835 | 57 000 | 57 200,16 |
| 960k | … | … | … |  | … |
| 960k | 18905 | 302480 | 2 792 491 | 71 000 | 70 799,52 |
| 66000-71000 |  |  |  |  |  |
|  | x16 | Nref Delta | Nref | Band LL/UL | CF |
| 120k | 13958 | 223328 | 2 713 339 | 66 000 | 66 050,40 |
| 120k | … | … | … |  | … |
| 120k | 19061 | 304976 | 2 794 987 | 71 000 | 70 949,28 |
|  | x16 | Nref Delta | Nref | Band LL/UL | CF |
| 480k | 14114 | 225824 | 2 715 835 | 66 000 | 66 200,16 |
| 480k | … | … | … |  | … |
| 480k | 18905 | 302480 | 2 792 491 | 71 000 | 70 799,52 |
|  | x16 | Nref Delta | Nref | Band LL/UL | CF |
| 960k | 14114 | 225824 | 2 715 835 | 66 000 | 66 200,16 |
| 960k | … | … | … |  | … |
| 960k | 18905 | 302480 | 2 792 491 | 71 000 | 70 799,52 |

This proposal can also be presented using the similar approach as in 38.101-2 table 5.3.2.3-1 as shown below.

Table 5.2-1: NR operating bands in FR2

| Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |
| n258 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz | TDD |
| n260 | 37000 MHz | – | 40000 MHz | 37000 MHz | – | 40000 MHz | TDD |
| n261 | 27500 MHz | – | 28350 MHz | 27500 MHz | – | 28350 MHz | TDD |
| … |  |  |  |  |  |  |  |
| n263 | 57000 MHz | – | 71000 MHz | 57000 MHz | – | 71000 MHz | TDD |

Table 5.4.2.3-1: Applicable NR-ARFCN per operating band

| Operating Band | ΔFRaster (kHz) | Uplink and Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- |
| n257 | 60 | 2054166 – <1> – 2104165 |
|  | 120 | 2054167 – <2> – 2104165 |
| n258 | 60 | 2016667 – <1> – 2070832 |
|  | 120 | 2016667 – <2> – 2070831 |
| n260 | 60 | 2229166 – <1> – 2279165 |
|  | 120 | 2229167 – <2> – 2279165 |
| n261 | 60 | 2070833 – <1> – 2084999 |
|  | 120 | 2070833 – <2> – 2084999 |
| … |  |  |
| n263 | 120 | 2563339– <16> – 2794987 |
|  | 480 | 2565835 – <16> – 2792491 |
|  | 960 | 2565835 – <16> – 2792491 |

# SSB raster for 52.6 – 71GHz frequency range

A global synchronization raster defined in 38.101-2 [3] can be used as starting point also for FR2-2. The specified GSCN frequency range already covers the 57 to 71GHz frequency range and 17.28MHz step size is suitable for 100 and 400MHz minimum channel bandwidths agreed for 120kHz and 480kHz SCS respectively.

Proposal #2:

As per agreement in [1] a floating raster is to be used for the licensed bands. Again, in order to harmonize and simplify the GSCN grid it is proposed that similar GSCN grid will be used for all licensed bands within RF2-2 and GSCN raster size of <3> is used for 120kHz SCS and <12> for 480kHz SCS. The GSCN locations for 480kHz SCS are down-selected from 120kHz locations.

GSCN= 23 898, which corresponds to CF= 52 623.84MHz has been selected as lowest sync channel location and highest sync channel location is GSCN= 24 960, which corresponds to CF= 70 975,20MHz.

Table 1. Example for defining the SS raster entries

| NR Operating Band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| 52.6-57GHz | 120kHz |  | 23898 < 3 > 24150 |
|  | 480kHz |  | 23904 < 12 > 24144 |
| 66-71GHz | 120kHz |  | 24675 < 3 > 24960 |
|  | 480kHz |  | 24681 < 12 > 24957 |
| NOTE1: SS Block pattern is defined in sub clause 4.1 in TS 38.213 [10] |  |  |  |

The location of these raster locations vs. fixed 100MHz channels for 52.6-57 and 66-71GHz is shown in Figure 1 below. Fixed 100MHz channel in these plots are artificial channels as floating ARFNC raster was proposed earlier, but this shows how the step=3 GSCN raster locations slide within such channel locations.

Figure 1. Floating step=3 raster vs. fixed 100MHz channels

Proposal #3:

For un-licensed band of n264 following GSCN grid is proposed:

- One SSB location is defined for each 100MHz channel

- Two SSB locations are defined for 480kHz SCS and 400MHz channel. SSB locations for 400MHz reuse the down selected center frequencies from 120kHz SSB. Two SSB locations is selected for each 400MHz of spectrum with 480kHz SCS.

Figure 2. High level description on selection of GSCN locations for 100MHz channels and further down-selection for 400MHz.

The down-selection of the global raster for 100MHz channels can be done with step size vector with length of 19 and values of < 6 6 6 6 5 6 6 6 6 5 6 6 6 5 6 6 6 6 5 >. In order to align the GSCN raster with ARFCN raster to support wide-band operation as in NRU a starting point of 24153 is selected. For 480k SCS with further down-selected entries (from 100MHz locations) the 70 locations can listed in the specification and are shown in Table 2 below.

Table 2. Example for defining the SS raster entries

| NR Operating Band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n263 57-71GHz | 120kHz |  | 24153 < NOTE2 > 24958 |
|  | 480kHz |  | NOTE3 |
| NOTE1: SS Block pattern is defined in sub clause 4.1 in TS 38.213 [10] NOTE2: Step size vector with length of 19: < 6 6 6 6 5 6 6 6 6 5 6 6 6 5 6 6 6 6 5 > NOTE3: Valid GSCN entries are 24159, 24165, 24182, 24188, 24206, 24211, 24229, 24234, 24252, 24258, 24275, 24281, 24298, 24304, 24321, 24327, 24344, 24350, 24368, 24373, 24391, 24397, 24414, 24420, 24437, 24443, 24460, 24466, 24483, 24489, 24507, 24512, 24530, 24536, 24553, 24559, 24576, 24582, 24599, 24605, 24622, 24628, 24646, 24651, 24669, 24674, 24692, 24698, 24715, 24721, 24738, 24744, 24761, 24767, 24784, 24790, 24808, 24813, 24831, 24837, 24854, 24860, 24877, 24883, 24900, 24906, 24923, 24929, 24947, 24952 |  |  |  |

With this approach the total number of SS raster entries is 210 as shown in Table 3.

Table 3. Number of SS raster entries

|  | Channels | Locations per channel | Total |
| --- | --- | --- | --- |
| 120k SCS | 140 | 1 | 140 |
| 480k SCS | 35 | 2 | 70 |
| TOTAL |  |  | 210 |

Figure 3 and Figure 4 below show how the SSBs falls inside the 100 and 400MHz channels when defined as in Table 3. Table 3 shows the width of the upper and lower guard-band (gap between SSB and channel edge), which are always larger than the minimum guard-bands currently defined in 38.101-2, which are 2.42MHz for 100MHz bandwidth and 9.86MHz for 400MHz wide bandwidth.

Table 4. Minimum guard bands

|  | Low | High |  |
| --- | --- | --- | --- |
| 120k SCS | 15,44 | 34,64 | MHz |
| 480k SCS | 73,28 | 92,48 | MHz |

Figure 3. 120k SCS SSB locations

Figure 4. 480k SCS SSB locations

# Conclusion

In this document channel raster and SSB raster for 52.6-71GHz frequency range is discussed and proposals for both are made for both un-licensed band n263 and also for foreseen bands for licensed operation.

Proposal #1:

In order to have common and simplified ARFCN scheme that could be applied to all FR2-2 bands a proposal to use channel raster step size of 960kHz is made, which is the same as the largest supported SCS.

Proposal #2:

As per agreement in [1] a floating raster is to be used for the licensed bands. Again, in order to harmonize and simplify the GSCN grid it is proposed that similar GSCN grid will be used for all licensed bands within RF2-2 and GSCN raster size of <3> is used for 120kHz SCS and <12> for 480kHz SCS. The GSCN locations for 480kHz SCS are down-selected from 120kHz locations.

Proposal #3:

For un-licensed band of n263 following GSCN grid is proposed:

- One SSB location is defined for each 100MHz channel

- Two SSB locations are defined for 480kHz SCS and 400MHz channel. SSB locations for 400MHz reuse the down selected center frequencies from 120kHz SSB. Two SSB locations is selected for each 400MHz of spectrum with 480kHz SCS.

Proposals are described in more detail in chapters 2 and 3 of this contribution.

List of SSB locations for 120kHz and 480kHz SCS are shown in Annex 1.

# References

[1] R4-2202365, WF on general aspects and system parameters of FR2-2, RAN4#101bis-e, January 2022

[2] R4-2200081, LS for the channelization for up to 71 GHz, RAN4#101bis-e, January 2022

[3] 3GPP TS 38.101-2, User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone

ANNEX 1:

SSB locations for n263

120k SCS

|  |  |  |  | 100MHz |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | N | GSCN | Fc | Low | High |  | N | GSCN | Fc | Low | High |
| 1 | 1897 | 24153 | 57030,24 | 57 000 | 57 100 | 36 | 2100 | 24356 | 60538,08 | 60 500 | 60 600 |
| 2 | 1903 | 24159 | 57133,92 | 57 100 | 57 200 | 37 | 2106 | 24362 | 60641,76 | 60 600 | 60 700 |
| 3 | 1909 | 24165 | 57237,6 | 57 200 | 57 300 | 38 | 2112 | 24368 | 60745,44 | 60 700 | 60 800 |
| 4 | 1915 | 24171 | 57341,28 | 57 300 | 57 400 | 39 | 2117 | 24373 | 60831,84 | 60 800 | 60 900 |
| 5 | 1921 | 24177 | 57444,96 | 57 400 | 57 500 | 40 | 2123 | 24379 | 60935,52 | 60 900 | 61 000 |
| 6 | 1926 | 24182 | 57531,36 | 57 500 | 57 600 | 41 | 2129 | 24385 | 61039,2 | 61 000 | 61 100 |
| 7 | 1932 | 24188 | 57635,04 | 57 600 | 57 700 | 42 | 2135 | 24391 | 61142,88 | 61 100 | 61 200 |
| 8 | 1938 | 24194 | 57738,72 | 57 700 | 57 800 | 43 | 2141 | 24397 | 61246,56 | 61 200 | 61 300 |
| 9 | 1944 | 24200 | 57842,4 | 57 800 | 57 900 | 44 | 2146 | 24402 | 61332,96 | 61 300 | 61 400 |
| 10 | 1950 | 24206 | 57946,08 | 57 900 | 58 000 | 45 | 2152 | 24408 | 61436,64 | 61 400 | 61 500 |
| 11 | 1955 | 24211 | 58032,48 | 58 000 | 58 100 | 46 | 2158 | 24414 | 61540,32 | 61 500 | 61 600 |
| 12 | 1961 | 24217 | 58136,16 | 58 100 | 58 200 | 47 | 2164 | 24420 | 61644 | 61 600 | 61 700 |
| 13 | 1967 | 24223 | 58239,84 | 58 200 | 58 300 | 48 | 2170 | 24426 | 61747,68 | 61 700 | 61 800 |
| 14 | 1973 | 24229 | 58343,52 | 58 300 | 58 400 | 49 | 2175 | 24431 | 61834,08 | 61 800 | 61 900 |
| 15 | 1978 | 24234 | 58429,92 | 58 400 | 58 500 | 50 | 2181 | 24437 | 61937,76 | 61 900 | 62 000 |
| 16 | 1984 | 24240 | 58533,6 | 58 500 | 58 600 | 51 | 2187 | 24443 | 62041,44 | 62 000 | 62 100 |
| 17 | 1990 | 24246 | 58637,28 | 58 600 | 58 700 | 52 | 2193 | 24449 | 62145,12 | 62 100 | 62 200 |
| 18 | 1996 | 24252 | 58740,96 | 58 700 | 58 800 | 53 | 2198 | 24454 | 62231,52 | 62 200 | 62 300 |
| 19 | 2002 | 24258 | 58844,64 | 58 800 | 58 900 | 54 | 2204 | 24460 | 62335,2 | 62 300 | 62 400 |
| 20 | 2007 | 24263 | 58931,04 | 58 900 | 59 000 | 55 | 2210 | 24466 | 62438,88 | 62 400 | 62 500 |
| 21 | 2013 | 24269 | 59034,72 | 59 000 | 59 100 | 56 | 2216 | 24472 | 62542,56 | 62 500 | 62 600 |
| 22 | 2019 | 24275 | 59138,4 | 59 100 | 59 200 | 57 | 2222 | 24478 | 62646,24 | 62 600 | 62 700 |
| 23 | 2025 | 24281 | 59242,08 | 59 200 | 59 300 | 58 | 2227 | 24483 | 62732,64 | 62 700 | 62 800 |
| 24 | 2031 | 24287 | 59345,76 | 59 300 | 59 400 | 59 | 2233 | 24489 | 62836,32 | 62 800 | 62 900 |
| 25 | 2036 | 24292 | 59432,16 | 59 400 | 59 500 | 60 | 2239 | 24495 | 62940 | 62 900 | 63 000 |
| 26 | 2042 | 24298 | 59535,84 | 59 500 | 59 600 | 61 | 2245 | 24501 | 63043,68 | 63 000 | 63 100 |
| 27 | 2048 | 24304 | 59639,52 | 59 600 | 59 700 | 62 | 2251 | 24507 | 63147,36 | 63 100 | 63 200 |
| 28 | 2054 | 24310 | 59743,2 | 59 700 | 59 800 | 63 | 2256 | 24512 | 63233,76 | 63 200 | 63 300 |
| 29 | 2060 | 24316 | 59846,88 | 59 800 | 59 900 | 64 | 2262 | 24518 | 63337,44 | 63 300 | 63 400 |
| 30 | 2065 | 24321 | 59933,28 | 59 900 | 60 000 | 65 | 2268 | 24524 | 63441,12 | 63 400 | 63 500 |
| 31 | 2071 | 24327 | 60036,96 | 60 000 | 60 100 | 66 | 2274 | 24530 | 63544,8 | 63 500 | 63 600 |
| 32 | 2077 | 24333 | 60140,64 | 60 100 | 60 200 | 67 | 2280 | 24536 | 63648,48 | 63 600 | 63 700 |
| 33 | 2083 | 24339 | 60244,32 | 60 200 | 60 300 | 68 | 2285 | 24541 | 63734,88 | 63 700 | 63 800 |
| 34 | 2088 | 24344 | 60330,72 | 60 300 | 60 400 | 69 | 2291 | 24547 | 63838,56 | 63 800 | 63 900 |
| 35 | 2094 | 24350 | 60434,4 | 60 400 | 60 500 | 70 | 2297 | 24553 | 63942,24 | 63 900 | 64 000 |

|  | N | GSCN | Fc | Low | High |  | N | GSCN | Fc | Low | High |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 71 | 2303 | 24559 | 64045,92 | 64 000 | 64 100 | 106 | 2505 | 24761 | 67536,48 | 67 500 | 67 600 |
| 72 | 2308 | 24564 | 64132,32 | 64 100 | 64 200 | 107 | 2511 | 24767 | 67640,16 | 67 600 | 67 700 |
| 73 | 2314 | 24570 | 64236 | 64 200 | 64 300 | 108 | 2517 | 24773 | 67743,84 | 67 700 | 67 800 |
| 74 | 2320 | 24576 | 64339,68 | 64 300 | 64 400 | 109 | 2523 | 24779 | 67847,52 | 67 800 | 67 900 |
| 75 | 2326 | 24582 | 64443,36 | 64 400 | 64 500 | 110 | 2528 | 24784 | 67933,92 | 67 900 | 68 000 |
| 76 | 2332 | 24588 | 64547,04 | 64 500 | 64 600 | 111 | 2534 | 24790 | 68037,6 | 68 000 | 68 100 |
| 77 | 2337 | 24593 | 64633,44 | 64 600 | 64 700 | 112 | 2540 | 24796 | 68141,28 | 68 100 | 68 200 |
| 78 | 2343 | 24599 | 64737,12 | 64 700 | 64 800 | 113 | 2546 | 24802 | 68244,96 | 68 200 | 68 300 |
| 79 | 2349 | 24605 | 64840,8 | 64 800 | 64 900 | 114 | 2552 | 24808 | 68348,64 | 68 300 | 68 400 |
| 80 | 2355 | 24611 | 64944,48 | 64 900 | 65 000 | 115 | 2557 | 24813 | 68435,04 | 68 400 | 68 500 |
| 81 | 2361 | 24617 | 65048,16 | 65 000 | 65 100 | 116 | 2563 | 24819 | 68538,72 | 68 500 | 68 600 |
| 82 | 2366 | 24622 | 65134,56 | 65 100 | 65 200 | 117 | 2569 | 24825 | 68642,4 | 68 600 | 68 700 |
| 83 | 2372 | 24628 | 65238,24 | 65 200 | 65 300 | 118 | 2575 | 24831 | 68746,08 | 68 700 | 68 800 |
| 84 | 2378 | 24634 | 65341,92 | 65 300 | 65 400 | 119 | 2581 | 24837 | 68849,76 | 68 800 | 68 900 |
| 85 | 2384 | 24640 | 65445,6 | 65 400 | 65 500 | 120 | 2586 | 24842 | 68936,16 | 68 900 | 69 000 |
| 86 | 2390 | 24646 | 65549,28 | 65 500 | 65 600 | 121 | 2592 | 24848 | 69039,84 | 69 000 | 69 100 |
| 87 | 2395 | 24651 | 65635,68 | 65 600 | 65 700 | 122 | 2598 | 24854 | 69143,52 | 69 100 | 69 200 |
| 88 | 2401 | 24657 | 65739,36 | 65 700 | 65 800 | 123 | 2604 | 24860 | 69247,2 | 69 200 | 69 300 |
| 89 | 2407 | 24663 | 65843,04 | 65 800 | 65 900 | 124 | 2610 | 24866 | 69350,88 | 69 300 | 69 400 |
| 90 | 2413 | 24669 | 65946,72 | 65 900 | 66 000 | 125 | 2615 | 24871 | 69437,28 | 69 400 | 69 500 |
| 91 | 2418 | 24674 | 66033,12 | 66 000 | 66 100 | 126 | 2621 | 24877 | 69540,96 | 69 500 | 69 600 |
| 92 | 2424 | 24680 | 66136,8 | 66 100 | 66 200 | 127 | 2627 | 24883 | 69644,64 | 69 600 | 69 700 |
| 93 | 2430 | 24686 | 66240,48 | 66 200 | 66 300 | 128 | 2633 | 24889 | 69748,32 | 69 700 | 69 800 |
| 94 | 2436 | 24692 | 66344,16 | 66 300 | 66 400 | 129 | 2638 | 24894 | 69834,72 | 69 800 | 69 900 |
| 95 | 2442 | 24698 | 66447,84 | 66 400 | 66 500 | 130 | 2644 | 24900 | 69938,4 | 69 900 | 70 000 |
| 96 | 2447 | 24703 | 66534,24 | 66 500 | 66 600 | 131 | 2650 | 24906 | 70042,08 | 70 000 | 70 100 |
| 97 | 2453 | 24709 | 66637,92 | 66 600 | 66 700 | 132 | 2656 | 24912 | 70145,76 | 70 100 | 70 200 |
| 98 | 2459 | 24715 | 66741,6 | 66 700 | 66 800 | 133 | 2662 | 24918 | 70249,44 | 70 200 | 70 300 |
| 99 | 2465 | 24721 | 66845,28 | 66 800 | 66 900 | 134 | 2667 | 24923 | 70335,84 | 70 300 | 70 400 |
| 100 | 2471 | 24727 | 66948,96 | 66 900 | 67 000 | 135 | 2673 | 24929 | 70439,52 | 70 400 | 70 500 |
| 101 | 2476 | 24732 | 67035,36 | 67 000 | 67 100 | 136 | 2679 | 24935 | 70543,2 | 70 500 | 70 600 |
| 102 | 2482 | 24738 | 67139,04 | 67 100 | 67 200 | 137 | 2685 | 24941 | 70646,88 | 70 600 | 70 700 |
| 103 | 2488 | 24744 | 67242,72 | 67 200 | 67 300 | 138 | 2691 | 24947 | 70750,56 | 70 700 | 70 800 |
| 104 | 2494 | 24750 | 67346,4 | 67 300 | 67 400 | 139 | 2696 | 24952 | 70836,96 | 70 800 | 70 900 |
| 105 | 2500 | 24756 | 67450,08 | 67 400 | 67 500 | 140 | 2702 | 24958 | 70940,64 | 70 900 | 71 000 |

480k SCS

|  |  |  |  | 400MHz |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | N | GSCN | Fc | Low | High |  | N | GSCN | Fc | Low | High |
| 1 | 1903 | 24159 | 57133,92 | 57 000 | 57 400 | 37 | 2320 | 24576 | 64339,68 | 64 200 | 64 600 |
| 2 | 1909 | 24165 | 57237,6 | 57 000 | 57 400 | 38 | 2326 | 24582 | 64443,36 | 64 200 | 64 600 |
| 3 | 1926 | 24182 | 57531,36 | 57 400 | 57 800 | 39 | 2343 | 24599 | 64737,12 | 64 600 | 65 000 |
| 4 | 1932 | 24188 | 57635,04 | 57 400 | 57 800 | 40 | 2349 | 24605 | 64840,8 | 64 600 | 65 000 |
| 5 | 1950 | 24206 | 57946,08 | 57 800 | 58 200 | 41 | 2366 | 24622 | 65134,56 | 65 000 | 65 400 |
| 6 | 1955 | 24211 | 58032,48 | 57 800 | 58 200 | 42 | 2372 | 24628 | 65238,24 | 65 000 | 65 400 |
| 7 | 1973 | 24229 | 58343,52 | 58 200 | 58 600 | 43 | 2390 | 24646 | 65549,28 | 65 400 | 65 800 |
| 8 | 1978 | 24234 | 58429,92 | 58 200 | 58 600 | 44 | 2395 | 24651 | 65635,68 | 65 400 | 65 800 |
| 9 | 1996 | 24252 | 58740,96 | 58 600 | 59 000 | 45 | 2413 | 24669 | 65946,72 | 65 800 | 66 200 |
| 10 | 2002 | 24258 | 58844,64 | 58 600 | 59 000 | 46 | 2418 | 24674 | 66033,12 | 65 800 | 66 200 |
| 11 | 2019 | 24275 | 59138,4 | 59 000 | 59 400 | 47 | 2436 | 24692 | 66344,16 | 66 200 | 66 600 |
| 12 | 2025 | 24281 | 59242,08 | 59 000 | 59 400 | 48 | 2442 | 24698 | 66447,84 | 66 200 | 66 600 |
| 13 | 2042 | 24298 | 59535,84 | 59 400 | 59 800 | 49 | 2459 | 24715 | 66741,6 | 66 600 | 67 000 |
| 14 | 2048 | 24304 | 59639,52 | 59 400 | 59 800 | 50 | 2465 | 24721 | 66845,28 | 66 600 | 67 000 |
| 15 | 2065 | 24321 | 59933,28 | 59 800 | 60 200 | 51 | 2482 | 24738 | 67139,04 | 67 000 | 67 400 |
| 16 | 2071 | 24327 | 60036,96 | 59 800 | 60 200 | 52 | 2488 | 24744 | 67242,72 | 67 000 | 67 400 |
| 17 | 2088 | 24344 | 60330,72 | 60 200 | 60 600 | 53 | 2505 | 24761 | 67536,48 | 67 400 | 67 800 |
| 18 | 2094 | 24350 | 60434,4 | 60 200 | 60 600 | 54 | 2511 | 24767 | 67640,16 | 67 400 | 67 800 |
| 19 | 2112 | 24368 | 60745,44 | 60 600 | 61 000 | 55 | 2528 | 24784 | 67933,92 | 67 800 | 68 200 |
| 20 | 2117 | 24373 | 60831,84 | 60 600 | 61 000 | 56 | 2534 | 24790 | 68037,6 | 67 800 | 68 200 |
| 21 | 2135 | 24391 | 61142,88 | 61 000 | 61 400 | 57 | 2552 | 24808 | 68348,64 | 68 200 | 68 600 |
| 22 | 2141 | 24397 | 61246,56 | 61 000 | 61 400 | 58 | 2557 | 24813 | 68435,04 | 68 200 | 68 600 |
| 23 | 2158 | 24414 | 61540,32 | 61 400 | 61 800 | 59 | 2575 | 24831 | 68746,08 | 68 600 | 69 000 |
| 24 | 2164 | 24420 | 61644 | 61 400 | 61 800 | 60 | 2581 | 24837 | 68849,76 | 68 600 | 69 000 |
| 25 | 2181 | 24437 | 61937,76 | 61 800 | 62 200 | 61 | 2598 | 24854 | 69143,52 | 69 000 | 69 400 |
| 26 | 2187 | 24443 | 62041,44 | 61 800 | 62 200 | 62 | 2604 | 24860 | 69247,2 | 69 000 | 69 400 |
| 27 | 2204 | 24460 | 62335,2 | 62 200 | 62 600 | 63 | 2621 | 24877 | 69540,96 | 69 400 | 69 800 |
| 28 | 2210 | 24466 | 62438,88 | 62 200 | 62 600 | 64 | 2627 | 24883 | 69644,64 | 69 400 | 69 800 |
| 29 | 2227 | 24483 | 62732,64 | 62 600 | 63 000 | 65 | 2644 | 24900 | 69938,4 | 69 800 | 70 200 |
| 30 | 2233 | 24489 | 62836,32 | 62 600 | 63 000 | 66 | 2650 | 24906 | 70042,08 | 69 800 | 70 200 |
| 31 | 2251 | 24507 | 63147,36 | 63 000 | 63 400 | 67 | 2667 | 24923 | 70335,84 | 70 200 | 70 600 |
| 32 | 2256 | 24512 | 63233,76 | 63 000 | 63 400 | 68 | 2673 | 24929 | 70439,52 | 70 200 | 70 600 |
| 33 | 2274 | 24530 | 63544,8 | 63 400 | 63 800 | 69 | 2691 | 24947 | 70750,56 | 70 600 | 71 000 |
| 34 | 2280 | 24536 | 63648,48 | 63 400 | 63 800 | 70 | 2696 | 24952 | 70836,96 | 70 600 | 71 000 |
| 35 | 2297 | 24553 | 63942,24 | 63 800 | 64 200 |  |  |  |  |  |  |
| 36 | 2303 | 24559 | 64045,92 | 63 800 | 64 200 |  |  |  |  |  |  |