3GPP TSG-RAN WG4 Meeting #102-e													R4-2205313

Electronic meeting, February 21 – March 3, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	System parameters for a NR band in the range 52.6GHz – 71GHz

Agenda item:	10.16.2

Document for:	Approval

# Introduction

Building on top of the earlier agreements, this contribution discusses open issues related to systems parameters for a NR band in the range 52.6GHz – 71GHz.

# Discussion

#### Synchronization raster and channel raster

The current WID for extending current NR operation to 71 GHz in [2] states the following on synchronization raster:

  - In addition to 120kHz, support 480 kHz SSB for initial access with support of CORESET#0/Type0-PDCCH configuration in the MIB with following constraints:

    - Limited sync raster entry numbers

      - It is assumed that RAN4 supports a channelization design which results in the total number of synchronization raster entries considering both licensed and unlicensed operation in a 52.6 – 71 GHz band no larger than 665 (Note: the total number of synchronization raster entries in FR2 for band n259 + n257 is 599). If the assumption cannot be satisfied, it’s up to RAN4 to decide its applicability to bands in 52.6 – 71 GHz.

In previous meeting RAN4 agreed to define fixed synchronization raster for unlicensed operating band n263 and floating synchronization raster for licensed operation. The discussion in this contribution concentrates mainly on the fixed raster design, which is done with the aim of maximizing the re-use of raster points between unlicensed and licensed network operation, i.e. fixed and floating raster.

While sync raster has agreed to be fixed, no limitations for the actual RF channel raster were agreed in RAN4. However, to derive the synchronization raster points in an optimal manner, we have considered fixed RF channel raster positions. For the RF channel raster following guiding principles were used.

- Select lowest possible ARFCN point for the first RF channel in the frequency range 57-71 GHz for each SCS and bandwidth pair

- {120 kHz, 100 MHz}, {120 kHz, 400 MHz}

- {480 kHz, 400 MHz}, {480 kHz, 800 MHz}, {480 kHz, 1600 MHz}

- {960 kHz, 400 MHz}, {960 kHz, 800 MHz}, {960 kHz, 1600 MHz}, {960 kHz, 2000 MHz}

- Minimize distance between adjacent RF channels by having

- Distance between center of the channels being integer multiple of 960 kHz

- Guard bands of different RF channels are not overlapping

- For 100MHz channel bandwidth the channel raster step is minimized

- For 400MHz, 800MHz, 1600MHz and 2GHz channel bandwidths the channel raster step sizes are aligned to enable smooth CA operation and minimize the gap between channels

      - Note: Only the cell-defining SSB of the PCell is always on the sync raster.

Figure 1 Principle of channel allocation in fixed channelization.

Following these principles result in RF raster displayed in Table 1.

Table 1: Center frequency of 1st channel in 57-71 GHz band and channel raster step size

| SCS [kHz] | Bandwidth [MHz] | 1st reference freq. [Hz] | Channel raster step [Hz] |
| --- | --- | --- | --- |
| 120 | 100 | 57050040000 | 100800000 |
| 120 | 400 | 57200040000 | 400320000 |
| 480 | 400 | 57200040000 | 400320000 |
| 480 | 800 | 57400020000 | 800640000 |
| 480 | 1600 | 57800040000 | 1601280000 |
| 960 | 400 | 57200040000 | 400320000 |
| 960 | 800 | 57400020000 | 800640000 |
| 960 | 1600 | 57800040000 | 1601280000 |
| 960 | 2000 | 58000020000 | 2001600000 |

When considering the RF raster points provided in the Table 1 it can be seen that defining the RF channel raster according these would not be fully optimal. For example, the spacing between 400 MHz channels is not a multiple of spacing of 100 MHz channels. This results in channel positions sliding relative to each other and making contiguous carrier aggregation between 100 and 400 MHz channels challenging if not impossible.

Secondly, if channel positions are set based on the lower frequency edge of the operating band, the highest feasible channel position may be far away from the upper edge of the operating band, resulting in a lot of spectrum not being available. For 2 GHz channel bandwidth this will mean almost 2 GHz being lost. Therefore, we think that the ‘fixed’ channel raster should be only considered in determination of reduced synchronization raster design (and RB offsets required in RAN1 specification) for the ‘fixed raster’, while leaving more flexibility to the actual RF channel raster. Overall, it is extremely challenging to define a fixed RF channel raster which would work for carrier aggregation as well as provide high spectrum efficiency.

When synchronization raster points are selected based on this RF raster design we get the points shown in table 2.

Table 2: GSCNs for fixed channelization.

| SCS | 120 kHz |  |  | 480 kHz |  |  |  | 960 kHz |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth | 100 MHz | 400 MHz |  | 400 MHz | 800 MHz | 1600 MHz |  | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
|  | 24153 | 24153 |  | 24157 | 24157 | 24157 |  | 24160 | 24160 | 24166 | 24166 |
|  | 24159 |  |  | 24181 |  |  |  | 24184 |  |  |  |
|  | 24165 |  |  | 24205 | 24205 |  |  | 24208 | 24208 |  |  |
|  | 24171 |  |  | 24229 |  |  |  | 24232 |  |  |  |
|  | 24177 | 24177 |  | 24253 | 24253 | 24253 |  | 24256 | 24256 | 24256 |  |
|  | 24183 |  |  | 24277 |  |  |  | 24280 |  |  | 24280 |
|  | 24189 |  |  | 24301 | 24301 |  |  | 24298 | 24304 |  |  |
|  | 24195 |  |  | 24325 |  |  |  | 24322 |  |  |  |
|  | 24201 | 24201 |  | 24349 | 24349 | 24349 |  | 24346 | 24346 | 24346 |  |
|  | 24207 |  |  | 24373 |  |  |  | 24370 |  |  |  |
|  | 24213 |  |  | 24397 | 24397 |  |  | 24394 | 24394 |  | 24394 |
|  | 24219 |  |  | 24421 |  |  |  | 24418 |  |  |  |
|  | 24225 | 24225 |  | 24445 | 24445 | 24445 |  | 24442 | 24442 | 24442 |  |
|  | 24231 |  |  | 24457 |  |  |  | 24460 |  |  |  |
|  | 24234 |  |  | 24481 | 24481 |  |  | 24484 | 24484 |  |  |
|  | 24240 |  |  | 24505 |  |  |  | 24508 |  |  | 24514 |
|  | 24246 | 24246 |  | 24529 | 24529 | 24529 |  | 24532 | 24532 | 24532 |  |
|  | 24252 |  |  | 24553 |  |  |  | 24556 |  |  |  |
|  | 24258 |  |  | 24577 | 24577 |  |  | 24580 | 24580 |  |  |
|  | 24264 |  |  | 24601 |  |  |  | 24604 |  |  |  |
|  | 24270 | 24270 |  | 24625 | 24625 | 24625 |  | 24622 | 24628 | 24628 | 24628 |
|  | 24276 |  |  | 24649 |  |  |  | 24646 |  |  |  |
|  | 24282 |  |  | 24673 | 24673 |  |  | 24670 | 24670 |  |  |
|  | 24288 |  |  | 24697 |  |  |  | 24694 |  |  |  |
|  | 24294 | 24294 |  | 24721 | 24721 | 24721 |  | 24718 | 24718 | 24718 |  |
|  | 24300 |  |  | 24745 |  |  |  | 24742 |  |  | 24742 |
|  | 24306 |  |  | 24769 | 24769 |  |  | 24766 | 24766 |  |  |
|  | 24312 | 24315 |  | 24781 |  |  |  | 24784 |  |  |  |
|  | 24318 |  |  | 24805 | 24805 | 24817 |  | 24808 | 24808 | 24814 |  |
|  | 24324 |  |  | 24829 |  |  |  | 24832 |  |  |  |
|  | 24330 |  |  | 24853 | 24853 |  |  | 24856 | 24856 |  |  |
|  | 24336 |  |  | 24877 |  |  |  | 24880 |  |  |  |
|  | 24339 | 24339 |  | 24901 | 24901 |  |  | 24904 | 24904 |  |  |
|  | 24345 |  |  | 24925 |  |  |  | 24928 |  |  |  |
|  | 24351 |  |  |  |  |  |  |  |  |  |  |
|  | 24357 |  |  |  |  |  |  |  |  |  |  |
|  | 24363 | 24363 |  |  |  |  |  |  |  |  |  |
|  | 24369 |  |  |  |  |  |  |  |  |  |  |
|  | 24375 |  |  |  |  |  |  |  |  |  |  |
|  | 24381 |  |  |  |  |  |  |  |  |  |  |
|  | 24387 | 24387 |  |  |  |  |  |  |  |  |  |
|  | 24393 |  |  |  |  |  |  |  |  |  |  |
|  | 24399 |  |  |  |  |  |  |  |  |  |  |
|  | 24405 |  |  |  |  |  |  |  |  |  |  |
|  | 24411 | 24408 |  |  |  |  |  |  |  |  |  |
|  | 24417 |  |  |  |  |  |  |  |  |  |  |
|  | 24423 |  |  |  |  |  |  |  |  |  |  |
|  | 24429 |  |  |  |  |  |  |  |  |  |  |
|  | 24435 | 24432 |  |  |  |  |  |  |  |  |  |
|  | 24441 |  |  |  |  |  |  |  |  |  |  |
|  | 24444 |  |  |  |  |  |  |  |  |  |  |
|  | 24450 |  |  |  |  |  |  |  |  |  |  |
|  | 24456 | 24456 |  |  |  |  |  |  |  |  |  |
|  | 24462 |  |  |  |  |  |  |  |  |  |  |
|  | 24468 |  |  |  |  |  |  |  |  |  |  |
|  | 24474 |  |  |  |  |  |  |  |  |  |  |
|  | 24480 | 24477 |  |  |  |  |  |  |  |  |  |
|  | 24486 |  |  |  |  |  |  |  |  |  |  |
|  | 24492 |  |  |  |  |  |  |  |  |  |  |
|  | 24498 |  |  |  |  |  |  |  |  |  |  |
|  | 24504 | 24501 |  |  |  |  |  |  |  |  |  |
|  | 24510 |  |  |  |  |  |  |  |  |  |  |
|  | 24516 |  |  |  |  |  |  |  |  |  |  |
|  | 24522 |  |  |  |  |  |  |  |  |  |  |
|  | 24528 | 24525 |  |  |  |  |  |  |  |  |  |
|  | 24534 |  |  |  |  |  |  |  |  |  |  |
|  | 24540 |  |  |  |  |  |  |  |  |  |  |
|  | 24546 |  |  |  |  |  |  |  |  |  |  |
|  | 24549 | 24549 |  |  |  |  |  |  |  |  |  |
|  | 24555 |  |  |  |  |  |  |  |  |  |  |
|  | 24561 |  |  |  |  |  |  |  |  |  |  |
|  | 24567 |  |  |  |  |  |  |  |  |  |  |
|  | 24573 | 24570 |  |  |  |  |  |  |  |  |  |
|  | 24579 |  |  |  |  |  |  |  |  |  |  |
|  | 24585 |  |  |  |  |  |  |  |  |  |  |
|  | 24591 |  |  |  |  |  |  |  |  |  |  |
|  | 24597 | 24594 |  |  |  |  |  |  |  |  |  |
|  | 24603 |  |  |  |  |  |  |  |  |  |  |
|  | 24609 |  |  |  |  |  |  |  |  |  |  |
|  | 24615 |  |  |  |  |  |  |  |  |  |  |
|  | 24621 | 24618 |  |  |  |  |  |  |  |  |  |
|  | 24627 |  |  |  |  |  |  |  |  |  |  |
|  | 24633 |  |  |  |  |  |  |  |  |  |  |
|  | 24639 |  |  |  |  |  |  |  |  |  |  |
|  | 24645 | 24642 |  |  |  |  |  |  |  |  |  |
|  | 24651 |  |  |  |  |  |  |  |  |  |  |
|  | 24654 |  |  |  |  |  |  |  |  |  |  |
|  | 24660 |  |  |  |  |  |  |  |  |  |  |
|  | 24666 | 24663 |  |  |  |  |  |  |  |  |  |
|  | 24672 |  |  |  |  |  |  |  |  |  |  |
|  | 24678 |  |  |  |  |  |  |  |  |  |  |
|  | 24684 |  |  |  |  |  |  |  |  |  |  |
|  | 24690 | 24687 |  |  |  |  |  |  |  |  |  |
|  | 24696 |  |  |  |  |  |  |  |  |  |  |
|  | 24702 |  |  |  |  |  |  |  |  |  |  |
|  | 24708 |  |  |  |  |  |  |  |  |  |  |
|  | 24714 | 24711 |  |  |  |  |  |  |  |  |  |
|  | 24720 |  |  |  |  |  |  |  |  |  |  |
|  | 24726 |  |  |  |  |  |  |  |  |  |  |
|  | 24732 | 24732 |  |  |  |  |  |  |  |  |  |
|  | 24738 |  |  |  |  |  |  |  |  |  |  |
|  | 24744 |  |  |  |  |  |  |  |  |  |  |
|  | 24750 |  |  |  |  |  |  |  |  |  |  |
|  | 24756 | 24756 |  |  |  |  |  |  |  |  |  |
|  | 24759 |  |  |  |  |  |  |  |  |  |  |
|  | 24765 |  |  |  |  |  |  |  |  |  |  |
|  | 24771 |  |  |  |  |  |  |  |  |  |  |
|  | 24777 | 24780 |  |  |  |  |  |  |  |  |  |
|  | 24783 |  |  |  |  |  |  |  |  |  |  |
|  | 24789 |  |  |  |  |  |  |  |  |  |  |
|  | 24795 |  |  |  |  |  |  |  |  |  |  |
|  | 24801 | 24804 |  |  |  |  |  |  |  |  |  |
|  | 24807 |  |  |  |  |  |  |  |  |  |  |
|  | 24813 |  |  |  |  |  |  |  |  |  |  |
|  | 24819 |  |  |  |  |  |  |  |  |  |  |
|  | 24825 | 24825 |  |  |  |  |  |  |  |  |  |
|  | 24831 |  |  |  |  |  |  |  |  |  |  |
|  | 24837 |  |  |  |  |  |  |  |  |  |  |
|  | 24843 |  |  |  |  |  |  |  |  |  |  |
|  | 24849 | 28489 |  |  |  |  |  |  |  |  |  |
|  | 24855 |  |  |  |  |  |  |  |  |  |  |
|  | 24861 |  |  |  |  |  |  |  |  |  |  |
|  | 24864 |  |  |  |  |  |  |  |  |  |  |
|  | 24870 | 24873 |  |  |  |  |  |  |  |  |  |
|  | 24876 |  |  |  |  |  |  |  |  |  |  |
|  | 24882 |  |  |  |  |  |  |  |  |  |  |
|  | 24888 |  |  |  |  |  |  |  |  |  |  |
|  | 24894 | 24894 |  |  |  |  |  |  |  |  |  |
|  | 24900 |  |  |  |  |  |  |  |  |  |  |
|  | 24906 |  |  |  |  |  |  |  |  |  |  |
|  | 24912 |  |  |  |  |  |  |  |  |  |  |
|  | 24918 | 24918 |  |  |  |  |  |  |  |  |  |
|  | 24924 |  |  |  |  |  |  |  |  |  |  |
|  | 24930 |  |  |  |  |  |  |  |  |  |  |
|  | 24936 |  |  |  |  |  |  |  |  |  |  |
|  | 24942 |  |  |  |  |  |  |  |  |  |  |
|  | 24948 |  |  |  |  |  |  |  |  |  |  |
|  | 24954 |  |  |  |  |  |  |  |  |  |  |

It has been confirmed that the coreset#0 offsets required for RAN1 specification are feasible with these synchronization raster points. The details can be found in [5].

Proposal 1: Adopt synchronization raster points shown in Table 2 for n263

Proposal 2: RF channel raster is floating based on 120 kHz SCS, network implementation will take care that SSB and coreset#0 fit within channel bandwidth

#### Spectrum utilization

To enable smooth implementation transfer from current FR2 to above 52.6 GHz, it is reasonable to minimize changes to 120 kHz design and apply the same spectrum utilization for 120 kHz SCS in FR2-2 as already agreed and implemented for FR2-1. In simulations submitted to RAN4 in study item phase [3] it was found that EVM and in-band emissions are typically the gating factor for output power and reducing spectrum utilization will not result in higher output power. As a result, RAN4#101bis-e endorsed the spectrum utilization for 120 kHz SCS in [4].

Same behavior was observed also for higher SCS, but given these will be new implementations, it is reasonable to consider other options for SU. FFT utilization is one issue that needs to be considered when defining the spectrum utilization for 480 kHz and 960 kHz SCS. The reason is that it is one factor defining the implementation complexity.

Based on that we think that for 800 MHz and 1600 MHz cases, the starting point would be to aim at similar spectrum utilization as 120 kHz SCS in FR2.

However, when considering 2 GHz CBW, the specification should facilitate smooth implementation based on 2k FFT. For example, 156 PRBs, which provides 90.0% spectrum utilization, means that the FFT utilization would be as high as 91.4%.  This would be too much e.g. from the filtering complexity point of view.

Hence, in order to facilitate smooth implementation also for 2k FFT, we propose to support reduced spectrum utilization for 2GHz CBW when using 960 kHz SCS.

Proposal 3: Consider similar spectrum utilization for scenarios with 800MHz and 1600MHz as already endorsed for 120 kHz SCS in FR2-2.

Proposal 4: Support reduced spectrum utilization for 960 kHz SCS & 2 GHz CBW

#### UE features

Discussion on UE features was kicked off in RAN4#101bis-e, with three different UE features raised for discussion: Support for UL 64QAM, optionality of channel bandwidth support, and ON-ON transient time. The discussion for 64QAM was concluded in previous meeting and therefore we concentrate here on channel bandwidths and ON-ON transient.

When it comes to channel bandwidth support, it is important to take into account that each subcarrier spacing is already optional: if this frequency range is not support at all, there is no need to support 120 kHz SCS, and 480 and 960 kHz SCS are listed to be optional already in the WID. The major added value of wider SCS is the ability to support wider channel bandwidths. Therefore, given the optionality of the subcarrier spacing, setting wider channel bandwidths of 480 and 960 kHz SCS would devalue the higher SCS greatly. If support of a single wide channel bandwidth is not desirable for UE manufacturer, it is still possible to support large amount of spectrum via carrier aggregation and 120 kHz SCS. As a conclusion, supporting all channel bandwidth for each SCS should be mandatory.

Proposal 5: As each SCS is optional to support, further optionality on maximum channel bandwidth support is not required.

For ON-ON transient time RAN4#101bis-e agreed to have 5 us minimum requirement, and continue to discuss possibility of having optional 1, 2 and/or 3 us ON-ON transient time. From network perspective it is greatly beneficial to manage UEs having the same behavior compared to different UE behavior/performance. On the other hand, improved UE performance is definitely desired. The optimal compromise between these somewhat competing points-of-view is to formalize the improved UE performance while simultaneously limiting the number of options. Therefore, we propose to specify only a single optional transient time value, and select between 1 us or 2us.

Proposal 6: For optional ON-ON transient time, only one value among 1 us or 2 us is specified.

# 3 Conclusion

In this contribution channel arrangement and spectrum utilization for NR operation above 52.6 GHz was discussed. The following proposals were made:

Proposal 1: Adopt synchronization raster points shown in Table 2 for n263

Proposal 2: RF channel raster is floating based on 120 kHz SCS, network implementation will take care that SSB and coreset#0 fit within channel bandwidth

Proposal 3: Consider similar spectrum utilization for scenarios with 800MHz and 1600MHz as already endorsed for 120 kHz SCS in FR2-2.

Proposal 4: Support reduced spectrum utilization for 960 kHz SCS & 2 GHz CBW

Proposal 5: As each SCS is optional to support, further optionality on maximum channel bandwidth support is not required.

Proposal 6: For optional ON-ON transient time, only one value among 1 us or 2 us is specified.

# References

[1] R4-2120061, “WF on NR extension to 71 GHz (Part 1) – System Parameters” Intel Corporation

[2] RP-213540, “Revised WID: Extending current NR operation to 71 GHz”, Qualcomm, Intel

[3] R4-2015444, “UE RF for NR beyond 52.6 GHz”, Nokia, Nokia Shanghai Bell

[4] R4-2202364, “Draft CR for TS 38.101-2: Introduction of system parameters for FR2-2”, Vivo

[5] R1-2201662, “Initial access aspects”, Nokia, Nokia Shanghai Bell