3GPP TSG-RAN WG4 Meeting # 102-e	R4-2205436

Electronic Meeting, February 21 – March 3, 2022

Source:	ZTE Corporation

Title:	On RAN2 LS on RRM relaxation for Rel-16 power saving

Agenda item:	13.2.6

Document for:	Approval

# 1	Introduction

In RAN4 102, there is an incoming RAN2 LS [2].

RAN2 would like to thank RAN4 for the LS on RRM relaxation in power saving. RAN2 has discussed the inconsistency raised in the LS between RAN2 and RAN4 specifications and concluded the following: 1) RAN2 agreed to follow the request from RAN4 for the change to 38.304 on RRM relaxation on inter-frequency layers/E-UTRA inter-RAT frequency layers of higher priority for the case when the lowMobilityEvalutation criterion is fulfilled and when highPriorityMeasRelax is configured, referring to clauses 4.2.2.10.2 and 4.2.2.11.2 in 38.133. 2) RAN2 also noted that in RAN4 specifications, for the case when both lowMobilityEvalutation and cellEdgeEvaluation criteria are fulfilled, referring to clauses 4.2.2.10.4 and 4.2.2.11.4 in 38.133, the UE shall search for inter-frequency layers/E-UTRA inter-RAT frequency layers at least every hour (for example, below extract for inter-frequency layers): ##### 4.2.2.10.4 Measurements for UE fulfilling low mobility and not-at-cell edge criterion This clause contains requirements for measurements on inter-frequency NR cells provided that: - T331 timer is not running for EMR measurements on inter-frequency NR carrier, and - UE is configured with both lowMobilityEvalutation [2] criterion and cellEdgeEvaluation [2] criterion, and - Has also fulfilled both criteria, and - less than 1 hour have passed since measurements for cell reselection were last performed In this case the UE is not required to meet Tdetect,NR_Inter, Tmeasure,NR_Inter and Tevaluate,NR_Inter as defined in Table 4.2.2.4-1.. Given the above includes the case when Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ this would result, for frequency layers of higher priority, in a less relaxed measurement requirement (1 hour measurement interval) compared with the case when only the lowMobilityEvalutation criterion is fulfilled (1 hour * Nlayers measurement interval), RAN2 kindly asks RAN4 whether this change (1 hour -> 1 hour * Nlayers) should also apply to the case when both lowMobilityEvalutation and cellEdgeEvaluation criteria are fulfilled, for consistency, or if RAN2 misinterpreted RAN4 specifications.

- The LS seeks RAN4 feedback if any on R16 power saving criterion.

- This paper discusses this issue.

2	Discussions

In our opinion, RAN2 made the correct observation on a previous RAN4 mistake in the spec 38.133. The highlighted content in yellow as shown in RAN2 LS is somehow contradictory to the case when both criteria are met, and it is only correct to have a more relaxed requirement when both criteria are met, not a stricter one.

Thus, we propose to adopt RAN2 observation and correct the text, without sending any LS back to RAN2.

Adopt RAN2 observation and correct the text, without sending any LS back to RAN2.

3	Conclusion

- Proposal 1: Adopt RAN2 observation and correct the text, without sending any LS back to RAN2.

References

- R2-2108877