3GPP TSG-RAN WG4 Meeting#102-e                              R4-2205466

E-meeting,21th Feb – 3rd Mar,2022

Agenda item:	10.5.3.3

Source: 	ZTE Corporation

Title: 	Further discussions on other requirements of radiated repeater

Document for:	Approval

Introduction

In the past RAN4 meetings, there were lots of progress made in last RAN4 meeting, however there are still lots of remaining issues. Therefore in this contribution, we want to share some further considerations on those remaining open issues.

Discussion

- 2.3. Input IMD requirements

The BW of the interfering signals is still open issue (currently opinion is split) between the 2 options: - Option 1: modulated signal to 50MHz; - Option 2: Use 2 CW signals

In the last RAN4 meeting, there were no further progress on the signal type of input IMD requirement. From our understanding, to generate the CW signal with lager output power, this would be a bit challenging and risky in the OTA chamber, for the simplicity of testing setup in the chamber, we are supportive to go with option 1 with one CW signal and another modulation signal up to 50MHz similar as FR2-1 BS Rx IMD requirements.

Proposal 1: to support option 1 with modulated signal to 50MHz;

- 2.4. Out of band gain requirements

In last RAN4 meeting, there are some discussion on out of band requirements, there are several options left for further discussion, therefore in the following section, we want to share some initial understandings on this issue.

These include: - In band gain assumption (FR1 assumes 90dB) - Antennas gain (e.g. BS assumes 10 to 32dBi) - BS side - UE side - BS to repeater MCL/PL including both: - Serving BS to repeater - Other (victim) BS to repeater - UE to repeater MCL/PL including both: - Served UE to repeater - Other (victim) UE(s) to repeater

Based on the initial simulation results as shown in Figure 1/2 with its simulation assumptions in TR38.803, if repeater with maximum in-band gain as 80dB, UE at the cell edge with the help of coverage extension with FR2 repeater, it can have the same experience as UE with best side conditions as cell center UE.

As shown in Figure 1, for urban macro scenario, the coupling loss for cell edge UEs could be improved up to 70dBc; Therefore if NR based repeater with in-band gain as 80dBc, it should be sufficient enough to ensure normal UE coverage by the repeater deployment.

In addition, assuming FR2 AAS BS with 100MHz and 30dBm TRP, then in-band PSD should be 10dBm/MHz and the emission in the 0.1*BWcontiguous should be -5dBm/MHz, therefore the effective ACLR within BWcontiguous should be 15dBc. If out of band gain within 0.1*BWcontiguous is 65dBc similar as FR1, then the interference signal amplified by repeater should be similar or lower than unwanted emissions.

Unwanted emission generated by itself= wanted signal+80dBi gain- 15dBc at 0.1*BWcontiguous;

interference amplified by repeater= interference signal+65dBi out of band gain at 0.1*BWcontiguous;

Where:

- The interference signal power level is the same as wanted signal power level;

Figure 1. Coupling loss for AAS NR Urban Macro at 30GHz [pathloss+BS antenna gain+UE antenna gain]

Figure 2. Coupling loss for AAS NR Indoor at 30GHz [pathloss+BS antenna gain+UE antenna gain]

Based on the above considerations, we think that in-band gain for FR2 could be 80dBc at most. For out of band gain requirement, it could be further discussed.

Proposal 2: in-band gain for FR2 could be 80dBc at most;

In addition, based on the comparison between LTE based repeater Cat B option 2 OBUE requirements and Out of band gain limit, it could be found that there are also some similarity between them, therefore gap between adjacent limits are close. For example, for OBUE requirement, at freq offset from carrier edge 0.2MHz, its PSD in 30kH is -14dBm with the assumption of 46dBm output power, the relative ACLR is around 31dBc (17dBm-(-14dBm)); for OOBB gain, at freq offset from carrier edge 0.2MHz, its OOBB gain is 60dBc with the assumption of 90dBc maximum in-band gain, then relative OOBB gain offset is 30dBc (90dBc-60dBc) which is close to effective ACLR 31dBc.

Similar for other frequency offset, at freq offset from carrier edge 10MHz, its PSD in 30kHz is -28.2dBm, its relative ACLR is around 45.2dBc (17dBm-(-28.2dBm)); for OOBB gain, at freq offset from carrier edge 10MHz, its OOBB gain is 45dBc with the assumption of 90dBc maximum in-band gain, then relative OOBB gain offset is 45dBc (90dBc-45dBc) which is close to effective ACLR 45.2dBc.

| Frequency offset of measurement filter -3dB point, f | PSD at the freq edge in dBm | Out of band gain limit in dB | Measurement bandwidth |
| --- | --- | --- | --- |
| In-band signal | 17 | 90 | 30 kHz |
| 0 MHz f < 0.2 MHz | -14 |  | 30 kHz |
| 0.2 MHz f < 1 MHz | -26 | 60 | 30 kHz |
| (Note 4) | -26 | 45 | 30 kHz |
| 1 MHz f min( 10 MHz, fmax) | -28.2 | 45 | 30 kHz |
| 10 MHz f fmax | -30.2 | 35 | 30 kHz |

To the follow the same approach as LTE based repeater or FR1 NR based repeater as analysis before to keep the similar offset between OBUE and OOBB limit at the different frequency offset.

Table 9.7.4.3.2-1: OBUE limits applicable

| Frequency offset of measurement filter -3B point, f | PSD at the freq edge in dBm | Limit | Measurement bandwidth |
| --- | --- | --- | --- |
| In-band signal | 10 | 80 |  |
| 0 MHz f < 0.1*BWcontiguous | -5 |  | 1 MHz |
| 0.1*BWcontiguous f < fB | -13 | 65 | 1 MHz |
| fB f < fmax | -15 | 57 | 1 MHz |
| NOTE 1: fB = 2*BWcontiguous when BWcontiguous ≤ 500 MHz, otherwise fB = BWcontiguous + 500 MHz. |  |  |  |

Based on the above considerations, the OOBB limit for FR2 is proposed as following:

Proposal 3:  to specify the OOBB limit for FR2 as following:

Table 1: Out of band gain limits

| Frequency offset, f_offset_CW | Maximum gain |
| --- | --- |
| 0.1*BWcontiguous f < fB | 65 dB |
| fB f < fmax | 57dB |
| NOTE 1: fB = 2*BWcontiguous when BWcontiguous ≤ 500 MHz, otherwise fB = BWcontiguous + 500 MHz. |  |

For fmax  f_offset_CW the out of band gain shall not exceed the maximum gain of table 2 or the maximum gain stated in table 1 whichever is lower.

Table 2: Out of band gain limits 2

| Frequency offset, f_offset_CW | Maximum gain |
| --- | --- |
| fmax f_offset_CW | Out of band gain minimum donor coupling loss |

- 2.5. ACRR

In last RAN4 meeting, it was agree to set the ACRR equal to ACLR requirement, indeed from ACRR requirement is tightly related with OOB gain requirement, as analyzed and proposed before , OOB gain is aligned with ACLR requirement, therefore we think that it should be okay to follow ACLR requirement for ACRR requirement and proposal in details for FR2 ACRR requirement is proposed as following:

Agreement: Take same approach as FR1, define both ACLR and ACRR core requirements. - Detailed test case(s) can be further discussed and decided in conformance phase. And Set ACRR equal to ACLR for FR2 as baseline assumption.

Proposal 4: to specify the OOBB limit for FR2 as following:

| Co-existence with other systems | Repeater maximum output power | Channel offset from the centre frequency of the first or last 50MHz channel within the pass band. | ACRR limit |
| --- | --- | --- | --- |
| NR | Declared maximum output power | 50 MHz | 28 (Note 1) 26 (Note 2) |
|  | Declared maximum output power | 100 MHz | 28 (Note 1) 26 (Note 2) |
| NOTE 1: Applicable to bands defined within the frequency spectrum range of 24.25 – 33.4 GHz. NOTE 2: Applicable to bands defined within the frequency spectrum range of 37 – 52.6 GHz. |  |  |  |

Conclusions

In this contribution, we want to share some further considerations on those remaining open issues of conducted repeater and proposals are made as following:

Proposal 1: to support option 1 with modulated signal to 50MHz;

Proposal 2: in-band gain for FR2 could be 80dBc at most;

Proposal 3: to specify the OOBB limit for FR2 as following:

Table 1: Out of band gain limits

| Frequency offset, f_offset_CW | Maximum gain |
| --- | --- |
| 0.1*BWcontiguous f < fB | 65 dB |
| fB f < fmax | 57dB |
| NOTE 1: fB = 2*BWcontiguous when BWcontiguous ≤ 500 MHz, otherwise fB = BWcontiguous + 500 MHz. |  |

For fmax  f_offset_CW the out of band gain shall not exceed the maximum gain of table 2 or the maximum gain stated in table 1 whichever is lower.

Table 2: Out of band gain limits 2

| Frequency offset, f_offset_CW | Maximum gain |
| --- | --- |
| fmax f_offset_CW | Out of band gain minimum donor coupling loss |

Proposal 4: to specify the OOBB limit for FR2 as following:

| Co-existence with other systems | Repeater maximum output power | Channel offset from the centre frequency of the first or last 50MHz channel within the pass band. | ACRR limit |
| --- | --- | --- | --- |
| NR | Declared maximum output power | 50 MHz | 28 (Note 1) 26 (Note 2) |
|  | Declared maximum output power | 100 MHz | 28 (Note 1) 26 (Note 2) |
| NOTE 1: Applicable to bands defined within the frequency spectrum range of 24.25 – 33.4 GHz. NOTE 2: Applicable to bands defined within the frequency spectrum range of 37 – 52.6 GHz. |  |  |  |

References

- R4-2120666, WF on repeater conducted requirements, approved.

- R4-2203027, WF on NR Repeater radiated requirements, Huawei, Approved.