3GPP TSG-RAN WG4 Meeting # 102-e                                                        R4-2205472			R4-22xxxxx

# Electronic Meeting, February 21 – March 3, 2022

Source:	ZTE Corporation

Title:	TP for TS 38.101-5: Genera(5.1) and Operating Band(5.2)

Agenda Item:	10.13.4.1

Document for:	Approval

- Introduction

In terms of the work split in the approved WF[1]. This contribution provides a text proposal to TS38.101-5 [2] on sub-clause 5.1(General) and sub-clause 5.2 (operating band)

- Reference

- [1] R4-2203080, Way Forward on NTN_solutions_Part1, THALES

- [2] R4-2203086,  Draft skeleton for TS 38.101-5, Samsung

- Text Proposal

----- Start of TP -----

- 5				Operating bands and channel arrangement

  - 5.1				General

The channel arrangements presented in this clause are based on the operating bands and channel bandwidths defined in the present release of specifications.

NOTE:	Other operating bands and SAN channel bandwidths may be considered in future releases.

Requirements throughout the RF specifications are in many cases defined separately for different frequency ranges (FR). The frequency ranges in which NTN satellite can operate according to this version of the specification are identified as described in Table 5.1-1.

Table 5.1-1: Definition of frequency ranges

| Frequency range designation |  | Corresponding frequency range |
| --- | --- | --- |
| FR1 |  | 410 MHz – 7125 MHz |
| FR2 | FR2-1 | 24250 MHz – 52600 MHz |
|  | FR2-2 | 52600 MHz – 71000 MHz |

The present specification covers FR1 operating bands.

  - 5.2				Operating bands

    - 5.2.1		General

    - NTN satellite covers FR1 operating bands in the present specification.When the operating bands for FR2 are defined for NTN satellite, the operating bands for FR2 shall be included in sub-clause 5.2.3.

    - 5.2.2		Operating bands with conducted requirements

NTN satellite is designed to operate in the operating bands defined in Table 5.2.2-1.

Table 5.2.2-1: NTN satellite bands in FR1

| NTN satellite Operating band | Uplink (UL) operating band Satellite Access Node receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band Satellite Access Node transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n256 | 1980MHz – 2010 MHz | 2170 MHz – 2200 MHz | FDD |
| n255 | 1626.5 MHz – 1660.5 MHz | 1525 MHz – 1559 MHz | FDD |
| NOTE: NTN bands are numbered in descending order from n256. |  |  |  |

    - 5.2.3		reserved (for radiated requirements)

----- End of TP -----