3GPP TSG-RAN WG4 Meeting #102-e                                         R4-2205581

Electronic Meeting, Feb 21- Mar 03, 2022

Source: 	Huawei, HiSilicon

Title: 	TP for TR 38.839: Update for simultaneous Rx/Tx capability

Agenda Item:	9.39.2

Document for:	Approval

# Background

This contribution provides the TP for TR of simultaneous Rx/Tx capability based on [1-3].

# References

[1] R4-2205579, TR38.839 v0.2.0

[2] R4-2202295 draft WF on simultaneous RxTx for higher order, NTT DOCOMO, INC

[3] R4-2205580, Further consideration on the simultaneous RxTx capability for FR1 TDD-FDD, Huawei, HiSilicon

# Text Proposal

## <Start of Text Poposal>

## 5.1	Applicability of simultaneous Rx/Tx capability

The following consensus was reached in RAN4 on applicability of simultaneous Rx/Tx capability.

- Simultaneous Rx/Tx capability for LTE CA band combination can be applied for the corresponding inter-band NR CA band combination.

- Same simultaneous Rx/Tx capability can be applied for the corresponding band combination among NR CA, NR DC and NR EN-DC

- If mandatory simultaneous Rx/Tx capability apply for a band configuration, mandatory simultaneous RxTx capability also apply for the band pair of the configuration when the applicable configuration is a subset of a higher order band configuration.

## <Next Change>

### 5.2.1	FR1+FR1 FDD-TDD band combination

Editor note: This band combination category also includes SUL band combination where SUL band is derived from FDD band.

For FR1+FR1 FDD-TDD band combination,

- Simultaneous Rx/Tx is the default capability in FDD-TDD FR1 band combinations

- For TDD-FDD combinations, the capability is mandatory without checking if there are no MSD issues and frequency range is not adjacent to each other for the band combinations

- For TDD-FDD combinations with REFSENS degradation due to self-interference, MSD caused by harmonics/IMD, etc. should be defined. For some band combinations that are close in frequency, e.g. bands n7 and n38, other methods to mitigate interference can be considered as well. Simultaneous Rx/Tx capability for this kind of special cases could be handled separately from those having MSD issues.

- If a FR1 FDD-TDD band combination is identified which cannot support simultaneous Rx/Tx operation, a note similar to FR1 TDD-TDD band combination shall be indicated in the specification, and for such operation the minimum requirements are not applicable for this band combination

- Simultaneous Rx/Tx capability can still be optional for FR1 FDD-TDD band combination with specified MSD

## <Next Change>

# Specific cases for TDD-TDD band combinations

<To be added>

# 7	Specific cases for FDD-TDD band combinations

<To be added>

## <End of Text Poposal>