3GPP TSG-RAN WG4 Meeting # 102-e 		                       R4-2205608

Electronic Meeting, February 21st – March 3rd, 2022

Title: 	TP to TS 38.101-5 on 7.3 Reference sensitivity

Source: 	Hughes/EchoStar

Agenda item:	10.13.4

Document for:	Approval

# Introduction

This contribution provides a text proposal on 7.3 Reference sensitivity for TS 38.101-5 [1] based on the approved WF in [2] and WF on UE RF requirement for NTN UE [3].

# Reference

[1] R4-2203086 TS 38.101-5 V0.0.1

[2] WF R4-2203080

[3] WF R4-2203036

[4] R4-2205654 – Rationale n256 to adopt n65

# Text proposal

---------------------------------------------------Start of Text proposal---------------------------------------------------------

## 7.3 Reference sensitivity

### 7.3.1	General

The reference sensitivity power level REFSENS is the minimum mean power applied to each one of the UE antenna ports for all UE categories, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel.

In later clauses of Clause 7 where the value of REFSENS is used as a reference to set the corresponding requirement:

in all bands, the UE shall be verified against those requirements by applying the REFSENS value in Table 7.3.2-1a.

### 7.3.2	Reference sensitivity power level

The throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels with parameters specified in Table 7.3.2-1a.

Table 7.3.2-1a: Two antenna port reference sensitivity QPSK PREFSENS for FDD bands

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
| n256 | 15 | -99.5 | -96.3 | -94.5 | -93.3 |  |  |  |  |  |  |
|  | 30 |  | -96.6 | -94.6 | -93.5 |  |  |  |  |  |  |
|  | 60 |  | -97.0 | -94.9 | -93.7 |  |  |  |  |  |  |
| n255 | 15 | -100.0 | -96.8 | -95.0 | -93.8 |  |  |  |  |  |  |
|  | 30 |  | -97.1 | -95.1 | -94.0 |  |  |  |  |  |  |
|  | 60 |  | -97.5 | -95.4 | -94.2 |  |  |  |  |  |  |

---------------------------------------------------End of Text proposal---------------------------------------------------------