3GPP TSG-RAN WG4 Meeting #102-e	R4-2205658

# Electronic Meeting, 21 February – 3 March, 2022

Agenda item:	10.19.2.2

Source:	Apple

Title:   On per-beam based P-MPR reporting in FeMIMO

Document for:	Discussion

# Introduction

In RAN4#101bise, the following agreements have been made on the per beam based P-MPR reporting

- Agreements

  - It will be further determined

    - whether to change current PCMAX definition based on the latest RAN1/2 agreement

    - whether to update the P-MPR NOTE based on potential CR proposed to the next meeting.

In this contribution, the RAN4 impacts due to per-beam based P-MPR reporting are discussed.

# RAN1 related agreements

Agreement

On Rel.17 enhancements to facilitate MPE mitigation, support N=1, 2, 3, and 4

- N is defined as the number of reported measurements

- UE reports supported largest N value as a UE capability

Agreement

On Rel.17 enhancements to facilitate MPE mitigation, confirm the following working assumption (in the midst of the previous agreement) as an agreement with the following refinement (highlighted in red):

On Rel.17 enhancements to facilitate MPE mitigation, support the following enhancement on the Rel-16 event-triggered P-MPR-based reporting (included in the PHR report when a threshold is reached, reported via MAC-CE): - In addition to the existing field in the PHR MAC-CE, N≥1 P-MPR values can be reported - The N P-MPR values are reported together with the following: - (Working Assumption) For each P-MPR value, up to M SSBRI(s)/CRI(s), where the SSBRI(s)/CRI(s) is selected by the UE from a candidate SSB/CSI-RS resource pool (FFS: how to perform the selection) - Support M=1 - FFS: The supported value(s) of M - FFS: Additional reporting quantities, e.g. SSBRI/CRI, MPR+DL RSRP, or modified virtual PHR - FFS: additional signaling (e.g. CSI triggering) from the NW

Agreement

On Rel.17 enhancements to facilitate MPE mitigation, the candidate resource pool corresponds to a CSI-RS/SSB resource set configured via RRC (details up to RAN2)

# RAN4 related impact

Existing P-MPR reporting is embedded in PHR reporting by 2 bits. The related reporting is per UE based. Based on RAN1’s agreements, up to 4 P-MPR, which are associated with different SSBRI/CRI, can be reported by the UE. With this mechanism introduced, P-MPR due to MPE can be factored in for UL Tx beam selection.

Observation 1: per-beam based P-MPR is introduced to enhance UL Tx beam selection with MPE impacts considered.

From RAN4 spec perspective, it is defined in 38.101-2 that the configured UE maximum output power PCMAX,f,c for carrier f of a serving cell c shall be set such that the corresponding measured peak EIRP PUMAX,f,c is within the following bounds

In the lower bound of this inequation, P-MPRf,c is derived based on the peak direction where minimum peak EIPR, i.e. Ppowerclass, can be achieved. In other words, the peak EIPR and the associated P-MPR should be paired.

In case of per-beam based P-MPR, it becomes less clear on the definition of P-MPRf,c. If P-MPR of the selected beam, which is necessarily the best beam in terms of EIRP, is used, it does not make a lot of sense to reuse Ppowerclass since it will artificially increase the lower bound.

For example, for PC3 UE, Ppowerclass is 22.4dBm for n257. Let’s also assume the per-UE based P-MPR is 6dB to meet MPE requirements when minimum peak EIRP is achieved. However, the UE can report two different per-beam based P-MPR, e.g. 6dB and 0dB, depending on the relative position between UE and proximate human body. It is possible 6dB PMPR is associated with a beam with EIRP of 22.4dBm and 0dB PMPR is associated with a beam with EIRP of 20dBm. Obviously, the beam with 0dB PMPR should be selected. However, if 0dB PMPR is used in the formular, the lower bound can be artificially tighten by 6dB.

Proposal 1: the corresponding measured peak EIRP PUMAX,f,c should be revisited to clarify the definition of P-MPRf,c when per-beam based P-MPR reporting is enabled.

# Conclusion

In this contribution, RAN4 impacts of the per-beam based PMRP reporting is evaluated with the following observation and proposals

Observation 1: per-beam based P-MPR is introduced to enhance UL Tx beam selection with MPE impacts considered.

Proposal 1: the corresponding measured peak EIRP PUMAX,f,c should be revisited to clarify the definition of P-MPRf,c when per-beam based P-MPR reporting is enabled.