3GPP TSG-RAN WG4 Meeting#102-e      	R4-2205662

E-meeting, 21 Feb – 3 Mar, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | <CR#> | rev | - | Current version: | 14.21.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for 36.101 Correction to Bands for NB-IoT in the USA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Dish Network |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOT_R14_bands-Core, NB_IOTenh-Core |  |  |  |  |  | Date: |  |  | 2022-02-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | A table for NB-IoT Bands for use in the USA was added with the intention of meeting FCC emission requirement (ending to skip the uppermost and lowermost 100KHz for UL on some bands). This requirement is for UL only, however, DL bands were also changed for the NB-IoT operation in the USA: Additionally, NB-IoT (in-band and guard band) operation mode TX-RX separation is flexible within the assigned LTE (or NR) channel bandwidth and therefore, the entire DL band should be possible to use in these cases. Band 70 is missing from the table. |  |  |  |  |  |  |  |  |
| Summary of change: |  | DL Bands changed to refer operating band in table 5.5-1 Band 70 added to table 5.5F-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Band 70 is missing from the US bands table DL frequency ranges for US bands cannot be fully used |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5F |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 36.521-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***************************** Start of changes ************************************

## 5.5F	Operating bands for category NB1 and NB2

Category NB1 and NB2 are designed to operate in the E-UTRA operating bands 1, 2, 3, 5, 8, 11, 12, 13, 17, 18, 19, 20, 21, 25, 26, 28, 31, 66 and 70 which are defined in Table 5.5-1. Category NB1 and NB2 systems operate in HD-FDD duplex mode.

In case UE receives network signaling value NS_04 on any of the operating bands listed in Table 5.5F-1 then the lower and upper limit of those bands are shown in Table 5.5F-1 to account for the USA emission requirements.

Table 5.5F-1 E-UTRA operating bands for NB-IoT in the USA

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 2 | 1850.1 MHz | – | 1909.9 MHz | (NOTE 1) |  |  | FDD |
| 4 | 1710.1 MHz | – | 1754.9 MHz | (NOTE 1) |  |  | FDD |
| 5 | 824.1 MHz | – | 848.9 MHz | (NOTE 1) |  |  | FDD |
| 12 | 699 MHz | – | 715.9 MHz | (NOTE 1) |  |  | FDD |
| 13 | 777 MHz | – | 786.9 MHz | (NOTE 1) |  |  | FDD |
| 17 | 704.1 MHz | – | 715.9 MHz | (NOTE 1) |  |  | FDD |
| 25 | 1850.1 MHz | – | 1914.9 MHz | (NOTE 1) |  |  | FDD |
| 26 | 814.1 MHz | – | 848.9 MHz | (NOTE 1) |  |  | FDD |
| 66 | 1710.1 MHz | – | 1779.9 MHz | (NOTE 1) |  |  | FDD |
| 70 | 1695.1 MHz | – | 1709.9 MHz | (NOTE 1) |  |  | FDD |
| NOTE 1: Downlink operating is defined as in Table 5.5-1 E-UTRA operating bands. |  |  |  |  |  |  |  |

***************************** End of changes ************************************