3GPP TSG-RAN WG4 Meeting #102-e	R4-2205688

Electronic Meeting, 21 February – 03 March 2022

Source:	Ericsson, Airtel

Title:	TP to TR 37.717-11-21: Addition of DC configurations for DC_5_n1-n78

Agenda item:	9.19.2

Document for:	Approval

# Background

This text proposal for TR 37.717-11-21 to add DC_5A_n1A-n78A configurations as defined in WID [1].

# Text Proposal

##### ---Start of changes---

## 6.x	DC_5_n1-n78

### 6.x.1	Operating bands for DC

Table 6.x.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_5_n1-n78 | 5 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.x.2	Channel bandwidths per operating band for DC

Table 6.x.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_5A_n1A-n78A | DC_5A_n1A DC_5A_n78A | 5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 170 |
|  |  | n1 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |

### 6.x.3	Co-existence studies

Based on co-existence studies IMD3 from UL 5_n1 may impact DL n78.

Based on co-existence studies IMD3 from UL 5_n78 may impact DL n1.

### 6.x.4	∆TIB and ∆RIB values

For DC_5_n1-n78, the TIB,c and RIB,c values are derived from CA_n1-n5-n78 and are given in the tables below.

Table 6.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5_n1-n78 | 5 | 0.6 |
|  | n1 | 0.6 |
|  | n78 | 0.8 |

Table 6.x.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5_n1-n78 | 5 | 0.2 |
|  | n1 | 0.2 |
|  | n78 | 0.5 |

### 6.x.5	MSD

Based on co-existence studies MSD need to be defined. Values from CA_n1-n5-n78 are reused.

Table 6.x.5-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_5_n1-n78 | 5 | 829 | 5 | 25 | 874 | N/A | FDD | N/A |
|  | n1 | 1932 | 5 | 25 | 2122 | 18.1 | FDD | IMD3 |
|  | n78 | 3780 | 10 | 50 | 3780 | N/A | TDD | N/A |
|  | 5 | 830 | 5 | 25 | 875 | N/A | FDD | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n78 | 3610 | 10 | 50 | 3610 | 15.7 | TDD | IMD3 |

##### ---End of changes---

# Reference

[1]		RP-212876, Revised WID on DC of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL) in Rel-17, LG Electronics