3GPP TSG-RAN WG4 Meeting #102-e	R4-2205725

Electronic Meeting, 21 February – 03 March 2022

Source:	Ericsson, Telstra

Title:	TP for TR 38.841 to add CA_n5-n78

Agenda item:			9.30.2

Document for:	Approval

# 1. Introduction

A text proposal for TR 38.841 to add CA_n5A-n78A.

# 2. Text Proposal

---Start of changes---

## 5.x 	CA_n5-n78

### 5.x.1	Configurations

Table 5.x.1-1: NR CA configurations and bandwidth combinations sets for supporting power class 2

| NR CA configuration | Uplink CA configuration | NR Band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n5A-n78A | CA_n5A-n78A | n5 | 15 | 5 | 10 | 15 | 20 | 25 |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 704 | 80 | 90 | 100 |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 704 | 80 | 90 | 100 |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 5.x.2	Maximum output power

| Uplink CA configuration | Power class 2 cases for CA_n5A-n78A | CA power class | Carrier n5 power class | Carrier n78 power class |
| --- | --- | --- | --- | --- |
| CA_n5A-n78A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

The tolerance +2/-3 dB is applied. Also when the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB.

### 5.x.3	REFSENS requirements

According to the PC3 CA_n5A-n78A study, the 4th IMD products from dual uplink of band n5 and n78 may fall into band n5 Rx frequency range. Thus, additional MSD should be considered to mitigate the impact of the interference for PC2 CA_n5A-n78A combination.

#### 5.x.3.1	Power class 2 Case A

The additional MSD due to intermodulation for PC2 CA_n5A-n78A are defined in table 5.x.3.1-1.

Table 5.x.3.1-1: MSD test points for PCell due to dual uplink operation for PC2 NR CA in NR FR1 (two bands)

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD for PC2 (dB) | Duplex mode | Source of IMD |
| CA_n5A-n78A | n5 | 844 | 5 | 25 | 889 | 18.6 | FDD | IMD4 |
|  | n78 | 3421 | 10 | 50 | 3421 | N/A | TDD | N/A |

#### 5.x.3.2	Power class 2 Case B

The additional MSD due to intermodulation for PC2 Case B CA_n5A-n78A are same as the Case A defined in table 5.x.3.1-1.

#### 5.x.3.3	OOB blocking exception requirements

Since band n5 is a low band and n78 is a wide band, the OOBB exception is needed.

Table 5.x.3.3-1: NR DC band combination with exceptions allowed

CA band combination

CA_n5-n78

### 5.x.4		∆TIB and ∆RIB values

For the ∆TIB,c and ∆RIB,c values, same PC3 CA_n5A-n78A requirements are applied for PC2 CA_n5A-n78A.

---End of changes---