3GPP TSG-RAN WG4 Meeting #102-e	R4-2205760

# Electronic Meeting, 21st Feb – 3rd Mar, 2022

Title: 	Simulation results on PRACH demodulation requirements for FR2 HST

Source: 	Huawei, HiSilicon

Agenda item:	10.9.4.3.3

Document for:	Discussion

Background

During RAN#101bis-e meeting, WF [2] on FR2 HST demodulation was approved. In this contribution, we provide our simulation results about PRACH demodulation requirements for FR2 HST.

Simulations

Here we provide the updated simulation results for FR2 HST PRACH for alignment as Figure 2-1 and Table 2-2 shows, based on simulation assumption as Table 2-1 shows.

Table 2-1 Simulation assumption for PRACH requirement

| Parameters | Value |
| --- | --- |
| Ncs | 0 |
| Logical sequence index | 0 |
| v | 0 |
| Channel | AWGN |
| Antenna configuration | 1T2R |
| SCS | 120KHz |
| Frequency offset | 19444Hz |
| Time error tolerance | 0.07us |
| Timing offset | 4.8us with TO range 0:0.48:4.8 |

Figure 2-1 Ideal simulation results for FR2 HST PRACH

Table 2-2 Ideal simulation results for FR2 HST PRACH

| Case number | Timing offset | SNR (dB) |
| --- | --- | --- |
| 1 | [0,0.48...4.8] | -14.07 |

Conclusion

In this contribution, we provide our updated simulation results for PRACH performance requirements for NR HST FR2.

Reference

R4-2120703, WF on BS demodulation requirement for FR2 HST, RAN4#101-e, Samsung

R4-2203006, WF on BS demodulation requirement for FR2 HST, RAN4#101bis-e, Nokia, Nokia Shanghai Bell