3GPP TSG-RAN WG4 Meeting # 102-e	R4-22xxxxx

# Electronic Meeting, Feb. 21st -Mar.3rd, 2022

Title: 	Simulation results on intra cell inter-user MMSE-IRC receiver

Source: 	Huawei, HiSilicon

Agenda item:	10.12.2.2

Document for:	Discussion

Background

In RAN4 #99-e meeting, WF [1] was approved. In this paper, we provide our simulation results.

Simulation results

The simulation assumptions derived from [1] are shown as Table 1:

Table 1: Simulation assumptions for MU-MIMO

| Parameter |  | Unit | Value |  |
| --- | --- | --- | --- | --- |
|  |  |  | Target UE | Co-scheduled UE |
| Channel Bandwidth/SCS |  | MHz/KHz | 10/15 for FDD and 40/30 for TDD |  |
| Duplex mode |  |  | FDD and TDD |  |
| TDD pattern |  |  | 7D1S2U |  |
| MCS |  |  | 13 | 16 QAM random symbols |
| Allocation for interference UE and target UE | Rank allocation |  | 1 | 1 |
|  |  |  | 2 | 2 |
|  | DMRS port allocation |  | For rank 1+1: Port 1000 For rank 2+2: Port 1000 and 1001 | For rank 1+1: Port 1001 For rank 2+2: Port 1002 and 1003 |
| MIMO configuration |  |  | 2T2R ULA Low and 2T4R ULA Low for case with rank1+1 and 4T4R ULA Low (Note 1) for case with rank 2+1 and rank 2+2 |  |
| Number of CDM groups without data |  |  | 1 for case with rank 1+1 and 2 for case with rank 2+2 |  |
| HARQ process number |  |  | 4 for FDD and 8 for TDD |  |
| Precoding model | Target UE |  | Random precoding with Single panel Type 1 per PRB bundling size per slot | - For case with rank 1+1: Select the PMI matrix randomly from the codebook of Co-scheduled UE to ensure that any column of precoding matrix of Co-scheduled UE is not equal to any column of precoding matrix of Target UE - For case with rank 2+2: Select the precoding matrix to ensure orthogonality with Target UE |
| PDSCH configuration | Mapping type |  | Type A |  |
|  | Starting symbol (S) |  | 2 |  |
|  | Length (L) |  | 12 |  |
|  | PRB bundling size |  | 2 |  |
|  | PRB bundling type |  | Static |  |
| PDSCH DMRS configuration | DMRS Type |  | DMRS Type 1 |  |
|  | Number of additional DMRS |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |  |
| Propagation conditions |  |  | For rank 1+1: TDLC300-100 For rank 2+2: TDLA30-10 |  |
| Receiver type |  |  | MMSE-IRC | N/A |
| Test metric |  |  | SNR @ %70 of maximum Throughput | N/A |

Simulation results for rank 1+1

Simulation results for case with rank 1(Target UE) + 1(interference UE) with same CDM group are shown in Figure 2-1 and Table 2-1.

Figure 2-1: Simulation results for rank 1+1

Table 2-1: SNR (dB) @ 70% of max TP for case with Rank 1+1

|  | 2x2 | 2x4 |
| --- | --- | --- |
| FDD | 14.95 | 8.36 |
| TDD | 14.34 | 7.58 |

Simulation results for rank 2+2

Simulation results for case with rank 2(Target UE) + 2(interference UE) with different CDM group are shown in Table 2-2:

Figure 2-2: Simulation results for rank 2+2

Table 2-1: SNR (dB) @ 70% of max TP for case with Rank 1+1

| FDD | 12.24 |
| --- | --- |
| TDD | 12.36 |

Conclusion

In this paper, we provide our simulation results for intra cell inter-user MMSE-IRC.

Reference

- R4-2203011, Way Forward on MMSE-IRC for intra-cell inter- user interference. Huawei, HiSilicon