3GPP TSG-RAN WG4 Meeting #102-e	R4-2205882

Feb 2022

Title:	Discussion on UE phase continuity requirements

Source:	Qualcomm Incorporated

Agenda item:	10.18.2.2

Release:	Rel-17

Work Item:	NR_cov_enh

Responsible WG:	RAN1

Document for:	Approval

# 1.   Introduction

IN RAN4#101-Bis-e a WF [19] was agreed. The WF agreed some assumptions for requirement setting for the UE. In summary, the UE requirements will be based on slot by slot channel estimation looking at the phase either referring to the previous slot or the first slot in the bundle. This paper discusses the two candidate options for the phase reference and other aspects in setting requirements for the UE.

We will also discuss the maximum duration with extended CP.

# 2. 	Discussion

## 2.1	UE behaviour and phase reference in requirements

Phase error in a UE is caused by random processes in the TX chain including PLL randomly locking in one the reference phases after every slot. UE does not have memory of what was the phase of the first slot in the bundle but the random process apply to ethe current state i.e. the reference is to the previous slot.

Proposal 1: The option 2 in [19] with phase of slot n will be referred to slot n-1 when setting the requirements for JCE

## 2.2	The amount of phase error

The discussed options (but not agreed) for requirement in WF [19] was made looking at the simulation data with amplitude variation included. Now RAN4 agreed not to specify the amplitude variation [19] but only phase variation so the link simulation results should be looked without any amplitude variation.

Observation 1: Link simulation results should be analysed reflecting the latest agreements how to set the UE requirements

The 0 dB amplitude variation results were presented in [22], one figure replicated below. Up to 40 degrees, the degradation in needed CINR for same throughput for JCE is less than 1 dB with option 2.  The solid and dashed brown curves show the difference with 8 slot bundles.

Figure 1. Increase in required CINR with same phase discontinuity for longer bundles

Proposal 2: The requirement for phase discontinuity with the agreed assumptions is +/- 40 deg and phase is referred to the previous slot.

The agreements in [19] and [20] state that the smallest maximum size of the bundle is 5 and UE requirements will be tested without gaps. What is not agreed if the testing will be done based on UE declared maximum duration.

Observation 2: The phase discontinuity test criteria should be agreed, is it based on maximum declared UE capability or something else?

## 2.3	Maximum gap duration with extended CP

RAN4 analysis on maximum duration when UE can maintain phase and amplitude consistency was analysed based on UE ability, as it was worded in the LS [10] asking this. If the time includes gaps or not was not discussed but it was assumed that the maximum gap length in time was the agrees 13 symbols. Having a gap less than a full slot would not force the next symbol in the bundle to start at the end of the slot which may not fit any feasible slot and frame format. The Figure 2 shows some examples and how if the number of PUSCH symbols + number of gap symbols is larger than one slot makes the first PUSCH symbol in the slot drift further. This might be impossible to schedule.

Figure 1. Different possible PUSCH+GAP patterns. a) 10 symbol PUSCH + 4 symbol GAP; b) maximum gap: 1 symbol PUSCH + 13 symbol GAP; c) 7 symbol PUSCH + 9 symbol GAP

Observation 3: Sum of scheduled PUSCH/PUCCH symbols with number of GAP symbols should equal = 14.

If we make the assumption that gap is inclusive in the slot that is scheduled part of the bundle, the total maximum duration in time can be calculated straight from number of slots and SCS. We can therefore tabulate a table:

Table 1. Maximum duration in msec for different numbers of slots and different SCS [msec]

|  | SCS |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Slots | 15 kHz | 30 kHz | 60 kHz | 60 kHz ECP | 120 kHz |
| 5 | 5.000 | 2.500 | 1.250 | 1.250 | 0.625 |
| 8 | 8.000 | 4.000 | 2.000 | 2.000 | 1.000 |
| 16 | 16.000 | 8.000 | 4.000 | 4.000 | 2.000 |
| 32 | 32.000 | 16.000 | 8.000 | 8.000 | 4.000 |

The RAN1 question if the gap is 11 symbols with extended CP can be analysed as follows. The gap length for 60 kHz is 231.8 usec. The length of 11 symbols with extended CP is 11 x 20.81 usec = 229 usec which is shorter than the 13 symbol gap with normal CP. This also allows the one symbols PUSCH/PUCCH (case b in Figure 2).

Observation 4: The 11 symbol gap with extended CP is shorter than 13 symbol gap with normal CP.

We can conclude that the maximum duration with the 11 extended symbol gap is shorter than any of the maximum duration for the maximum number of slots in discussion.

Proposal 3: Respond to RAN1 that the maximum duration with 11 symbols gap with extended CP is within the agreed maximum duration and maximum gap length agreements.

# Conclusion

We discussed, observed and proposed:

Proposal 1: The option 2 with phase of slot n will be referred to slot n-1 when setting the requirements for JCE

Observation 1: Link simulation results should be analysed reflecting the latest agreements how to set the UE requirements

Proposal 2: The requirement for phase discontinuity with the agreed assumptions is +/- 40 deg and phase is referred to the previous slot.

Observation 2: The phase discontinuity test criteria should be agreed, is it based on maximum declared UE capability or something else?

Observation 3: Sum of scheduled PUSCH/PUCCH symbols with number of GAP symbols should equal = 14.

Observation 4: The 11 symbol gap with extended CP is shorter than 13 symbol gap with normal CP.

Proposal 3: Respond to RAN1 that the maximum duration with 11 symbols gap with extended CP is within the agreed maximum duration and maximum gap length agreements.

# References

[1]	R4-2100014, ”LS on PUCCH and PUSCH repetition”, RAN1, 3GPP TSG-RAN WG4 Meeting #98-e, 24 Jan – 5 Feb 2021

[2]	R4-2103393, “Reply LS on PUCCH and PUSCH repetition”, RAN4 (Qualcomm), 3GPP TSG-RAN WG4 Meeting #98-e, 24 Jan – 5 Feb 2021

[3]	R4-2103363, “Way forward on phase continuity and power consistency for PUCCH and PUSCH repetition”, Huawei, 3GPP TSG-RAN WG4 Meeting #98-e, 24 Jan – 5 Feb 2021

[4]	R4-2105417, “Reply LS on PUCCH and PUSCH repetition”, RAN4 (Qualcomm), RAN4#98-Bis-e

[5]	R4-2105418, “Way forward on phase continuity and power consistency for PUCCH and PUSCH repetition” Huawei, RAN4#98-Bis-e

[6]	R4-2107611, “Reply LS on PUCCH and PUSCH repetition”, RAN1, RAN4#99e

[7]	R4-2107880, “Reply LS on PUCCH and PUSCH repetition”, RAN4 (Qualcomm), RAN4#99e

[8]	R4-2107881, “WF on phase continuity and power consistency for PUCCH and PUSCH repetition”, Huawei, RAN4#99e

[9]	R4-2111194, “Reply LS to RAN1 latest question on phase discontinuity”, Ericsson, RAN4#99e

[10]	R4-2111706, “LS on joint channel estimation for PUSCH and PUCCH”, RAN1, RAN4#100-e, Online, Aug 2021

[11] 	R4-2114991, “Reply LS on PUCCH and PUSCH transmissions”, Qualcomm, RAN4#100-e, Online, Aug 2021

[12]	R4-2114992, “WF on phase continuity and power consistency for PUCCH and PUSCH transmissions”, Huawei, HiSilicon, RAN4#100-e, Online, Aug 2021

[13]	R4-2117006, “Reply LS on PUCCH and PUSCH repetition”. RAN1, RAN4#101e, Nov 2021

[14]	R4-2120002, ”Reply LS on PUCCH and PUSCH transmissions”, RAN4 (Qualcomm Incorporated), 3GPP TSG-RAN WG4 Meeting #101-e, Nov 2021

[15]	R4-2120003, “WF on phase continuity and power consistency for PUCCH and PUSCH transmissions”, Huawei, 3GPP TSG-RAN WG4 Meeting #101-e, Nov 2021

[16]	R4-2119494, “UE requirement for phase continuity”, Qualcomm, 3GPP TSG-RAN WG4 Meeting #101-e, Nov 2021

[17] R4-2200339, “draft CR for EVM based requirements”, Qualcomm, RAN4#101Bis-e, Jan, 2022

[18]	R4-2119495, “Simulation results and criterias for setting requriements for UE”, Qualcomm Incorporated, RAN4#101e, Nov 2021

[19]	R4-2202418, “WF on phase continuity and power consistency for PUCCH and PUSCH transmissions”, Huawei, 3GPP TSG-RAN WG4 Meeting # 101-bis-e, Electronic Meeting, January 17-25, 2022

[20]	R4-2202368, “Reply LS on Maximum duration for DMRS bundling” RAN4(Qualcomm), 3GPP TSG-RAN WG4 Meeting # 101-bis-e, Electronic Meeting, January 17-25, 2022

[21]	R4-2200020, “Updated RAN4 RF work plan for NR coverage enhancements WI”, China Telecom, 3GPP TSG-RAN WG4 Meeting # 101-bis-e, Electronic Meeting, January 17-25, 2022

[22]	R4-2200338, “Discussion on UE requirement for JCE”, Qualcomm Incorporated, 3GPP TSG-RAN WG4 Meeting #101Bis-e, Jan 2022