3GPP TSG-RAN WG4 Meeting #102-e 	R4-2205931

Electronic Meeting, Feb 21st - March 3rd 2022

Source:	T-Mobile USA

Title:	TP for TR38.841: PC1.5 n77 for CA_n71A-n77A

Agenda item:	9.30.2

Document for:	Approval

- Introduction

This contribution is a text proposal to introduce PC1.5 n77 for DL CA_n71A-n77A. There is no impact of n77 PC1.5 to MSD.

Table 5.14.1-1 is updated with Note 9 for PC1.5, and also with Note 8 for PC2, which reflects what is in 38.101-1 already.

- Reference

[1]

- Text Proposal

<Start of text proposal>

## 5.14		CA_n71-n77

### 5.14.1	Configurations

Table 5.14.1-1: NR CA configurations and bandwidth combinations sets for supporting power class 2

| NR CA configuration | Uplink CA configuration | NR Band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n71A-n77A | n778,9 CA_n71A-n77A8 | n71 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 5.14.2	Maximum output power

Table 5.14.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_n71A-n77A | CA power class | Carrier n71 power class | Carrier n77 power class |
| --- | --- | --- | --- | --- |
| CA_n71A-n77A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

The tolerance +2/-3 dB is applied. Also when the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB.

### 5.14.3	REFSENS requirements

According to the PC3 CA_n71A-n77A study, there is MSD due to harmonic mixing and cross band isolation.

#### 5.14.3.1	Power class 2 case a

Comparing with PC3 CA_n71A-n77A, no additional MSD are expected for this PC2 CA_n71A-n77A with dual or single uplink carrier.

#### 5.14.3.2	Power class 2 case b

Comparing with PC3 CA_n71A-n77A, additional MSD is expected for this PC2 CA_n71A-n77A with dual uplink due to higher Tx power in n77. The MSD was calculated by starting with 5.5 dB MSD for PC3, calculating the interference noise power, and then adding 3 dB to the interference noise power.

Table 5.14.3.2-1: 2DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC3 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n71-n77 | n71 | 681.5 | 5 | 25 | 635.5 | 11.4 | FDD | IMD5 |
|  | n77 | 3361.5 | 10 | 50 | 3361.5 | N/A | TDD | N/A |

#### 5.14.3.3	Single uplink Power class 1.5

Comparing with PC2 CA_n71A-n77A, no additional MSD is expected with single uplink n77 PC1.5

Change for Table 5.5A.3.1-1 highlighted in yellow:

Table 5.14.3.3-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) |  |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 |  | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n71A-n77A | n778,9 CA_n71A-n77A8 | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 |  | 10 | 15 | 20 | 25 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |

### 5.14.4		∆TIB and ∆RIB values

For the ∆TIB,c and ∆RIB,c values, same PC3 CA_n71A-n77A requirements are applied for PC2 CA_n71A-n77A.

<End of text proposal>