3GPP TSG-RAN WG4 Meeting # 102-e 		R4-2205942

# Electronic Meeting, February 21 – March 3, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on BS demodulation requirements for Additional enhancements for NB-IoT

Agenda item:	12.9.6.2.2

Document for:	Discussion

Introduction

The Rel-17 work item for additional enhancements for NB-IoT and LTE-MTC was approved at TSG RAN#88-e with latest WID revision [1]. At RAN4 #101bis-e, the technical discussion on demodulation requirements for this WI was started. The WID1 [1] lists following objective for NB-IoT involving RAN4 effort:

- Specify 16-QAM for unicast in UL and DL, including necessary changes to DL power allocation for NPDSCH and DL TBS. This is to be specified without a new NB-IoT UE category. For DL, increase in maximum TBS of e.g. 2x the Rel-16 maximum, and soft buffer size will be specified by modifying at least existing Category NB2. For UL, the maximum TBS is not increased. [NB-IoT] [RAN1, RAN4] - Extend the NB-IoT channel quality reporting based on the framework of Rel-14—16, to support 16-QAM in DL. [NB-IoT] [RAN2, RAN1, RAN4] - Specify necessary performance requirements, measurement accuracy requirements and test cases related to the above-mentioned enhancements and core requirements.

Based on RAN4 #101bis-e discussions [2] and the approved WF [3], this contribution provides our view on BS demodulation requirements for NB-IoT 16QAM.

Discussion

Regarding BS demodulation requirements, requirements for NPUSCH format 1 need to be defined as agreed at RAN4 #101bis-e [3].

Proposed new FRCs

The UL TBS agreed in RAN1 for 16QAM in NB-IoT uplink is provided in Table 1 below.

Table 1: TBS indices are introduced for 16QAM in NB-IoT uplink

| I_TBS | I_RU |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 14 | 256 | 552 | 840 | 1128 | 1416 | 1736 | 2280 |  |
| 15 | 280 | 600 | 904 | 1224 | 1544 | 1800 | 2472 |  |
| 16 | 328 | 632 | 968 | 1288 | 1608 | 1928 | 2536 |  |
| 17 | 336 | 696 | 1064 | 1416 | 1800 | 2152 |  |  |
| 18 | 376 | 776 | 1160 | 1544 | 1992 | 2344 |  |  |
| 19 | 408 | 840 | 1288 | 1736 | 2152 | 2536 |  |  |
| 20 | 440 | 904 | 1384 | 1864 | 2344 |  |  |  |
| 21 | 488 | 1000 | 1480 | 1992 | 2536 |  |  |  |

It has also been agreed in RAN1 that repetition is not used for 16QAM in NB-IoT uplink, where it can be used for 3, 6 and 12 subcarriers NPUSCH format 1. Note that 16QAM cannot be used for 1 subcarrier NPUSCH format 1. Considering the agreements in RAN1, and to ensure satisfactory BS receiver performance with 3, 6 and 12 subcarriers for 16QAM in NB-IoT uplink, as specified for QPSK, Nokia proposal [4] is to define the following three new uplink FRCs for 16QAM for the BS demodulation requirements as shown in Table 2 below, which also includes proposals made by Ericsson [5] and Huawei [6].

Table 2: Proposed new FRCs for 16QAM in NB-IoT uplink

| Reference channel | Nokia A16-x [4] | Nokia A16-y [4] | Nokia A16-z [4] | Ericsson [5] | Huawei [6] |
| --- | --- | --- | --- | --- | --- |
| Sub carrier spacing (kHz) | 15 | 15 | 15 | 15 | 15 |
| Number of tones | 3 | 6 | 12 | 12 | 12 |
| Modulation | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM |
| Diversity | No | No | No | No | No |
| Frequency offset | 0 | 0 | 0 | 0 | 0 |
| I_TBS / I_RU | 19 / 3 | 19 / 3 | 19 / 3 | 15 / 0 | 14 / 0 |
| Payload size (bits) | 1736 | 1736 | 1736 | 280 | 256 |
| Allocated resource units | 4 | 4 | 4 | 1 | 1 |
| Transport block CRC (bits) | 24 | 24 | 24 | 24 | 24 |
| Coding rate (target) | 3/4 | 3/4 | 3/4 | 1/2 | 1/2 |
| Coding Rate (effective) | 0.76 | 0.76 | 0.76 | 0.53 | 0.49 |
| Code block CRC size (bits) | 0 | 0 | 0 | 0 | 0 |
| Number of code blocks – C | 1 | 1 | 1 | 1 | 1 |
| Total symbols per resource unit | 144 | 144 | 144 | 144 | 144 |
| Total number of bits per resource unit | 576 | 576 | 576 | 576 | 576 |
| Tx time (ms) | 16 | 8 | 4 | 1 | 1 |
| Timing error (us) | 0 | 0 | 0 | 0 | 0 |
| Channel estimation length (ms) Note 1 | 12 | 8 | 4 | 1 | 1 |
| Propagation channel | ETU 1Hz Low | ETU 1Hz Low | ETU 1Hz Low | ETU 1Hz Low | ETU 1Hz Low |
| Antenna configuration | 1T2R | 1T2R | 1T2R | 1T2R | 1T2R |
| Performance metric | SNR @70% of maximum TP | SNR @70% of maximum TP | SNR @70% of maximum TP | SNR @70% of maximum TP | SNR @70% of maximum TP |
| Note 1: Channel estimation lengths are included in the table for information only. |  |  |  |  |  |  |

Simulation Results

Link-level simulations have been performed using the parameters of the proposed new FRCs in Table 2 as proposed by Nokia [4], Ericsson [5] and Huawei [6]. Simulation results for SNR required at 70% of maximum throughput are provided in Figure 1 below for all proposals, and the required SNR figures are summarized in Table 3.

(a) Nokia A16-x, 3 tones

(b) Nokia A16-y, 6 tones

(c) Nokia A16-z, 12 tones

(d) Ericsson [4], 12 tones

(e) Huawei [5], 12 tones

Figure 1: SNR required at 70% of maximum throughput for 16QAM in NB-IoT uplink

Table 3: SNR at 70% of maximum TP of proposed new FRCs for 16QAM in NB-IoT uplink

| Reference channel | Nokia A16-x [4] | Nokia A16-y [4] | Nokia A16-z [4] | Ericsson [5] | Huawei [6] |
| --- | --- | --- | --- | --- | --- |
| SNR [dB] | 10.79 | 10.96 | 11.05 | 7.49 | 6.80 |

It can be seen from Table 3 for the Nokia proposal, that the required SNR figures, as observed in the simulations, are quite close, around 11 dB, for the different number of tones.

For Ericsson and Huawei proposals, the required SNR for 12 tones is observed to be lower by about 3.5 to 4.25 dB versus the Nokia proposal, due to lower code rate used for the shorter payload sizes 280 bits [5] and 256 bits [6] (1/2 versus 3/4 for Nokia proposal). We acknowledge that this setup will be beneficial for performance testing, as less sensitive to any external interference source. We have a preference to use a payload size of 280 bits versus 256 bits, since better corresponding to the expected higher SNR operating point for NB-IoT 16-QAM. Thus, we support the proposal in [5] for 12 subcarriers being extended to 3 and 6 subcarriers. It is estimated that a similar SNR is required for 3 or 6 subcarriers as for 12 subcarriers, as observed for the longer payload of 1736 bits.

The following proposal is made.

To define new uplink FRCs for 16QAM NPUSCH format 1 demodulation requirements in TS 36.104, based on payload size proposal of 280 bits, for 3, 6 and 12 subcarriers.

Conclusion

This contribution has provided our view on BS demodulation core requirements for NB-IoT 16QAM.

The following proposal is submitted for discussion:

- To define new uplink FRCs for 16QAM NPUSCH format 1 demodulation requirements in TS 36.104, based on payload size proposal of 280 bits, for 3, 6 and 12 subcarriers.

References

- RP-211340, WID revision: Additional enhancements for NB-IoT and LTE-MTC, Huawei, HiSilicon, RAN #92e

- R4-2203121, Email discussion summary for [101-bis-e][323] NB-IOT_MTC_Demod_NWM, Moderator (Huawei), RAN4 #101bis-e

- R4-2203041,	WF on Rel-17 NB-IOT and eMTC performance requirements, Huawei, RAN4 #101bis-e

- R4-2201984, Discussion on BS demodulation requirements for Additional enhancements for NB-IoT, Nokia, Nokia Shanghai Bell, RAN4 #101bis-e

- R4-2201433, View on Rel-17 NB-IoT BS demodulation requirements, Ericsson, RAN4 #101bis-e

- R4-2200996, Discussion on BS demodulation requirements for additional enhancements of NB-IoT, Huawei, HiSilicon, RAN4 #101bis-e