3GPP TSG-RAN WG4 Meeting #102-e                 	R4-2205964

Electronic Meeting, February 21 – March 3, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	HST FR2 PRACH Impairment Simulation Results

Agenda item:	10.9.4.3.3 PRACH requirements

Document for:	Discussion

Introduction

At RAN4#101-e meeting all of the open issues in relation to the PRACH requirements in HST FR2 deployments were resolved [1]:

Agreements: - One timing offset configuration - Configure the maximum timing offset (i.e., the end of the tested range) in FR2 HST testing setup equal to 4.8us - Value of Timing offset: 0 - Step of Timing offset increase: 0.48us

In this contribution we share the impairment PRACH simulation results for the alignment and definition of performance requirements.

Discussion

We have performed link-level PRACH simulations following the reference parameters from Table 1.

Table 1: Parameters for PRAHC requirements in HST FR2 deployment

| Parameters | Value |
| --- | --- |
| Ncs | 0 |
| Logical sequence index | 0 |
| v | 0 |
| Channel | AWGN |
| Antenna configuration | 1T2R |
| SCS | 120KHz |
| Frequency offset | 19444Hz |
| Time error tolerance | 0.07us |
| Carrier frequency | 28 GHz |
| Timing offset | 4.8us with TO range 0:0.48:4.8 |

PRACH detection requirements define the SNR levels corresponding to the probability of detection of the preamble equal to or exceeding 99% when the signal is present. The SNR level inferred from link-level simulations is presented in Table 2.

Table 2: SNR for PRACH missed detection requirements in HST FR2 deployment.

| Number of TX antennas | Number of demodulation branches | Propagation conditions | Frequency offset | Maximum timing offset, us | SNR, dB Burst format C2, Ideal | SNR, dB Burst format C2, Impairment |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | AWGN | 19444Hz | 4.8 | -6.91 | -3.81 |

Use the impairment results in the table above for the alignment PRACH requirements.

Conclusion

In this contribution we shared the impairment PRACH simulation results for the alignment and definition of performance requirements in HST FR2 deployments.

The following proposal is made:

| Number of TX antennas | Number of demodulation branches | Propagation conditions | Frequency offset | Maximum timing offset, us | SNR, dB Burst format C2, Ideal | SNR, dB Burst format C2, Impairment |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | AWGN | 19444Hz | 4.8 | -6.91 | -3.81 |

- Use the impairment results in the table above for the alignment PRACH requirements.

- References

- R4-2120703, WF on BS demodulation requirement for FR2 HST, Nokia, Samsung, RAN4#101-e.