3GPP TSG-RAN4 Meeting #102-e		R4-2206038

# Electronic Meeting, February 21 – March 03, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | DraftCR | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | RRC connection release with redirection for redcap in TS 36.133 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap-Core |  |  |  |  |  | Date: |  |  | 2022-02-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To introduce RRC connection release with redirection requirements for redcap UE from E-UTRA serving cell to NR |  |  |  |  |  |  |  |  |
| Summary of change: |  | RRC connection release with redirection requirements for redcap UE from E-UTRA serving cell to NR are specified for RedCap UE supporting 1Rx, 2Rx, FDD, TDD or HD-FDD bands. The draft CR is based on the approved work split in R4-2202718 (6. RRC Connection Mobility Control). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Performance of RRC connection release with redirection for redcap UE from LTE to NR cannot be guaranteed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------START OF CHANGES----------------------------

#### 6.3.2.6	RRC connection release with redirection to NR Redcap

The RedCap UE shall be capable of performing the RRC connection release with redirection to the target NR cell within Tconnection_release_redirect_NR_RedCap.

The time delay (Tconnection_release_redirect_NR_RedCap) is the time between the end of the last TTI containing the RRC command, “RRCConnectionRelease” (TS 36.331 [2]) on the E-UTRAN PDSCH and the time the UE starts to send random access to the target NR cell. The time delay (Tconnection_release_redirect_NR_RedCap) shall be less than:

Tconnection_release_redirect_NR_RedCap = TRRC_procedure_delay + Tidentify-NR_Redcap + TSI-NR_RedCap + TRACH_RedCap

The target NR cell shall be considered detectable for RedCap UE supporting 1Rx when for each relevant SSB:

-	SSB_RP and SSB Ês/Iot according to Annex B.2.X of TS 38.133 [50] for a corresponding NR Band.

The target NR cell shall be considered detectable for RedCap UE supporting 2Rx when for each relevant SSB:

-	SSB_RP and SSB Ês/Iot according to Annex B.2.X of TS 38.133 [50] for a corresponding NR Band.

TRRC_procedure_delay: It is the RRC procedure delay for processing the received message “RRCConnectionRelease” as defined in clause 6.2.2 of TS 36.331 [2]. It shall be less than 110 ms.

Tidentify-NR_RedCap: It is the time to identify the target NR cell and depends on the frequency range (FR) of the target NR cell. It is defined in table 6.3.2.6-1 for RedCap UE capable of 1Rx and in table 6.3.2.6-2 for RedCap UE capable of 2Rx. Tidentify-NR_RedCap = TPSS/SSS-sync_RedCap + Tmeas_RedCap, whereTPSS/SSS-sync_RedCap is the cell search time and Tmeas_RedCap is the measurement time due to cell selection criteria evaluation. The RedCap UE operating in NR HD-FDD shall meet the requirements in this section provided that the SSB is available at the UE at least once every SMTC period during Tidentify-NR_RedCap.

TSI-NR_RedCap: It is the time required for acquiring all the relevant system information of the target NR cell. This time depends upon whether the UE is provided with the relevant system information of the target NR cell or not by the old NR cell before the RRC connection is released.

TRACH_RedCap: It is the delay caused due to the random access procedure when sending random access to the target NR cell. TRACH_RedCap can be up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in the table 8.1-1 of TS 38.213 [39]. The RedCap UE operating in NR HD-FDD mode shall perform the PRACH transmission on the PRACH resource of the target NR cell provided that the UE has received at least one SSB associated with the PRACH resource during the last [TBD] period in the target NR cell.

Trs is the SMTC periodicity of the target NR cell if the UE has been provided with an SMTC configuration for the target cell in the redirection command, otherwise Trs is the SMTC periodicity configured in the measObjectNR having the same SSB frequency and subcarrier spacing configured for the RRC connection release with redirection. If the UE is not provided with SMTC configuration or measurement object for the frequency which is also configured for the RRC connection release with redirection then:

-	the requirement in this section is applied with Trs = 20 ms assuming the SSB transmission periodicity is not larger than 20 ms,

-	there is no requirement if the SSB transmission periodicity is larger than 20 ms.

Table 6.3.2.6-1: Time to identify target NR cell for RRC connection release with redirection to NR for RedCap UE supporting 1Rx

| Frequency range (FR) of target NR cell | Tidentify-NR_RedCap |
| --- | --- |
| FR1 | MAX (TBD ms, TBD x Trs) |
| FR2 | MAX (TBD ms, TBD x Trs) |

Table 6.3.2.6-2: Time to identify target NR cell for RRC connection release with redirection to NR for RedCap UE supporting 2Rx

| Frequency range (FR) of target NR cell | Tidentify-NR_RedCap |
| --- | --- |
| FR1 | MAX (680 ms, 11 x Trs) |
| FR2 | MAX (880 ms, 88 x Trs) |

----------------------END OF CHANGES----------------------------